3GPP TSG RAN WG1 #96 	R1-1901607

Athens, Greece, 25th February – 1st March, 2019

Title:	Considerations on DL reference signals and channels design for NR-U

Source: 	ZTE, Sanechips

Agenda Item:	7.2.2.1.2

Document for:	Discussion and Decision

# Introduction

At the RAN #82 meeting, the new WID on NR-U (NR-based Access to Unlicensed Spectrum) [1] aims to identify solutions and techniques for next generation wireless systems operating on unlicensed bands. And in the WID, one of the objectives is to specify DL physical layer channels design. And in the RAN1 Ad-hoc meeting, the below agreements were reached on DL signals and channels design for NR-U.

Agreement:

- The UE may assume the presence of a signal, such as the DMRS in any [PDCCH or GC-PDCCH] transmission, to detect transmission bursts by the serving gNB, to enable power saving by not necessitating performing blind decodes to detect the transmission burst (Note: The power saving possibility by not necessitating blind decodes assumes performance relaxation for PDCCH decoding is not needed. Also, this does not mandate a two-step PDCCH decoding process for the UE with respect to DMRS detection).

- If a preamble transmitted at the start of a burst is agreed to be specified (this does not preclude usage of preambles by implementation), it may be used in addition to the DMRS of PDCCH/GC-PDCCH or any other signals in the gNB transmission to detect the start of transmission bursts by the serving gNB and potentially for power saving

  - Note: Whether a preamble, if defined, can be used for power saving in all cases depends on the details of the design.

- Note: Other signals present in the transmission burst may also be used for the purpose of detection of the transmission burst

- FFS: Potential enhancements to DMRS design to address issues with detection probability

- The payload of a PDCCH and/or GC-PDCCH transmission can contain information regarding COT structure that may be used by the UE for power saving

In this contribution, we discuss potential DL reference signals and physical layer channels design for NR-U, including dynamic PDCCH monitoring, COT structure indication, PDSCH transmission and mechanism to detect DL starting point and so on.

# DL Reference Signals and Physical Channels Design

## Dynamic PDCCH Monitoring

In NR, the PDCCH monitoring information per search space is configured via RRC, such as PDCCH monitoring periodicity, offset and symbols for PDCCH monitoring in the slots. For the first slot of the COT, if small monitoring periodicity is configured, then NR unlicensed systems will have better performance due to the finer time granularity for PDCCH transmission. From the second slot of the COT, the PDCCH monitoring granularity can be bigger, such that slot-level PDCCH monitoring can be performed for UE complexity and power consumption reduction. This approach provides more flexibility, and saving power. One example is shown in Figure 1.

Figure 1 PDCCH monitoring pattern in a COT

In current Rel-15 specification, UE needs to perform the same PDCCH monitoring pattern configured by RRC. In order to have different PDCCH monitoring patterns for a COT, some enhancement for PDCCH monitoring configuration should be considered.  Pre-defined or semi-static configuration method can be considered to realize the two detection patterns above. However, for semi-static configuration, as the slot detection PDCCH is an absolute slot index configured through search space configuration in monitoringSlotPeriodicityAndOffset, it may not be the first slot of the COT. Therefore, the slot parameter definition in monitoringSlotPeriodicityAndOffset should be changed to the slot position in the COT, or the slot index should be started from the first slot of the COT.

Proposal 1:

- Different PDCCH monitoring patterns can be performed for the first slot of the COT and the remaining slot(s) of the COT. And the PDCCH monitoring occasion granularity of the first slot is smaller than the remaining slot(s) of the COT.

- In NR-U, the slot index indicated in parameter monitoringSlotPeriodicityAndOffset of the search space shall be offset to the starting slot of the COT.

## COT Structure Indication

In licensed band, NR supports dynamic and semi-statically configured slot configuration and the configuration period can be {0.5, 0.625, 1, 1.25, 2, 2.5, 5, 10} ms.  The semi-static DL/UL assignment is done via cell-specific and UE-specific RRC configuration. With the cell-specific configuration, a UE is allocated a DL-unknown-UL pattern. The "unknown" resources are flexible resources that can be assigned DL or UL direction using UE-specific configuration or dynamic signaling. The UE-specific RRC signaling or dynamic signaling includes the indication as per slot basis that can override the "unknown" allocation in the cell-specific configuration. Resources in the slot configuration that are without DL/UL indication remain flexible resources. For NR-U, dynamic and semi-statically configured UL/DL configuration as in licensed band should also be considered. For example, some semi-static UL/DL assignments can be used to indicate DL slots for UE to receive SSB, and some other slots can be assigned for UE to transmit PRACH or configured grant transmission.

As NR R15 PDCCH can be configured to start from any symbol, and gNB can start downlink transmission from the nearest symbol after a successful LBT, such as in the middle of the slot transmission through the search space configuration, therefore once gNB performs LBT successfully on the semi-statically configured DL slot and symbols, gNB can indicate the COT structure through DCI format 2_0 dynamically. Besides, as DCI format 2_0 can indicate multiple slots and the partial slot structure, therefore, DCI format 2_0 in Rel-15 NR can be directly used for the indication of the COT structure.

Observation 1: DCI format 2_0 in Rel-15 NR can be directly used for the indication of the COT structure.

Proposal 2:  Rel-15 DCI format 2_0  for licensed band is sufficient to indicate the COT structure.

## PDSCH Transmission

In Rel-13 LAA, eNB can only start transmission at slot boundaries (symbol 0/7) with 15 kHz subcarrier spacing. The reservation signal is transmitted before the DL transmission start position, i.e., the slot boundary, to reserve the carrier and prevent other devices from occupying the spectrum, which is the overhead of LAA system. Therefore, the overhead of reservation signal should be minimized. In NR, PDSCH can support typeA and typeB scheduling, which have much finer granularities (e.g. 2/4/7 OFDM symbols), therefore, more flexible transmission can be reached.

For NR-U, such different slot types including slot and mini-slot (non-slot-based scheduling) can be utilized to minimize the reservation signal length. The NR gNB should be able to send DL burst as soon as possible whenever it finishes LBT/CCA, resulting in different slot duration. As illustrated in Figure 2, the gNB can choose the type of first slot i.e. mini-slot based on LBT/CCA success location and slot boundary, and then select the type of following slots with a larger size e.g. slot. The type of last slot(s) is/are chosen according to the remaining time within the MCOT. In Figure 2, the rest of the maximum COT is not enough for a normal slot, but enough for two whole mini-slots.

Figure 2: multiple slot durations in an MCOT

For PDSCH transmission in the partial slot at least for the first PDSCH(s) transmitted in the DL transmission burst, below options have been identified as possible candidates.

Option 1: PDSCH(s) as in Rel-15 NR

Option 2: Punctured PDSCH depending on LBT outcome

Option 3: PDSCH mapping type B with durations other than 2/4/7 symbols

Option 4: PDSCH across slot boundary

Option 5: Option 3+ Option 4

For Option 1, gNB can transmit PDSCH 2/4/7 symbols according to the remaining symbols of the first slot, which has no specification impact. If further considering less reservation signals overhead, more choices of PDSCH duration should be defined, which is Option 3, but only limited additional symbol number(e.g.3/9/10 symbols) should be considered for type B mapping in order to reduce blind detection complexity to UE.

For Option 2, if punctured data too much, the decoding performance would be degraded and retransmission would be needed. Besides, it was also proposed that the PDCCH is not punctured, however its DRMS can be shifted to the actual starting position of the DL transmission. However for this method, UE PDCCH monitoring complexity will be extremely increased as UE does not know when gNB perform CCA success. And the DL grant supports any start symbol, any length within the slot and the puncturing and/or rate-matching indication should be introduced.

For Option 4, as the DMRS sequence generation is based on the absolute symbol index and the slot number, some influence on DMRS sequence generation and design should be considered. Therefore, this option should be de-prioritized. Besides, the same problem can be seen in Option 5.

Proposal 3: Among the candidates for PDSCH transmission in the partial slot,

- Option 1 is preferred

- If  Option 3 is considered, only limited additional symbol number should be supported for type B mapping.

## DL Burst Detection

In the ad-hoc meeting, following candidates were discussed to detect gNB’s DL transmission for UE power saving. And it reached the agreement that option 1 was at least to be supported (further details need to be discussed) and option 2 is still FFS.

Option 1: detection of PDCCH using DMRS

Option 2: detection of a preamble or any other signals

It should be noted that the preamble in WiFi is constructed of a training sequence and control information which cannot be identified by LAA/NR-U transmission. Besides, detection of WiFi preamble can lead to significantly higher implementation complexity considering different numerology between NR and Wi-Fi. Furthermore, it seems undesirable or strange that NR-U operates with a signal relying on other standardization. Therefore, the mechanism to detect gNB’s transmission should be based on existing signals/channels in NR, such as PRACH preamble and PSS/SSS.

For option1, wideband DMRS and narrow band DMRS are supported in NR R15. And the wideband DMRS can have better detection probability than narrow band DMRS due to more PRBs used for wideband DM-RS. Besides, for narrow band DMRS, as it combined with PDCCH position, UE needs to detect the narrow band DM-RS blindly in all candidate PDCCH positions which leads to higher complexity. Wideband DM-RS could alleviate this since DM-RS can span over the whole CORESET. Therefore, wideband DMRS is more appropriate for used to detect gNB’s DL transmission. Since wideband DM-RS is optional in the sub-set resource of the CORESET in NR Release 15, it needs to become mandatory if it is used for NR-U as DL burst detection signal. But this can also be a implementation problem.

Observation 2: the preamble in WiFi does not fit perfectly in the NR numerology. And NR-U operates with a signal relying on other standardization is undesirable.

Proposal 4:

- NR-U UE is not expected to receive WiFi preamble.

- Wideband DMRS is more appropriate for used to detect gNB’s DL transmission.

- No further enhancement is needed for wideband DMRS.

# Conclusion

In this contribution, we discuss some potential solutions and techniques for NR-U DL reference signal design and physical channel design, and have the following proposals:

Proposal 1:

- Different PDCCH monitoring patterns can be performed for the first slot of the COT and the remaining slot(s) of the COT. And the PDCCH monitoring occasion granularity of the first slot is smaller than the remaining slot(s) of the COT.

- In NR-U, determine slot index in parameter monitoringSlotPeriodicityAndOffset  of the search space relative to the starting slot of the COT.

Proposal 2:  Rel-15 DCI format 2_0 for licensed band is sufficient to indicate the COT structure.

Proposal 3: Among the candidates for PDSCH transmission in the partial slot,

- Option 1 is preferred

- If Option 3 is considered, only limited additional symbol number is additionally supported for type B mapping.

Proposal 4:

- NR-U UE is not expected to receive WiFi preamble.

- Wideband DMRS is more appropriate for used to detect gNB’s DL transmission.

- No further enhancement is needed for wideband DMRS.

# References

- 3GPP, RP-182878, New WID on NR-based Access to Unlicensed Spectrum, Qualcomm, RAN#82, December, 2018.

- 3GPP, R1-1810326, Discussion on channel access mechanism for NR-U, ZTE, RAN1#95, November, 2018.