3GPP TSG RAN WG1 Meeting #96 R1-1901765 Athens, Greece, 25th February – 1st March, 2019 CR-Form-v11.2. CHANGE REQUEST. 38.213; CR; DRAFT; rev; -; Current version:; 15.4.0. For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests.. Proposed change affects:; UICC apps; ME; X; Radio Access Network; X; Core Network. Title:; Draft CR on identifying transmission occasion after resetting a PC closed loop. Source to WG:; ZTE. Source to TSG:; RAN1. Work item code:; NR_newRAT-Core; Date:; 2019-2-15. Category:; F; Release:; Rel-15. Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900.; Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16). Reason for change:; In TS38.213, if P0 or alpha which is related to j is re-configured by gNB, UE shall reset closed-loop power control value f(i=0, l)=0, where l and j are associated with the same SRI. f(i, l) is maintained independently for each of closed loop l, but uplink transmission occasion i is shared among all PUSCH transmissions in one BWP. If only one closed-loop power control value, say l=0, needs to be reset, it should be f(i, l=0)=0 rather than f(i=0, l=0)=0. - In this case, if transmission occasion index i is also reset, it also affects the other closed loop process f(i, l=1). There is no description about how to determine the value of f(i=0, l=1) in TS 38.213. Straightforwardly, f(i, l=1) should maintanin its historical value and is decoupled with the resetting process of f(i, l=0). - Besides, even for closed loop l=0, resetting the transmission occasion i is still meaningless. Therefore it is proposed to remain the transmission occasion index i when resetting f(i,l). PUCCH transmission also support up to 2 closed-loop power control loops. So the similar changes are recommended. SRS transmission only supports 1 closed-loop power control loop, but for consistency, the transmission index is suggested not to be reset when resetting h(i).. Summary of change:; When resetting closed-loop power control value f(i,l),g(i,l) or h(i), do not reset transmission index i to 0, and just keep it as naturally accumulated.. Consequences if not approved:; In case of two closed-loop power control loops, resetting the transmission occasion index i of one closed loop causes unclear procedure for the other loop.. Clauses affected:; 7, 7.1.1, 7.2.1, 7.3.1. Y; N. Other specs; X; Other core specifications; TS/TR ... CR .... affected:; X; Test specifications; TS/TR ... CR .... (show related CRs); X; O&M Specifications; TS/TR ... CR .... Other comments:.

# 7	Uplink Power control

Uplink power control determines a power for PUSCH, PUCCH, SRS, and PRACH transmissions.

A UE does not expect to simultaneously maintain more than four pathloss estimates per serving cell for all PUSCH/PUCCH/SRS transmissions as described in Subclauses 7.1.1, 7.2.1, and 7.3.1.

A PUSCH/PUCCH/SRS/PRACH transmission occasion  is defined by a slot index  within a frame with system frame number , a first symbol  within the slot, and a number of consecutive symbols .

<Unchanged part omitted>

### 7.1.1	UE behaviour

<Unchanged part omitted>

-	 is the PUSCH power control adjustment state  for active UL BWP  of carrier  of serving cell  and PUSCH transmission occasion  if the UE is not provided tpc-Accumulation, where

-	The  values are given in Table 7.1.1-1

-	 is a sum of TPC command values in a set  of TPC command values with cardinality  that the UE receives between  symbols before PUSCH transmission occasion  and  symbols before PUSCH transmission occasion  on active UL BWP  of carrier  of serving cell  for PUSCH power control adjustment state , where  is the smallest integer for which  symbols before PUSCH transmission occasion  is earlier than  symbols before PUSCH transmission occasion

-	If a PUSCH transmission is scheduled by a DCI format 0_0 or DCI format 0_1,  is a number of symbols for active UL BWP  of carrier  of serving cell  after a last symbol of a corresponding PDCCH reception and before a first symbol of the PUSCH transmission

-	If a PUSCH transmission is configured by ConfiguredGrantConfig,  is a number of  symbols equal to the product of a number of symbols per slot, , and the minimum of the values provided by k2 in PUSCH-ConfigCommon for active UL BWP  of carrier  of serving cell

-	If the UE has reached maximum power for active UL BWP of carrier  of serving cell  at PUSCH transmission occasion  and , then

-	If UE has reached minimum power for active UL BWP of carrier  of serving cell  at PUSCH transmission occasion  and , then

-	A UE resets accumulation of a PUSCH power control adjustment state  for active UL BWP  of carrier  of serving cell  to

-	If a configuration for a corresponding  value is provided by higher layers

-	If a configuration for a corresponding  value is provided by higher layers

-	If  and the PUSCH transmission is scheduled by a DCI format 0_1 that includes a SRI field, and the UE is provided higher SRI-PUSCH-PowerControl, the UE determines the value of  from the value of  based on an indication by the SRI field for a sri-PUSCH-PowerControlId value associated with the sri-P0-PUSCH-AlphaSetId value corresponding to  and with the sri-PUSCH-ClosedLoopIndex value corresponding to

-	If  and the PUSCH transmission is scheduled by a DCI format 0_0 or by a DCI format 0_1 that does not include a SRI field or the UE is not provided SRI-PUSCH-PowerControl,

-	If ,  is provided by the value of powerControlLoopToUse

-	 is the PUSCH power control adjustment state for active UL BWP  of carrier  of serving cell  and PUSCH transmission occasion  if the UE is provided tpc-Accumulation, where

-	 absolute values are given in Table 7.1.1-1

-	If the UE receives a random access response message in response to a PRACH transmission on active UL BWP  of carrier  of serving cell  as described in subclause 8

-	, where  and

-	 is a TPC command value indicated in the random access response grant of the random access response message corresponding to the PRACH transmission on active UL BWP  of carrier  in the serving cell , and

-	 and  is provided by higher layers and corresponds to the total power ramp-up requested by higher layers from the first to the last random access preamble for carrier  in the serving cell ,  is the bandwidth of the PUSCH resource assignment expressed in number of resource blocks for the first PUSCH transmission on active UL BWP  of carrier  of serving cell , and  is the power adjustment of first PUSCH transmission on active UL BWP  of carrier  of serving cell .

<Unchanged part omitted>

### 7.2.1	UE behaviour

<Unchanged part omitted>

-	is a downlink pathloss estimate in dB calculated by the UE using RS resource index  as described in Subclause 7.1.1 for the active DL BWP of carrier  of the primary cell  as described in Subclause 12

-	If the UE is not provided pathlossReferenceRSs or before the UE is provided dedicated higher layer parameters, the UE calculates  using a RS resource obtained from the SS/PBCH block that the UE uses to obtain MIB

<Unchanged part omitted>

-	For the PUCCH power control adjustment state  for active UL BWP  of carrier  of primary cell  and PUCCH transmission occasion

-	 is a TPC command value and is included in a DCI format 1_0 or DCI format 1_1 for active UL BWP  of carrier  of the primary cell  that the UE detects for PUCCH transmission occasion  or is jointly coded with other TPC commands in a DCI format 2_2 with CRC scrambled by TPC-PUCCH-RNTI [5, TS 36.212], as described in Subclause 11.3

-	 if the UE is provided twoPUCCH-PC-AdjustmentStates and PUCCH-SpatialRelationInfo and  if the UE is not provided twoPUCCH-PC-AdjustmentStates or PUCCH-SpatialRelationInfo

-	If the UE obtains a TPC command value from a DCI format 1_0 or a DCI format 1_1 and if the UE is provided PUCCH-SpatialRelationInfo, the UE obtains a mapping, by an index provided by p0-PUCCH-Id, between a set of pucch-SpatialRelationInfoId values and a set of values for closedLoopIndex that provide the  value(s). If the UE receives an activation command indicating a value of pucch-SpatialRelationInfoId, the UE determines the value closedLoopIndex that provides the value of  through the link to a corresponding p0-PUCCH-Id index

-	If the UE obtains one TPC command from a DCI format 2_2 with CRC scrambled by a TPC-PUCCH-RNTI, the  value is provided by the closed loop indicator field in DCI format 2_2

-	 is the current PUCCH power control adjustment state  for active UL BWP  of carrier  of serving cell  and PUCCH transmission occasion , where

-	The  values are given in Table 7.1.2-1

-	 is a sum of TPC command values in a set  of TPC command values with cardinality  that the UE receives between  symbols before PUCCH transmission occasion  and  symbols before PUCCH transmission occasion  on active UL BWP  of carrier  of serving cell  for PUCCH power control adjustment state, where  is the smallest integer for which  symbols before PUCCH transmission occasion  is earlier than  symbols before PUCCH transmission occasion

-	If the PUCCH transmission is in response to a detection by the UE of a DCI format 1_0 or DCI format 1_1,  is a number of symbols for active UL BWP  of carrier  of serving cell  after a last symbol of a corresponding PDCCH reception and before a first symbol of the PUCCH transmission

-	If the PUCCH transmission is not in response to a detection by the UE of a DCI format 1_0 or DCI format 1_1,  is a number of  symbols equal to the product of a number of symbols per slot, , and the minimum of the values provided by k2 in PUSCH-ConfigCommon for active UL BWP  of carrier  of serving cell

-	If the UE has reached maximum power for active UL BWP  of carrier  of primary cell  at PUCCH transmission occasion  and , then

-	If UE has reached minimum power for active UL BWP  of carrier  of primary cell  at PUCCH transmission occasion  and , then

-	If a configuration of a  value for a corresponding PUCCH power control adjustment state  for active UL BWP  of carrier  of serving cell  is provided by higher layers,

-

If the UE is provided PUCCH-SpatialRelationInfo, the UE determines the value of  from the value of  based on a pucch-SpatialRelationInfoId value associated with the p0-PUCCH-Id value corresponding to  and with the closedLoopIndex value corresponding to ; otherwise,

-	Else,

-	, where , and  is the TPC command value indicated in a random access response grant corresponding to a PRACH transmission or is the TPC command value in a DCI format with CRC scrambled by C-RNTI or MCS-C-RNTI that the UE detects in a first PDCCH reception in a search space set provided by recoverySearchSpaceId if the PUCCH transmission is a first PUCCH transmission after 28 symbols from a last symbol of the first PDCCH reception, and, if the UE transmits PUCCH on active UL BWP  of carrier  of serving cell ,

;

otherwise,

where  is provided by higher layers and corresponds to the total power ramp-up requested by higher layers from the first to the last preamble for active UL BWP  of carrier  of primary cell , and  corresponds to PUCCH format 0 or PUCCH format 1

<Unchanged part omitted>

### 7.3.1	UE behaviour

<Unchanged part omitted>

-	If a configuration for a  value or for a  value for a corresponding SRS power control adjustment state  for active UL BWP  of carrier  of serving cell  is provided by higher layers

-

-	Else

-

where

is the TPC command value indicated in the random access response grant corresponding to the random access preamble that the UE transmitted on active UL BWP  of carrier  of the serving cell , and

;

where  is provided by higher layers and corresponds to the total power ramp-up requested by higher layers from the first to the last preamble for active UL BWP  of carrier  of serving cell .

<Unchanged part omitted>