3GPP TSG RAN WG1 #96										R1- 1901820

Athens, Greece, Feb 25th 1st March 2019

Agenda Item:	7.2.10.1.4

Source:	MediaTek

Title:	 System level performance with explicit link level simulation for downlink angle based technique

Document for:	Discussion, Decision

# 1 Introduction

At RAN1#1901ah, the system level performance report template has been agreed. Base on the template, in this contribution we show simulation results on DL-AoD horizontal positioning performance for Umi FR1 & Uma FR1 scenarios.

# 2 System Level Simulation Results

## 2.1 Scenario 2 – Urban micro

The text proposal capturing our results for scenario 2 is provided in appendix.

### 2.1.1 Simulation results

The simulation setting can be found in appendix. In the following, we show the horizontal positioning error CDF curve for DL-AoD positioning technique:

Figure 1: horizontal positioning error CDF curve for DL-AoD positioning technique for Umi

Observation 1: For Umi, the DL-AoD positioning technique need to be further studied to check if it can be used for positioning since the requirement that horizontal positioning error < 10m for 80% of UEs is not meet, at least in our simulation setting.

### 2.1.2 Summary of results for Scenario 2

The DL-AoD positioning technique need to be further studied to check if it can be used in Umi scenario for positioning.

## 2.2 Scenario 3 – Urban macro

The text proposal capturing our results for scenario 3 is provided in appendix.

### 2.2.1 Simulation results

The simulation setting can be found in appendix. In the following, we show the horizontal positioning error CDF curve for DL-AoD positioning technique:

Figure 2: horizontal positioning error CDF curve for DL-AoD positioning technique for Uma

Observation 2: The DL-AoD positioning technique is not suitable for UMa scenario since the requirement that horizontal positioning error < 50m for 80% of UEs is not met.

### 2.2.2 Summary of results for Scenario 3

The DL-AoD positioning technique is not suitable for UMa scenario since the performance cannot meet the regulatory requirement.

# 3 Conclusion

In this contribution, we show simulation results on DL-AoD horizontal positioning performance for Umi FR1 & Uma FR1 scenarios. We have the following observations:

Observation 1: For Umi, the DL-AoD positioning technique need to be further studied to check if it can be used for positioning since the requirement that horizontal positioning error < 10m for 80% of UEs is not meet, at least in our simulation setting.

Observation 2: The DL-AoD positioning technique is not suitable for UMa scenario since the requirement that horizontal positioning error < 50m for 80% of UEs is not met.

# Appendix: TR skeleton for section 8

# 8 Evaluation Results of NR Positioning

## 8.1 Downlink evaluations

### 8.1.1 System simulations for Scenario 1 – Indoor Open Office

(void)

### 8.1.2 System simulations for Scenario 2 - Umi

Table 1: Parameters for Downlink evaluations in Scenario 2

| Parameter | [MTK, FR1] |
| --- | --- |
| Channel model (baseline, otherwise state any modifications) | AWGN, assume all LOS |
| Carrier frequency | 4Gz |
| Subcarrier spacing | 15kHz |
| Reference Signal Transmission Bandwidth | 20MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | In a two-symbol PRS occasion: 1st symbol is comb-4 starting at RE 0 2nd symbol is comb-4 starting at RE 2 |
| Reference signal (type of sequence, number of ports, …) | PN sequence, one port |
| Number of sites | 7 |
| Number of symbols used per occasion | 2 |
| number of occasions used per positioning estimate | 10 |
| Power-boosting level | 6.02 dB |
| Uplink power control (applied/not applied) | Not applied |
| interference modelling (ideal muting, or other) | Ideal muting |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | RSRP = average power per RE |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, taylor series, etc) | AoD estimation: gNB beam pattern is known in advance, so a LUT table saving the AoD and power of each beam can be constructed. UE reports the measured RSRP ratio of beams to gNBs (see method 1 in our companion contribution R1-1901817) Transforming AoD estimation results to UE location: Least-square estimator |
| Network synchronization assumptions | Ideal |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | Each gNB perform beamsweeping with LCS α angle = [-52.5 -37.5 -22.5 -7.5 7.5 22.5 37.5 52.5] degrees. Note these angles are equally 15-degree spaced and centered at zero |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | N/A |
| Additional notes, if any | Assume UE use 1 omni-directional rx antenna element |

Table 2: results for downlink methods evaluations of Scenario 2 – UMi horizontal positioning

| Percentile | 50 | 67 | 80 | 90 |
| --- | --- | --- | --- | --- |
| DL-AoD positioning | 12m | 16m | 29m | 42m |

### 8.1.3 System simulations for Scenario 3 - Uma

The simulation setting basically follow the setting for UMa FR1 as captured in TR38.855. The modified parameters for UMa are the same as those in Table 1.

Table 3: results for downlink methods evaluations of Scenario 3 – Uma horizontal positioning

| Percentile | 50 | 67 | 80 | 90 |
| --- | --- | --- | --- | --- |
| DL-AoD positioning | 31m | 43m | 73m | 130m |