3GPP TSG RAN WG1 Meeting #96 	R1-1901822

Athens, Greece, February 25th– March 1st, 2019

Agenda Item: 7.2.6.1.1

Source: MediaTek Inc.

Title: Evaluations and enhancements of NR PDCCH for URLLC

Document for: Discussion and Decision

# Introduction

On the enhancement of NR-PDCCH for Rel-16 URLLC, the following agreement has been made during RAN1#AH1901 [1]:

Agreements:

For the DCI format scheduling Rel-16 NR URLLC,

- Support potential reduction of the number of bits for at least one of the following fields compared to Rel-15 DCI

  - Frequency domain resource assignment

  - Time domain resource assignment

  - Modulation and coding scheme

  - HARQ process number

  - Redundancy version

  - PUCCH resource indicator

  - PDSCH-to-HARQ feedback timing indicator

  - Downlink assignment index

  - Note: Reduction of other fields are not precluded

- Down-select one of the following options for the DCI format size – targeting down-selection in RAN1#96 (not to be captured in the TR for now)

  - Option 1: Fixed DCI size targeting a reduction of 10~16 bits reduction compared to the DCI format size of Rel-15 fallback DCI

  - Option 2: aligned with Rel-15 fallback DCI

  - Option 3: configurable DCI size with the limitation as below

    - Minimum DCI size should target 10~16 bits reduction compared to the DCI format size of Rel-15 fallback DCI

    - Maximum size should be equal to the DCI format size of Rel-15 fallback DCI

  - Option 4: DCI with configurable sizes for some fields, while

    - The maximum DCI size can be larger than Rel-15 fallback DCI

    - The minimum DCI size target a reduction of 10~16 bits less than the DCI format size of Rel-15 fallback DCI

    - Provide the possibility to align with the size of the Rel-15 fallback DCI (including possible zero padding if any)

  - Option 5: no introduction of new DCI format due to this SI

- Note: The DCI format may be impacted by other objectives in this study item and/or the following work item, e.g. PDCCH repetition mechanism and/or UCI enhancement, or may be impacted by objectives in other study item and/or work item, e.g. multi-TRP transmission from Rel-16 work item

In this contribution, we evaluate the performance of NR-PDCCH for URLLC in terms of reliability and latency. Also, we provide detailed evaluation and analysis of the possible enhancements for NR DL control channels.

# DL Control channel design

With the smallest subcarrier spacing of 15 kHz the slot duration is 1ms. In this case, due to the URLLC latency constraint, a single transmission is available only, thus the control and data reliability need to be very high. The overall reliability of (10-6) requires control and data reliability better than (0.5*10-7). The flexibility of DL control allocation is very limited compared to the DL data allocation due to the following reasons;

- The control channel uses selected aggregation levels (that are powers of 2) to increase user packing

- No user-specific beamforming can be applied to the DL control channel, hence no SNR improvements

- No opportunity for soft combining, hence no SNR improvements from retransmissions

- The maximum number of monitored candidates by a UE per slot is fixed

- The maximum number of non-overlapped control channel elements (CCEs) per slot is fixed

Observation 1: There is limited flexibility in improving the reliability of DL control channel compared to the DL data channel.

The following options can be explored for PDCCH enhancement.

## Reducing the DCI size

One important approach to enhance the PDCCH reliability is by reducing the DCI payload size, which will lower the effective code rate of the DL control. Considering that the fallback DCI (DCI formats 0_0 and 1_0), relatively, has small payload size, it is logical to use it as a starting point. For URLLC, due to the low latency requirement, some of the fields can be removed as they are either not essential or can be fixed to predefined values.

- Some fields can be fixed to a pre-defined value, such as ‘VRB-to-PRB mapping’ and ‘PDSCH-to-HARQ feedback timing indicator’, and they can be removed from the DCI.

- The granularity of the frequency and time domain resource allocations can be reduced, which results in less number of bits to indicate the allocation

- Given the required low latency, the possible number of HARQ retransmissions will be low compared to eMBB service. Thus, the redundancy version and HARQ process number fields’ size can be reduced.

Figure 1 and Figure 2 show the performance of NR PDCCH for carrier frequency 700MHz and 4GHz using the simulation parameters in Table 4 agreed in RAN1#94b and RAN1#95. Simulation results show that reducing the DCI size from 40 bits down to 24 bits provides gains of ~1dB and ~1.2dB for both of AL16 and AL8, respectively.

Figure 1: Performance of NR-PDCCH with different DCI payloads for carrier frequency 4GHz.

Figure 2: Performance of NR-PDCCH with different DCI payloads for carrier frequency 700MHz.

In addition to meeting the URLLC requirements, compact DCI will reduce the required AL to achieve a specific PDCCH BLER target.

The design of the compact DCI could be optimized according to the system operation settings. For example, when operating in 15 kHz SCS, only a single-shot transmission would be possible within the latency restriction. Thus, for this case, the DCI fields can be optimized for operating in a single-shot approach. On the other hand, when operating in 60kKHz SCS, several (re)transmissions are possible within the latency restriction, and the DCI fields can be optimized to take advantage of this flexibility.

In Table 1 and Table 2 and based on the LLS simulation results shown in Figure 1 and Figure 2 and the geometry CDF in Figure 8 and Figure 9, we present the percentage of UEs in the cell using each AL for a target BLER of 10-5  and 10-6   and we analyse the average used CCE resources for PL40 and PL24.

| BLER | PL | AL16 | AL8 | AL4 | AL2 | AL1 | Average NR-PDCCH resources (CCE) | NR-PDCCH resources saving of PL24 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1e-5 | PL40 | 6% | 38% | 24% | 16% | 16% | 5.44 | 20.22% |
|  | PL24 | 1% | 29% | 33% | 17% | 20% | 4.34 |  |
| 1e-6 | PL40 | 14% | 41% | 20% | 16% | 9% | 6.73 | 18.12% |
|  | PL24 | 7% | 36% | 27% | 13% | 17% | 5.51 |  |

Table 1 : 700MHz, 2Rx, 30 kHz, 2 OS CORESET, TDL-C 300ns

| BLER | PL | AL16 | AL8 | AL4 | AL2 | AL1 | Average NR-PDCCH resources (CCE) | NR-PDCCH resources saving of PL24 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1e-5 | PL40 | 0% | 5% | 32% | 29% | 34% | 2.6 | 13.07% |
|  | PL24 | 0% | 2% | 28% | 28% | 42% | 2.26 |  |
| 1e-6 | PL40 | 0% | 11% | 34% | 26% | 29% | 3.05 | 14.42% |
|  | PL24 | 0% | 5% | 33% | 27% | 35% | 2.61 |  |

Table 2: 4GHz, 4Rx, 30 kHz, 2 OS CORESET, TDL-C 300ns

We observe that using DCI with PL24 allows for 18% CCE resource saving compared to using PL40 for target BLER 10-6 at 700MHz carrier frequency. It also allows for 14% CCE resource saving for target BLER 10-6 at 4GHHz carrier frequency.

Observation 2: The Use of compact DCI allows to improve the reliability and also enhances the spectral efficiency.

Also, the maximum number of blind decodes per slot is a function of SCS. In order not to increase the blind decodes with additional DCI format, we can have following options:

- We can consider having compact DCI operation as SCS dependent, because the blind-decoding budget for small SCS is higher than the one for large SCS.

- The UE is not expected to monitor both the compact DCI and normal DCI in the same monitoring occasion for certain SCS to meet the #BDs budget.

Proposal 1: The increased complexity of blind detection by having a new DCI format should be taken into account when evaluating the advantage of the compact DCI.

Although we think that the fallback DCI should be considered as a starting point for the DCI size reduction, it is important to note that some fields are not available in the fallback DCI but could be very important for URLLC performance and scheduling. For example, MIMO related DCI fields (antenna ports, TCI fields, etc.) are not available in the fallback DCI but could be very beneficial for URLLC performance.

A configurable DCI size can guarantee better scheduling flexibility and allow for more options for the gNB. Some fields could be useful or not depending on the deployment scenario.

We therefore favour option 3 and option 4.

Proposal 2: Explore options 3 and 4 for the design of the compact DCI.

In general, the design of the compact DCI shall be realized based on the fallback DCI as a baseline and then to consider any possible optimization of the different fields. The approach we would recommend to be adopted is to study the need of the different DCI fields and remove the unnecessary ones and optimize the size of the remaining fields while ensuring good performance and good scheduling flexibility.

Proposal 3: The design of the compact DCI should be based on the following aspects:

- Removing unnecessary fields that are not essential for URLLC scheduling

- Optimizing/reducing the size of the remaining fields based on the URLLC traffic characteristics

- No new fields to be added unless agreed in other AIs.

## CCEs/BDs Limits

### Increase of CCEs/BDs

In this section, we will provide some latency analysis for different numerologies using the parameters given in Table 5. In this analysis, we compare the CDF of latency for different PDCCH monitoring configuration periodicities. The aim is to determine the number of required PDCCH monitoring occasions to meet the URLLC latency requirement.

Based on the number of required PDCCH monitoring occasions and assuming at least the support of two non-overlapping PDCCH candidates of AL16 per occasion, the number of required CCEs is derived. The assumption of two non-overlapping PDCCH candidates of AL16 per occasion is needed to allow for low blockage probability.

Figure 3 and Figure 4 show the CDF of the latency for a single shot transmission for SCS = 15 kHz and SCS = 30 kHz. Various PDCCH monitoring periodicities are evaluated to determine the periodicity needed to meet the latency requirement.

For example for SCS = 15 kHz and with a single shot transmission, PDCCH monitoring configuration with 4 OS periodicity is needed to meet the 1ms latency required, which is equivalent to at least 3 PDCCH monitoring occasions configured.

| Figure 3: SCS =15 kHz | Figure 4: SCS =30 kHz |
| --- | --- |

Table 3 shows the number of CCEs required to meet the 1ms latency target while accommodating two non-overlapping AL16 candidates or only one AL16 candidate per each monitoring occasion.

The Table shows that 96 CCEs and 32 CCES are required respectively for SCS = 15 kHz and 30 kHz with two non-overlapping AL16 candidates per occasion.

As a result, we can deduce that the number of CCEs need to be increased for 15 kHz to 96 CCEs instead of 56 CCEs in Rel-15 and the number of CCEs could be maintained the same for the remaining numerologies.

|  | Monitoring occasions | CCEs required (2xAL16) | CCEs required (1xAL16) | Rel-15 |
| --- | --- | --- | --- | --- |
| SCS=15kHz | 3 | 96 | 48 | 56 |
| SCS=30kHz | 1 | 32 | 16 | 56 |

Table 3 : CCEs required assuming two non-overlapping AL16 candidates per monitoring occasion

Proposal 4: Increase the number of CCEs to 96 for SCS = 15 kHz and keep the same Rel-15 numbers for the remaining numerologies.

Another concern about increasing the BDs/CCEs limits is the impact on the eMBB-only UEs. In fact, increasing the CCEs and BDs limits for all the UEs including the eMBB-only UEs will add extra complexity for the UE implementation. eMBB-only UEs are not required to support the increased CCEs/BDs limits intended mainly to the UEs supporting the URLLC service. One possibility to tackle this issue is to have more flexibility in defining the CCEs/BDs limits. One approach is to specify multiple CCEs/BDs numbers for each numerology (E.g. one intended for eMBB and one for the URLLC service) and define them as UE capabilities (E.g. Rel-15 as the baseline capability and Rel-16 new capability).

Proposal 5:  Multiple BDs/CCEs limits could be specified per numerology and defined as UE capabilities.

### CCEs/BDs Budget distribution

It is very likely that the eMBB scheduling and the group-common DCI will take place most of the time in the first half of the slot. However, the URLLC scheduling could be in the first or the second half of the slot with equal probability. Splitting the budget equally between half-slots could penalize the traffic scheduled in the first-half.

Therefore, multiple options are possible to distribute the new CCEs/BDs budget. One approach is to have an asymmetric limits of CCEs/BDs budgets between the first and the second half of the slot, which means different CCEs/BDs budgets for the first half-slot and the second half-slot. E.g. 48 BDs and 20BDs for the first and second half slot respectively. 64 CCEs and 32 CCEs for the first and second half slot respectively for SCS = 15 kHz.

Another approach is to split the budget between monitoring occasions/search spaces/CORESETs. The budget could be split equally between monitoring occasions (or search spaces or CORESETs), or also some priorities could be given to some monitoring occasions (or search spaces or CORESETs).  E.g. higher priority given to monitoring occasions (or search spaces or CORESETs) scheduling URLLC traffic. Or E.g. allocate specific budget for the 1st monitoring occasion (since it usually has the GC-PDCCH) and then split the remaining budget between the remaining occasions.

Proposal 6: Adopt one of the two following options for CCEs/BDs limits:

- Option 1: Asymmetric limits of BDs/CCEs per half slot.

- Option 2: Split the budget between monitoring occasions/search spaces.

### PDCCH overbooking and dropping rules

A pseudo-code is defined in Rel-15 specs to handle the PDCCH overbooking. In the specified pseudo-code, CCEs/BDs allocation starts with search space (SS) with the lowest index. Therefore, priorities is currently given to the SS according to their indexes. This is not ideal for the URLLC traffic which could be scheduled by an SS with high index.

To handle this issue multiple options are possible:

- Option 1: Allocate priorities to search spaces.

- Option 2: Split the CCEs/BDs budget equally (or with some priorities to the URLLC traffic) to avoid the risk of the CCEs being consumed from the initial monitoring occasion/CORESETs/search space.

# PDCCH monitoring enhancements

In this section, we will perform some latency analysis for the UL and DL transmissions, and we will assess if any link direction is particularly impacting the latency performance more than the other link.

Once the UE has a packet to be transmitted in UL, the UE needs to generate and send an SR to the gNB to be allocated some resources for the UL transmission. The SR is not transmitted immediately since the UE needs to comply with the configured SR periodicity. The gNB needs also some time to process the SR and generate the UL grant. The DCI carrying the UL grant is not transmitted immediately since the PDCCH transmission needs some time alignment to comply with configured PDCCH periodicity and bitmap. Therefore, the SR transmission and decoding procedure will clearly affect the overall UL latency.

Using the parameters and assumptions given in Table 5, we show in that Figure 5 UL latency is higher than DL latency and this is due to the UL control aspects fully dependent on the SR-based UL-SCH operation. As the processing delays and the transmission procedures are not the same in UL and DL this leads to concluding that the overall latencies in UL and DL are not symmetric and the assumption of symmetric latency for UL and DL is invalid. Therefore, Different techniques could be envisaged to handle each link differently to improve its latency.

Observation 3: The incurred latency in UL transmission is higher than that for DL transmission.

Figure 5: DL/UL transmission latency.

Therefore, one possible approach to reduce the latency in UL is to have more frequent PDCCH monitoring occasions to meet the latency requirement and compensate for the additional delay caused by the scheduling request procedure. There is however no current mechanism to differentiate between PDCCH monitoring occasions to target UL or DL traffic during the monitoring and the decoding of the control channels.

One possible proposal, instead of permanently increasing the PDCCH monitoring occasions, it is possible to define an additional PDCCH monitoring configuration that is used only when there is UL data to be transmitted. Hence, the UE could be configured with a secondary PDCCH monitoring configuration (or secondary Search Space) that is monitored at certain period of times based on specific triggers (e.g. SR transmission) as shown in Figure 6.

Figure 6: Secondary PDCCH monitoring configuration triggered by SR.

For example, based on the parameters and assumptions in Table 5, latency analysis for UL and DL is established in

Figure 7  for various PDCCH monitoring configurations. To meet the latency requirement, one possible PDCCH monitoring adaptation based on the SR is:

- Default Configuration: PDCCH monitoring configuration with 4 OS periodicity,

- Secondary Configuration: PDCCH monitoring configuration with 2 OS periodicity (applied when there is SR).

Figure 7: UL and DL latencies based on various PDCCH monitoring configurations.

The primary and secondary PDCCH monitoring configurations should be taken into account to restrict the UE blind decoding to maintain it within the reasonable range. The UE blind decoding is controlled by higher layer RRC parameters. The gNB can select the proper settings for secondary PDCCH monitoring configuration to restrict the UE blind decoding. E.g. reduce number of monitored ALs, DCI formats and/or PDCCH candidates when secondary configuration is used.

Proposal 7: The UE is to be configured with two PDCCH monitoring configurations where the secondary configuration has more PDCCH monitoring occasions and it is triggered when the SR is transmitted.

# Conclusion

In this contribution, we evaluated the performance of NR DL control channel in terms of reliability and latency. Based on the discussions and the analysis, we have the following observations and proposals:

Observation 1: There is limited flexibility in improving the reliability of DL control channel compared to the DL data channel.

Observation 2: The Use of compact DCI allows to improve the reliability and also enhances the spectral efficiency.

Observation 3: The incurred latency in UL transmission is higher than that for DL transmission.

Proposals for PDCCH enhancement:

Proposal 1: The increased complexity of blind detection by having a new DCI format should be taken into account when evaluating the advantage of the compact DCI.

Proposal 2: Explore options 3 and 4 for the design of the compact DCI.

Proposal 3: The design of the compact DCI should be based on the following aspects:

- Removing unnecessary fields that are not essential for URLLC scheduling

- Optimizing/reducing the size of the remaining fields based on the URLLC traffic characteristics

- No new fields to be added unless agreed in other AIs.

Proposal 4: Increase the number of CCEs to 96 for SCS = 15 kHz and keep the same Rel-15 numbers for the remaining numerologies.

Proposal 5:  Multiple BDs/CCEs limits could be specified per numerology and defined as UE capabilities.

Proposal 6: Adopt one of the two following options for CCEs/BDs limits:

- Option 1: Asymmetric limits of BDs/CCEs per half slot.

- Option 2: Split the budget between monitoring occasions/search spaces.

Proposal 7: The UE is to be configured with two PDCCH monitoring configurations where the secondary configuration has more PDCCH monitoring occasions and it is triggered when the SR is transmitted.

# References

- RAN1 # AH1901 Chairman Notes.

- RP-180524, “Summary of calibration results for IMT-2020 self-evaluation”.

# Appendix A

| Figure 8 : Urban Macro - URLLC, 4GHz Model B | Figure 9 : Urban Macro - URLLC, 700MHz Model B |
| --- | --- |

Table 4 : LLS simulation parameters

| Parameter | Value |
| --- | --- |
| Carrier Frequency | 4GHz and 700MHz |
| BW | 40MHz |
| SCS | 30kHz |
| gNB antenna configuration | 4 Tx antenna |
| UE antenna configuration | 4 Rx antenna for 4GHz 2 Rx antenna for 700MHz |
| Channel Model | TDL-C, 300ns, 3kM/h |
| CORESET | 2 OS |

Table 5: Assumptions and parameters for UL/DL latency analysis

| SR periodicity | 2 OS |
| --- | --- |
| SR duration | 1 OS |
| SR processing | SR duration |
| UE processing | Rel-15 capability 2 |
| gNB processing for DL | N2 |
| gNB processing for UL | N1 |
| gNB decodes HARQ-ACK | PUCCH duration |
| COREST duration | 1 OS |
| PUSCH duration | 2 OS |
| PDSCH duration | 2 OS |
| PUCCH Duration | 2 OS |
| N1 {15kHz, 30kHz, 60kHz} | { 3, 4.5 , 9 } |
| N2 {15kHz, 30kHz, 60kHz} | { 5, 5.5 , 11 } |