3GPP TSG RAN WG1 #96			R1-1901910

Athens, Greece, 25th February – 1st March, 2019

## Agenda item:	7.2.6.1.2

Source: 	AT&T

Title: 	UCI Enhancements for NR

Document for:	Discussion/Decision

# Introduction

In RAN #80, a new study item on Physical layer enhancements for NR URLLC was approved.  The objective of the study item is to investigate different URLLC L1 improvements to further improve reliability/latency. In the RAN1#AH1901 meeting, the following agreements were made.

Agreements:

- Down-select in RAN1#96 for potential A-CSI on PUCCH

  - Opt.1: A-CSI report on PUCCH triggered by DL-scheduling DCI.

    - For measurement source

      - Alt.1: Based on CSI-RS/CSI-IM measurement

      - Alt.2: Based on DMRS/PDSCH/PDCCH measurement

    - For report quantity

      - Alt.1: R15 baseline

      - Alt.2: Delta CQI

      - Alt.3: Delta SINR

    - For report timeline

      - Alt.1: R15 timeline

      - Alt.2: New timeline

  - Opt.2: A-CSI report on PUCCH based on group-common PDCCH (similar to A-SRS triggering in GC-PDCCH in Rel-15) using Rel-15 mechanisms for measurement source, report quatity, and timeline (A-CSI triggered to transmit on PUSCH)

  - Opt.3: No A-CSI on PUCCH due to this SI

Companies are encouraged to perform more evaluations/analysis w.r.t. the above options to facilitate coming up with observations and eventually drawing conclusion

In this contribution, we provide detailed analysis of DMRS/PDSCH SNR based CSI estimation for URLLC.

# Fast CSI Computation Using DMRS

NR Release 15 supports two BLER thresholds (10-1, 10-5) for CQI reporting. The second threshold improves the reliability of URLLC. However, NR release 15 supports CSI estimation using CSI-RS and reporting using a dedicated PUCCH/PUSCH resources for CSI reporting.  Note that in practice CSI-RS is transmitted periodically (say for every 5 slots) and CSI is reported periodically with a period say 5 slots (for periodic and semi-persistent reporting).  In our view the conventional procedure for CSI estimation and reporting is very inefficient for accurate CSI and delay sensitive applications such as NR-URLLC. Hence we propose to use DM-RS based CSI estimation. Note that DMRS is transmitted along with PDSCH and uses the same precoding as that of PDSCH.

Another drawback with conventional CSI-RS (only) based technique is that with target BLER of 10-5, small variations in the channel causes significant BLER. This is because, the BLER curves are very steep around BLER of 10-5 compared to the BLER targets of 10-1. Hence we need an accurate CSI with these low BLER targets.

## Motivation for DMRS based CSI Estimation:

Figure 1 and 2 show a  snapshot of RI reported using conventional CSI-RS based CSI estimation for 2 and 4 CSI-RS ports with 2 and 4 receiving antennas, respectively. Since the rank information (second order statistics) is computed over the whole wideband, RI does not change instantaneously. This means that if, for example, the network schedules the UE with a certain number of layers at time T1, it is unlikely that the number of layers changes significantly for the next scheduling  interval T2 = T1+ 1, 2.. n.

Figure 1 Snapshot of RI reported with 2 CSI-RS ports

Figure 2 Snapshot of RI reported with 4 CSI-RS ports

Figure 3 and 4 shows the PMI reported using conventional CSI-RS based CSI estimation for 2 and 4 CSI-RS ports with 2 and 4 receiving antennas respectively. Similar to RI, in this case also the PMI reported is almost constant over a period of time. Hence we can conclude that if the UE can estimate the SINR on the scheduled rank and scheduled PMI (via precoded DMRS), the SINR variation is not significant. There are many benefits of such a scheme such as reduced CSI reporting delay, reduced number of resources for PUCCH, reduced CSI-RS transmissions etc. We will explain the detailed procedure in the next subsections.

Figure 3 Snapshot of PMI reported with 2 CSI-RS ports

Figure 4 Snapshot of PMII reported with 4 CSI-RS ports

Observation 1: Since the RI and PMI do not change significantly over a period of time, the UE can estimate the channel and the SINR using precoded DMRS and can report the CSI along with HARQ-ACK of the current transmission.

## Details of DMRS based CSI computation

Figure 5 shows the message sequence chart for DMRS based CSI computation. Note that for our method to work, we need CSI-RS transmission albeit less frequently for estimating the complete CSI. During the muting period of CSI-RS, the UE can estimate the SINR using DMRS and feedback the CSI to the network.

Figure 5 Message sequence chart using DMRS based CSI estimation

Note that with DMRS based CSI estimation the UE can estimate CQI, LI. Assuming wideband PDSCH transmission, Table 1 shows the possible CSI reporting entries using DMRS based CSI estimation.

Table 1 Possible CSI entries using DMRS based CSI estimation

| PMI-FormatIndicator=widebandPMI and CQI-FormatIndicator=widebandCQI | PMI-FormatIndicator= subbandPMI or CQI-FormatIndicator=subbandCQI |  |  |
| --- | --- | --- | --- |
|  | CSI Part I | CSI Part II |  |
|  |  | wideband | Subband |
| - CRI | - CRI | - Wideband CQI for the second TB | - Subband differential CQI for the second TB of all even subbands |
| - Rank Indicator | - Rank Indicator | - PMI wideband (X1 and X2) | - PMI subband information fields of all even subbands |
| - Layer Indicator | - Layer Indicator | --------- | - Subband differential CQI for the second TB of all odd subbands |
| - PMI wideband (X1 and X2) | - Wideband CQI for the first TB | --------- | - PMI subband information fields of all odd subbands |
| - Wideband CQI | - Subband differential CQI for the first TB | --------- | --------- |

Where  indicates that the configuration is not possible with DMRS based CSI estimation.

## Simulation Results with DMRS based CSI Computation

To check the benefits of DMRS based CSI estimation, we performed link simulations with 4 CSI-RS ports and 4 receiving antennas.  Figure 6 shows the spectral efficiency as a function of SINR for two schemes namely CSI estimation using CSI-RS only and the other CSI estimation using DMRS+CSI-RS. Note that in this case, the spectral efficiency is computed as

Spectral efficiency = TBS*(1-BLER)/(T*BW)

Where, TBS is the transport block size in bits, BLER is the block error rate, T is the time duration of one subframe, and BW is the actual bandwidth. The link level simulation assumptions are shown in Table 2.

We consider 4 port MIMO configurations with single codeword with link adaptation. In our simulations, the feedback is assumed to have 5 slot delay and is assumed to be error free. Similarly, we assumed CSI-RS is transmitted for every 5 slots. Simulations are run for a UE with different SNRs and the wireless channel assumed is TDL-A channel. The velocity of the mobile is assumed to be 3 Kmph.

Table 2 Link Level simulation assumptions

| Parameter | Value |
| --- | --- |
| Carrier frequency for evaluation | 2GHz |
| Slot length | 1 msec |
| Channel model | TDL-A as in 38.901 |
| UE speed | 3 km/h |
| BS TX antenna configuration | 4 TX ports |
| UE RX antenna configuration | 4 RX ports |
| System bandwidth | 10 MHz, with 10 RBs are scheduled for transmission |
| Sub-carrier spacing | 15 kHz |
| Number of UEs | 1 UE |
| Channel estimation | Practical |
| Receiver type | MMSE |
| DMRS configuration | Type 1 with single symbol front loaded |
| CSI-RS configuration | CDM in frequency |
| CQI/MCS | Link adaptation with QPSK, 16 QAM and 64 QAM |
| CSI Reporting period | 5msec |
| CSI-RS periodicity | 5 msec |
| FEC | LDPC code with 25 decoding iterations |
| Precoding codebook | NR Release 15 |

It can be observed from Figure 6 that DMRS+CSI-RS based CSI estimation outperforms at all SINRs. This is because the UE is feeding back the accurate CSI at no additional overhead.

Figure 6 Spectral efficiency comparison between DMRS based CSI estimation and conventional CSI estimation

Note that in our simulations for Figure 6 we used 5 msec reporting period for both CSI-RS transmission and CSI reporting. However with this DMRS based technique, we can relax that constraint and the gains with higher reporting periods are significant.  Figures 7 and 8 shows the performance benefits with the proposed schemes when CSI-RS is transmitted every 10 msec and 20 msec, respectively.  It can be observed that the gains are more considerable for larger CSI transmission and reporting. We thus propose that RAN1 should study DMRS based CSI computation for fast CSI computation and reporting.

Figure 7 Spectral efficiency comparison between DMRS based CSI estimation and conventional CSI estimation with transmission and reporting period is set to 10 slots

Figure 8 Spectral efficiency comparison between DMRS based CSI estimation and conventional CSI estimation with transmission and reporting period is set to 20 slots

Proposal 1:  RAN1 should study enhanced CSI estimation techniques using DMRS for fast CSI estimation and reporting as with very low BLER targets the accuracy of CSI is very low

# Enhanced HARQ Reporting with DMRS based CSI estimation

## Main Principle

In Release 15, HARQ is conveyed using PUCCH format 0-4. Formats 0 and 1 can convey information up to 2 bits. However with DMRS based CSI estimation, we can envision that format 0 and 1 can support more than 2 bits. As an example, consider PUCCH format 0. Since only ZC sequences with different phase shifts (m_cs) are used to differentiate between HARQ possibilities, we propose that RAN1 should study enhanced HARQ mechanisms such that different sequences can indicate HARQ+CSI information.  For example let’s say the HARQ is combined with differential CQI (say WB) and let’s define the offset level as

Offset level (s) = CQI index computed using CSI-RS –CQI index computed using DM-RS

Then we can use the following table as an example to report both HARQ-ACK and differential CQI.

Note that with the proposed scheme the signalling overhead for reporting the CQI is zero, i.e. we are reusing the NR Release 15 framework to report HARQ. In the next section, we analyse the probability of misdetection due to the multiplexing of HARQ-ACK+CSI.

Table 3: Proposed Mapping of HARQ-ACK and CQI to offset level with DMRS based CSI estimation

| Sequence | HARQ-ACK | Offset level |
| --- | --- | --- |
| Sequence1 (m_cs= 0) | ACK | 0 |
| Sequence2 (m_cs= 2 ) | ACK | 1 |
| Sequence3(m_cs=4) | ACK | ≥ 2 |
| Sequence4(m_cs =1) | ACK | ≤-1 |
| Sequence5(mcs= 5) | NAK | 0 |
| Sequence6(m_cs =7) | NAK | 1 |
| Sequence7(m_cs =9) | NAK | ≥ 2 |
| Sequence8(m_cs=8) | NAK | ≤-1 |

## Simulation Results

To check the performance impact of using more sequences for sending CSI (say differential CQI), we did link simulations with PUCCH format 0.  As a performance metric we used probability of misdetection of HARQ-ACK i.e. the first entry in Table 3. At the receiver we used non coherent maximum likelihood detection.  The link simulation assumptions are shown in Table 4. It can be observed that with 4 and 8 sequences the impact of probability of misdetection is very minimal.

Figure 9  Probability of HARQ-ACK misdetection with 16 Receiver antennas

Table 4 Link simulations for HARQ-ACK performance

| Assumptions | Value |
| --- | --- |
| Carrier frequency | 2 GHz |
| System Bandwidth | 10 MHz |
| Slot length | 1 ms |
| Subcarrier spacing | Single numerology case: 15KHz |
| FFT size | 1024 for 15KHz subcarrier spacing |
| Channel model | TDL-A, 3 Kmph |
| Antenna config. | 1T NR, where N is 4,16 |
| Channel estimation | MMSE |
| PUCCH Format | 0 |

Figure 10 shows the probability of misdetection with 4 receiver antennas. In this case too, we observed the performance impact is very minimal if we increase the number of sequences to 4 or 8.

Figure 10 Probability of HARQ-ACK misdetection with 4 Receiver antennas

Based on above observations, we recommend RAN1 to study techniques to multiplex HARQ-ACK with differential CQI for DMRS based CSI estimation.

Proposal 2:  RAN1 should study mechanism to multiplex HARQ-ACK with CSI for DMRS based CSI estimation for PUCCH formats 0 and 1

# Conclusions

In this contribution we described our views on Physical layer enhancements for NR URLLC.

Based on our observations, we have the following proposal:

Proposal 1:  RAN1 should study enhanced CSI estimation techniques using DMRS for fast CSI estimation and reporting as with very low BLER targets the accuracy of CSI is very low

Proposal 2:  RAN1 should study mechanism to multiplex HARQ-ACK with CSI for DMRS based CSI estimation for PUCCH formats 0 and 1