3GPP TSG RAN WG1 Meeting #96		                                                      R1-1901997

Athens, Greece, 25th February – 1st March, 2019

Source:	CATT

Title:	Discussion On Uu-based sidelink resource allocation/configuration in NR V2X

Agenda Item:	7.2.4.3

Document for:	Discussion and Decision

Introduction

In RAN1#95 meeting [1], the following agreements have been achieved for Uu-based sidelink resource allocation/configuration in NR V2X:

Agreements: The following NR sidelink resource allocation techniques by NR Uu for mode-1 are supported: - Dynamic resource allocation - Configured grant. - FFS whether type-1 and/or type-2 Agreements: - In continuing evaluating NR Uu scheduling of LTE sidelink mode-3, consider at least: - What will be required on the UE side to support such feature - DCI design (e.g., whether DCI 5A can be reused) - Deployment scenarios where it is beneficial

In RAN1 AH#1901 meeting [2], the following agreements have been achieved:

Agreements: - When NR Uu schedules NR SL mode 1, both type 1 and type 2 configured grants are supported for NR SL Agreements: - LTE Uu to schedule NR sidelink mode 1 is supported: - The support is done based on type 1 configured grant with configuration restricted to time/frequency resources & periodicity, with the condition that no additional function/procedure is to be introduced for LTE Uu - Both DCI based scheduling and type 2 configured grant scheduling are not supported for scheduling NR sidelink mode 1 - Send an LS to RAN2 – Phillipe (HW), R1-1901410, which is approved with the final LS in R1-1901445

This contribution discusses the potential enhancements of LTE & NR Uu to control NR sidelink and NR Uu to control LTE sidelink from the cellular network.

Discussion

NR Uu to control NR sidelink

Support of high data rate services

Another issue of mode-1 resource allocation is to support the high data rate e.g. 25Mbps. Packets are transmitted every 20ms, and the average packet is 62500byte. While in 3GPP V2X Phase 2, the maximum transmission capability can be up to 9186 byte within a TTI, for a SL-C-TX Category 5 UE working in 30 MHz band combination and adopting 64 QAM. If supporting the packet size of 62500 byte within a slot, the much wider bandwidth should be supported, which is around 100MHz. While in FR 1, it can hardly be achieved. Therefore, the carrier aggregation and slot aggregation should be studied in NR V2X to allocate more resource in frequency and time domains for a large size packet. For example, resources of multiple continuous slots can be allocated in multiple carriers by SPS scheduling.

Proposal 1: For resource allocation mode-1, the scheduling solution for carrier aggregation and slot aggregation should be further studied, in order to support high data rate services.

Support of sidelink resource allocation on shared carrier

When the frequency resources are shared by Uu-based and PC5-based sidelink transmission, TDD mode can be used to mitigate the transmission collision. For in-coverage scenario, UE would only need to decode the DL common channels that include SSBs, CORESETS (PDCCH) and group-common PDCCH. Other resources used by UE would be scheduled by gNB through DCI or by higher layer through semi-static allocation. Therefore, the V2X UEs should use any resources (e.g. UL, DL, X or any symbol of them) allocated by the gNB without concerning if it is DL/UL/GP in the slot format configuration in the shared carrier. UEs should not even consider any potential interference management scheme to other cells (e.g. power control), since UEs would be controlled by the network when any sidelink resource is allocated either through mode-1 or mode-2 resource allocation.

Observation 1: For TDD and in-coverage scenario, V2X UEs should use any resources scheduled by gNB or semi-statically allocated by higher layer without concerning in the shared carrier.

Proposal 2: At least, UL symbols can be used for sidelink transmission on shared carrier.

NR Uu to control LTE sidelink

Network control

Resource pool of LTE sidelink under the NR network control is more flexible than that being pre-configured because the NR network can configure UE with the resource collectively to adapt to the usage of the resource pools. The NR network also can adjust the resource of the LTE mode-3/4 resource pools if needed. If LTE sidelink is not controlled by LTE Network, as the UE is out of the LTE network coverage but in the coverage of NR Network, LTE sidelink resource controlled by NR Network seems to be helpful for improving the flexibility of resource pool configuration.

Observation 3: LTE sidelink resource controlled by NR Network may improve the flexibility of resource pool configuration.

LTE sidelink and NR sidelink may be deployed with non-co-channel deployment.  Several issues, such as the in-device interference and half duplex impact for adjacent channel transmission, should be taken into consideration for the NR and LTE V2X resource allocation. One solution of in-device interference mitigation is that LTE sidelink and NR sidelink are transmitted in TDM. However, the resource utilization is significantly degraded by the TDMed resource pool partition among LTE and NR V2X resources for PC5 communication.

If both LTE sidelink and NR sidelink are controlled by NR network, NR network can coordinate the resource allocation and transmission of LTE sidelink and NR sidelink to be TDMed. Moreover, the LTE mode-3 resource allocation for LTE sidelink transmission and NR mode-1 resource allocation for NR sidelink transmission can be further coordinated by NR network based on traffic types and QoS requirements, such as latency and data rate. If the NR network could collectively allocate the resources for LTE sidelink and NR sidelink transmission based on the inter-band or intra-band deployment scenario, the resource utilization would be more efficient and the in-device cross-interference could be minimized.

Observation 2: If the NR network could collectively allocate the resources for LTE sidelink and NR sidelink transmission based on the inter-band or intra-band deployment scenario, the resource utilization would be more efficient and the in-device cross-interference could be minimized.

Proposal 3: NR network controlling the resource allocation for the LTE sidelink transmission should be supported to improve the resource utilization.

DCI design

When NR Uu controlling LTE sidelink mode-3 is supported, DCI format designing should be considered (e.g. LTE DCI 5B). The new DCI format for NR, such as DCI 3_0, can include the same information that of LTE DCI 5A to manage LTE sidelink mode-3. Besides, NR Uu managing NR sidelink mode-1 also needs to define a DCI format (DCI 0_2). When UEs do blind decoding of DCI, different DCI formats for LTE sidelink mode-3 and NR sidelink mode-1 may increase the blind decoding complexity since DCI format for NR sidelink mode-1 will be quite different from that of LTE sidelink mode-3. To solve this issue, DCI 3_0 can be padded until the size to match with NR DCI format 0_2.

Proposal 4: A new DCI format for NR (e.g. DCI 3_0) can be designed with the same contents as that of LTE DCI 5A for NR Uu managing LTE sidelink mode-3.

Proposal 5: To solve blind decoding issue, the new DCI format (e.g. DCI 3_0) can be padded until the size is the same as the DCI format that used by NR Uu to manage NR sidelink mode-1.

Coordination between eNB and gNB on schedule signaling

When NR UE is in coverage of LTE network and NR network, the coordination between LTE network and NR network for the resource allocation is needed avoiding the resource collision and performing interference mitigation. The backhaul signaling of Xn+ interactive interface can be further studied according to specific message for the coordination of resource allocation. If NR network coordinate with LTE network, NR network can inform the resources allocated to the LTE network, and the LTE network indicate the LTE mode-3 resources to the UE by DCI format 5A. Even the NR V2X UE is out-of-coverage of gNB but in-coverage of eNB, gNB can configure/schedule resources for NR sidelink transmission. Not only can this coordination extend the coverage of gNB, but also provide flexibility of network to control NR sidelink transmission. However, additional latency and signaling overhead are introduced when the signaling is transferred between eNB and gNB.

Proposal 6: The coordination between the LTE network and the NR network is essential in avoiding resource collision and mitigating the interference. The signaling IE on Xn+ should be studied by RAN3

Coexistence of LTE Uu, NR Uu and NR sidelink

When NR UE is in coverage of LTE network and NR network the aspects of the system design of NR network controlling the resource allocation for the LTE sidelink transmission are as follows,.

- Issue 1: For a dual mode V2X UE is in the coverage of both LTE and NR network, the sidelink transmission is controlled individually by the LTE network and the NR network respectively. The collective resource allocation by one network would improve the efficient of resource utilization for both LTE and NR V2X sidelink transmission.   Which network would be a better candidate for the collective resource allocation?

- Issue 2: When the resource allocation for the LTE and NR sidelink transmissions for a dual module V2X UE are controlled by the NR network, how is the controlled signaling design to enable the dual-mode UE differentiate the control signaling designated for LTE sidelink or NR sidelink transmission?

For issue 1, the resource allocation and the control of LTE V2X sidelink transmission in the LTE-NR V2X dual connectivity will be by the access network, which is the primary cell group (PCG) in LTE-NR dual connectivity.  The access network can be pre-configured by the operator or can be selected by UE according to a pre-configured rule. If the advanced V2X services are expected for the dual-mode UE, UE may choose the NR network as the access network for the coordination of the resource allocation and the control of the NR and LTE sidelink dual connectivity.

Proposal 7: For a dual mode V2X UE in the coverage of the LTE and the NR network, the selection of the primary cell group (PCG) for the LTE-NR V2X dual connectivity shall be pre-configured by the operator.

For issue 2, when the NR network allocated both LTE and NR V2X resources, the DL control signaling needs to include the field of carrier index to indicate the resource designated for LTE or NR sidelink transmission. Cross carrier indication field (CIF) in CA could be reused as the carrier index of the LTE V2X resource allocation, and carrier indication can be used for the differentiation of resource allocation for LTE or NR sidelink transmission. Other methods such as RNTI scrambling or additional 1 bit indication are also can be considered.

Proposal 8: Carrier indication, RNTI or additional indicator could be used for the differentiation of resource allocation for LTE or NR sidelink transmission.

Conclusion

In this contribution, the LTE & NR Uu to control NR sidelink and NR Uu to control LTE sidelink transmission is discussed. We have the following observations and proposals:

Proposal 1: For resource allocation mode-1, the scheduling solution for carrier aggregation and slot aggregation should be further studied, in order to support high data rate services.

Observation 1: For TDD and in-coverage scenario, V2X UEs should use any resources scheduled by gNB or semi-statically allocated by higher layer without concerning in the shared carrier.

Proposal 2: At least, UL symbols can be used for sidelink transmission on shared carrier.

Observation 2: If the NR network could collectively allocate the resources for LTE sidelink and NR sidelink transmission based on the inter-band or intra-band deployment scenario, the resource utilization would be more efficient and the in-device cross-interference could be minimized.

Proposal 3: NR network controlling the resource allocation for the LTE sidelink transmission should be supported to improve the resource utilization.

Proposal 4: A new DCI format for NR (e.g. DCI 3_0) can be designed with the same contents as that of LTE DCI 5A for NR Uu managing LTE sidelink mode-3.

Proposal 5: To solve blind decoding issue, the new DCI format (e.g. DCI 3_0) can be padded until the size is the same as the DCI format that used by NR Uu to manage NR sidelink mode-1.

Proposal 6: The coordination between the LTE network and the NR network is essential in avoiding resource collision and mitigating the interference. The signaling IE on Xn+ should be studied by RAN3

Proposal 7: For a dual mode V2X UE in the coverage of the LTE and the NR network, the selection of the primary cell group (PCG) for the LTE-NR V2X dual connectivity shall be pre-configured by the operator.

Proposal 8: Carrier indication, RNTI or additional indicator could be used for the differentiation of resource allocation for LTE or NR sidelink transmission.

References

  - 3GPP TSG RAN WG1 Meeting RAN1 #95 chairman notes, November 2018.

  - 3GPP TSG RAN WG1 Meeting RAN1 AH#1901 chairman notes, January 2019.