3GPP TSG RAN WG1 #96							R1-1902065

Athens, Greece, 25th February – 1st March, 2019

Agenda Item:	6.2.2.3

Source: 	LG Electronics

Title: 	Discussion on multiple transport blocks scheduling in NB-IoT

Document for:	Discussion and decision

- Introduction

In the RAN#90 meeting, new Rel-15 work item of NB-IoT enhancement was agreed [1]. Also, several agreements on scheduling of multiple DL/UL transport blocks were made in the last RAN1 meeting as follows:

Agreements For unicast, scheduling multiple DL/UL transport blocks with single DCI is supported. Agreements For Unicast, the possibility of scheduling multiple DL/UL transport blocks is configured via RRC. Details TBD Agreements For unicast, the number of TBs scheduled should be dynamically indicated in the DCI, the maximum number of TBs is FFS Agreements For UE supporting multiple TBs, the soft buffer size stays the same as that of the legacy UE Agreements Individual feedback for each HARQ process is supported. FFS if HARQ bundling/multiplexing can be optionally supported. Agreements The UE should only monitor one DCI size in the UE specific search space. Agreements One DCI to schedule multiple TBs for SC-MCCH is not supported Agreements Using one DCI to schedule multiple TBs for SC-MTCH is supported, and it is configured and enabled per SC-MTCH via SC-PTM configuration message in SC-MCCH. Agreements Maximum UL/DL HARQ process supported is 2 Agreements For unicast, when multiple DL/UL transport blocks are assigned by a single DCI, the relationship(s) between HARQ process and TB is/are selected from the following two candidates(multiple choices are allowed) • Relationship 1: 1 HARQ process corresponds to 1 TB • Relationship 2: 1 HARQ process corresponds up to 2 TBs Agreements For UL/DL unicast, at least consecutive resource allocation in time is supported when multiple TBs are scheduled by one single DCI. • ‘consecutive resource allocation in time’ means no new scheduling gap between the end of previous TB and the start of the next TB FFS: Whether scheduling gaps is also supported FFS: How to schedule repetitions within the consecutive resource allocation

In this contribution, we discuss and provide our view on scheduling of multiple DL/UL transport blocks for NB-IoT.

- Discussions

  - Unicast

Relationship between HARQ process and TB

In unicast, HARQ process ID can be used to handle transmission / retransmission of multiple TBs. In NB-IoT maximum 2-HARQ process can be configured if a UE has a 2-HARQ capability. According to the agreement in the last meeting, multi-TB scheduling can support maximum 2 HARQ processes in NB-IoT. Thus it seems natural supporting multi-TB scheduling only for 2-HARQ capable UE. If so, likewise the most of other LTE systems, 1-HARQ process corresponds to 1 TB can be supported.

On the contrary, there were some proposals for supporting multi-TB scheduling for 1-HARQ process capable UEs. For example splitting TBS to multiple TBs can be used to meet the soft buffer restriction of 1-HARQ process capability while supporting multi-TB scheduling using single DCI. It might be useful if 2 TBs can be handled separately. However, current HARQ process mechanism is not suitable to support TBS splitting which may cause significant specification impact in other working groups. Also, it should be noted that major purpose of the multi-TB scheduling is to reduce the network resource overhead especially in control channel.

In another way, supporting 2 TBs without TBS limitation can be considered if HARQ-ACK can be transmitted during a gap between 2 TBs. In this method, the first scheduled TB can be retransmitted without DL grant. Instead, UE expect new data transmission at the next TB when it reports ACK during a gap between TBs while it expect retransmission when NACK is reported. However, handling retransmission without DL grant may cause significant problem when eNB misunderstand HARQ-ACK report.

In this point of views, supporting multi-TB scheduling for 2-HARQ process only is preferred and each TB should correspond to a unique HARQ process.

Proposal 1: For unicast, when multiple DL/UL transport blocks are assigned by a single DCI, each transport block corresponds to a unique HARQ process.

DCI design

For unicast when multi-TBs are scheduled, whether to schedule initial and retransmission TBs within one DCI should be decided. In some cases initial and retransmission TBs might require different scheduling information such as TBS, MCS, and/or RV. In scheduling flexibility point of view, using separate DCI might be beneficial. However, it would not beneficial in network resource overhead point of view, because DCI should be transmitted multiple times if eNB want to schedule initial and retransmission at once. Also, if separate DCI is transmitted for initial and retransmission, there is no big difference from the current 2-HARQ process mechanism which is introduced in Rel-14. Thus scheduling initial and retransmission TBs with a single DCI should be supported.

Proposal 2: Support scheduling of initial and retransmission TBs within one DCI.

To support multi-TB scheduling using single DCI, some additional information should be conveyed compare to the legacy DCI format. In scheduling flexibility point of view, supporting scheduling parameters with separate DCI field for each TBs might be beneficial. However, conveying scheduling information separately may increase required number of DCI bits. To guarantee target MCL while preserving number of repetition for NPDCCH transmission in multi-TB scheduling DCI design, number of DCI bits should also be maintained as much as possible. In this point of view, joint encoding could be considered for some information.

For multi-TB scheduling, at least number of HARQ process, HARQ process ID, and NDI of each scheduled HARQ process should be guaranteed. These information can be represented by 3 bits joint encoding. Among 3 bits, one bit is used as a flag to represent number of scheduled TBs by the DCI. If one bit flag indicates 1 TB scheduling, another one bit represent HARQ process ID while the other bit is used for NDI information. If one bit flag indicates 2 TB scheduling, other 2 bits represent NDI information of two HARQ process IDs each. Note that legacy DCI format for 2-HARQ process uses 2 bits to represent HARQ process ID and NDI. Compare to the legacy DCI format only additional one bit is required for scheduling multi-TB using single DCI.

Proposal 3: HARQ process ID, the number of scheduled HARQ processes, and NDI are signaled by 3 bit-long joint encoding scheme as follows:

- 1 bit is used as a flag indicating the number of scheduled TBs between 1 and 2

- when the flag indicates 1 TB scheduling, another bit signals a scheduled HARQ process ID and the other bit conveys NDI information

- when the flag indicates 2 TBs scheduling, the other 2 bits convey NDI information of each HARQ process ID

DL/UL gap between repetitions

To improve the resource efficiency and power consumption reduction of the UE, using gap between repetitions can be considered. It can be expected that configuring gap in the middle of NPDSCH/NPUSCH transmission can increase the time diversity. According to the evaluation results in [2], SNR gain can be achieved when gap is configured between repetitions. It is obvious that larger gap can serve higher time diversity, while it may cause lower data transmission speed. Thus suitable size of gap should be discussed carefully.

In another point of view, configuring gap on a UE-specific search space can be considered. If multiple TBs are scheduled by a single DCI, TBs transmission would require much longer time than single TB scheduling. If UE miss a DCI, it cannot be scheduled until the end of the transmissions of TBs which are scheduled by a previous DCI. If a gap is configured on a UE-specific search space where UE monitors, UE which miss the DCI can have a chance to be scheduled for the remaining TB transmissions. Note that this gap can be used for other purpose also; eNB can use the gap to transmit any signal/channel for other UEs.

As we discussed above, configuring gap within a multiple TBs transmission can be beneficial because it helps increasing the time diversity gain and/or preventing DCI missing case. However, it should be also noted that transmission latency would be increased if the gap is configured larger or frequently. Details of gap configuration and usage of the gap should be discussed further.

Proposal 4: In case of multi-TB scheduling via single DCI, gap can be configured for following purposes: .

- Utilizing DL/UL gap(s) for the time diversity gain.

- Configuring DL/UL gap(s) on UE-specific search space(s) to reduce the impact on missing of multi-TB scheduling DCI.

HARQ-ACK feedback

One of the major differences from the legacy HARQ process could be a method of HARQ-ACK reporting. According to the agreement in a previous meeting, individual HARQ-ACK feedback for each HARQ process is supported for multi-TB scheduling. In a simplest way, reusing legacy HARQ-ACK feedback channel design with some modification on scheduling can be considered. However, it should be noted that power consumption efficiency and resource overhead problem of multiple HARQ process are not considered in legacy HARQ-ACK feedback channel design. Thus, enhanced HARQ-ACK feedback design should be discussed further.

Unlike legacy HARQ process with individual DCI, UE could notice exact number of HARQ process at once using single DCI. So, transmitting multiple HARQ-ACK feedback using single HARQ-ACK channel can be work in this method. HARQ-ACK bundling or HARQ-ACK multiplexing could be considered. UE in deep coverage may require HARQ-ACK bundling, which represents one-bit bundled HARQ-ACK information, to reduce the latency due to the HARQ-ACK reporting. On the other hand, HARQ-ACK multiplexing, which represents multi-bit information within a single physical channel, would be more beneficial in case of UE in normal coverage level. It seems reasonable to support at least one of these HARQ-ACK methods to enhance the efficiency of multiple HARQ process with single DCI.

Proposal 5: Efficient HARQ-ACK feedback mechanisms (e.g. HARQ-ACK bundling and/or multiplexing) corresponding to multiple transport blocks scheduled via single DCI needs to be introduced for unicast channels .

If individual HARQ-ACK is supported, HARQ-ACK resources are scheduled for the number of scheduled HARQ processes. In this case, it would be beneficial to transmit HARQ-ACK feedback for ACK reporting only, while DTX is used for NACK representation. In case of DTX, UE can save the power for a HARQ-ACK transmission. However, if DTX is used for representing NACK for individual HARQ processes, eNB may not distinguish all NACK case from DCI missing case. To solve the problem, representing NACK on a scheduled HARQ-ACK resource only for all NACK case can be considered.

Proposal 6: If individual HARQ-ACK is used in multi-TB scheduling, UE transmits HARQ-ACK for ACK reporting while DTX is used for NACK representation.

- Explicit NACK transmission can be considered to represent NACK for all scheduled HARQ processes

  - SC-PTM

In NB-IoT, SC-PTM can be used for multicast. It can be used for various types of service. In some SC-PTM applications, data traffic to be transmitted by multicast channel may exceed the maximum TBS which can be used for SC-MTCH transmission (e.g. firmware update). Also, SC-PTM can be used for periodic data which may have stable traffic pattern (e.g. billboard). In these cases, eNB can expect the required traffic pattern to be transmitted. Thus, instead of transmitting DCI for every SC-MTCH transmission, configure pre-defined transmission pattern for multicast purpose would be beneficial in some scenario.

To indicating scheduling information of multiple TBs for SC-MTCH two candidate approaches could be considered. First, indicating scheduling information using DCI could be considered. For example, number of TBs and their resource allocation can be conveyed by DCI. In this approach, it seems necessary to introduce new DCI design. Thus, search space configuration and G-RNTI value should be configured separately only for multi-TB capable Rel-16 UEs. New DCI based dynamic multiple TBs scheduling can have benefits in scheduling flexibility point of view. In this case, design principle of multi-TB scheduling DCI for unicast can be shared to reduce the effort of standardization. However, legacy multi-TB non-capable UEs that requires same SC-PTM service with multi-TB capable UEs cannot recognize the new DCI. Thus eNB may need to transmit same SC-MTCH multiple times, resulting in increased the DL resource overhead. It should be noted that one of the main purpose of introducing the multi-TB transmission is to reduce the network overhead.

Observation 1: It is beneficial to the network in terms of scheduling flexibility but the network overhead would be increased if new DCI with separate G-RNTI which can dynamically schedule multiple NPDSCHs for SC-MTCH is introduced.

Alternatively, using same DCI and G-RNTI with legacy UE for multi-TB scheduling in SC-MTCH can be considered. If the same DCI and G-RNTI is used for Rel-16 multi-TB capable UEs, SC-MTCH transmission for legacy SC-MTCH would be easily shared with Rel-16 multiple TBs capable UEs by configuring same G-RNTI. Thus, in resource efficiency point of view, reusing same DCI and G-RNTI may be more beneficial than new DCI design approach. Note that there is no remaining bit or state in legacy DCI format for SC-MTCH, so dynamic scheduling of multiple SC-MTCH cannot be performed. Instead, pre-defined DCI skipping rule in UE perspective can be introduced that can be configured by SC-PTM configuration message in SC-MCCH. For example, if Rel-16 multi-TB capable UE obtain grant for SC-MTCH, it can skip following search spaces but can expect multiple SC-MTCH transmission. Number of SC-MTCH transmission that can be expected by UE with a single DCI can be configured by SC-MCCH. In this approach, SC-MTCH transmission in eNB perspective is same as for legacy UEs. In UE perspective, power consumption efficiency could be increased by reducing blind decoding attempts. Figure 1 shows an example of Rel-16 UE behavior with DCI skipping method.

Observation 2: It is beneficial to both UE and network in terms of power and downlink resource efficiency if UE is allowed to periodically skip monitoring NPDCCHs scrambled with G-RNTI in the Type-2A common search space and directly read NPDSCHs for SC-MTCH based on the scheduling information obtained by a DCI which schedules NPDSCH for SC-MTCH in the preceding Type-2A common search space.

Proposal 7: For multiple SC-MTCH transmission, introduce DCI skipping mechanism which allows UE to periodically skip monitoring NPDCCHs scrambled with G-RNTI in the Type-2A common search space and directly read NPDSCHs for SC-MTCH based on the scheduling information obtained by a DCI which schedules NPDSCH for SC-MTCH in the preceding Type-2A common search space.

Figure 1 Example of skipping DCI method for SC-MTCH

  - Interleaved transmission

In NB-IoT, subframe level repetition can be scheduled to extend coverage level. If the repetition level is long enough, time diversity gain can be achieved. To enhance the time diversity gain in multiple transport block transmission, interleaved transmission of transport blocks could be considered. Also, this kind of interleaved pattern would be more suitable for early termination process. For example, if UE could succeed decoding of whole HARQ-process, early HARQ-ACK feedback could be used to reduce the latency for the next DCI grant. Note that interleaved transmission pattern can be considered not only for shared channels(e.g. NPDSCH and NPUSCH format 1) but also for control channel(NPUSCH format 2).

It is clear that UE power consumption efficiency can be improved if NPDSCH/NPUSCH decoding can be finished as soon as possible. In interleaved TB pattern point of view, improving decoding performance of each sub-block (i.e. interleaved TB) would be beneficial to finish the decoding with small repetition number. To enhance the decoding performance within a sub-block, at least one repetition of NPDSCH/NPUSCH should be contained in each sub-block. Also, cyclic repetition structure for symbol level combining, which is already used for legacy NPDSCH/NPUSCH repetition, should be considered in interleaved TBs structure as well. Figure 2 shows an example of multiple transport block transmission patterns.

Proposal 8: Interleaved transmission of multiple transport blocks scheduled via single DCI should be introduced.

- Each interleaved transport blocks should contain at least one repetition of NPDSCH/NPUSCH.

- Cyclic repetition pattern should be considered in designing interleaving pattern

Figure 2  Example of interlaced transmission pattern

- Conclusion

In this contribution, we discuss and provide our view on the multiple transport blocks scheduling for NB-IoT. The proposals of this contribution are summarized as follows.

Observation 1: It is beneficial to the network in terms of scheduling flexibility but the network overhead would be increased if new DCI with separate G-RNTI which can dynamically schedule multiple NPDSCHs for SC-MTCH is introduced.

Observation 2: It is beneficial to both UE and network in terms of power and downlink resource efficiency if UE is allowed to periodically skip monitoring NPDCCHs scrambled with G-RNTI in the Type-2A common search space and directly read NPDSCHs for SC-MTCH based on the scheduling information obtained by a DCI which schedules NPDSCH for SC-MTCH in the preceding Type-2A common search space.

Proposal 1: For unicast, when multiple DL/UL transport blocks are assigned by a single DCI, each transport block corresponds to a unique HARQ process.

Proposal 2: Support scheduling of initial and retransmission TBs within one DCI.

Proposal 3: HARQ process ID, the number of scheduled HARQ processes, and NDI are signaled by 3 bit-long joint encoding scheme as follows:

- 1 bit is used as a flag indicating the number of scheduled TBs between 1 and 2

- when the flag indicates 1 TB scheduling, another bit signals a scheduled HARQ process ID and the other bit conveys NDI information

- when the flag indicates 2 TBs scheduling, the other 2 bits convey NDI information of each HARQ process ID

Proposal 4: In case of multi-TB scheduling via single DCI, gap can be configured for following purposes: .

- Utilizing DL/UL gap(s) for the time diversity gain.

- Configuring DL/UL gap(s) on UE-specific search space(s) to reduce the impact on missing of multi-TB scheduling DCI.

Proposal 5: Efficient HARQ-ACK feedback mechanisms (e.g. HARQ-ACK bundling and/or multiplexing) corresponding to multiple transport blocks scheduled via single DCI needs to be introduced for unicast channels .

Proposal 6: If individual HARQ-ACK is used in multi-TB scheduling, UE transmits HARQ-ACK for ACK reporting while DTX is used for NACK representation.

- Explicit NACK transmission can be considered to represent NACK for all scheduled HARQ processes

Proposal 7: For multiple SC-MTCH transmission, introduce DCI skipping mechanism which allows UE to periodically skip monitoring NPDCCHs scrambled with G-RNTI in the Type-2A common search space and directly read NPDSCHs for SC-MTCH based on the scheduling information obtained by a DCI which schedules NPDSCH for SC-MTCH in the preceding Type-2A common search space.

Proposal 8: Interleaved transmission of multiple transport blocks scheduled via single DCI should be introduced.

- Each interleaved transport blocks should contain at least one repetition of NPDSCH/NPUSCH.

- Cyclic repetition pattern should be considered in designing interleaving pattern

- Reference

- RP-181451, New WID on Rel-16 enhancements for NB-IoT, Ericsson, Huawei

- R1-1808356, “Multiple Transport Block Grant for Unicast,” RAN1#94, Sierra Wireless