3GPP TSG RAN WG1 Meeting #96	R1-1902196

Athens, Greece, 25th February – 1st March, 2019

Agenda Item:	6.2.2.1

Source: 	Sony

Title:	UE-group wake-up signal for NB-IoT

Document for:	Discussion and Decision

# Introduction

In RAN1 meeting #94bis [1], the following related agreements on UE-group wake up signal for NB-IoT were made:

- Multiplexing of Rel-16 UE groups is down-selected among

- Single sequence CDM is supported

- Single sequence CDM is supported and TDM can be additionally applied

  - FFS: Configurability of CDM and/or TDM

- A common WUS is used to wake up all group WUS UEs monitoring the same WUS (time-frequency) resource if more than one UE group occupies the WUS resource.

- FFS: Whether the above is also applicable for Rel-15 WUS UEs

- FFS: Whether to support waking up a subset of all WUS UE groups

- For multiplexing between Rel-16 UE-group WUS and Rel-15 legacy WUS within the same carrier, further evaluate and down select among the following options

- TDM

- single-seq. CDM

- single-seq. CDM+TDM

FFS whether legacy WUS is the common WUS for all new UEs or only a part of new UEs.

In this contribution, we discuss some aspects on the operation of UE-group wake-up signal for NB-IoT (NWUS).

# Discussion

Subgrouping of UEs within a NWUS allows only a subgroup of UEs to be woken up by the NWUS for a paging occasion (PO). The UE subgrouping operation will reduce the number of UEs having to wake up unnecessarily to detect NPDCCH and decode NPDSCH. Thus, it reduces UE power consumption for such UEs.

# Multiplexing of NWUS

UE-group NWUS in Rel-16 should consider coexistence with NWUS operation introduced in Rel-15 (legacy). One of the easiest solutions is to allocate different WUS physical resources for UE-group NWUS and legacy NWUS. NWUS occupies 1 PRB and that means the FDM approach can only be supported if the other(s) NWUS is allocated on another carrier.

CDM and TDM have also been considered for the WUS multiplexing scheme. In our understanding CDM represented with multiple sequences entails many issues (UE complexity, largest delay, performance issues). The main issue with TDM is a potentially larger delay. The main issue with FDM is the NWUS location can be in another NB-IoT carrier for the subsequent paging occasion (PO).

Proposal 1: On multiplexing between legacy Rel-15 NWUS and Rel-16 NWUS: support FDM (in different NB-IoT carrier) and TDM (in case of the same NB-IoT carrier).

Proposal 2: On multiplexing of Rel-16 UE groups NWUS: Support Single sequence CDM is supported. TDM can be additionally applied is FFS (i.e. depending on the number of supported groups).

There was a discussion during the online session in RAN1#94 on the introduction of a super group NWUS or common NWUS. In such case, basically there is a dedicated NWUS to wake up all UEs assigned to the corresponding paging occasion. Super-group is in addition to the individual UE group NWUS. The benefit of super-group NWUS is that the eNB only needs to transmit the super-group NWUS in case the eNB needs to wake up all UEs. However, from the UE point of view, this could increase UE power consumption and complexity as the UE needs to detect both the super-group NWUS and group specific NWUS. This will make NWUS Rel-16 worse than NWUS Rel-15 in terms of UE power consumption and complexity. In case the eNB wants to wake up all groups, the eNB must transmit all UE-group NWUS.

Proposal 3: NB-IoT Rel-16 does not support common NWUS operation which requires the UE to receive and detect NWUS in multiple NWUS resources.

Introducing many UE-groups can be beneficial in a dense network. In a legacy LTE system, the probability of UEs having the same paging frame as a function of the number of paging frames is given in Table 1. While a reduction of the occasions of many UEs being paged at the same time can be achieved by allocating many PFs, this solution might not be the optimal one as more resources are being used to carry paging information.

Table 1: Number of PF versus Probability UEs with same PF

| N | UE_ID mod N | % UEs have same paging frame |
| --- | --- | --- |
| 1 | 0 | 100% |
| 2 | 0,1 | 50% |
| 4 | 0,1,2,3 | 25% |
| 8 | 0,1,2,…,7 | 12.5% |
| 16 | 0,1,2,…,15 | 6.75% |

In scenarios where the number of paging frames is limited or can be configured to be a small number, introducing many UE-groups would be clearly beneficial. The advantage is that the network can have more flexibility and the number of UEs being paged unnecessarily can be further reduced. In order to maximize the benefit of the UE-group mechanism and limit the probability of overhearing to below 15% we propose the maximum number of UE groups supported is at least 6.

Proposal 4: The introduction of UE-group should be designed to maximize the benefit of UE-group mechanism. We propose the maximum number of UE groups supported is at least 6.

The usage of UE-group ID should support the operation where the eNB can page a group, some of the groups, or all groups. This information should be carried out in UE-group ID indication embedded in NWUS. The UE needs to decode the group ID information in order to decide whether to continue with paging reception or go to sleep. The UE-group ID can be included in a form of bitmap operation as illustrated in Table 2. Here, an example is given when the supported number of groups is 4. It can be observed that the table size is significantly bigger in proportional to the maximum number of UE groups. Furthermore, most of the possible operation/combination is to page some of the groups. The number of operations/combinations will be significantly less in case MWUS with single-seq CDM for some of the groups is not supported. However, it may have some potential issues if NWUS group-ID is designed to page a group and all groups. This can be further investigated. We consider RAN1 needs to decide whether NWUS with single-seq CDM supports paging for some groups or not. This will affect the UE-group NWUS design and UE receiver to detect the NWUS.

Proposal 5: RAN1 needs to decide whether NWUS with single-seq CDM supports paging for some groups or not.

Table 2: UE-group ID with bitmap operation

| No. | Bitmap | UE-Group ID being paged |
| --- | --- | --- |
| 1 | 1111 | 1,2,3,4 |
| 2 | 1110 | 2,3,4 |
| 3 | 1101 | 1,3,4 |
| 4 | 1100 | 1,2,4 |
| 5 | 1011 | 1,2,3 |
| 6 | 1010 | 3,4 |
| 7 | 1001 | 2,4 |
| 8 | 1000 | 2,3 |
| 9 | 0111 | 1,4 |
| 10 | 0110 | 1,3 |
| 11 | 0101 | 1,2 |
| 12 | 0100 | 4 |
| 13 | 0011 | 3 |
| 14 | 0010 | 2 |
| 15 | 0001 | 1 |

# 3	Conclusions

In this contribution, the following proposals are made:

Proposal 1: On multiplexing between legacy Rel-15 NWUS and Rel-16 NWUS: support FDM (in different NB-IoT carrier) and TDM (in case of the same NB-IoT carrier).

Proposal 2: On multiplexing of Rel-16 UE groups NWUS: Support Single sequence CDM is supported. TDM can be additionally applied is FFS (i.e. depending on the number of supported groups).

Proposal 3: NB-IoT Rel-16 does not support common NWUS operation which requires the UE to receive and detect NWUS in multiple NWUS resources.

Proposal 4: The introduction of UE-group should be designed to maximize the benefit of UE-group mechanism. We propose the maximum number of UE groups supported is at least 6.

Proposal 5: RAN1 needs to decide whether NWUS with single-seq CDM supports paging for some groups or not.

# References

- RAN1 #95 Chairman notes, Spokane, USA, November 2018

- R1-1811664, “Feature summary for NB-IoT WUS UE-grouping,” Ericsson, Chengdu, Oct 2018