3GPP TSG RAN WG1 #96	R1-1902269

Athens, Greece, 25th February – 1st March, 2019

Agenda item:	7.2.3.2

Source: 	Samsung

Title: 	Support of SSBs for IAB Node Discovery and Measurement

Document for:	Discussion and Decision

# Introduction

A new work item on “Integrated Access and Backhaul for NR” was approved in RAN#82 [1] and the following agreement was made in RAN1#94 [2], RAN1#95 [3] and RAN1#Ad-Hoc 1901 [4] as follows.

Agreements:

- Solution 1-B means SSB, that may get muted, for inter-IAB cell search and measurement in stage 2 is not on the currently defined sync raster for a SA frequency layer, while for a NSA frequency layer the SSBs are transmitted outside of the SMTC configured for access UEs

- Solution 1-A means SSB for inter-IAB cell search in stage 2 is on the currently defined sync raster for a SA frequency layer, while for a NSA frequency layer the SSBs are transmitted inside of the SMTC configured for access UEs

Agreements:

An IAB node should not mute SSB transmissions targeting UE cell search and measurement when doing inter-IAB cell search in stage 2

- For SA, means that SSBs transmitted on the currently defined sync raster follows the currently defined periodicity for initial access

- Means that Solution 1-B implies SSB, that may get muted, for inter-IAB stage 2 cell search is at least TDM with SSB used for UE cell search and measurements

Agreements:

- Solution 1-A and Solution 1-B are both supported

  - Enhancements for off-raster SSB, e.g. new periodicities and time-domain mapping can be considered

Agreements:

Capture the following conclusions for the IAB SI TR:

RAN1 has studied various physical layer aspects for Integrated Access and Backhaul, and from a RAN1 perspective, support for the following features and solutions has been determined to be beneficial and feasible:

- Mechanisms for discovery of IAB nodes and management of backhaul links in both SA and NSA deployments, taking into account the half-duplex constraint at an IAB node and multi-hop topologies, including:

  - Solutions reusing the same set of SSBs used for access UEs and solutions which use of SSBs which are orthogonal (TDM and/or FDM) with SSBs used for access UEs

For the SSB based solution, it has been agreed that two sub-solutions will be supported:

-	Solution 1-A) Reusing the same set of SSBs used for access UEs:

-	In this case, the SSBs for inter-IAB cell search in stage 2 are on the currently defined sync raster for a SA frequency layer, while for a NSA frequency layer the SSBs are transmitted inside of the SMTC configured for access UEs.

-	Solution 1-B) Use of SSBs which are orthogonal (TDM and/or FDM) with SSBs used for access UEs:

-	In this case, the SSBs, that may get muted, for inter-IAB cell search and measurement in stage 2 are not on the currently defined sync raster for a SA frequency layer, while for a NSA frequency layer the SSBs are transmitted outside of the SMTC configured for access UEs.

Agreements:

- SSB transmission configuration information (i.e. the necessary information for the DU to determine when to transmit SSB) and SSB reception configuration information (i.e. the necessary information for the MT to determine when to search for SSB) for IAB inter-node measurements are determined in a centralized manner, i.e. not locally by the IAB node.

- An IAB node is provided with configuration of SSB transmission and reception information for IAB inter-node measurements.

  - FFS details

In this contribution, we will discuss the necessary enhancements for inter IAB nodes discovery via SSBs.

# Solution 1-A

For solution 1-A, the same set of SSBs will be used for both access UEs and inter IAB node discovery. In such a case, a mechanism is needed to make sure that one IAB node can hear from other IAB node subject to half-duplex constraint. A muting pattern can be in SSB burst set level. For the SSB burst set level muting, an entire SSB burst set needs to be muted to listen to SSB from other IAB nodes and it will cause significant impact on access UEs. However, it has been agreed that the impact on Rel-15 UE should be minimized so that such muting might not be optimal for access UEs. On the contrary, within one SSB burst set the positons of SSB from multiple IAB nodes can be coordinated to enable inter node discovery subject to the half-duplex constraint. Within each SSB burst set, there are multiple positions to transmit SSBs. However, the SSB positions are only possible positions for SSB transmission, that is, it is not mandated that an SSB should be transmitted in all the possible locations. Anything from a single SSB up to maximum number of SSBs within one SSB burst set is possible depending on beam sweeping requirements. In such a case, one IAB node A can transmit on one subset of all possible positions and another IAB node B can transmit on another subset not overlapping with subset of IAB node A. In this regard, with appropriate subsets chosen mechanism, e.g., semi-persistent or dynamic ones, there could be minimal or even no impact on access UEs.

Proposal 1: For solution 1-A, SSB positions within one SSB burst set can be coordinated across multiple IAB nodes to enable inter IAB node discovery subject to half-duplex constraint.

# Solution 1-B

For solution 1-B, SSBs which are orthogonal (TDM and/or FDM) with SSBs for access UEs are used for inter IAB node discovery so that there is no impact on access UE. Subject to half-duplex constraint, each IAB node might need to mute some of its own SSBs to listen to SSBs from other IAB nodes. Orthogonal resources for backhaul and access link detection and measurement can be achieved via SSB muting across IAB nodes. During those muted SSBs, IAB nodes can listen to SSBs from other IAB nodes to detect potential candidate backhaul links. However, it should be noted that such muting is only needed when the SSB transmission are aligned for those IAB nodes that require measurement of SSBs to monitor the quality of backup backhaul links between each other. In particular, IAB nodes within the same hop could align their SSB transmissions to optimize the signalling overhead. However, between different hops, other reference signals and more flexible measurement can be considered.

Observation 1: Muting is only needed when the SSB transmission are aligned for those IAB nodes that require measurement of SSBs to monitor the quality of backup backhaul links between each other.

In order to introduce muting pattern, we can consider following alternatives:

- Alt1: Muting across all IAB nodes;

- Alt2: Muting based one hop order, i.e., muting pattern within each hop.

The first solution is illustrated in the below figure (tree and muting). The muting is across all IAB nodes no matter which hop they belong to. The benefit of such configuration is that one IAB node can always measure all possible backhaul links including both primary and backup ones and it is independent of topology change. However, the scalability of such configuration might be limited since it requires higher signalling overhead scaling with total number of IAB nodes involved in inter IAB node SSB measurement.

Figure. 1 Muting across all IAB nodes

Figure. 2 Muting based on hop order

The second solution is shown in Figure 2, where the muting is constrained between IAB nodes belonging to the same hop. In such a case, the SSB transmission are either aligned across all IAB nodes as shown in Fig. 2 or across IAB nodes belonging to the same hop only as shown in Fig. 3. For solution 2, one IAB node can measure all backup backhaul links via SSBs irrelevant of hop order. Such configuration can reduce the signalling overhead since the required number of SSBs is equal to the total number of IAB nodes involved in inter IAB node SSB measurement within each hop. Therefore, the scalability of solution 2 is improved compared with the first solution. It should be noted that the configuration of SSBs depends on topology and any topology change might incur re-configuration of SSBs as discussed next.

Figure. 3 Muting based one hop order

As aforementioned, for the first solution, the configuration of SSBs, e.g., number of SSBs within one measurement cycle, depends on the total number of IAB nodes involved in inter-IAB node discovery and measurement. The topology change does not have impact on this number. However, for the second solution, the total number of SSBs within one measurement cycle is equal to the maximum number of IAB nodes involved in inter-IAB node discovery and measurement belonging to the same hop so that topology change has impact on the configuration of SSBs. At least semi-persistent configuration/reconfiguration of SSBs for inter-IAB node discovery and measurement should be supported and dynamic configuration can further be discussed.

The TDM or muting pattern mentioned above needs to be repeated in a time domain and it can be configured either by multiple sets of SSBs or otherwise, one set of SSB for inter-node discovery can be configured for each IAB node, and then a periodicity for SSB measurement can be configured together with a bitmap indicating TDM pattern within each period. The periodicity for SSB can be derived from the length of the bitmap to further reduce singling overhead. The same level of flexibility can be achieved by both approaches but the signalling overhead of configuring multiple sets of SSBs could be much higher than the latter approach. For measurement of such SSBs, a single SMTC can be configured for inter-node discovery and when it collides with inter-node SSB transmission from itself, it can be overridden so the IAB node can transmit SSB.

Proposal 2: For SSB TDM pattern, one set of SSB for inter-node discovery can be configured for each IAB node, and then a periodicity for SSB measurement can be configured together with a bitmap indicating TDM pattern within each period. Accordingly, a single SMTC can be configured for inter-node discovery and when it collides with inter-node SSB transmission from itself, it can be overridden so the IAB node can transmit SSB. Such configuration should depend on hop order.

Propose 3:  The following solutions for muting of SSBs should be considered for inter IAB node discovery and measurement

- Alt1: Muting across all IAB nodes;

- Alt2: Muting based one hop order, i.e., muting pattern within each hop.

Proposal 4: At least semi-persistent configuration/reconfiguration of SSBs for inter-IAB node discovery and measurement should be supported.

In particular, two Alts should be considered as follows,

- Alt1: Predefined muting;

- Alt2: Semi-persistent or dynamic SSB muting.

For Alt1, SSB muting pattern is predefined so that less signaling overhead is caused but it also provides low flexibility and configurability. For Alt2, SSB muting can be configurable to provide high flexibility, thus better adapting itself to the variations of channel and traffic. For example, a relay IAB node can indicate to its parent IAB node the SSBs that it has detected. Based on this report, the parent IAB node can then indicate to the relay IAB node a candidate set of SSBs that the relay IAB node shall choose from to transmit its own synchronization signals. It provides a means for the parent IAB node to coordinate the SSB resources by multiple relay IAB nodes. The signaling procedure needs to be further studied. In both Alts, the cell detection/measurement performance of the UE before RRC_CONNECTED will be impacted due to loss of SSB occasions because of muting. For the pre-defined solution, the level of impact cannot be controlled by the network and in some circumstances might be significant. On the contrary, the level of impact can be controlled and mitigated by the network in Alt2 by changing the muting pattern configuration semi-persistently or dynamically.

Proposal 5: Following Alts can be considered for SSB muting

- Alt1: Predefined muting;

- Alt2: Semi-persistent or dynamic SSB muting.

- FFS: Signaling procedure.

# Conclusions

This contribution considered the necessary enhancements for inter IAB nodes discovery via SSBs. In particular, the following is proposed.

Observation 1: Muting is only needed when the SSB transmission are aligned for those IAB nodes that require measurement of SSBs to monitor the quality of backup backhaul links between each other.

Proposal 1: For solution 1-A, SSB positions within one SSB burst set can be coordinated across multiple IAB nodes to enable inter IAB node discovery subject to half-duplex constraint..

Proposal 2: For SSB TDM pattern, one set of SSB for inter-node discovery can be configured for each IAB node, and then a periodicity for SSB measurement can be configured together with a bitmap indicating TDM pattern within each period. Accordingly, a single SMTC can be configured for inter-node discovery and when it collides with inter-node SSB transmission from itself, it can be overridden so the IAB node can transmit SSB. Such configuration should depend on hop order.

Propose 3:  The following solutions for muting of SSBs should be considered for inter IAB node discovery and measurement

- Alt1: Muting across all IAB nodes;

- Alt2: Muting based one hop order, i.e., muting pattern within each hop.

Proposal 4: At least semi-persistent configuration/reconfiguration of SSBs for inter-IAB node discovery and measurement should be supported.

Proposal 5: Following Alts can be considered for SSB muting

- Alt1: Predefined muting;

- Alt2: Semi-persistent or dynamic SSB muting.

- FFS: Signaling procedure.

# References

[1] RP-182882 New WID Proposal: WID on Integrated access and backhaul for NR.

[2] 3GPP RAN1#94 Chairman’s note.

[3] 3GPP RAN1#95 Chairman’s note.

[3] 3GPP RAN1#Ad-Hoc 1901 Chairman’s note.