3GPP TSG RAN WG1 #96                          	                                    R1-1902332

Athens, Greece, 25th February – 1st March, 2019

Source:	CMCC

Title:	Discussion on UE-UE cross link interference measurements

Agenda Item:	7.2.5.1

Document for:	Discussion and Decision

- Introduction

In 3GPP RAN1 AH1901 meeting, the following agreements on UE-UE CLI measurements and reporting  achieved [1]:

Agreement

Measurement resource for SRS-RSRP measurement is configured by SRS resource configuration.

FFS: Whether REs for PDSCH is rate matched around the REs for SRS for SRS-RSRP measurement or not.

FFS: Whether OFDM symbol(s) for PDSCH is rate-matched or not.

- Note: If Rel-15 already supports this functionality, no additional specification support will be introduced

Agreement

For SRS-RSRP measurement report, L3 measurement reporting is applied

Agreement

For CLI-RSSI, L3 measurement reporting is applied

- FFS: Whether SRS-RSRP measurement report and CLI-RSSI measurement report can be configured together for a UE

  - Whether UE should be required to perform SRS-RSRP measurement and CLI-RSSI measurement simultaneously

Agreement

For L3 measurement reporting of RSSI/SRS-RSRP, the specification detail on the reporting and triggering mechanism is discussed in RAN2

- Send an LS to RAN2 (LGE, Hyunsoo)

In this contribution, we will present some views on measurement SRS resource configuration and the CLI measurement. Besides, discussion on back-haul information exchanged among multiple cells for CLI management can be found in our companion contribution R1-1902333[2]

- CLI Measurement through SRS configuration

2.1 Discussion on SRS configuration

In this subsection, we mainly discuss SRS-RSRP based CLI management mechanism. In 3GPP RAN1 AH1901 meeting, it has been agreed that “Measurement resource for SRS-RSRP measurement is configured by SRS resource configuration.” However, in the existing specification, the usage of one SRS resource set can only be set to “Beam Management”, “Antenna Switching”, “Codebook” or “NonCodebook”. In addition, different restrictions are defined in the specification for the different usages of SRS resource set.  For example, only one SRS resource set can be configured when higher layer parameter SRS-SetUse is set to “Codebook” or “NonCodebook”, but more than one SRS resource sets can be configured when higher layer parameter SRS-SetUse is set to “Beam Management” or “Antenna Switching”. Hence, considering the SRS resource set for CLI measurement should be distinguished from the other usages, we have the following proposal:

Proposal 1: To reuse  SRS resource set for CLI measurement, the higher layer parameter SRS-SetUse can be set as “CLI measurement”.

2.2 Discussion on SRS resources based joint transmit/receive mechanism

For SRS-RSRP based CLI measurement, not only resource configuration for SRS transmit is required, but also corresponding resource configuration for SRS receive is needed. That is, a UE needs to know the time-frequency resources where to transmit SRS and where to receive SRS from UEs of other cells. Herein, we present a joint CLI measurement mechanism through SRS resources configuration, for long-term periodic/semi-persistent CLI measurement.

In current specification, the UE only transmit SRS resources within the configured time-frequency resources. In terms of the joint transmit/receive mechanism, the transmit or receive states corresponding to the SRS resources can be indicated by a new higher layer parameter. For example, for one SRS resource set with 3 SRS resources and a corresponding 3-bit bitmap as “110”, then the states of the 3 SRS resources is “[transmit transmit receive]”. That is, some SRS resources can be reused to perform the CLI measurement. Thus, based on the certain coordination information (e.g. the SRS resource configuration and bitmap configuration information) exchanged among neighboring TRPs, within the configuration period of the SRS resource set, all the UEs/UE groups of multiple cells can perform CLI measurement simultaneously. This mechanism can reduce the Xn/back-haul signaling overhead, as only the SRS configuration information needs to be exchanged one time among TRPs for long-term periodic/semi-persistent CLI measurement.

Fig 1 aims to present this mechanism more clearly. Assume that there are 3 neighboring TRPs and only one UE group in each TRP is to be measured. In this case, only one CLI measurement SRS resource set with 3 SRS resources is configured for each UE group. The time domain behavior of this SRS resource set is periodic/semi-persistent. All SRS resources within one set are configured with the same periodicity (e.g. 10 slots). Specifically, for each SRS resource only one OFDM symbol is configured. For different UE groups among neighboring TRPs the time-frequency resources of the SRS resource set and the slot offset configuration  are consistent. Within a SRS resource set, the transmit or receive state of the symbols corresponding to the SRS resources can be indicated by a 3-bit bitmap. In Fig 1, for UE group 1 served by TRP1, if the 3-bit bitmap set to “110”, this refers to that the UE group 1 will transmit the first and the second SRS resource, and do the operation of measurement at the resource corresponding to the third SRS resource. Meanwhile, through network coordination, the bitmaps of the UE group 2 and UE group 3 will  set as “101” and “011”, respectively.

Fig 1.  SRS resources based joint transmit/receive mechanism

Besides, considering the switch time between transmit and receive, a guard period (GP) of Y symbols between any two SRS resources is needed, specially for the case that the SRS resources of a SRS resource set are transmitted in the same slot. The value of Y corresponds to the numerology of the system, and the minimum value is 1. Also, note that the GP can avoid the timing misalignment.

Hence, we have the following proposal:

Proposal 2: For long-term periodic/semi-persistent CLI measurement, support SRS resources based joint transmit/receive  mechanism. The transmit/receive  states can be indicated by a bitmap, and a GP between any two SRS resources  needs to be considered.

- CLI Measurement through RateMatchPattern Configuration

Alternatively, CLI can be measured by reusing RateMatchPattern configuration, which could minimize the spec impacts. As shown in Fig 2, the resources corresponding to RateMatchPattern can be configured with RB-Symbol level. The TRP can indicate the interfered UEs to perform the CLI measurement in the OFDM symbols configured by RateMatchPattern. Specifically, through network coordination, the gNB could configure UE the CLI measurement resource by RateMatchPattern which is same as the SRS resource transmitted by the UEs in the neighbor cell. That is, in this scheme, the transmit resource for  measurement RS (SRS) and  the receive resource for CLI measurement are configured independently.

Fig 2. Reusing RateMatchPattern for CLI measurement

Proposal 3: Support reusing the RateMatchPattern for  CLI measurement.

- Conclusion

In this contribution, we have presented some discussions on UE-UE cross-link interference management. The following proposals are given:

Proposal 1: To reuse  SRS resource set for CLI measurement, the higher layer parameter SRS-SetUse can be set as “CLI measurement”.

Proposal 2: For long-term periodic/semi-persistent CLI measurement, support SRS resources based joint transmit/receive  mechanism. The transmit/receive  states can be indicated by a bitmap, and a GP between any two SRS resources  needs to be considered.

Proposal 3: Support reusing the RateMatchPattern for CLI measurement.

References

- Chairman Notes, 3GPP TSG RAN WG1 Meeting #AH1901

- R1-1902333 Discussion on backhaul information exchange for CLI management