3GPP TSG-RAN WG1 #96	R1-1902364

Athens, Greece, February 25th – March 1st, 2019

Agenda Item:	6.1.4

Source:	Qualcomm Incorporated, Ericsson

Title:	Details for Draft CR R1-1902364 of WUS EPRE

Document for:	Discussion and Decision

# 1	Introduction

- Current specification does not state the WUS EPRE. WUS uses single antenna port while CRS may use one or multiple antenna ports. CRS may be power boosted, resulting in different power ρA and ρB for PDSCH REs. But WUS EPRE should be same for all WUS REs. Considering impact of CRS port number as well as ρA and ρB, the WUS EPRE relative to CRE EPRE should be defined.

# 2	Discussion

- Figure 1 illustrated the power in case of 1, 2 or 4 CRS ports, where ρA and ρB relative to CRS power per port is the power for PDSCH. The ratio of ρA and ρB is cell-specific, indicated by PB as specified in Table 5.2-1 of [1].

- Figure 1 Power for different CRS port number

- In case of single CRS port (p=1), assuming same power for symbol with or without CRS RE, we have 5ρB+PCRS=6ρA. If assuming PCRS=1, the default value relative to CRS power is ρAd=(1/(6-5ρ) with ρ=ρB/ρA.

- In case of two CRS ports (p=2), CRS port 0 and port1 are FDMed. Assuming same power for symbol with or without CRS REs, we have 4ρB+PCRS=6ρA. If assuming PCRS=1, the default value relative to CRS power is ρAd=(1/(6-4ρ) with ρ=ρB/ρA.

- In case of four CRS ports (p=4), CRS port2 and port 3 are FDMed, but TDMed with the symbol with CRS port 0 and port1. Assuming same power for symbol with or without CRS REs, we have 4ρB+PCRS=6ρA. If assuming PCRS=1, the value relative to CRS power is same as 2-port case.

- Different from PDSCH, the WUS power should be constant per RE. To keep it within max transmit power per symbol, we take the smaller power for WUS REs. Therefore, accumulating the power on each antenna port as that of a virtual antenna port, the WUS EPRE relative to CRS EPRE is 10log10(p×minρAd, ρBd), where ρAd and ρBd for different PB are given by Table 5.2-1a and p is the number of CRS ports. Note that ρAd and ρBd  allow for uniform power allocation across all RBs.  In case of ρAd=ρBd, the WUS EPRE relative to CRS EPRE is 0dB for 1- or 2-port CRS (similar with the agreed NB-IoT WUS EPRE relative to NRS EPRE), and 3dB for 4-port CRS, respectively.

- By default, we assume power boosting on the 2PRB WUS by blanking the other 4 RBs within a 6PRB narrowband, the WUS EPRE relative to CRS EPRE is 10log10(3×p×minρAd, ρBd).

Table 5.2-1: The cell-specific ratio  for 1, 2, or 4 cell specific antenna ports

|  | One Antenna Port | Two and Four Antenna Ports |
| --- | --- | --- |
| 0 | 1 | 5/4 |
| 1 | 4/5 | 1 |
| 2 | 3/5 | 3/4 |
| 3 | 2/5 | 1/2 |

Table 5.2-1a: ρAd and ρBd for 1, 2, or 4 cell specific antenna ports assuming the same transmitted power for symbols with or without CRS

| PB | One Antenna Port |  | Two and Four Antenna Ports |  |
| --- | --- | --- | --- | --- |
|  | ρAd | ρBd | ρAd | ρBd |
| 0 | 1 | 1 | 1 | 5/4 |
| 1 | 1/2 | 2/5 | 1/2 | 1/2 |
| 2 | 1/3 | 1/5 | 1/3 | 1/4 |
| 3 | 1/4 | 1/10 | 1/4 | 1/8 |

- Proposal:

- The WUS EPRE relative to CRE EPRE is defined as 10log103×p×min⁡(ρAd,ρBd), where ρAd and ρBd are the default values for ρA and ρB relative to CRS EPRE assuming the same transmitted power for symbols with or without CRS, and p is the number of CRS ports.

# 3	Conclusion

- Proposal:

- The WUS EPRE relative to CRE EPRE is defined as 10log103×p×min⁡(ρAd,ρBd), where ρAd and ρBd are the default values for ρA and ρB relative to CRS EPRE assuming the same transmitted power for symbols with or without CRS, and p is the number of CRS ports.

# Reference

[1] 3GPP TS36.213.