3GPP TSG-RAN WG1 Meeting #96	R1- 1902771

# Athens, Greece, 25th February – 1st March, 2019

Agenda Item:	7.2.9.2

Source:	Apple Inc.

Title:	UE Power Saving Techniques

Document for:	Discussion, Decision

# Introduction

To reduce the power consumption at UE side, UE can take advantage of the information available inside handset such as file size, traffic arrival rate, application layer traffic’s throughput/delay requirements, implementation specific power consumption characteristics of RF and BB for different frequency, bandwidth, number of carriers, scheduling time line etc. Some of this information could be directly used at UE side for reducing power, but in most cases this information should be transferred to network to help network configure/schedule UE for power saving. This is because the power consumption of UE highly depends on UE configuration, and the configuration is done by the network.

In this paper, we discuss UE-assisted techniques for UE power saving.

# UE-assisted Techniques for Power Saving

## UE-preferred Bandwidth Part Configuration and Change Request

### Motivation

NR supports higher data rate with wider bandwidth (up to 400MHz for single carrier). This requires much higher rate of data processing in both RF side and baseband side; supporting higher sampling rate, smaller slot lengths, higher encoder/decoder throughput, faster clocks, higher voltage (with higher leakage), larger memory, and LTE-NR dual connectivity require significantly higher power consumption. Especially, the support of mmWave requires additional power consumption for beam forming operation and beam management.

The BWP were introduced to overcome the increased power consumption. Network can use BWPs to adapt bandwidth to meet data transmission/reception demand. Network can configure narrow BWP for monitoring PDCCH only or wider BWP for receiving large-sized data. Channel bandwidth adaptation using BWP is an effective power saving technique.

However, choosing right size of BWP size for UE power saving is quite challenging problem. The first order question is whether to use large bandwidth part or narrow bandwidth part to support a given amount of data for a given time duration. Two strategies could be considered : i) using large BWP and long deep sleep vs ii) using narrow BWP and short shallow sleep. This is not easy to answer the total projected power consumption depends on many factors such as data amount to transfer, power consumption in different BWP, sleep power, duration T of interest, etc.

Figure 1 BWP scheduling problem with two strategies: i) large BWP and long deep sleep, ii) narrow BWP and short shallow sleep

The second order question is how large the BWP should be. If the selected BWP is too large compared to required, then RF components could consumes too much power (more than required) for unnecessary sampling/filtering/ADC. While, if the selected BWP is too small compared to offered traffic load, then network has to schedule PDSCHs to UE for longer time, which requires UE awake / processing for longer time. Thus, it could potentially increase UE power consumption.

Furthermore, whether UE can expect power saving or not is up to network configuration / scheduling decision. If network does not configure multiple BWPs with different sizes or if network does not change BWPs considering traffic demand and UE power consumption, then, even with multiple different BWPs configured, there could be no power saving at UE side (e.g., the case when BWP is used just for load balancing purpose). Similar example in LTE is DRX configuration. If only very simple configuration is used due to network complexity, then it is hard to expect intended power saving gain at UE side.

UE having better knowledge on channel condition, traffic demand/pattern/usage and its internal power consumption status/level may help network to make better decisions for both BWP configuration and switching. For example, UE knows file sizes to download or upload. Thus, it could estimate expected power consumption/throughput for different BWPs using its internal information. This estimate could be sent to network to help network make BWP configuration in favor of UE’s power saving. Figure 2 captures current configuration mechanism, which is one way (left), and suggested new configuration mechanism based on UE-preferred BWP configuration, which is two way (right).

Observation 1

- UE having better knowledge on channel status and traffic demand/pattern/usage could help network to make better decisions for BWP configuration.

Figure 2 Current BWP framework (left) and improved BWP framework 
with UE preferred BWP configuration (right)

### Proposals

If UE can indicate its preferred BWP configuration, then UE can potentially achieve higher power saving. It is clear that UE has more information about device owner’s preference on applications, types/amount/duration of traffics to be received/transmitted, etc. This kind of information could be used in conjunction with internal power consumption statistics to determine the right size of BWP. Another reason is that different UE could have different RF capabilities and different implementations. This means optimal BWP for a certain traffic could be different for different UEs.

We see that there are strong motivation for network to configures larger BWP (and potentially larger number of component carriers (CC)). It is because that, with larger BWPs and larger number of CCs, network can keep its scheduling flexibility high to improve network capacity. Thus, in many cases, we see that component carriers are active without data transmission. In such case, UE should be able to request to use a smaller BWP or turn off unused/idling component carriers.

Proposal 1

- NR supports that UE can send UE-preferred BWP configuration to network.

  - UE can indicate it’s preferred BWP configuration including a set of BWP sizes and starting RB indices, and BWP inactivity timer value.

  - NR supports that network can trigger UE to send UE-preferred BWP configuration by sending triggering message to UE potentially including side information to help UE to make better estimate of its UE-preferred BWP.

Another related topic is UE-preferred BWP change request. It is different from UE-preferred BWP configuration in the sense that UE can indicate network one specific BWP which it wants to operate among already configured BWPs. Considering that different operator/network vendors may use different policies for BWP switching, it may be hard for UE to expect consistent power saving gain across different networks. Thus, it could be quite beneficial to have UE initiated BWP change request to provide consistent power saving gain.

Proposal 2

- NR supports UE sends a BWP change request to network to initiate the change of active DL/UL BWP.

### Procedure

The UE-preferred BWP configurations could depend on UE’s traffic situation. If UE expect large amount of traffic arrival, then UE may prefer to be configured with BWPs including at least one very wide BWP. Thus, ideally, UE should be able to send message including preferred configurations whenever UE wants to change its set of configured BWPs.

Figure 3 (a) shows the potential signaling procedure for UE preferred BWP configuration message transmission. Network can optionally send triggering message to UE to get UE-preferred configuration from UE. The Figure 3 (b) shows the signaling flow for BWP change triggered by UE’s BWP change request.

Figure 3 (a) shows the signaling flow to support UE-preferred BWP configuration. Figure 2 (b) shows the signaling flow for BWP change based on the BWP change request from UE.

General procedure could further improved as follows based on agreed text in TR[7].

The UE assistance information for the power saving schemes for further studies are as follows,

  - UE assistance information/feedback to assist network in configurations for UE adaptation

    - UE preferred processing timeline parameters, e.g., K0, K1, K2 values

    - UE preferred BWP information/configuration

      - UE can indicate its preferred BWP configuration to network, which is determined by UE based on e.g., total amount of data to download/upload from higher layer, required throughput, transmission time requirement, etc.

    - UE preferred antenna configuration, including MIMO layers, antenna panel awareness information

    - UE assistance/feedback on the DRX configurations/parameters

    - UE preferred BWP provided to assist network in BWP switching

      - UE can request gNB to change its active BWP in case traffic pattern is changed based on higher layer user activity (e.g., application initiation / termination by user)

    - UE request on SCell/SCG activation/de-activation/configuration

    - UE preferred PDCCH monitoring parameters/search space configuration/maximum number of blind decoding

# Other UE assistance information for the power saving schemes is not precluded.

## UE-preferred Timeline Adaptation for Power Saving

### Motivation

NR has very flexible HARQ scheduling timing. Unlike LTE with fixed HARQ timing, the transmission timing for NR’s PDSCH and corresponding ACK can be dynamically determined and indicated by DCI. The increased flexibility makes network scheduler much easier to serve multiple UEs with different processing capability, different traffic demand, different latency requirement, etc.

We see that a certain scheduling pattern (we call it close-in-time scheduling) could potentially help UE power saving. For example, note that a UE which receives PDCCH in slot N, PDSCH in slot N+4, and sends ACK in slot N+8 can enter micro sleep state between slot N and N+4 and between N+4 and N+8 as shown in the Figure 4 (a). If network can schedule this UE with PDCCH, PDSCH, and ACK all very close in time (e.g., in the consecutive N slots ), then the UE can quickly finish data reception and enter deeper sleep mode for longer time, which save more power as shown in Figure 4 (b). Since there is transition power cost for ramp up and ramp down, it is better to reduce those power cost and make modem sleeps longer in deeper sleep state. This can be achieved by network’s choice K0, K1, K2 and at the same time UE’s indication of preferred K0,K1,K2 values to network could be help achieving such scheduling pattern.

Figure 4 Reduced power consumption of close-in-time scheduling compared slot scheduling with K0>0 and K1>0 illustrating the reduced power consumption (dark green) and lower power consumption during sleep state

Another strategy for achieving close-in-time scheduling is daisy chain like scheduling as shown in Figure 5. In this case, ACK, PDCCH, and PDSCH belonging to different HARQ processes are scheduled back-to-back to avoid unnecessary ramp down/up and maximize sleep duration between tx/rx activities. In addition to that, cross-slot is enabled with K0>0 and processing time of PDSCH could be relaxed with K1>0, which could potentially helpful. However, compared to previous strategy, this strategy introduces additional delay due to increased K0 and K1 values and can be used only when there is consistent regular traffic arrivals.

Figure 5 Daisy chain scheduling with K0=P, K1=P-1, where P is PDCCH monitoring periodicity, could achieve close-in-time scheduling. Corresponding PDCCH, PDSCH and ACK have the same color.

In either cases shown in Figure 4 and Figure 5, UE-preferred K0, K1, K2 values to network could help UE’s power saving.

Proposal 3

- NR supports the UE’s indication of preferred set of K0, K1, K2 values to network for UE power saving.

### Procedure

The UE indication of its preferred set of K0, K1, and K2 values could be sent any time the UE wants. And, actual power saving gain would depend on network’s decision on K0,K1 and K2 values. Since it does not require any specific activation/deactivation or start/end signaling, the application of such scheduling policy to UE is transparent to UE’s signaling point of view.

Figure 6 Singling procedure for UE-preferred K0/K1/K2 indication and application of close-in-time scheduling

General procedure could further improved as follows based on agreed text in TR[7].

The UE assistance information for the power saving schemes for further studies are as follows,

  - UE assistance information/feedback to assist network in configurations for UE adaptation

    - UE preferred processing timeline parameters, e.g., K0, K1, K2 values

      - UE can further indicate sleep operation related parameters together with K0, K1, K2.

    - UE preferred BWP information/configuration

    - UE preferred antenna configuration, including MIMO layers, antenna panel awareness information

    - UE assistance/feedback on the DRX configurations/parameters

    - UE preferred BWP provided to assist network in BWP switching

    - UE request on SCell/SCG activation/de-activation/configuration

    - UE preferred PDCCH monitoring parameters/search space configuration/maximum number of blind decoding

# Other UE assistance information for the power saving schemes is not precluded.

## UE-preferred Time Domain Resource Allocation Table Configuration

### Motivation

In NR, 3GPP 38.331 defines very flexible PDCCH/PDSCH time-domain resource allocation (TDRA) pattern. In particular, RRC can configure one table per BWP, which includes a list of possible (up-to 16) TDRA patterns. Each entry of the TDRA table includes three fields, namely, K0, PDSCH mapping type, and StartSymbolAndLength. K0 defines the distance between PDCCH and PDSCH in slot (i.e., K0=0 indicates same-slot scheduling, whereas K0>0 indicates cross-slot scheduling); mapping type defines the scheduling to be either slot-based or mini-slot based; and startSymbolAndLength defines the location and span of PDSCH allocation within the slot. At PHY layer, PDCCH DCI decoding provides an entry index of the table, indicating to UE which entry of the TDRA table should be used in current slot. In theory, gNB can change TDRA from slot to slot by DCI. UE does not know the pattern used for current slot before PDCCH decoding finishes.

The fundamental problem of this approach is that UE cannot avoid unnecessary PDSCH buffering if there is potential PDSCH scheduling before PDCCH decoding finishes, as shown in Figure 7. This is because UE does not know whether PDSCH is scheduled or not until PDCCH decoding finishes. Considering that PDCCH decoding finishes in the middle of slot, there is gap between the last PDCCH symbol and the symbol PDCCH decoding finishes during which UE has to buffer potential PDSCH data, which requires RF in ON state and some baseband processing for buffering. To avoid unnecessary buffering, there should be enough gap between PDCCH and PDSCH for UE to finish PDCCH decoding.

Figure 7 Unnecessary PDSCH buffering is required when there could be PDSCH transmission before PDCCH decoding finishes.

Observation 2

- UE buffers potential PDSCH unnecessarily when potential PDSCH transmission can occur before corresponding PDCCH decoding finishes.

Based on our study, we found that many different TDRA patterns, including both same-slot scheduling and cross-slot scheduling patterns, can provide significant power savings for UE, provided that the potential set of patterns (i.e., the entries of the table) are known by UE before DCI decoding in advance. In following sections, we provide detailed proposals and procedures to avoid unnecessary PDSCH buffering.

### Proposals

One simple current standard based solution is to use BWP framework. For example, among all BWPs configured, one BWP could be configured to have TDRA entries with K0>0. This will ensure that there is always enough gap for UE to finish PDCCH decoding before actual PDSCH starts. However, considering that there is only 4 BWPs allowed in R15, duplicating one BWP with only different K0 value for power saving purpose is actually significant waste of configuration resource reducing flexibility of network/UE operation. Thus, direct control of PDCCH-to-PDSCH gap is preferred design choice.

The solution for ensuring enough gap between PDCCH and PDSCH are related to TDRA of PDSCH. TDRA basically has all the information about PDSCH allocation ; in which slot the PDSCH is transmitted (i.e., K0), from which symbol it starts and how long it lasts (SLIV information). In most cases ensuring K0>0 is good enough, however there are certain cases where i) K0>0 does not guarantee avoiding unnecessary PDSCH buffering and ii) with K0=0 where we can still avoid unnecessary PDSCH buffering.

- Case i) A search space configured in the middle of slot and actual PDSCH transmission occurs in the beginning of next slot. In this case, cross slot scheduling (with K0=1) may not guarantee avoiding unnecessary PDSCH buffering.

- Case ii) A PDSCH is transmitted in the last two OFDM symbols in the slot where corresponding PDCCH is transmitted.

Observation 3

- Controlling K0 only does not always ensure UE to avoid unnecessary PDSCH buffering.

Thus, we propose to control TDRA entries or table taking into account of both K0 and PDSCH symbol allocation. Depending on PDSCH allocation and K0 value, network can enable / disable certain set of entries in TDRA table.

Proposal 4

- NR supports a MAC CE which can activate/deactivate a subset of TDRA entries. A scheduling DCI can indicate one of activated entries only.

Proposal 5

- NR supports that UE report its preferred TDRA entries back to gNB among the entries in the RRC-configured TDRA table.

Proposal 6

- NR supports that UE sends a TDRA entry change request.

General procedure could further improved as follows based on agreed text in TR[7].

Section 5.1.6

The UE assistance information for the power saving schemes for further studies are as follows,

  - UE assistance information/feedback to assist network in configurations for UE adaptation

    - UE preferred processing timeline parameters, e.g., K0, K1, K2 values

    - UE preferred BWP information/configuration

    - UE preferred antenna configuration, including MIMO layers, antenna panel awareness information

    - UE assistance/feedback on the DRX configurations/parameters

    - UE preferred BWP provided to assist network in BWP switching

    - UE request on SCell/SCG activation/de-activation/configuration

    - UE preferred PDCCH monitoring parameters/search space configuration/maximum number of blind decoding

    - UE can indicate its preferred TDRA table, or indices of entries of configured TDRA table.

# Other UE assistance information for the power saving schemes is not precluded.

## UE-initiated SCell Activation/Deactivation

### Motivation

It has been noted that LTE CA consumes significant amount of power. But, the real problem is not that it consumes high power but that a lot portion of power for activating secondary carrier is being wasted instead of being used for data transmission. In typical CA scenarios, the following problems have been observed.

- Activation delay : SCells are typically activated only after network DL queue builds up beyond a certain threshold, which delays quick data transfer due to the delay for activation.

- Deactivation delay : Network does not quickly deactivate after data transfer finishes although there is no active data transfer happening. Even though there is MACCE-based dynamic activation/deactivation mechanism, it is hardly used in the field and deactivation mostly rely on SCell deactivation timer which is typically configured in the order of hundreds of ms (typically on the next on-duration interspersed by a long CDRX cycle)

- SCells causing low overall achievable throughput: Sometimes it is observed that SCells with low achievable throughput do not always contribute to the overall achievable throughput by a huge margin. In such situations, the advantages of additional carriers for higher achievable throughput is limited by the power penalty of CA activation itself.

In such situations, a UE based input to the network will always be advantageous in terms of CA activation/de-activation. The network can make better decisions based on these UE provided inputs.

### Proposals

Fast activation / deactivation could help UE in power savings. To reduce the required delay for activation and deactivation, the first approach we can think is DCI-based signaling. Since DCI is processed in the physical layer, it is faster than current MAC-CE based scheme.

UE-initiated Fast SCell Activation / Deactivation

Another aspect of improvement is UE-initiated activation/deactivation approach. This is quite useful for reducing the delay for activation/deactivation since UE can use information available in application layer such as file size to download or upload.

- SCell activation in file downloading : When a UE wants to download a file, UE can know that it will start downloading it soon from application layer and sends an explicit request to network to turn on additional component carriers in advance. This is really effective in reducing delay since network usually does not have access to this information primarily due to the data transfers being performed within TLS sessions. Network usually turns on additional component carrier only after downlink queue starts to build up beyond a certain threshold, which is a bit slow. From our tests, we observe that activation delay can be reduced from 35ms to 12ms.

- SCell deactivation in file downloading : Current LTE/NR supports sCellDeactivationTimer which is used to deactivate SCells. The timer is reset every time a DL/UL grant is received. It expires when there is no DL/UL grant is received for sCellDeactivationTimer duration. The configurable timer values ranges from 20ms to 1280ms and usually end up with values which are of the range of a long DRX cycle for best effort flows. If this parameter is not configured, then a value of infinity is used and SCell can be deactivated by MACCE only. In the field logs, we indeed see wide ranges of values for this timer from 20ms to infinity. Due to such large de-activation values, even after file downloading is completed, UE has to wait until the timer expires in order for SCells to be de-activated. This problem is exaggerated especially when the downloaded file is smaller in size. For small file downloads, the actual active transfer time of data is much smaller compared to the PDCCH monitoring time during which the SCell is active yielding to highly inefficient RRC connections on the UE that consume significant power. In this scenario, if the UE has a mechanism to indicate to the network a request for SCell deactivation, the UE can then avoid the power penalty of PDCCH monitoring with CA. From our tests, we observe that deactivation delay is reduced on an average from 130ms to 20ms.

- SCell activation in file uploading : When UE starts sending data in UL from RRC IDLE (which is typical case when uploading large file), it first has to connect to network to get to RRC-CONNECTED mode. In this case, MSG3 could include BSR. It is kind of earliest time that a UE in out of sync can tell network that it has a lot of data to transmit to network. So, in this case, there is not much benefit of UE-initiated SCell activation. However, when UE starts transmitting during RRC-CONNECTED mode from PCell only, UE can proactively send SCell activation request before BSR transmission occasion comes. In this case, there is some reduced delay, which is on average the half of BSR reporting period. So, in this case, the reduced delay depends on BSR reporting period.

- SCell deactivation in file uploading : This case is very similar to SCell deactivation in file downloading. Since UE knows whether file uploading has been finished or not, the UE can immediately send SCell deactivation request to network to avoid wasting energy for SCell cells. From our tests, we observe that deactivation delay is reduced from 192ms to 16ms.

- SCell deactivation for handing coexistence issue : When there are a set of component carriers which UE might have coexistence issue with other RATs, UE could indicate this information to network so that network can avoid configuring or activating those carriers in the set. UE’s explicit deactivation request could be effectively used in this case to notify network that the UE want to avoid those carriers.

Observation 4

- UE can significantly reduce SCell activation/deactivation delay using UE-initiated SCell activation/deactivation mechanism based on UE’s knowledge in UE’s application layer (e.g., file size to download/upload). Since such information is available only in the UE, UE-initiated approach is better than network-initiated approach.

Based on above observation, we make following proposal.

Proposal 8

- NR supports UE-initiated SCell activation/deactivation request mechanism.

Job(or File) Size Reporting

Together with UE-initiated SCell activation signal, job size is also useful information helping network to make decision on SCell activation/deactivation. When a user initiates application update(e.g., download) or video uploading to cloud server, the size of job/file to be downloaded/uploaded is known to UE even before actual data transfer starts at physical layer. This job size information can be used in network in determining the number of required DL/UL component carriers to activate/deactivate.

Proposal 9

- NR supports UE’s job size reporting for DL/UL.

General procedure could further improved as follows based on agreed text in TR[7].

Section 5.1.6

The UE assistance information for the power saving schemes for further studies are as follows,

  - UE assistance information/feedback to assist network in configurations for UE adaptation

    - UE preferred processing timeline parameters, e.g., K0, K1, K2 values

    - UE preferred BWP information/configuration

    - UE preferred antenna configuration, including MIMO layers, antenna panel awareness information

    - UE assistance/feedback on the DRX configurations/parameters

    - UE preferred BWP provided to assist network in BWP switching

    - UE request on SCell/SCG activation/de-activation/configuration

      - Before file downloading starts, based on higher layer information (such as file size) UE can determine how many SCells are required. Then, UE can send SCell activation request to gNB (optionally with job size).

      - Once file downloading/uploading is finished or application is terminated by user,  UE can determine that there will be no imminent data transfer. UE can send SCell deactivation request to gNB for quick deactivation.

      - NR supports MACCE based SCell activation/deactivation request.

    - UE preferred PDCCH monitoring parameters/search space configuration/maximum number of blind decoding

# Other UE assistance information for the power saving schemes is not precluded.

## MIMO Layer Restriction

### Motivation

NR supports up to two codewords with variable number of MIMO layers. First four layers are mapped to the first codeword and remaining 4 layers are mapped to the second codeword. Once the number of codeword supported is RRC-configured, then the actual number of MIMO layers for PDSCH transmission is dynamically indicated in DCI.

The number of layers indicated in DCI is up to network implementation, but typically it is determined based on the rank indication reported from UE. However, note that the rank indication from UE does not guarantee that network schedules PDSCH only the layers less or equal to the reported rank. It is possible that network schedules number of layers larger than reported rank, although it is less likely. Due to this reason, UE is supposed to be ready for receiving up to maximum number of layers always which is RRC-configured; 4 for a single codeword and 8 for dual codewords.

At UE side, buffering up to maximum number of layers consumes significant amount power/energy. RFIC includes a dedicated receiver chain for each layer and some blocks in BB such as ADC is also layer specific. Thus, power consumed by those blocks linearly increases as the number of max layer increases.

The problem is that UE has to make maximum number of RF chains ready even when reported rank is lower than the maximum. In addition to that, the RRC-configured maximum number of layers is not adaptable to dynamic traffic arrival, which makes UE waste power in buffering unnecessary signals for unused layers.

For UE power saving, we propose to restrict the maximum number of MIMO layers. Since the number of layers is directly related to the amount of data rate to be supported, the maximum number should be controlled based on traffic arrival. When traffic arrival rate is high, and rank is high, then large maximum number could be configured. When traffic arrival rate is low, the small maximum number of layers (equal or lower than rank) could be configured.

### Proposal

Proposal 10

- NR supports limiting the maximum number of MIMO layers.

- The possible maximum number ranges from 1 to 4 for single codeword and 1 to 8 for dual codeword.

## UE-assisted DRX configuration

### Motivation

DRX mechanisms is one of the key component mechanism for UE power saving, but it is less utilized in LTE. For example, it has been observed that many LTE networks use only limited set of DRX parameters and they are hardly changed. Long inactivity timer value (~200ms) is one of such examples. Another important observation we made is that in many cases the same DRX configuration is applied to all the UEs implying that network does not usually take into account different characteristics of UE’s traffic. The only exception we have observed is VoLTE case. When VoLTE call is going on, network changes DRX parameters to support it in more power efficient manner. Considering that NR DRX design is essentially is not different from that of LTE, we expect that NR network would have similar configurations as LTE network.

However, since the scope of NR use case / scenarios are much wider than that of LTE, we expect much more variation in traffics arrival / demand in NR. For power efficient operation at UE, it is critical to quickly adapt to such variations. The DRX parameters are one of key parameters which enables such adaptation. But, as pointed out above, it is quite static and not fully utilized in most of cases. Thus, we propose to utilize it further for adaptation and to support better, we think UE can provide assistance information to network based on the knowledge inside UE (such as traffic arrival statistics, user behavior statistics, power consumption statistics).

Proposal 11

- NR supports that UE’s indication of its preferred DRX parameters.

### Procedure

The simple procedure we can think is as follow.

- Network configures multiple DRX configurations.

- UE can indicate its preferred DRX configuration index based on application currently running in handset.

- Network determines DRX configuration considering UE’s indication.

It is important to point out here that it is UE’s responsibility to figure out the mapping between traffic pattern of a given application and recommended DRX configuration. The simple approach we can consider is to use delay requirement of the application traffic to determine recommended DRX cycle value. Higher amount of expected traffic could require longer inactivity timer value. DRX On duration timer could be determined within less than 10ms considering scheduler’s flexibility.

General procedure could further improved as follows based on agreed text in TR[7].

Section 5.1.4

  - Dynamic DRX configuration including at least the following

    - UE is configured with multiple DRX configurations

    - Dynamic selection of DRX configuration by gNB from multiple DRX configurations (e.g., traffic, mobility)

      - UE assistance information may be considered ; UE could indicate its preferred DRX configuration (either by index or set of parameters) based on higher layer application performance requirement.

    - Adaptive parameters setting of one DRX configuration

      - UE assistance information may be considered

      - DRX parameters are indicated by gNB

      - Adaptive UE behavior in the DRX operation (e.g, restart the inactivity timer)

Section 5.1.6

The UE assistance information for the power saving schemes for further studies are as follows,

  - UE assistance information/feedback to assist network in configurations for UE adaptation

    - UE preferred processing timeline parameters, e.g., K0, K1, K2 values

    - UE preferred BWP information/configuration

    - UE preferred antenna configuration, including MIMO layers, antenna panel awareness information

    - UE assistance/feedback on the DRX configurations/parameters

      - UE can determine its preferred DRX configuration based on performance requirement (e.g., delay, traffic inter-arrival pattern, throughput) of applications currently running in device.

      - gNB can configure UE with a DRX configuration based on UE’s recommendation.

    - UE preferred BWP provided to assist network in BWP switching

    - UE request on SCell/SCG activation/de-activation/configuration

    - UE preferred PDCCH monitoring parameters/search space configuration/maximum number of blind decoding

# Other UE assistance information for the power saving schemes is not precluded.

## Explicit Signal based Sleep Indication

### Motivation

When a UE in RRC CONNECTED mode is scheduled by gNB to receive DL/UL data, the grant transmission time to the UE of interest could be random. This is because that traffic arrival for the UE from application server is random and the gNB should serve multiple UEs in the cell. Figure 8 shows an exemplary CDF of grant inter arrival time of video and web-browsing in LTE. It is observed the mean grant-inter arrival time is 5ms for video traffic and 6.2ms for web-browsing. Note that the tail of distribution reaches to ~ 20ms for video and ~40ms for web-browsing. This indicates that there could be time gap of order of ms to tens of ms during which UE can enter sleep state for power saving. The further analysis could found in [4].

Figure 8 CDF of grant inter-arrival time for video (left) and web-browsing (right)

### Proposals

To take advantage of such time gap between grants, we could think of explicit sleep indication based UE power saving mechanism as shown in Figure 9. Due to the dynamic change of traffic arrivals, configuring static PDCCH monitoring period or CDRX parameters may not be enough to effectively handle dynamic data transmission / reception in power efficient manner. With explicit signaling approach, network can dynamically handle such situation while maximizing power saving gain.

Figure 9 Expected UE power consumption with explicit signal based sleep indication

Observation 5

- The explicit signal-based sleep mechanism is effective in adapting to dynamic traffic arrivals for UE power saving.

Proposal 12

- NR supports explicit-signal based sleep indication for UE power saving.

### Procedure

In this section, we discuss the channel structure, signal content, and UE behavior aspect for supporting network-initiated sleep indication.

Channel/Signal Structure

There could be multiple options for sleep indication signaling.

- O1) New signal/channel

- O2) Existing channel

There could be some benefit of having new signal or channel such as low power operation. However, considering that sleep indication is most useful when there are dynamic arrival of traffic (when active data transfer happening) or CDRX ON duration with potentially with long inactivity timer values, it is better to use existing channel such as PDCCH because UE is already in the state where it can monitor PDCCH. So, having new signal/channel could just increase complexity without much gain.

Proposal 13

- NR supports sleep indication using existing channel structure (e.g., PDCCH)

Signal Content

The most simplest approach would be to include a single bit indicating sleep indication for a certain time interval. Multiple bits could be signaled to signal more information. However, this would require more complex processing / operation at network side. For example, if length of sleep is indicated, then it network need to more accurately predict sleep duration until next grant arrival. Thus as a first step toward dynamic sleep operation, we think including single bit indicator should be good enough, which can also minimize any necessary DCI size change and its negative impact on coverage. The sleep duration could be configured by RRC or MACCE.

Proposal 14

- NR supports single bit sleep indication to indicate sleep for a specified time duration.

- The sleep duration is configured by RRC or MACCE.

UE behavior

Currently there is no clear definition of “sleep”. One possible definition is skipping of configured PDCCH monitoring. In this case, UE is still supposed to handle RRC configured activities such CSI-RS monitoring, SRS transmission, CSI reporting, etc during sleep duration. This approach leaves power saving as an UE implementation while minimizing the impact on network operation.

Another possible definition is literal sleep allowing UE to ignore RRC configured activities. This approach allows UE to enter sleep with higher chance but requires additional handling of RRC configured signals. For example, UE should take into account missing samples of RSRP (if any) in its filtering operation. Skipping CSI-RS or SSB could have negative impact on beam management process, which could potentially delay/slow beam failure recovery as well. Network should also take into account the missing SRS transmission (if any) in its UL channel sounding procedure. The missing CSI reporting, if any, could be also taken into account for network for DL channel status tracking. At this point in time, we see that many of such issues arising by ignoring RRC scheduled activities. For these reason, it may be good choice to define sleep as PDCCH monitoring skipping.

Proposal 15

- Sleep signal from network indicates that UE is not supposed to monitor PDCCHs during sleep duration.

General procedure could further improved as follows based on agreed text in TR[7].

Section 5.1.4

# For power saving scheme with UE adaptation to the DRX operation for further study

  - UE adaptation of its behavior to the DRX operation for UE power consumption reduction

    - When is configured with power saving signal/channel, power saving signal/channel as the indication whether to wakeup or not before or at the beginning of DRX ON duration

      - At least for the indication of PDCCH monitoring

    - Preparation period is used for (e.g., to perform channel tracking, CSI measurements, beam tracking).

      - Preparation period can be used in preparation for the PDCCH decoding

      - Preparation period could be before or during the DRX ON duration

      - Network can indicate UE to report CSI before or after the power saving signal/channel (if configured) during the preparation period

      - Network can indicate additional RS transmission (e.g., CSI-RS, TRS, SSB and power saving signal) at the preparation period

    - Go-to-sleep signaling as the indication allowing UE going back to sleep state

      - MAC-CE / DCI / Power saving signal/channel

      - Sleep duration could be preconfigured and/or indicated by gNB.

    - Constraints on scheduling DCI during DRX_ON

Section 5.1.5

The power saving schemes to reduce PDCCH monitoring and blind decoding for further studies are as follows,

  - Triggering of PDCCH monitoring – dynamic trigger through L1 signal/signaling

    - Power saving signal triggering PDCCH monitoring

    - Go-to-sleep signaling to skip PDCCH monitoring

  - PDCCH skipping -

    - DCI based indication  for PDCCH skipping (e.g., indication in DCI content, new SFI state).

      - gNB can indicate the length of PDCCH skipping duration.

      - NR support Go-to-sleep signal field in scheduling DCI and new DCI.

    - L1 signal/signaling (other than DCI) based triggering  -

  - Multiple CORESET/search space configurations

    - Configuration of different PDCCH periodicities with dynamic signaling

    - Adaptation of CORESET/search space configuration – DCI/timer/HARQ-ACK based indication

    - Dynamic/semi-persistent CORESET/search space ON/OFF

    - Adaptation between DRX ONduration timer and inactivitytimer

    - Separated PDCCH monitoring of DL and UL

  - L1 signaling triggering to assist  UE in reducing the number of PDCCH blind decoding

  - Reduced PDCCH monitoring on SCell (including cross carrier scheduling)

  - Network assistance –  RS is dynamically transmitted based on the need to assist UE performing synchronization, channel tracking, measurements and  channel estimations before PDCCH decoding

## Other issues

### BWP-DRX association

In above discussion, DRX cover both Connected mode DRX (CDRX) and Idle mode DRX. But CDRX is much more relevant in the context of adaptation to dynamically varying traffic.

Considering the most common use case of BWP is power saving, we see the connection between BWP and DRX. Given that both features can be configured for quick adaptation to traffic arrival while trying to reduce power consumption, we can consider the joint operation of the two features. For example, NR could support the per BWP DRX configuration. For example, a narrow bandwidth part BWP1 and wide bandwidth part BWP2 could be configured with different values for short/long DRX cycle timer, on duration timer, inactivity timer, etc. The values for BWP1 could be configured as different from than that for BWP2 to further reduce power consumption when there is not much traffic coming in BWP1. This kind of association between BWP and DRX parameters could provide increased flexibility enabling better adaptation.

Proposal 16

- NR study the association between a BWP and DRX operation.

# Conclusions

In this contribution, we have identified a few important UE-assisted power saving mechanisms in the area of bandwidth part, UE-preferred Timeline Adaptation for Power Saving, cross-slot scheduling, UE-initiated SCell activation/ deactivation, MIMO layer restriction and BWP-DRX parameter association. We made following observations and proposals based on the discussion.

Observation 1

- UE having better knowledge on channel status and traffic demand/pattern/usage could help network to make better decisions for BWP configuration.

Proposal 1

- NR supports that UE can send UE-preferred BWP configuration to network.

  - UE can indicate it’s preferred BWP configuration including a set of BWP sizes and starting RB indices, and BWP inactivity timer value.

  - NR supports that network can trigger UE to send UE-preferred BWP configuration by sending triggering message to UE potentially including side information to help UE to make better estimate of its UE-preferred BWP.

Proposal 2

- NR supports UE sends a BWP change request to network to initiate the change of active DL/UL BWP.

Proposal 3

- NR supports the UE’s indication of preferred set of K0, K1, K2 values to network for UE power saving.

Observation 2

- UE buffers potential PDSCH unnecessarily when potential PDSCH transmission can occur before corresponding PDCCH decoding finishes.

Observation 3

- Controlling K0 only does not always ensure UE to avoid unnecessary PDSCH buffering.

Proposal 4

- NR supports a MAC CE which can activate/deactivate a subset of TDRA entries. A scheduling DCI can indicate one of activated entries only.

Proposal 5

- NR supports that UE report its preferred TDRA entries back to gNB among the entries in the RRC-configured TDRA table.

Proposal 6

- NR supports that UE sends a TDRA entry change request.

Proposal 7

- NR supports the scheduling DCI-based carrier activation mechanism.

Observation 4

- UE can significantly reduce SCell activation/deactivation delay using UE-initiated SCell activation/deactivation mechanism based on UE’s knowledge in UE’s application layer (e.g., file size to download/upload). Since such information is available only in the UE, UE-initiated approach is better than network-initiated approach.

Proposal 8

- NR supports UE-initiated SCell activation/deactivation request mechanism.

Proposal 9

- NR supports UE’s job size reporting for DL/UL.

Proposal 10

- NR supports limiting the maximum number of MIMO layers.

- The possible maximum number ranges from 1 to 4 for single codeword and 1 to 8 for dual codeword.

Proposal 11

- NR supports that UE’s indication of its preferred DRX parameters.

Observation 5

- The explicit signal-based sleep mechanism is effective in adapting to dynamic traffic arrivals for UE power saving.

Proposal 12

- NR supports explicit-signal based sleep indication for UE power saving.

Proposal 13

- NR supports sleep indication using existing channel structure (e.g., PDCCH)

Proposal 14

- NR supports single bit sleep indication to indicate sleep for a specified time duration.

- The sleep duration is configured by RRC or MACCE.

Proposal 15

- Sleep signal from network indicates that UE is not supposed to monitor PDCCHs during sleep duration.

Proposal 16

- NR study the association between a BWP and DRX operation.

# References

- R1-166368 UE Power Consideration based on Days-of-Use, Qualcomm, Aug, 2016

- R1-1609557 On Techniques for Enhanced UE Power Efficiency, MediaTek, Oct, 2016

- RP-172318 Motivation for Study on UE Power Saving in NR, VIVO, Dec, 2017

- R1- 1714092 Control Information for UE Power Saving, Apple

- R1-xxxxxx Chairman’s Note RAN1 94b, Cheng-du, Oct, 2018

- R1- 1902770 Evaluation of UE power saving techniques, Apple, Feb, 2019

- TR38.840 Study on UE Power Saving v0.1.1, Feb, 2019