3GPP TSG RAN WG1 Meeting #96	R1-1903085

Athens, Greece, 25th February – 1st March, 2019

Agenda Item:	7.2.7.2

Source:	Huawei, HiSilicon

Title:	Considerations and evaluation results for IMT-2020 for eMBB spectral efficiency in NR

Document for:	Discussion and Decision

Introduction

According to 3GPP work plan on IMT-2020 submission, the final submission will be made in June 2019. More test environments or configurations for NR can be evaluated for the final submission. In this contribution, the evaluation results for eMBB spectral efficiency (average and the 5th percentile user spectral efficiency) for NR in Indoor Hotspot - eMBB test environment with configuration C are provided.

Evaluated technical features

The IMT-2020 eMBB spectral efficiency requirement is three times higher compared to IMT-Advanced. Therefore, it is a challenging requirement and the NR features that could contribute to the improvement of spectral efficiency should be selected.

On the other hand, it is recalled that for LTE-Advanced evaluation towards IMT-Advanced submission, the evaluation is basically applied based on duplexing schemes, i.e., to FDD and TDD, respectively. This is due to the fact that, duplexing scheme is one of the fundamental features among the other features that impact spectral efficiency performance.

Duplexing scheme

In NR design, the flexible duplexing scheme is available, e.g.,

- Different transmission directions in either part of a paired spectrum,

- TDD operation on an unpaired spectrum where the transmission direction of most time resources can be dynamically changing.

In this document, the following duplexing schemes are considered

- FDD (for paired spectrum): NR features that can be applied to FDD operation are invoked.

- TDD (for unpaired spectrum): NR features that can be applied to TDD operation are invoked.

Spectral efficiency calculation for duplexing scheme

The spectral efficiency of different duplexing schemes can be calculated according to Report ITU-R M.2412.

For DL average spectral efficiency and 5th percentile spectral efficiency,

- In case of FDD, the simulation bandwidth is 10 MHz for DL and 10 MHz for UL. The DL average spectral efficiency is given by

(1)

where W is the DL bandwidth of 10 MHz; Ri (T) denotes the number of correctly received bits of user i, and the overhead of DL control and DL reference signals on the DL bandwidth of 10 MHz is taken into account when deriving Ri (T); and T is the simulation time. Similar notations are applied to 5th percentile user spectral efficiency.

- For TDD in FR1, the simulation bandwidth is 20 MHz for DL and UL. For TDD in FR2, the simulation bandwidth can be 80 MHz with 60 kHz subcarrier spacing for DL and UL. The DL average spectral efficiency is given by Eq. (1), where W is the effective DL bandwidth that accounts for the time-frequency resource used for DL transmission (including GP symbols); Ri (T) denotes the number of correctly received bits of user i, and the overhead of DL control, DL reference signal on the DL effective bandwidth is taken into account; and T is the simulation time. Similar notations are applied to 5th percentile user spectral efficiency.

For UL average spectral efficiency and 5th percentile spectral efficiency, similar way is employed to derive the evaluation results for these two metrics.

Spectral efficiency calculation for different bandwidth

To reflect the benefit of reduced guard band ratio and overhead for larger bandwidth in NR, i.e. when the system bandwidth is larger than simulation bandwidth (10 MHz in FDD, 20 MHz in TDD in FR1 and 80 MHz in TDD in FR2), the spectral efficiency can be derived from Eq. (2)

(2)

where  and is the guard band ratio and the overhead at given number of RB N, respectively, and  is calculated by Eq. (1). For FDD,  for 10 MHz simulation bandwidth and 15 kHz subcarrier spacing. For TDD,  for 20 MHz simulation bandwidth and 30 kHz subcarrier spacing.

The overhead reduction for the larger bandwidth mainly comes from the PDCCH. In addition, SSB and TRS overhead will be reduced slightly. By assuming M0 OFDM symbols for PDCCH at the bandwidth BW0, the number of OFDM symbol for PDCCH at bandwidth BW could be

M= BW0/ BW×M0                                                                                                  (3)

For example, if we assume M0 = 2 for 20 MHz bandwidth system, then M=1 for 40 MHz bandwidth system. The value of M could be a non-integer since NR supports PDCCH sharing with PDSCH.

For the evaluation results in Section 3, the guard band ratio and PDCCH overhead reduction model for larger bandwidth based on Eq. (2) and based on Eq. (3) is considered in DL.

Evaluation results

Based on the above discussion, evaluation results for average spectral efficiency and 5th percentile spectral efficiency is evaluated in Indoor Hotspot - eMBB test environment with configuration C. The detailed evaluation assumptions can be found in Appendix.

NR spectral efficiency evaluation results

The detailed evaluation assumptions for NR DL and UL spectral efficiency evaluation can be found in Appendix A.

DL spectral efficiency evaluation results

The DL spectral efficiency evaluation results for NR are given in Table 1.

TABLE 1 TDD DL spectral efficiency evaluation for FR2

| Test env. | Evaluation config. | Average spectral efficiency (bit/s/Hz/TRxP) |  |  | 5th percentile spectral efficiency (bit/s/Hz) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BW=80 MHz | BW=200 MHz | Req. | BW=80 MHz | BW=200 MHz | Req. |
| Indoor Hotspot | Config. C (70 GHz); 32T8R | 9.611 | 11.062 | 9.0 | 0.509 | 0.586 | 0.3 |

Observation 1: NR with above 6 GHz can fulfill DL spectral efficiency requirements in Indoor Hotspot – eMBB test environment with configuration C.

UL spectral efficiency evaluation results

The UL spectral efficiency evaluation results for NR are given in Table 2.

TABLE 2 TDD UL spectral efficiency evaluation for FR2

| Test env. | Evaluation config. | Average spectral efficiency (bit/s/Hz/TRxP) |  | 5th percentile spectral efficiency (bit/s/Hz) |  |
| --- | --- | --- | --- | --- | --- |
|  |  | BW=80 MHz | Req. | BW=80 MHz | Req. |
| Indoor Hotspot | Config. C (70 GHz); 8T32R | 7.368 | 6.75 | 0.366 | 0.21 |

Observation 2: NR with above 6 GHz can fulfill UL spectral efficiency requirements in Indoor Hotspot – eMBB test environment with configuration C using 8TXU UEs.

Conclusion

In this document, the evaluation of eMBB spectral efficiency for NR operating on frequency range 2 (FR2, i.e., above 6 GHz) is provided. Based on the evaluation, the following observations are made.

Observation 1: NR with above 6 GHz can fulfill DL spectral efficiency requirements in Indoor Hotspot – eMBB test environment with configuration C.

Observation 2: NR with above 6 GHz can fulfill UL spectral efficiency requirements in Indoor Hotspot – eMBB test environment with configuration C using 8TXU UEs.

- Reference

- R1-1803386, “Summary of offline discussion for IMT-2020 self evaluation”, Huawei, February 2018.

- Appendix A: Evaluation assumptions for NR

Table A-1 Evaluation assumptions for NR DL

| Parameter |  | Value |
| --- | --- | --- |
| Test environment |  | Indoor Hotspot – eMBB |
| Evaluation configuration |  | Configuration C |
| Channel model |  | InH_B |
| ISD |  | 20 m (12TRxP) |
| TDD frame structure |  | DDDSU, S(10DL:2GP:2UL) |
| Carrier Frequency |  | 70 GHz |
| System bandwidth |  | 80MHz |
| Subcarrier spacing |  | 60kHz |
| Symbols number per slot |  | 14 |
| Number of antenna elements per TRxP |  | 256Tx cross-polarized antennas (M,N,P,Mg,Ng) = (8,16,2,1,1); |
| Number of TXRU per TRxP |  | 32TXRU: Vertical 1-to-2 |
| Number of antenna elements per UE |  | 8Rx with 0°,90° polarization (M,N,P,Mg,Ng; Mp,Np) = (2,4,2,1,2; 1,2) |
| Transmit power per TRxP |  | 23 dBm |
| TRxP number per site |  | 1 |
| Mechanic tilt |  | 180deg in GCS (pointing to the ground) |
| Electronic tilt |  | According to Zenith angle in "Beam set at TRxP" |
| Beam set at TRxP |  | Azimuth angle φi = [-pi/4,pi/4], Zenith angle θj = [pi/4, 3*pi/4] |
| Beam set at UE |  | Azimuth angle φi = [-pi/4, pi/4]; Zenith angle θj = [pi/4, 3*pi/4] |
| UT attachment |  | Based on RSRP (Eq. (8.1-1) in TR 36.873) from port 0 |
| Scheduling |  | MU-PF |
| ACK/NACK delay |  | Next available UL slot |
| MIMO mode |  | MU-MIMO with rank 2/4 adaptation per user; Maximum MU layer = 6 |
| Guard band ratio |  | 5.5% (for 80 MHz); |
| BS receiver type |  | MMSE-IRC |
| CSI feedback |  | 5 slots period based on CSI-RS with delay |
| Precoder derivation |  | TDD: SRS based |
| Overhead | PDCCH | 2 complete symbols |
|  | DMRS | Type II, based on MU-layer (dynamic in simulation) |
|  | CSI-RS | FDD: 32 ports per 5 slots |
|  |  | TDD: 4 ports per UE per 5 slots |
|  | CSI-RS for IM | ZP CSI-RS with 5 slots period; 4 RE/PRB/5 slots |
|  | SSB | 12 SSB per 20 ms |
|  | TRS | 2 consecutive slots per 20 ms, 1 port, maximal 52 PRBs |
|  | PTRS | 2 ports PT-RS, (L,K) = (1,4) L is time density and K is frequency density |
|  | GP | 50% length of GP symbols |
| Channel estimation |  | Non-ideal |
| Waveform |  | OFDM |

Table A-2 Evaluation assumptions for NR UL

| Parameter |  | Value |
| --- | --- | --- |
| Test environment |  | Indoor Hotspot – eMBB |
| Evaluation configuration |  | Configuration C |
| Channel model |  | InH_B |
| Subcarrier spacing |  | 60kHz |
| TDD frame structure |  | DDDSU, S(10DL:2GP:2UL) |
| Symbols number per slot |  | 14 |
| Number of antenna elements per TRxP |  | 256Rx cross-polarized antenna (M,N,P,Mg,Ng) = (8,16,2,1,1); |
| Number of TXRU per TRxP |  | Configuration B: 32TXRU Vertical 1-to-2, Horizontal 1-to-2 |
| Number of antenna elements per UE |  | 8Tx with 0°,90° polarization (M,N,P,Mg,Ng; Mp,Np) = (2,4,2,1,2; 1,2) |
| UE power class |  | 23 dBm |
| Mechanic tilt |  | 180deg in GCS (pointing to the ground) |
| Electronic tilt |  | According to Zenith angle in "Beam set at TRxP" |
| Beam set at TRxP |  | Azimuth angle φi = [-pi/4,pi/4], Zenith angle θj = [pi/4, 3*pi/4] |
| Beam set at UE |  | Azimuth angle φi = [-pi/4, pi/4]; Zenith angle θj = [pi/4, 3*pi/4] |
| UT attachment |  | Based on RSRP (Eq. (8.1-1) in TR36.873) from port 0 |
| Scheduling |  | SU-PF |
| MIMO mode |  | SU-MIMO with rank 4 adaptation; |
| BS receiver type |  | MMSE-IRC |
| UE precoder scheme |  | Codebook based |
| UL CSI derivation |  | Non-precoded SRS based, with delay |
| Power control |  | α=0.9， P0=-86 dBm |
| Power backoff model |  | Continuous RB allocation: follow TS 38.101 in Section 6.2.2; Non-continuous RB allocation: additional 2 dB reduction |
| Overhead | PUCCH | 3 RBs, 14 OFDM symbols |
|  | DMRS | Type II, 2 symbols (including one additional DMRS symbol), multiplexing with PUSCH |
|  | SRS | 2 symbols per 5 slots, 8 RBs per symbol |
|  | PTRS | 2 ports PT-RS, (L,K) = (1,4) L is time density and K is frequency density |
|  | GP | 50% length of GP symbols |
| Channel estimation |  | Non-ideal |
| Waveform |  | OFDM |