3GPP TSG RAN WG1 Meeting #96 	R1-1903097

Athens, Greece, 25th February – 1st March, 2019

Agenda Item:	7.2.8.6

Source:	Huawei, HiSilicon

Title:	Phase Randomization and Correction for CSI quantization in frequency domain

Document for:	Discussion and Decision

- Introduction

It has been agreed in previous meetings [1-2] that:

Agreement

For Rel-16 NR, agree on Alt1 (DFT-based compression) in Table 1 of R1-1813002 as the adopted Type II rank 1-2 overhead reduction (compression) scheme as formulated in Alt1.1 of R1-1813002

- Note: The same DFT-based compression scheme is extended for Type II port selection codebook

- Codebook subset restriction (CBSR) is supported when DFT-based compression is utilized for Type II codebooks with overhead reduction (compression) scheme

  - FFS: detailed signaling mechanism

- Note: Additional compression scheme(s) are not precluded

Working Assumption

On the choice of oversampling factor O3, O3 = 4 is supported.

In this contribution, a problem of phase randomization and correction for DFT-based compression codebook is discussed which is for UE implementation and affects the performance if this point is ignored. In addition, the impact of oversampling factor O3 on the system performance is also discussed.

- Space-frequency compression codebook

The design of type II codebook with DFT-based compression is introduced in [3-4]. In this design, the space-frequency matrix is compressed in both spatial domain and frequency domain, by a set of spatial basis vectors and a set of frequency basis vectors.

The space-frequency matrix W can be represented and approximated by the following formulation

W=W1W2WfH

where W1 and Wf are composed of selected basis vectors from the spatial codebook and frequency codebook, respectively. The dimension of the coefficients matrix W2 is 2L×M, with L and M as the number of selected spatial and frequency basis vectors, respectively. The UE only needs to feedback the indices of selected spatial and frequency basis vectors, i.e. W1 and Wf, as well as the combination coefficients W2.

- Oversampling on frequency domain

The oversampled Wf N3×M can be expressed as the multiply of rotation matrix R and non-oversampled Wf,o:

Wf=R* Wf,o

where:

R=diagej2πq3OfN30ej2πq3OfN31⋯ej2πq3OfN3N3-1 q3=1~Of

Wf,o=f1f1⋯fM

fm=ej2πpmN30ej2πpmN31⋯ej2πpmN3N3-1T pm=1~N3,m=1~M

With oversampled frequency beam, the coefficient matrix W2 generally can be obtained by:

W2=W1HHWf=W1HHR Wf,o

where H is ideal space-frequency matrix. It can be found that the matrix of R would have an impact on determining feedback coefficients of  Wf,o and other coefficients related to it.

The space-frequency matrix gNB reconstruct by the PMI components that UE has reported:

W=W1W2WfH=W1W1HHR Wf,oWfH=W1W1HHR Wf,oWf,oHRH

From the equation above, we can find that oversampling factor has an impact over reconstructed channel with multiple candidate values of q3 in the matrix R at the middle side of the equation. Choosing an appropriate oversampling factor, system performance can be further improved. For the matrix RH at right side of the equation due to the reporting of q3, only a phase rotation per column within W is applied and it seems have no impact on the precoder for each PMI subband. However, phase difference between precoders of two adjacent PMI quantization units, i.e. with R=2, would lead to different CQI value for a CQI subband. Then different phase rotation per column within W would lead to different value of CQI and q3 needs to be reported accordingly.

- Phase correction for eigenvectors

In the design of space-frequency compression codebook, the space-frequency matrix W is obtained by concatenating the precoding vectors of different subbands. Then the space-frequency matrix is compressed in both spatial domain and frequency domain. The effect of frequency domain compression highly depends on the smoothness of the coefficients along subbands.

In one implementation, the leading eigenvector is utilized as the precoding vector for each subband. Note that if a vector v is the eigenvector of a matrix A, ejθ*v is also its eigenvector. We should be cautious that eigenvectors, as the output of a SVD solver, may have a “random” phase rotation. Whether such phase rotation exists may depend on the numerical algorithm of the SVD solver. To achieve more efficient frequency domain compression, random phase rotation should be corrected.

To implement phase correction, the eigenvector for each subband should be multiplied by a proper phase. Then the space-frequency matrix is obtained with rotated eigenvectors. It should be noted that phase correction between adjacent subbands will provide more efficient frequency domain compression. A better phase correction will lead to the result that a larger proportion of power concentrates within the selected frequency components and then a smaller M is enough in the frequency domain compression. A potential phase correction implementation is to compensate the phase of eigenvector corresponding to the (i+1)-th subband (ei+1) with the phase of the inner product of the i-th and (i+1)-th eigenvectors (phase of ei+1H*ei), i.e., ei+1=phaseei+1H*ei*ei+1.

This procedure has no impact over codebook structure and associated PMI quantization. Whilst PMI quantization is purely a UE implementation, a proper phase correction will ensure PMI coefficient compression in the frequency domain more efficiently.

  - CDF curves for power ratio

We define power ratio as the ratio of preserved power after spatial-frequency compression and the total power before the compression. The power ratio reflects the level that the power of coefficients concentrates on the selected spatial and frequency basis vectors.

Fig.1 illustrates the CDF curves of power ratio with and without phase correction. With the help of a proper phase correction, the power concentrates on the selected frequency basis more, which leads to a more efficient frequency domain compression.

Observation 1: Phase correction by a proper UE implementation should be considered to avoid random phase rotation caused by a SVD solver in order to achieve efficient frequency domain compression.

Figure 1. CDF curves for power ratio with and without phase correction.

  - Insight on the phase correction

From the analysis in section 3, the reconstructed space-frequency matrix at gNB side can be expressed as:

W=W1W2WfH=W1W1HHR Wf,oWfH=W1W1HHR Wf,oWf,oHRH

The phase correction essentially is multiplying a phase correction matrix to the ideal space-frequency matrix H:

H'=H∆

where

∆=diagφ1φ2…φN3

and φr is the phase correction multiplied to the r-th subband. With phase correction, the reconstructed space-frequency matrix at gNB side can be expressed as:

W=W1W2WfH=W1W1HH'R Wf,oWf,oHRH=W1W1HH∆R Wf,oWf,oHRH

It can be found in the equation above, phase correction can seriously impact on the reconstructed space-frequency matrix due to the matrix ∆. Essentially, the aim of phase correction is to strive to reduce reported coefficients of space-frequency matrix so that PMI quantization can be concentrated or constrained to the selected W1 and  Wf.

- Simulation results

System-level simulation results for the proposed space-frequency compression codebook with or without phase correction are included in this section.

The spatial basis reuses the oversampled 2D-DFT beam, whilst the frequency basis uses the oversampled DFT beam with oversampling factor of 4. Each coefficient within matrix W2 is quantized with A bits for amplitude and P bits for phase. Detailed parameters are shown in Appendix I.

The trade-off between performance and overhead with and without phase rotation is shown in Fig. 2, wherein oversampling factor of 4 is used. If phase rotation is not conducted, the performance of frequency domain compression is even worse than Type II codebook. The phase rotation procedure will significantly improve the performance and compress the coefficients in the frequency domain more efficiently. The trade-off between performance and overhead with and without oversampling is shown in Fig. 3, wherein phase correction is conducted. The oversampling on frequency beam can bring a considerable gain compared with non-oversampling.

Figure 2. The trade-off between performance and overhead with/without phase rotation for (L, M) = (6, 4).

Figure 3. The trade-off between performance and overhead with/without oversampling for (L, M) = (6, 4).

Observation 2: Without proper phase correction, frequency domain compression shows worse performance than type II codebook assuming the same feedback overhead.

Observation 3: Oversampling on frequency beam can bring a considerable gain compared with non-oversampling.

- Conclusions

The contribution discusses the codebook design or enhancement for Rel-16, based on which the following observation and proposal are made.

Observation 1: Phase correction by a proper UE implementation should be considered to avoid random phase rotation caused by a SVD solver in order to achieve efficient frequency domain compression.

Observation 2: Without proper phase correction, frequency domain compression shows worse performance than type II codebook assuming the same feedback overhead.

Observation 3: Oversampling on frequency beam can bring a considerable gain compared with non-oversampling.

- References

- “RAN1 Chairman’s Notes”, Spokane, USA, November 12-16, 2018.

- “RAN1 Chairman’s Notes”, Taipei, January 21-25, 2018.

- R1-1812242, Huawei, HiSilicon, “Discussion on CSI enhancement”, Spokane, USA, November 12-16, 2018.

- R1-1813002, Samsung, “Summary of CSI enhancement for MU-MIMO support”, Spokane, USA, November 12-16, 2018.

- Appendix I

| Parameters | Dense Urban (Macro layer only) |
| --- | --- |
| Duplex mode | FDD |
| Carrier frequency | 4GHz |
| Subcarrier spacing | 15kHz |
| System bandwidth | 10MHz (13 subbands, 4 PRB for each subband) |
| Layout | Hexagonal grid, 3 sectors per site, 19 macro sites |
| Channel model | SCM-3D-UMa |
| Inter-BS distance | 200m |
| Minimum distance | 35m |
| BS antenna height | 25m |
| BS Tx power | 41dBm |
| BS antenna configuration | (M, N, P, Mg, Ng, Mp, Np) = (8,8,2,1,1,2,8), (dH, dV) = (0.5, 0.8) λ |
| UE antenna configuration | (M, N, P, Mg, Ng, Mp, Np) = (1,1,2,1,1,1,1) for overhead reduction; (M, N, P, Mg, Ng, Mp, Np) = (1,2,2,1,1,1,2) for higher rank of Type II; the polarization angles are 0 and 90 |
| UE distribution | 80% indoor, 3km/h; 20% outdoor, 30km/h |
| UE receiver | MMSE-IRC |
| MIMO scheme | SU/MU-MIMO switch for overhead reduction; SU-MIMO for higher rank of Type II |
| Scheduler | PF |
| Traffic model | FTP model 1 with packet size 0.5 Mbytes |
| Feedback assumption | Realistic |
| Channel estimation | Realistic |