3GPP TSG RAN WG1 Meeting #96	                      R1-1903196

Athens, Greece, February 25 – March 1, 2019

Agenda Item:	7.2.14

Source:	Huawei, HiSilicon

Title:	Discussion on data transmission related procedures for NTN

Document for:	Discussion and Decision

Introduction

RAN#80 approved a new SI on solutions evaluation for NR to support Non-Terrestrial Network [1]. The objectives of the SI for physical-layer and high-layer are reported as follows respectively.

Physical layer

Consolidation of potential impacts as initially identified in TR 38.811 and identification of related solutions if needed  [RAN1]:

- Physical layer control procedures (e.g. CSI feedback, power control)

- Uplink Timing advance/RACH procedure including PRACH sequence/format/message

- Making retransmission mechanisms at the physical layer more delay-tolerant as appropriate. This may also include capability to deactivate the HARQ mechanisms.

Layer 2 and above, and RAN architecture

- Study the following aspects and identify related solutions if needed: Propagation delay: Identify timing requirements and solutions on layer 2 aspects, MAC, RLC, RRC, to support non-terrestrial network propagation delays considering FDD and TDD duplexing mode. This includes radio link management. [RAN2]

- Handover: Study and identify mobility requirements and necessary measurements that may be needed for handovers between some non-terrestrial space-borne vehicles (such as Non Geo stationary satellites) that move at much higher speed but over predictable paths [RAN2, RAN1]

- Architecture: Identify needs for the 5G’s Radio Access Network architecture to support non-terrestrial networks (e.g. handling of network identities) [RAN3]

  - Paging: procedure adaptations in case of moving satellite foot prints or cells

In this document, we provide some initial considerations on data transmission related procedures for NTN.

Discussion

The long round-trip delay (RTD) and fast moving cell are of the main physical phenomena in a satellite communication system that makes the design different from that of a terrestrial mobile system. As shown in TR 38.821 [2], NTN should deal with maximum RTD (propagation delay only) up to 541ms, and the relative motion between UE and NTN platform can exceed 1000km/h. Considering these two key features in NTN communication, several data transmission related procedures shall be re-evaluated, including adaptive modulation and coding, UL power control, HARQ procedure and transmission resource scheduling.

Adaptive modulation and coding

Adaptive Modulation and Coding (AMC) is one of the most key techniques to cope with the dynamic fading of wireless channels. In 5G network, gNB determines the modulation scheme and coding rate according to the channel quality indicator (CQI) fed back by the User Equipment (UE) in the uplink, maximizing the throughput while maintaining the target BLER performance of the transmission.

However, due to the characteristic of long round trip delay, the channel state feedback in NTN communication cannot be obtained in time, which limits the performance of the AMC to some extent. To overcome this issue, several strategies for determining the AMC of the present moment and the following adjustment can be considered. A study shows that in certain scenario, the CQI data with larger scale fading does not differ very much with fast variation CQI data [3], so it’s worth considering scheduling AMC based on large scale CQI data without much performance loss. Besides, prediction-based AMC scheduling shall also be evaluated. For reference, many existing prediction methods with different prediction ability have been introduced in terrestrial system [4]. To guarantee AMC performance in NTN, proper model for prediction and method for generating approximate data to do prediction shall be re-considered.

Another issue for AMC scheme is the adjustment period. Longer AMC adjustment period will deteriorate the system performance, but the benefits gained by shorter adjustment period ask for higher requirement of the feedback link. Therefore, the adjustment period requires compromise in order to achieve the optimal system performance.

Observation 1: Refined AMC scheme is necessary for NTN communication.

UL power control

To resist large scale shadowing and long distance path loss in NTN communication, a power control scheme is essential. With a properly designed scheme implemented on the uplink, all user’s signal arrive at the NTN platform with equal average power as they move within the NTN cell, and co-channel interference can be minimized.

Generally, uplink power control can be implemented in several forms, including:

- Closed loop;

- Open loop;

For the closed-loop scheme, UE receives the feedback signal from NTN and modifies the transmitted power to compensate the path loss for uplink. In the terrestrial communication, closed-loop scheme is feasible under most conditions. However, because of the long RTD, accurate closed-loop control for NTN communication is of great challenge and may be only of margin benefit.

An alternate form of power control is an open-loop scheme, it can be deployed without any system-wide consideration and additional resource, but the power control accuracy is a major concern. New design compromising the merits of closed loop power control and open loop power control shall be evaluated.

Observation 2: Uplink power control in NTN communication shall be further studied.

HARQ procedure

HARQ procedure is a combination of high-rate forward error-correcting coding and ARQ error- control. With HARQ procedure, the whole communication quality become more reliable and efficient. In NTN communication, the HARQ procedure shall be re-designed to accommodate the challenge of long RTD.

Many considerations and challenges of HARQ design are mentioned in some companies’ previous proposals [5] [6]. Simply increasing the number of HARQ processes linearly maybe not feasible due to the restriction of UE’s memory and limited processing capabilities, new HARQ scheme including bundling and repetition shall be evaluated. Besides, disabling HARQ has also been considered as an option, consequently, detailed disabling HARQ schemes and potential challenges brought to upper layer protocol need to be studied [7].

Observation 3: HARQ procedures should be further studied based on NR in NTN.

Conclusions

In summary, we discuss some key considerations on data transmission related procedures for NTN communication and have following observations:

Observation 1: Refined AMC scheme is necessary for NTN communication.

Observation 2: Uplink power control in NTN communication shall be further studied.

Observation 3: HARQ procedures should be further studied based on NR in NTN.

- References

RP-181370, Study on solutions evaluation for NR to support Non-Terrestrial Network, RAN#80.

TR 38.821, Solutions for NR to support non-terrestrial networks.

Y. Zheng, M. Dong, and J. Wu, “Channel quality approximation based on shadow fading model in GEO mobile satellite system”, Advanced Materials Research, vol. 791-793, pp. 1941-1944, Sep. 2013.

Y. Zheng, M. Dong, Y. Jin, and J. Wu, “CQI prediction for shadow fading in LTE-compatible GEO mobile satellite communications system”, Advanced Materials Research, vol. 756-759, pp. 2137-2141, Sep. 2013.

R1-1802613, NTN NR impacts on the HARQ Operation, Fraunhofer IIS, RAN1#92, Athens, Greece.

R1-1805848, Consideration on HARQ Impact for NTN, Nokia, RAN1#93, Busan, Korea.

R1-1903203, Discussion on HARQ procedures for NTN, Huawei, RAN1 #96, Athens, Greece.