3GPP TSG-RAN WG1 Meeting #96	R1-1903247

Athens, Greece, February 25th – March 1st, 2019

Agenda Item:	6.2.2.1

Source:	Ericsson

Title:	Feature lead summary for Group wake-up signal for NB-IoT

Document for:	Discussion, Decision

# Introduction

This document presents the different contributions in RAN1 #96 in the agenda item 6.2.2.1, Group wake-up signal for NB-IoT [1]-[14].

# Discussion

## WUS multiplexing

How to multiplex group WUS and legacy WUS?

Legacy WUS partitioning:	Ericsson

Carrier:				Ericsson, Sony

SS-CDM+TDM:			Huawei, Nokia, Qualcomm

TDM:				Intel, Lenovo, MediaTek, Docomo, Samsung, Sharp, Sony, ZTE, vivo

Possible agreement

Group WUS and legacy WUS are time multiplexed, i.e., they do not share the same gap lengths. FFS defining the gap lengths for group WUS.

How to multiplex UE groups in group WUS?

SS-CDM:			Ericsson, Intel, LG, MediaTek, Docomo, Sharp, Sony

SS-CDM (+ TDM):	Huawei, Qualcomm, ZTE, vivo

Possible agreement

UE groups sharing the same resource set are multiplexed by single sequence CDM. FFS if more than one resource set is configurable.

Possible agreement

Up to two group WUS resources may be configured time multiplexed. SS-CDM is applied within the respective resources if more UE groups are configured. FFS whether one of the WUS resources may be shared with legacy WUS and, if so, how to achieve acceptable false paging performance for both the legacy WUS UEs and group WUS UEs that are allocated to the shared resource.

What should be the WUS sequence of the common group if group WUS and legacy WUS share resources?

Partitioned legacy WUS seq.:				Ericsson

Legacy WUS seq. in legacy WUS loc.:		Huawei, LG, Qualcomm

Non-legacy WUS seq. in legacy WUS loc.:	Nokia, ZTE

## Number of UE groups

Should group WUS support paging of more than one but less than all UE groups sharing a resource set? (That is, how many UE groups should a UE be required to detect?)

No (2 seq.):	Huawei

Yes (>2 seq):	Sharp, ZTE

Maybe (FFS):	Sony, vivo

Possible agreement

Further evaluate the benefits and costs with more grouping levels, i.e., subgroups of subgroups.

What is the maximum number of UE groups supported by group WUS?

2:		MediaTek

4:		Intel, MediaTek, Nokia, Samsung, ZTE

≥6:		Sony

≥8:		Huawei

Many:	Ericsson

Possible agreement

The maximum number of UE groups is determined when the group WUS sequence is agreed.

## Sequence design

How is the group WUS sequence designed? Legacy WUS +

Phase shift:	Ericsson, ZTE

Scrambling:	Intel, LG, Qualcomm

Cover codes:	LG

Gold codes:	Qualcomm

Under what conditions should the sequence design be evaluated (shared with LTE-MTC)?

- [xx, yy] µs detection window

- 50 Hz frequency error

- EPA Channel model with 1 Hz Doppler

- 1% false detection rate w.r.t. AWGN

- Assessed in terms of CDF vs. SNR for maximum cross sequence correlation value within detection window in relation to AWGN threshold

Possible agreement

The different candidates for the group WUS sequence design are evaluated according to the following parameters (possibly shared with LTE-MTC):

- [2.33 CP, X CP] detection window

- 50 Hz frequency error

- EPA Channel model with 1 Hz Doppler

- 1% false detection rate w.r.t. AWGN

- Presented as a CDF of maximum cross sequence correlation values within detection window in relation to AWGN threshold.

## Miscellaneous

Should a UE be required to monitor WUS in multiple resource locations?

No:	Sony, Ericsson

Possible agreement

A UE is only required to monitor WUS in one WUS (T/F) resource location.

Bases for UE grouping (RAN2):

Mobility:			Ericsson

Non-uniform distribution:	LG

Service-based:		Lenovo

DRX vs eDRX:		Nokia, Sharp

Coverage levels:		Nokia

WUS gaps:			Sharp

WUS for SI updates:	ZTE

UE group configuration aspects:

Single group configuration:						Nokia

Configuration valid for all WUS gaps:					Nokia

Configure number of groups per WUS gap:					Samsung, ZTE (ffs)

Relative group sizes controlled by eNB:					Nokia

Consider ratio of group WUS UEs to legacy WUS UEs:			Qualcomm

Use paging carrier weight to determine number of UE groups:		Sharp

Number of UE groups configured in SIB:					ZTE

Use legacy WUS resources if legacy WUS not configured:		vivo

Support WUS for Enhanced Coverage Restricted UEs			LG

# References

R1-1901746, “UE-group wake-up signal in NB-IoT,” Ericsson, RAN1 #96, Athens, Greece, February 2019.

R1-1901502, “UE-group wake-up signal,” Huawei, HiSilicon, RAN1 #96, Athens, Greece, February 2019.

R1-1902457, “UE-group wake-up signal for NB-IoT,” Intel Corp., RAN1 #96, Athens, Greece, February 2019.

R1-1902063, “Discussion on UE-grouping wake up signal in NB-IoT,” LG Electronics, RAN1 #96, Athens, Greece, February 2019.

R1-1902148, “UE-group wake-up signal for Rel-16 NB-IoT,” Lenovo, Motorola Mobility, RAN1 #96, Athens, Greece, February 2019.

R1-1901732, “UE-Group WUS in NB-IoT,” MediaTek Inc., RAN1 #96, Athens, Greece, February 2019.

R1-1902778, “Discussion on UE-group wake up signal in NB-IoT,” NTT DOCOMO, INC., RAN1 #96, Athens, Greece, February 2019.

R1-1901963, “UE group wake-up signal for NB-IoT,” Nokia, Nokia Shanghai Bell, RAN1 #96, Athens, Greece, February 2019.

R1-1902374, “UE-group wake-up signal,” Qualcomm Inc., RAN1 #96, Athens, Greece, February 2019.

R1-1902216, “UE-group wake-up signal for NB-IoT,” Samsung, RAN1 #96, Athens, Greece, February 2019.

R1-1902642, “Consideration on UE-group wake-up signal for Rel-16 NB-IoT,” Sharp, RAN1 #96, Athens, Greece, February 2019.

R1-1902196, UE group wake up signal for NB-IoT, Sony, RAN1 #96, Athens, Greece, February 2019.

R1-1901865, Discussion on Wake-up signal for NB-IoT, ZTE, RAN1 #96, Athens, Greece, February 2019.

R1-1901648, UE-group wake-up signal for NB-IoT, vivo, RAN1 #96, Athens, Greece, February 2019.