3GPP TSG-RAN WG1 #96	R1-1903252

Athens, Greece, February 25th – March 1st, 2019

Agenda Item:	6.2.1.1

Source:	Qualcomm Incorporated

Title:	Summary of 6.2.1.1 UE group MWUS

Document for:	Discussion and Decision

# 1	Introduction

In RAN1 #95, we have following agreements:

Agreement

For multiplexing between Rel-16 UE-group WUS and Rel-15 WUS, further evaluate and down select among the following options

- TDM

- FDM

- single-seq CDM

- single-seq CDM+TDM

- single-seq CDM+FDM

- FFS whether legacy WUS is the common WUS for all new UEs or only a part of new UEs.

Agreement

For multiplexing between different Rel-16 UE-group WUS, further evaluate and down select among the following options

- single-seq CDM

- FDM

- single-seq CDM+TDM

- single-seq CDM+FDM

Note: At least the maximum number of UE groups should be considered.

Agreement

The number of UE groups is configurable and broadcasted in SIB.

- FFS: Further details on the number of UE groups. For example, whether it is per PO or per gap configuration of a PO

Agreement

UE group ID is used as a parameter to generate WUS UE group sequence(s).

Agreement

One group WUS is designed as a single sequence

- Agreement

Further study false detection (cross/auto correlation) performance properties for the following designs:

- legacy WUS + cover codes,

- legacy WUS + shifted scrambling codes,

- legacy WUS + phase shift + cover code + scrambling bits

  - Including combinations of phase shift, cover code, and/or scrambling bits

Other designs are not precluded.

Agreement

Rel-16 group WUS uses the same gap configurations as for Rel-15 legacy WUS except for differences from possible TDM.

- No new gap higher layer signaling will be introduced for TDM

In RAN1 #94, we have following agreements:

Agreement

UE-group WUSs are only multiplexed in the same NB as associated PO

- FFS TDM/FDM/CDM for UE-group MWUS multiplexing

Agreement

- UE-group MWUS is supported based on eNB’s and UE’s capability.

- Whether the network supports UE-group MWUS is done by higher layer signalling.

- FFS: The number of UE groups is configured by SIB.

- Note that the UE-group MWUS is UE optional

Agreement

Rel-16 UE-group MWUS sequence should consider at least

- Fallback to legacy UE behavior

- Inter-cell interference randomization

- UE group ID for different UE-group MWUS

- Reuse of Rel-15 sequences is not precluded

- Effect of sequence detection on UE complexity

Agreement

Study the RAN1 consequence of UE-grouping on the following basis:

- UE ID

- Coverage

- DRX/eDRX

- Gap configuration

- Services

In RAN1#94bis meeting, we have the following agreements for UE-group WUS:

Agreement

UE grouping is based on at least UE ID or some function of UE ID

Agreement

The legacy UE should not be prevented from using legacy WUS even in the case of Rel-16 group WUS is enabled

- Performance impact on legacy WUS should be carefully considered

Agreement

Group WUS is based on at least legacy WUS and UE-group ID.

Agreement

Configuration of group WUS is at least signaled in SI

Agreement

A Rel-16 group WUS capable UE shall also be capable of Rel-15 legacy WUS

This is to summarize the issues for the 6.2.1.1 providing potential proposals on UE group MWUS based on the views expressed by companies in the contributions listed in the appendix.

# 2	Discussion

## 2.1	Multiplexing schemes

|  | Pros | Cons |
| --- | --- | --- |
| TDM | No power sharing; short duration; orthogonal; no impact on legacy MWUS; | Large delay/overhead |
| FDM | Same delay/duration as legacy WUS only if power boosting; Orthogonal; | Power boosting issue; Large delay/duration if no power boosting and potential impact on legacy MWUS |
| Single-sequence CDM | No power sharing No additional overhead/delay | Lower power saving since common WUS wake up all UEs |
| Single-sequence CDM+TDM | Improved power saving than CDM only; Less overhead/delay than TDM only | Larger delay/overhead than CDM; Slight impact on power saving of legacy UE if sharing with new UE in same resource |
| Single-sequence CDM+FDM | Improved power saving than CDM only; Same delay/duration as legacy WUS only if power boosting | Larger delay/duration if no power boosting and potential impact on legacy MWUS; Slight impact on power saving of legacy UE if sharing with new UE in same resource; |

Note: Fragmentation should be a smaller concern in MTC due to possibility of multiplexing with wideband UEs.

| Company | tdoc | Multiplexing between legacy and new WUS | Multiplexing of new WUS | Notes |
| --- | --- | --- | --- | --- |
| HW, HiSi | R1-1901514 | FDM and TDM | FDM if <=3groups FDM+Single-seq CDM if >3groups |  |
| Vivo | R1-1901647 | TDM and/or FDM | Single-seq CDM or Single-seq CDM+TDM if >X groups Not support FDM | Threshold X configurable |
| Ericsson | R1-1901737 | FDM | single-seq CDM Not support TDM+single-seq CDM | Limited TDM resources for DRX UEs between eDRX short gap and DRX gap if different |
| ZTE | R1-1901857 | FDM | Single-seq CDM if <=4 groups; Single-seq CDM+FDM if >4 groups | If single-seq CDM is used, a sequence can be used for one group, group combinations and all groups. |
| Nokia, NSB | R1-1901953 | FDM or FDM+Single-seq CDM | FDM if small G; FDM+Single-seq CDM if large G | A common WUS for Rel-16 UEs in all groups is supported; Configure a single group for Rel-16 UEs to separate with Rel15 UEs |
| LGE | R1-1902055 | TDM or single-seq CDM (Not FDM due to power boosting issue) | Single-seq CDM+TDM | Rel-15 WUS is used for group common WUS for UEs sharing same resource |
| Lenovo, Motorola Mobility | R1-1902145 | TDM TDM+FDM if large number of groups | Single-seq CDM or Single-seq CDM+FDM | FFS whether legacy WUS is common or not |
| Sony | R1-1902191 | FDM | Single-seq CDM+FDM | No common WUS to avoid multi-seq detection |
| Samsung | R1-1902209 | FDM | FDM | 2-PRB or 1-PRB WUS |
| Qualcomm | R1-1902366 | TDM+single-carrier WUS (Not FDM due to power sharing issue) | single-seq CDM+TDM (load balancing in up to two TDM resources if large ratio of new UEs) | Evaluation of power consumption: - TDM+single-seq CDM has up to +13~22% gain than single-sequence CDM only for low paging p=0.1~0.2 and up to 44~59% relative to no UE grouping; Negative impact on legacy UEs is marginal by grouping partial new UEs together to use legacy WUS as a common WUS; |
| Intel | R1-1902450 | TDM if max 5 groups (up to 2TDM) TDM+FDM if max 8 groups | FDM if max 5 groups (up to 2FDM) TDM+FDM if max 8 groups | A common WUS for all Rel16 UEs (not Rel15 WUS) is supported if max 5 groups; Two common UE group WUSes are supported for Rel-16 UEs each transmitted in one of WUS time resources if max 8 groups |
| Sharp | R1-1902641 |  | FDM if <=3 groups FDM+CDM if >3 groups | Common WUS configured by RRC between legacy WUS or a new WUS |
| DOCOMO | R1-1902775 | TDM | FDM and/or single-seq CDM |  |

Observation1: for multiplexing between different Rel-16 UE-group WUS:

- FDM only allow max 3 groups (each FDM resource with same size as legacy WUS resource)

  - Power sharing by more than one FDM resource results in larger overhead/delay for both Rel15 and Rel16 UEs

    - Two/three FDM WUS resources share the power to send WUS simultaneously, requiring 2x/3x WUS duration or additional 3dB/4.8dB power boosting to keep same WUS duration.

- Single-sequence CDM only has worse power saving gain than single-sequence CDM+TDM/FDM.

- Single-sequence CDM+TDM

  - An additional TDM WUS resource, if configured, uses the same 2PRB in frequency as legacy WUS resource and max WUS duration in time before the legacy WUS resource.

  - No more than one additional TDM WUS resource besides legacy WUS resource may be considered to limit overhead/delay.

  - Single-sequence CDM is used in each TDM WUS resource.

- Single-sequence CDM+FDM

  - An additional FDM WUS resource, if configured, uses 2PRB non-overlapped with legacy WUS resource in frequency and same max WUS duration in time.

  - No more than two additional FDM WUS resources can be configured besides legacy WUS resource within the same narrowband as associated PO.

  - Single-sequence CDM is used in each FDM WUS resource.

Proposal1a: Transmit power for Rel16 WUS sequence is same as that of Rel15 WUS sequence per WUS resource.

Proposal1: For multiplexing between different Rel-16 UE-group WUS, further evaluate and down select among the following options

- Alt1a: Single-sequence CDM+TDM

  - An additional TDM WUS resource, if configured, uses the same 2PRB in frequency as legacy WUS resource and max WUS duration in time before the legacy WUS resource.

  - No more than one additional TDM WUS resource besides legacy WUS resource may be considered to limit overhead/delay.

  - Single-sequence CDM is used in each TDM WUS resource if more than one group is configured.

- Alt1b: Single-sequence CDM+FDM

  - An additional FDM WUS resource, if configured, uses 2PRB non-overlapped with legacy WUS resource in frequency and same max WUS duration in time.

  - No more than [two] additional FDM WUS resources can be configured besides legacy WUS resource within the same narrowband as associated PO.

  - Single-sequence CDM is used in each FDM WUS resource if more than one group is configured.

- Alt1c: only single-seq CDM

- Alt1a+Alt1b

- FFS: Combinations by eNB configuration

- FFS whether one of the WUS resources may be shared with legacy WUS

Observation2: for multiplexing between Rel-16 UE-group WUS and Rel-15 WUS:

- FDM only

  - Power sharing by more than one FDM resource results in larger overhead/delay for both Rel15 and Rel16 UEs

  - Limited utilization of legacy WUS resource in case of low ratio of legacy UE

- TDM only

  - No impact on legacy UEs

  - Limited utilization of legacy WUS resource in case of low ratio of legacy UE

- Single-sequence CDM only has worst power saving gain than single-sequence CDM+TDM/FDM.

- Single-sequence CDM+TDM

  - Legacy WUS resource can be configured to multiplex Rel15 WUS and/or Rel16 WUS by single-sequence CDM.

- Single-sequence CDM+FDM

  - Legacy WUS resource can be configured to multiplex Rel15 WUS and/or Rel16 WUS by single-sequence CDM.

Proposal2: For multiplexing between Rel-16 UE-group WUS and Rel-15 WUS, further evaluate and down select among the following options

- Alt2a: Single-sequence CDM+TDM

  - Legacy WUS resource can be configured to multiplex Rel15 WUS and/or Rel16 WUS by single-sequence CDM.

- Alt2b: Single-sequence CDM+FDM

  - Legacy WUS resource can be configured to multiplex Rel15 WUS and/or Rel16 WUS by single-sequence CDM.

- Alt2c: TDM only

- Alt2d: FDM only

- FFS: Combinations by eNB configuration

Proposal3: A common WUS is used to wake up all group WUS UEs monitoring the same WUS (time-frequency) resource if more than one UE group occupies the WUS resource.

- FFS: Whether the above is also applicable for Rel-15 WUS UEs

- FFS: Whether to support waking up a subset of all WUS UE groups

## 2.3	Configurations for UE group MWUS

- The number of UE groups is configured per DRX/eDRX gap

  - Yes (4): Nokia, NSB (grouping for DRX UEs and eDRX UEs together if eDRX short gap is same as DRX gap), ZTE, QC

  - FFS (1): Samsung

Proposal 4: The number of UE groups is configured per DRX/eDRX gap.

- If DRX gap is same as eDRX short gap, same number of UE groups is configured for DRX UEs and eDRX UEs with eDRX short gap.

- Max number of UE group MWUS

  - Ericsson: number of UE groups should be an evaluation criterion for group WUS sequence

  - HW, HiSi: up to 8 for MTC

  - Sony: at least 6

  - ZTE: at least 4

  - Samsung: up to 4

Observation: No clear majority view yet, related with multiplexing scheme

## 2.4 UE grouping

- Up to RAN2: Ericsson, Sony, QC

Besides UE ID, consider

- Coverage: Nokia, NSB, LGE (separate grouping for Enhanced Coverage Restricted UEs)

- Service: Lenovo, Moto

- Mobility or Stationary UEs: Ericsson

Up to RAN2: Ericsson, Sony

Observation: Up to RAN2 discussion

## 2.4	Sequence design for UE group MWUS

- UE-group MWUS sequence design is same as UE-group NWUS per RB and same sequence is repeated in two PRBs

  - Yes: QC, Intel, Ericsson

Observation: No objection to reuse NWUS sequence design for MWUS

Proposal 5: UE-group MWUS sequence is same as UE-group NWUS per RB and same sequence is repeated in two PRBs.

# 3	Summary

Potential proposals are summarized as follows:

Proposal1: For multiplexing between different Rel-16 UE-group WUS, further evaluate and down select among the following options

- Alt1a: Single-sequence CDM+TDM

  - An additional TDM WUS resource, if configured, uses the same 2PRB in frequency as legacy WUS resource and max WUS duration in time before the legacy WUS resource.

  - No more than one additional TDM WUS resource can be configured besides legacy WUS resource within the same carrier.

  - Single-sequence CDM is used in each TDM WUS resource.

- Alt1b: Single-sequence CDM+FDM

  - An additional FDM WUS resource, if configured, uses 2PRB non-overlapped with legacy WUS resource in frequency and same max WUS duration in time.

  - No more than two additional FDM WUS resources can be configured besides legacy WUS resource within the same narrowband.

  - Single-sequence CDM is used in each FDM WUS resource.

- Alt1a or Alt1b by configuration

- Alt1c: only single-seq CDM

- Combinations by eNB configuration

- Transmit power for Rel16 WUS is same as that of Rel15 WUS sequence per WUS resource.

Proposal2: For multiplexing between Rel-16 UE-group WUS and Rel-15 WUS, further evaluate and down select among the following options

- Alt2a: Single-sequence CDM+TDM

  - Legacy WUS resource can be configured to multiplex Rel15 WUS and/or Rel16 WUS by single-sequence CDM.

- Alt2b: Single-sequence CDM+FDM

  - Legacy WUS resource can be configured to multiplex Rel15 WUS and/or Rel16 WUS by single-sequence CDM.

- Alt2a or Alt2b by configuration

- Alt2c: TDM only

- Alt2d: FDM only

- Combinations by eNB configuration

Proposal3: A common WUS is used to wake up all group WUS UEs monitoring the same WUS (time-frequency) resource if more than one UE group occupies the WUS resource.

- FFS: Whether the above is also applicable for Rel-15 WUS UEs

- FFS: Whether to support waking up a subset of all WUS UE groups

Proposal 4: The number of UE groups is configured per DRX/eDRX gap.

- If DRX gap is same as eDRX short gap, same number of UE groups is configured for DRX UEs and eDRX UEs with eDRX short gap.

Proposal 5: UE-group MWUS sequence is same as UE-group NWUS per RB and same sequence is repeated in two PRBs.

# References

| R1-1901514 | Support of sub-groups for MWUS | Huawei, HiSilicon |
| --- | --- | --- |
| R1-1901647 | UE-group wake-up signal for eMTC | vivo |
| R1-1901737 | UE-group wake-up signal in LTE-MTC | Ericsson |
| R1-1901857 | Discussion on Wake-up signal for MTC | ZTE |
| R1-1901953 | UE-group wake-up signal for MTC | Nokia, Nokia Shanghai Bell |
| R1-1902055 | Discussion on UE-grouping wake up signal in MTC | LG Electronics |
| R1-1902145 | UE-group wake-up signal for Rel-16 MTC | Lenovo, Motorola Mobility |
| R1-1902191 | UE group wake up signal for MTC | Sony |
| R1-1902209 | UE-group wake-up signal for eMTC | Samsung |
| R1-1902366 | UE-group wake-up signal | Qualcomm Incorporated |
| R1-1902450 | UE-group wake-up signal for eMTC | Intel Corporation |
| R1-1902641 | Consideration on UE-group wake-up signal for Rel-16 MTC | Sharp |
| R1-1902775 | Discussion on UE-group wake up signal in MTC | NTT DOCOMO, INC. |