3GPP TSG RAN WG1 Meeting #96	R1-1903255

Athens, Greece, February 25th – March 1st, 2019

Agenda Item:	6.2.2.4

Source:	Huawei, HiSilicon

Title:	Feature lead summary of Coexistence of NB-IoT with NR

Document for:	Discussion and decision

# Introduction

Coexistence with NR

- Study NR and LTE specifications to identify possible issues related to coexistence of FDD/TDD NB-IoT with NR [RAN4, RAN1, RAN2]

At RAN#82, the Rel-16 work item on additional enhancements for NB-IoT was updated [1]. One of the objectives in this work item is to study NR and LTE specifications to identify possible issues related to coexistence of FDD/TDD NB-IoT with NR.

Observation

From RAN1 perspective, no issues were identified that would prevent the coexistence of NR and NB-IoT

Agreement

RAN1 studies additional specification enhancement for improving the performance of coexistence of NB-IoT with NR.

Agreements on coexistence of NB-IoT with NR in RAN1 94 [2], RAN1 94b [3] and RAN1 95 follow.

RAN1#94

Agreement

The following aspects are studied to improve the performance of coexistence of NB-IoT with NR

- Resource reservation in NB-IoT

- Overlap of NR SSB with NB-IoT

RAN1#94b

Agreement

RAN1 continues to study the following techniques for performance improvements of NB-IoT resource allocation until the next meeting:

- Resource reservation at symbol level/slot level/subframe level/subcarrier level

- Whether the resource reservation for NB-IoT is dynamic or semi-static (if supported)

- Whether and how to support NB-IoT transmission in a portion of the subframe

- Impact of resource reservation to legacy UEs

- Whether NB-IoT transmission is postponed or dropped in reserved resources

- Whether resource reservation is used for anchor/non-anchors

Conclusion

Overlap of NR SSB with NB-IoT anchor carrier can be avoided.

RAN1#95

In this meeting, there are 6 contributions [5]-[10] discussing the coexistence of NB-IoT and NR. This contribution summarizes the views and observations/proposals from all companies.

# Performance improvement of coexistence of NB-IoT with NR

## Resource reservation in NB-IoT

In RAN1#96, six companies [5-10] show their views on resource reservation in NB-IoT to improve the performance of the coexistence of NB-IoT with NR as summarized in Table 1.

Table 1: Views on resource reservation in NB-IoT

| Huawei, HiSilicon [5] | Proposal 2: To improve the performance of NB-IoT coexistence with NR, - Dynamic resource reservation is supported. - Symbol-level and subframe-level resource reservation are further considered. - NB-IoT UE can be configured with postponed or dropped operation in reserved resources. - Resource reservation is used for NB-IoT non-anchor carriers. |
| --- | --- |
| MediaTek [6] | Proposal 3: Resource reservation at symbol level, slot level, or subcarrier-level for NR is not supported. Observation 1: Semi-static reservation of a portion of a valid or an invalid subframe for NR is not backward compatible and requires rel-16 UE to rate matching around partial NPDSCH or NPUSCH within a subframe. Observation 2: Semi-static resource reservation with invalid subframes via DL-Bitmap-NB-r13 is backward compatible with for legacy NB-IoT. Proposal 4: Reserving a portion of a subframe for NR is not supported. Observation 3: Dynamic resource reservation for NB-IoT is not backward compatible. Observation 4: Dynamic resource reservation for NB-IoT requires Rel-16 NB-IoT to monitor DCI to determine if future subframes are valid, then monitor another DCI in valid subframes to get DL assignment or UL grant. Proposal 5: Dynamic resource reservation for NB-IoT is not supported. |
| Ericsson [7] | Observation 7 It may be beneficial to introduce reserved resources in NB-IoT to enable rate matching around NR resources, such as, for example initial CORESET, CSI-RS and TRS. Proposal 2 RAN1 to study suitable mechanism for reserving resources in NB-IoT to accommodate at least some of an NR initial CORESET, NR CSI-RS and NR TRS. |
| ZTE [8] | Proposal 3: To improve the coexistence performance of NB-IoT with NR, symbol level resource reservation in non-anchor NB-IoT carriers can be considered to avoid the overlap of NR SSB and NB-IoT transmission in the serving cell. - Symbol level resource reservation can be semi-statically configured. Proposal 4: For NB-IoT coexistence with NR, dynamic signaling in DCI is used to indicate downlink puncturing information to NB-IoT UEs. Proposal 5: For NB-IoT coexistence with NR, a dedicated control information is used to indicate uplink muting information to NB-IoT UEs. |
| Nokia, Nokia Shanghai Bell [9] | Observation 1: NB-IoT reserved resource (e.g. symbol-level reservation, slot format indicator) can be used to allow NB-IoT transmission in a portion of the subframe. Observation 2: Invalid subframe bitmap can be used by legacy UEs to avoid NR transmissions and NB-IoT reserved resources. Observation 3: NB-IoT transmission can be dropped in reserved resources. Observation 4: NB-IoT reserve resource can be configured for both anchor and non-anchor carriers. |
| Qualcomm [10] | Observation 1: Resource reservation at subframe level for NB-IoT may not efficiently support NR URLLC. Proposal 1: For TDD NB-IoT coexistence with NR, the slot or symbol level resource reservation shall be supported to allow NB-IoT transmission in a portion of the subframe. Observation 2: Reserving fixed resources for NB-IoT may not efficiently support dynamic TDD for NR. Proposal 2: Support resource reservation for NB-IoT using flexible NR resources. Proposal 3: The time domain granularity of the valid subframe configuration for Rel-16 NB-IoT UE can also include 0.5ms, 0.25ms or 0.125ms dependent on the subcarrier spacing of NR. |

Most of the companies give observations/proposals for the detailed FFS parts of resource reservation in NB-IoT which were agreed in RAN1 #95.

### Resource reservation at symbol level/slot level/subframe level/subcarrier level

For resource reservation,

- [10] supports symbol or slot level resource reservation in TDD NB-IoT for coexistence of TDD NB-IoT with NR.

- [6] does not support resource reservation at symbol level, slot level, or subcarrier-level.

- [5][8][9] consider symbol-level resource reservation.

- [5] considers also subframe-level resource reservation.

Based on the inputs from all contributions, the following potential agreements should be discussed:

Potential Agreement#1: To improve the performance of coexistence of FDD/TDD NB-IoT with NR, NB-IoT resource reservation is supported.

Potential Agreement#2: Subframe-level resource reservation in NB-IoT is supported.

Next, it can be further discussed the support of

- slot-level and symbol-level resource reservation.

### Dynamic/semi-static resource reservation

Current views on dynamic/semi-static resource reservation are not aligned: [5] supports dynamic, [6] does not support dynamic, [8] suggests that symbol level resource reservation can be semi-statically configured. Further discussion is needed taking into account also the progress of Sec.2.1.1 on the granularity of the resource reservation.

### NB-IoT transmission is postponed or dropped in reserved resources

Regarding whether NB-IoT transmission is postponed or dropped in reserved resources: [5] supports that it can be configured by eNB; [9] supports that drop can be used; [7] considers rate matching. The following potential agreement can be considered.

Potential Agreement#3: NB-IoT transmission is dropped in reserved resources.

### Resource reservation for anchor/non-anchor carriers

The views on supporting resource reservation for anchor and/or non-anchor carriers are as follows: [5][8] consider resource reservation for NB-IoT non-anchor carriers, while [9] consider it for both anchor and non-anchor carriers. The following potential agreement can be considered.

Potential Agreement#4: Resource reservation in FDD/TDD NB-IoT is used at least for non-anchor carriers.

### Other aspects of NB-IoT resource reservation

Besides the FFS parts of resource reservation from RAN1#95 meeting, the following aspects are discussed in contributions to RAN1#96 meeting:

[8] supports to use DCI to indicate puncturing information in downlink, and a dedicated control information to indicate muting information in uplink.

[7] suggests RAN1 to study resource reservation in NB-IoT for NR initial CORESET, NR CSI-RS and NR TRS.

[10] supports a time domain granularity of 0.5ms, 0.25ms or 0.125ms for the valid subframe configuration for Rel-16 NB-IoT UE.

Further study is needed.

## Overlap of NR SSB with NB-IoT

The inputs on the overlap of NR SSB with NB-IoT are shown in Table 2.

Table 2: Views on overlap of NR SSB with NB-IoT

| Huawei, HiSilicon [5] | Proposal 1: Overlap of NR SSB and NB-IoT non-anchor carrier(s) can be avoided to support the coexistence of NR and NB-IoT. Observation 1: NR SSB would collide with NRS if a NB-IoT non-anchor carrier is placed within a NR SSB. |
| --- | --- |
| Ericsson [7] | Observation 1 It is possible to configure NR reserved resources to avoid collision with NB-IoT transmissions. |
| ZTE [8] | Proposal 3: To improve the coexistence performance of NB-IoT with NR, symbol level resource reservation in non-anchor NB-IoT carriers can be considered to avoid the overlap of NR SSB and NB-IoT transmission in the serving cell. - Symbol level resource reservation can be semi-statically configured. |

Based on the expressed views, the following conclusion can be considered to be agreed:

Potential Conclusion#1: Overlap of NR SSB with NB-IoT non-anchor carrier(s) can be avoided.

# Other aspects of coexistence of NB-IoT with NR

Views on other aspects of coexistence of NB-IoT with NR are summarized in Table 3.

Table 3: Views on other aspects of coexistence of NB-IoT with NR

| MediaTek [6] | Proposal 1: Semi-static resource reservation is used for anchor/non-anchors with RE-level rate-matching is used for to indicate NRS ports, v_shift, BW, and MBSFN for NR-PDSCH rate matching around NRS for NB-IoT coexistence with NR. Proposal 2: Semi-static resource reservation is used for anchor/non-anchors with rateMatchPatterns at RB symbol level granularity with resourceBlocks indicating 1 or several RBs and symbolsInResourceBlocks indicating all subframes #0, #1, .., #9 in radio frame reserved for NB-IoT, and not configuring periodicityAndPattern is supported to rate match around - Persistent NB-IoT NPSS, NSSS, NPBCH, SIB1-NB - Non-persistent NB-IoT NPDCCH, NPDSCH on NR carrier |
| --- | --- |
| Ericsson [7] | Observation 2 The NR resource utilization in coexistence with NB-IoT increases as the NR system bandwidth increases. Observation 3 With Rel-15 PRB-level NR resource reservation, the minimum NB-IoT overhead is already quite small, for example less than 1% for all NR system bandwidth larger than 15 MHz. Observation 4 With Rel-15 PRB- and OS-level NR resource reservation, the minimum NB-IoT overhead is even smaller, for example less than 1% for all NR system bandwidth larger than 5 MHz. Observation 5 Additional gains from RE-level NR resource reservation are relatively marginal compared to Rel-15 PRB- and OS-level NR resource reservation mechanisms. Observation 6 To properly use dynamic or semi-static resource reservation schemes for anchor and non-anchor NB-IoT carriers, various factors should be considered: resource utilization requirement, number of non-anchor carriers, NR system bandwidth, and scheduling complexity. Observation 8 When deploying NB-IoT inside NR with 30-kHz subcarrier spacing, it is possible to avoid interference from an NR BS on NB-IoT UE by intelligent deployment of the NB-IoT carrier. However, guard band will be required around the NB-IoT carrier to mitigate interference from an NB-IoT BS on an NR UE. The guard band can be minimized by placing NB-IoT near the center of an NR RB. Proposal 1 RAN1 to study NR resource utilization when deploying multi-carrier NB-IoT for various NR system bandwidths, and number of non-anchor carriers. |
| ZTE [8] | Proposal 1: To improve the coexistence performance of NB-IoT with NR, non-overlapping frequency regions are deployed for anchor NB-IoT and NR. Observation 1: For coexistence of NB-IoT with NR, if multiple non-anchor NB-IoT carriers are deployed, the coexistence impact would be minimized if they can be deployed in adjacent PRBs. Proposal 2: To improve the coexistence performance of NB-IoT with NR, if multiple non-anchor NB-IoT carriers are supported, multiple non-anchor carriers are deployed in adjacent PRBs. |
| Nokia, Nokia Shanghai Bell [9] | Observation 5: The NR PRB for placement of the NB-IoT carrier may be selected to allow for adequate guard bands and optimally use any excess guard band. |
| Huawei, HiSilicon [5] | Observation 2: Subcarrier and PRB alignment can be achieved when deploying TDD NB-IoT carrier(s) within a NR bandwidth. Observation 3: The uplink-downlink subframe configurations supported by TDD NB-IoT can be achieved by NR configurations when TDD NB-IoT is coexisted with NR. Observation 4: From RAN1 perspective, no issues were identified that would prevent the coexistence of NR and TDD NB-IoT. |

## Reservation of NR resources for NB-IoT

Some companies show views on how to reserve NR resources for NB-IoT.

[7] observes that other factors such as resource utilization requirement, number of non-anchor carriers, NR system bandwidth, and scheduling complexity should be considered to use dynamic or semi-static properly.

[8] supports that non-overlapping frequency regions are deployed for anchor NB-IoT and NR.

When deploying multiple NB-IoT carriers in NR:

[7] proposes to study NR resource utilization considering various NR system bandwidths, and number of non-anchor carriers.

[8] supports that multiple non-anchor carriers are deployed in adjacent PRBs.

From the feature lead point of view, it is up to the gNB implementation on how to reserve NR resources for NB-IoT anchor and/or non-anchor carrier(s) based on Rel-15 NR and Rel-15 NB-IoT specifications.

Potential Conclusion#2: From RAN1 perspective, it is up to the gNB implementation on how to reserve NR resources for NB-IoT anchor and/or non-anchor carrier(s) based on Rel-15 NR and Rel-15 NB-IoT specifications.

## Coexistence of NB-IoT and NR with 30-kHz subcarrier spacing

Two companies [7] [9] show views on coexistence of NB-IoT and NR with 30-kHz subcarrier spacing.

[7] observes that it is possible to avoid interference from an NR BS on NB-IoT UE but guard band is required around the NB-IoT carrier to mitigate interference from an NB-IoT BS on an NR UE.

[9] observes that the NR PRB for placement of the NB-IoT carrier may be selected to allow for adequate guard bands and optimally use any excess guard band.

No convergence, further study is needed.

## On TDD NB-IoT coexistence with NR

Besides the aspects above, [5] observes that no issues were identified that would prevent the coexistence of NR and TDD NB-IoT.

From feature lead point of view, similar to the conclusion for FDD NB-IoT, RAN1 can conclude that there are no identified issues for the coexistence of TDD NB-IoT with NR, while further performance improvements are studied. In RAN1#94 an observation was already agreed, and since there is no explicit mentioning of FDD or TDD, it is understood that the observation applies to both TDD and FDD NB-IoT. If preferred, the observation can be amended by adding “FDD/TDD” as follows.

Potential Observation:

From RAN1 perspective, no issues were identified that would prevent the coexistence of NR and FDD/TDD NB-IoT

# Conclusion

Aspects of coexistence of NB-IoT with NR to improve performance are addressed in [5]—[10]. A summary of companies’ views is given in the above Sections, and potential agreements are proposed.

# References

- RP-182902	WID revision: Additional enhancements for NB-IoT, December 2018.

- Chairman’s Notes RAN1#94, August 2018.

- Chairman’s Notes RAN1#94b, October 2018.

- Chairman’s Notes RAN1#95, November 2018

- R1-1901505	Performance enhancements for NB-IoT coexistence with NR	Huawei, HiSilicon

- R1-1901731	Coexistence of NB-IoT with NR	MediaTek Inc.

- R1-1901749	Coexistence of NB-IoT with NR	Ericsson

- R1-1901868	Coexistence of NB-IoT with NR	ZTE

- R1-1901966	Coexistence of NB-IoT with NR	Nokia, Nokia Shanghai Bell

- R1-1902377	Coexistence of NB-IoT with NR	Qualcomm Incorporated