3GPP TSG-TSG RAN WG1 Meeting # 96	R1-1903271

# Athens, Greece,  25th Feb - 1st March, 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.213 | CR | 1219 | rev |  | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for TDD UL/DL configuration 0 and PUSCH with repetitions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_MTCe2_L1-Core, LTE_VoLTE_ViLTE_enh-Core |  |  |  |  |  | Date: |  |  | 2019-02-15 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Current specification defines only the behavior for single repetition for the case of TDD UL/DL configuration 0 – for multiple repetitions, it is only mentioned that MSB and LSB cannot be set simultaneoulsy to 1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove the constraint of more than one repetition (N=1) in the procedural text. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Specification is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.0 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

## 8.0	UE procedure for transmitting the physical uplink shared channel

<Unchanged parts are omitted>

A BL/CE UE shall upon detection on a given serving cell of an MPDCCH with DCI format 6-0A/6-0B intended for the UE, adjust the corresponding PUSCH transmission in subframe(s) n+ki with i = 0, 1, …, N-1 according to the MPDCCH, where

-	subframe n is the last subframe in which the MPDCCH is transmitted; and

-	x≤k0<k1<…,kN-1 and the value of  is determined by the repetition number field in the corresponding DCI, where are given in Table 8-2b and Table 8-2c; and

-	in case N>1, subframe(s) n+ki with i=0,1,…,N-1 are N consecutive BL/CE UL subframe(s) starting with subframe n+x, and in case N=1, k0=x;

-	for FDD, x = 4;

-	for TDD UL/DL configurations 1-6, or for TDD UL/DL configuration 0 and a BL/CE UE in CEModeB, the value of x is given as the value of k in Table 8-2 for the corresponding TDD UL/DL configuration; If the value x is not given in Table 8-2 for subframe n, denote subframe n' as the first downlink/special subframe which has a value in Table 8-2 after subframe n, and substitute n with n' in the above procedure for adjusting the PUSCH transmission.

-	for TDD UL/DL configuration 0 and a BL/CE UE in CEModeA, if the MSB of the UL index in the MPDCCH with DCI format 6-0A is set to 1, the value of x is given as the value of k in Table 8-2 for the corresponding TDD UL/DL configuration; if the LSB of the UL index in the MPDCCH with DCI format 6-0A is set to 1, x = 7. The UE is not expected to receive DCI format 6-0A with both the MSB and LSB of the UL index set to 1 when N>1. In case both the MSB and LSB of the UL index are set to 1, the HARQ process number of the PUSCH corresponding the MSB of the UL index is  and the HARQ process number of the PUSCH corresponding the LSB of the UL index is , where  is determined according to the HARQ process number field in DCI format 6-0A

-	The higher layer parameter ttiBundling is not applicable to BL/CE UEs.

-	For a BL/CE UE, in case a PUSCH transmission with a corresponding MPDCCH collides with a PUSCH transmission without a corresponding MPDCCH in a subframe n, the PUSCH transmission without a corresponding MPDCCH is dropped from subframe n.

-	For a BL/CE UE, in case of collision between at least one physical resource block to be used for PUSCH transmission and physical resource blocks corresponding to configured PRACH resources for BL/CE UEs or non-BL/CE UEs (defined in [3]) in a same subframe, the PUSCH transmission is dropped in that subframe.

-	For a BL/CE UE in half-duplex FDD operation, in case a PUSCH transmission including half-duplex guard subframe without a corresponding MPDCCH collides partially or fully with a PDSCH transmission with a corresponding MPDCCH, the PUSCH transmission without a corresponding MPDCCH is dropped.

<Unchanged parts are omitted>

A UE configured with parameter pusch-EnhancementsConfig shall upon detection on a given serving cell of an PDCCH/EPDCCH with DCI Format 0C intended for the UE, perform a corresponding PUSCH transmission in subframe(s) n+ki if a transport block corresponding to the HARQ process of the PUSCH transmission is generated as described in [8] with i = 0, 1, …, N-1 according to the PDCCH/EPDCCH, where

-	subframe n is the last subframe in which the PDCCH/EPDCCH is transmitted; and

-	x≤k0<k1<…,kN-1 and the value of N is given by Table 8-2k based on the repetition number field in the corresponding DCI Format 0C; and

-	in case N>1, subframe(s) n+ki with i=0,1,…,N-1 are N consecutive UL subframe(s) starting with subframe n+x, and in case N=1, k0=x;

-	for FDD, x = 4;

-	for TDD UL/DL configurations 1-6, the value of x is given as the value of k in Table 8-2 for the corresponding TDD UL/DL configuration; If the value x is not given in Table 8-2 for subframe n, denote subframe n' as the first downlink/special subframe which has a value in Table 8-2 after subframe n, and substitute n with n' in the above procedure for performing the PUSCH transmission.

-	for TDD UL/DL configuration 0 and, if the MSB of the UL index in the PDCCH with DCI format 0C is set to 1, the value of x is given as the value of k in Table 8-2 for the corresponding TDD UL/DL configuration; if the LSB of the UL index in the PDCCH with DCI format 0C is set to 1, x = 7. The UE is not expected to receive DCI format 0C with both the MSB and LSB of the UL index set to 1 when N>1. In case both the MSB and LSB of the UL index are set to 1, the HARQ process number of the PUSCH corresponding the MSB of the UL index is  and the HARQ process number of the PUSCH corresponding the LSB of the UL index is , where  is determined according to the HARQ process number field in DCI format 0C

<Unchanged parts are omitted>