3GPP TSG RAN WG1 Meeting #96	R1-1903295

Athens, Greece, February 25 – March 2, 2019

Agenda Item:	6.2.3.1

Source:	Huawei, HiSilicon

Title:	Feature summary on LTE DL MIMO efficiency enhancement

Document for:	Discussion and decision

Introduction

In this contribution, the discussion on enhancing SRS capacity and coverage for LTE [2-23] are summarized. Some suggested proposals are also provided.

WID objectives and previous agreements

At RAN#80 meeting, the following objectives were agreed for enhancement in LTE Rel-16 [1].

The work item aims to specify the enhancements identified for further improving network performance. The detailed objectives are as follows.

- Enhance SRS capacity and coverage [RAN1]

  - Introduce more than one symbol for SRS for one UE or for multiple UEs on a UL normal subframe

      - Baseline: the minimum SRS resource allocation granularity for a cell is one slot, when more than one symbol in a normal subframe is allocated for SRS for the cell

      - Enhancements on PUCCH and PUSCH are not in scope

  - Introduce virtual cell ID for SRS

- Specify higher layer support of enhancements listed above [RAN2]

- Specify RF requirement of UE for the enhancements listed above [RAN4]

At RAN1#95 meeting, the following agreements were achieved:

Agreement

Both intra-subframe frequency hopping and repetition are supported for aperiodic SRS in additional symbols

- FFS whether above is supported for periodic SRS in additional SRS symbols

Agreement

Intra-subframe antenna switching is supported for aperiodic SRS in additional SRS symbols.

- FFS: Whether intra-subframe antenna switching and intra-subframe frequency hopping/repetition can be concurrently configured

- FFS: Whether the above is supported for periodic SRS in additional SRS symbols

Conclusion

The term additional SRS symbols are those further introduced in Rel-16, as indicated in the title of the AI “Additional SRS symbols”. The last symbol is not part of the additional SRS symbols.

Agreement

Both legacy SRS and additional SRS symbol(s) can be configured for the same UE.

- In case of legacy SRS is aperiodic, UE can transmit one of legacy SRS or additional SRS symbol(s) in the same subframe

- In case of legacy SRS is periodic, UE can transmit legacy SRS and additional SRS symbol(s) in the same or different subframes

- FFS: In case of legacy SRS is aperiodic, UE can transmit legacy SRS and additional SRS symbol(s) in the same subframes

  - Including triggering mechanism

Agreement

Down-select among the following alternatives in the next RAN1 meeting

- Alt1: When a virtual cell ID is configured, it applies to both legacy and additional SRS symbol(s)

- Alt2: When a virtual cell ID is configured, eNB has the flexibility to apply the virtual cell ID to

  - Only legacy SRS

  - Or only additional SRS symbol(s)

  - Or both legacy and additional SRS symbol(s)

If virtual cell ID is not configured, physical cell ID is used.

Summary

SRS resources in a cell

The views of companies are listed as below:

| Company | Proposals and comments |
| --- | --- |
| Huawei, HiSilicon | Observation 1: There is additional SRS starvation for downlink heavy UEs, as the additional SRS symbols collides with PUCCH transmission. Observation 2: Uplink data transmission would be interrupted by additional SRS transmissions. Proposal 1: Support the configuration of all symbols in one slot of a subframe used for SRS from a cell perspective. - Support PUCCH/PUSCH rate matching around additional SRS. Proposal 2: Introduce sPUSCH capability for Rel-16 UEs to enable sPUSCH transmission in the SRS subframe. - FFS sPUCCH capability for Rel-16 UEs. |
| Vivo | Observation1: SRS resource definition is useful for additional SRS in LTE, dynamic allocation of number of symbols to a UE for SRS transmission could be beneficial for efficient multiplexing with sTTI data. Proposal2: all symbols in a normal UL subframe can be configured for additional SRS, multiplexing of sTTI on same subframe and same PRBs with additional SRS on symbols where SRS is not transmitted is supported. Proposal3: dynamic time and frequency domain resource allocation for additional SRS is supported |
| MTK | Observation 1: In case all symbols in one subframe can be used for SRS from cell perspective (Option 2), it is reasonable to drop occasionally SRS transmission if it collides with an UL HARQ A/N feedback. Observation 2: In case all symbols in only one slot of one subframe can be used for SRS from cell perspective (option 1), it would require the introduction of a UE capability for slot-sPUSCH to avoid the collision between PUSCH and additional SRS symbols. Proposal 1: All symbols in one subframe can be used for SRS from cell perspective (Option 2). |
| LG Electronics | Proposal 1: Introduce separate RRC signalling for Rel-16 cell-specific SRS subframe configuration. Proposal 2: 1-slot granularity is sufficient for Rel-16 cell-specific SRS subframe configuration as enhancement on normal UL subframe. |
| Lenovo, Moto | Proposal 1: All symbols in one subframe can be used for SRS from cell perspective. |
| Samsung | Proposal 1: Define the baseline by using a combination of Rel-14 SRS enhancements (i.e. up to 6 symbols in UpPTS + SRS carrier switching). Proposal 2: Cell-specific configuration of SRS resources in normal subframes is not specified in Rel-16. Proposal 4: All symbols in one subframe can be used for SRS from cell perspective. - FFS, location of SRS symbols of an SRS resource within a subframe from UE perspective - FFS, the maximum number of SRS symbols within a subframe from UE perspective |
| QC | Observation 1: Subframe-based SRS creates issues of collision between SRS and HARQ-ACK. Observation 2: Collision between legacy PUSCH and subframe-based SRS in different CCs results in spectrum utilization degradation. Proposal 2: Support slot-level SRS configuration for time location of possible SRS symbols from cell perspective. Proposal 3: Support slot-level PUSCH/PUCCH in the same subframe of additional SRS symbols. |
| Intel | Proposal 1: All symbols in one subframe can be used for SRS from cell perspective. This does not preclude configuring only one slot for additional SRS. Observation 1: There seems to be no need to specify cell-specific configuration parameters for additional SRS symbol Rel-16 resource. |
| Mitsubishi | Proposal 1 : Support Option 2, “All symbols in one subframe can be used for SRS from cell perspective” Proposal 2 : cell-specific configuration of SRS resources in slot-level granularity Proposal 3 : Both slots in a subframe can be used for SRS transmission |
| Nokia, NSB | Proposal 1: All symbols in one subframe can be used for SRS from cell perspective. |
| Ericsson | Proposal 2 In Rel-16, all symbols in one normal UL subframe can be used for SRS from cell perspective. Proposal 3 Cell-specific SRS configuration is not modified in Rel-16. |

For SRS resources in a cell, the views of companies are summarized as below:

-   Option 1: All symbols in only one slot of one subframe can be used for SRS from cell perspective

-	With cell-specific configuration of SRS resources in slot-level granularity

-	LGE, QC(at least one slot), Huawei, HiSilicon

-	Without cell-specific configuration of SRS resources in slot-level granularity

-	Intel,

-   Option 2: All symbols in one subframe can be used for SRS from cell perspective.

-	With cell-specific configuration of SRS resources in slot-level granularity

-	Mitsubishi

-	Without cell-specific configuration of SRS resources in slot-level granularity

-	ZTE, Vivo, MTK, Lenovo, Moto, Samsung, Intel, Nokia, NSB, Ericsson

The reasons of option 1 include UCI transmission (impacts to DL performance), impacts to uplink transmission (UL throughput/PUSCH, PRACH), flexibility in SRS resource allocation etc. The reasons of option 2 include specification complexity, WID description etc.

As the concerns on the preference on additional SRS resource in a cell are generally related to impacts between additional SRS and PUSCH/PUCCH. Therefore, the inputs in this section should be combined with inputs in section 3.2.7, where the following are proposed:

- Avoid collision between UCI/UL-SCH with additional SRS symbols by eNB configuration/scheduling, i.e., no spec impacts.

- Simultaneous transmission of additional SRS symbols and other UL channels.

- PUSCH/PUCCH rate matching to slot-level.

- Independent sPUSCH capability, FFS sPUCCH capability.

- Introduce HARQ reference configuration, similar to eIMTA and SRS carrier based switching, for HARQ feedback.

- Priority rules, e.g., drop additional SRS transmission when it collides with HARQ-ACK feedback.

To move forward, the following is proposed

Proposal 1: Down-selection between following options for the time location of possible additional SRS symbols in one normal UL subframe for a cell:

- Option 1: All symbols in only one slot of one subframe can be used for SRS from cell perspective, and support

  - PUSCH/PUCCH rate matching to slot-level.

- Option 2: All symbols in one subframe can be used for SRS from cell perspective, and down-select between:

  - Alt 1:

    - Avoid collision between UCI/UL-SCH with additional SRS symbols by eNB configuration/scheduling.

    - Independent sPUSCH capability, and independent sPUCCH capability.

  - Alt 2:

    - Independent sPUSCH capability, and independent sPUCCH capability.

    - Priority rules, e.g., drop additional SRS transmission when it collides with HARQ-ACK feedback.

UE specific SRS configuration and transmission

Periodic and aperiodic of additional SRS symbols

The views of companies are listed as below:

| Company | Proposals and comments |
| --- | --- |
| Huawei, HiSilicon | Proposal 3: Support periodic SRS transmission for additional SRS symbols. - FFS semi-persistent SRS to achieve the tradeoff between overhead and performance Proposal 7: Support intra-subframe frequency hopping and repetition for periodic SRS in additional SRS symbols. |
| ZTE | Proposal 1: Not support periodic SRS in additional symbols. |
| Vivo | Proposal1: periodic transmission of additional SRS symbol(s) is not supported. |
| MTK | Proposal 6: Intra-subframe frequency hopping and repetition are not supported for periodical additional SRS symbols. |
| Lenovo, Moto | Proposal 2: Periodic SRS transmission for additional SRS symbol(s) should also be supported. Proposal 3: Frequency hopping and repetition should be supported for periodic SRS transmission for additional SRS symbol(s). |
| Samsung | Proposal 3: Periodic SRS transmission in the additional SRS symbols is not supported in Rel-16. |
| QC | Proposal 6: Support periodic SRS transmission in additional SRS symbols. - Intra-subframe antenna switching/frequency hopping/repetition are supported for periodic SRS in additional symbols. |
| Nokia, NSB | Proposal 4: Periodic SRS transmission is supported in the additional SRS symbols Proposal 5: UE drops SRS transmission if it collides with PUCCH/PUSCH/PRACH in the same carrier |
| Ericsson | Proposal 4 The trade-off between channel sounding accuracy from multi-symbol SRS transmissions of Rel-16 UEs and UL throughput impact on legacy UEs should first be carefully evaluated before specifying periodic multi-symbol SRS enhancements in Rel-16. |

On the support of periodic SRS transmission for additional SRS, the views of companies are summarized as below:

- Periodic:

  - Support: Lenovo, Moto, QC, Nokia, NSB, Huawei, HiSilicon, LGE

  - Not support: ZTE, Vivo, Samsung, MTK

  - FFS: Ericsson (trade-off between channel sounding accuracy and UL throughput impact)

The reasons to support periodic SRS transmission include further delay with only AP SRS in frequency hopping or antenna switching, configuration flexibility, DL intermittent traffic UEs. The reasons to not support periodic SRS transmission include impacts to uplink transmission, usage of legacy SRS, spec complexity. Based on the inputs, the following observation is made:

Proposal 2: Further study on periodic SRS triggering for additional SRS considering the gain and impacts to DL/UL performance.

Frequency hopping and repetition for additional SRS

The views of companies are listed as below

| Company | Proposals and comments |
| --- | --- |
| Huawei, HiSilicon | Proposal 6: For aperiodiccaly triggered additional SRS symbols, frequency hopping and repetition are supported by , where l∈0,1,…,NsymbSRS-1 is the OFDM symbol number, NsymbSRS is the number of configured SRS symbols, and R is the repetition factor for the configured UE. |
| ZTE | Proposal 3: For aperiodic SRS frequency hopping, to keep the same hopping pattern as legacy UE in the last symbol, Rel-16 SRS frequency hopping order can start from the last symbol to the first symbol of additional SRS symbols. |
| Vivo | Proposal4: intra-subframe frequency hopping and repetition transmission for additional SRS is supported, frequency hopping using both additional SRS and legacy SRS together is not supported. |
| QC | Proposal 5: Support intra-subframe SRS repetition with flexible configuration (e.g., comb/comb offset configuration) for capacity enhancement. |
| Intel | Proposal 3: When additional SRS symbols are configured with repetition they sound identical sub-carriers in all the symbols corresponding to the repetition. Proposal 4: LTE Rel-15 framework for frequency hopping and antenna switching can be reused for supporting frequency hopping and antenna switching for additional SRS symbols with appropriate changes. |

For details of frequency hopping and repetition for additional SRS symbols, companies provide proposals:

- For support of repetition and frequency hopping, , where l∈0,1,…,NsymbSRS-1  is the OFDM symbol number, NsymbSRS is the number of configured SRS symbols, and R is the repetition factor for the configured UE

- Consider whether legacy SRS and additional SRS symbols have the same hopping pattern, and Rel-16 SRS frequency hopping order can start from the last symbol to the first symbol of additional SRS symbols

- Consider whether flexible configuration (e.g., comb/comb offset configuration) is supported for repetition of additional SRS symbols.

- LTE Rel-15 framework for frequency hopping and antenna switching can be reused for supporting frequency hopping and antenna switching for additional SRS symbols with appropriate changes. FFS on the appropriate changes.

Based on the input, the following is proposed:

Proposal 3: For intra-symbol frequency hopping and repetition of additional SRS symbols, for further study:

- For support of repetition and frequency hopping, , where l∈0,1,…,NsymbSRS-1  is the OFDM symbol number, NsymbSRS is the number of configured SRS symbols, and R is the repetition factor for the configured UE

- Consider whether legacy SRS and additional SRS symbols have the same hopping pattern.

- Consider whether flexible configuration (e.g., comb/comb offset configuration) is supported for repetition of additional SRS symbols.

- LTE Rel-15 framework for frequency hopping and antenna switching can be reused for supporting frequency hopping and antenna switching for additional SRS symbols with appropriate changes. FFS on the appropriate changes.

Antenna switching for additional SRS

The views of companies are listed as below:

| Company | Proposals and comments |
| --- | --- |
| Huawei, HiSilicon | Proposal 4: Intra-subframe antenna switching and intra-subframe frequency hopping/repetition can be concurrently configured. Proposal 5: Introduce the guard period for antenna switching when additional SRS symbols are allocated for the same UE. |
| ZTE | Proposal 4: If both SRS antenna switching and frequency hopping are configured within one subframe, the gap should be minimized. |
| Vivo | Observation2: it is possible for a UE to transmit additional SRS with antenna switching within a sub-frame. |
| MTK | Proposal 5: Concurrent configuration of Intra-subframe antenna switching and intra-subframe frequency hopping/repetition for aperiodic SRS in additional SRS symbols is not supported. |
| Lenovo, Moto | Proposal 4: Intra-subframe antenna and intra-subframe frequency hopping cannot be concurrently configured. |
| QC | Proposal 4: Support intra-subframe antenna switching/frequency hopping/repetition with flexible configuration to enable the following - Intra-subframe AS with no repetition/frequency hopping on narrowband or wideband - Intra-subframe AS with frequency hopping - Intra-subframe AS with repetition |
| Intel | Proposal 5: Intra-subframe antenna switching and intra-subframe frequency hopping/repetition can be concurrently configured, where, frequency hopping/repetition is performed before switching antenna ports. |

On whether intra-subframe antenna switching and intra-subframe frequency hopping/repetition can be concurrently configured, the views are summarized as below:

- Support: ZTE, QC, Intel, Huawei, HiSilicon

- Not support: MTK, Lenovo, Moto, LGE

The reasons to support concurrently configuration include guaranteeing of SRS coverage, fast sounding of all antennas, flexibility. The reasons to not support concurrently configuration include difference use cases for frequency hopping/repetition and antenna switching, limited number of symbols.

Based on the majority view, the following is proposed to move forward:

Proposal 4: Intra-subframe antenna switching and intra-subframe frequency hopping/repetition can be concurrently configured.

Considering the RF impact due to antenna switching, the guard period was proposed and solutions to minimize the RF impact are proposed that frequency hopping/repetition is performed before antenna switching. Therefore, the following is proposed:

Proposal 5: On intra-symbol antenna switching of additional SRS symbols, further study on

- Whether the guard period of one symbol for antenna switching of the same UE is supported

- Whether frequency hopping/repetition is performed before switching antenna ports if Intra-subframe antenna switching and intra-subframe frequency hopping/repetition can be concurrently configured to a UE.

Number of SRS symbols configured to a UE within a subframe/slot

The views of companies are listed as below:

| Company | Proposals and comments |
| --- | --- |
| MTK | Proposal 2: The UE can be allocated a maximum of 6 SRS symbols within one subframe |
| Nokia, NSB | Proposal 2: UE can be configured transmit 1 – 7 additional SRS symbols in a subframe |
| Ericsson | Proposal 1 For LTE SRS enhancements in Rel-16, support up to 4 SRS symbols per subframe for a UE. |

Based on the inputs, companies are encouraged to provide inputs on the candidate numbers of additional SRS symbols that can be configured to a UE within a subframe.

Proposal 6: The numbers of additional SRS symbols that can be configured to a UE

- Within a subframe is 1-13 if subframe-level SRS resources can be supported

- Within a slot is 1-6 for the second slot, and 1-7 for the first slot if slot-level SRS resources can be supported

Concurrent transmission of legacy SRS symbols and additional SRS symbols

The views of companies are listed as below:

| Company | Proposals and comments |
| --- | --- |
| ZTE | Proposal 5: UE can transmit legacy aperiodic SRS and additional SRS symbol(s) in the same subframes. |
| MTK | Observation 3: There is no significant benefit for UE to transmit aperiodic legacy SRS symbol and aperiodic additional SRS at the same time. Proposal 3: In case legacy SRS is aperiodic, UE cannot transmit legacy SRS and additional SRS symbol(s) in the same subframes. Proposal 4: one bit in a re-interpreted field in DCI format 0 can indicate if SRS request is for legacy SRS symbol or additional SRS symbols – It is FFS which re-interpreted field in DCI format 0. |
| LG Electronics | Proposal 5: Additional and legacy AP-SRS can be triggered in the same subframe by extending legacy AP-SRS triggering mechanism. |
| Lenovo, Moto | Proposal 5: Aperiodic legacy SRS transmission cannot be triggered in the subframes containing additional SRS symbol(s). |
| Samsung | Proposal 5: Support aperiodic SRS transmission across legacy and additional SRS symbols within a subframe. |
| QC | Proposal 1: In case of legacy SRS is aperiodic, UE can transmit legacy SRS and additional SRS symbol(s) in the same subframes. |
| Intel | Proposal 2: UE does not support concurrent transmission of legacy aperiodic SRS and additional SRS symbol(s) in the same subframe. |
| Mitsubishi | Proposal 5 : Allow aperiodic legacy and additional SRS transmission in the same subframe |
| Nokia, NSB | Proposal 3: Reuse existing SRS request field in the relevant DCI formats to trigger aperiodic SRS transmission in additional SRS symbols. |

For the concurrent triggering/transmission of aperiodic legacy SRS symbols and aperiodic additional SRS symbols, the views are summarized as:

In case of legacy SRS is aperiodic, UE can transmit legacy SRS and additional SRS symbol(s) in the same subframes

- Support: ZTE, LGE, Samsung, QC, Mitsubishi, Nokia, NSB

- Not support: MTK, Lenovo, Moto, Intel, Ericsson

Based on the inputs, the following is proposed:

Proposal 7: For further study on whether and the triggering on transmission of aperiodic legacy SRS and aperiodic additional SRS symbol(s) in the same subframes for a UE.

Power control

The views of companies are listed as below:

| Company | Proposals and comments |
| --- | --- |
| Huawei, HiSilicon | Proposal 1: For additional SRS, re-use the power control mechanism defined for carriers without PUSCH/PUCCH, where higher layer parametersPO_SRS,c, aSRS,c andPO_PUSCH,c, ac can be independently configured for SRS and PUSCH. Proposal 2: Apply DCI format 3B to indicate the power adjustment of SRS even PUSCH/PUCCH is configured for the carrier. |
| Vivo | Observation3: simultaneous SRS and other uplink channels transmission can be considered, and SRS power control mechanisms need to be enhanced. |
| QC | Proposal 7: Consider flexible power control (i.e., different open loop/close loop parameters) at least for additional SRS symbols. Proposal 8: Further consider SRS configurations based on reducing the number of power changes in a subframe. |
| Nokia, NSB | Proposal 6: Separate PC loop for additional SRS symbols should be considered. |

Based on the inputs, the following is proposed:

Proposal 8: Further study on power control enhancements of additional SRS symbols:

- Independent power control of additional SRS symbols from legacy SRS symbols and PUSCH, re-use SRS power control for carriers without PUSCH/PUCCH.

- Separate power control of additional SRS symbols.

- Consider SRS configurations to reduce the number of power changes in a subframe.

UCI and UL-SCH transmission

The views of companies are listed in following table

| Sourcing | Related proposals/comments |
| --- | --- |
| Huawei, HiSilicon | Observation 1: There is additional SRS starvation for downlink heavy UEs, as the additional SRS symbols collides with PUCCH transmission. Observation 2: Uplink data transmission would be interrupted by additional SRS transmissions. Proposal 1: Support the configuration of all symbols in one slot of a subframe used for SRS from a cell perspective. - Support PUCCH/PUSCH rate matching around additional SRS. Proposal 2: Introduce sPUSCH capability for Rel-16 UEs to enable sPUSCH transmission in the SRS subframe. - FFS sPUCCH capability for Rel-16 UEs. Proposal 8 Introduce HARQ reference configuration, similar to eIMTA and SRS carrier based switching, for HARQ feedback when configured with additional SRS symbols. |
| Vivo | Observation3: simultaneous SRS and other uplink channels transmission can be considered, and SRS power control mechanisms need to be enhanced. Proposal2: all symbols in a normal UL subframe can be configured for additional SRS, multiplexing of sTTI on same subframe and same PRBs with additional SRS on symbols where SRS is not transmitted is supported. Proposal3: dynamic time and frequency domain resource allocation for additional SRS is supported |
| MTK | Observation 1: In case all symbols in one subframe can be used for SRS from cell perspective (Option 2), it is reasonable to drop occasionally SRS transmission if it collides with an UL HARQ A/N feedback. Observation 2: In case all symbols in only one slot of one subframe can be used for SRS from cell perspective (option 1), it would require the introduction of a UE capability for slot-sPUSCH to avoid the collision between PUSCH and additional SRS symbols. |
| LGE | Proposal 3: PUSCH rate matching behaviour is extended considering both legacy and Rel-16 cell-specific SRS regions together on a given scheduled UL subframe. |
| QC | Observation 1: Subframe-based SRS creates issues of collision between SRS and HARQ-ACK. Observation 2: Collision between legacy PUSCH and subframe-based SRS in different CCs results in spectrum utilization degradation. Proposal 3: Support slot-level PUSCH/PUCCH in the same subframe of additional SRS symbols. |
| Mitsubishi | Proposal 6 : Define collision rules between SRS and other channels |
| Nokia, NSB | Proposal 5: UE drops SRS transmission if it collides with PUCCH/PUSCH/PRACH in the same carrier |

Sequence generation with introduction of virtual cell ID

And the details of group/sequence hopping of proponents are:

| Sourcing | Proposals |
| --- | --- |
| ZTE | NR introduces symbol level generation of the pseudo-random sequence for both the sequence-group number and the base sequence number because multiple symbols in one slot can be used for SRS transmission. Proposal 6: For SRS pseudo-random sequence in additional SRS symbol(s), support the same symbol-level generation of the pseudo-random sequence for both and as NR. To be more specific, we provide the modified formula based on proposal 4 as below. Where is the symbol number within the subframe . |
| QC | Proposal 2: When nIDRS is configured as cell ID or virtual cell ID, the time-varying SRS sequence for multiple SRS symbols in a UL normal subframe is defined by changing (u, v) for SRS ZC roots as - u=fghl0+l',ns+nIDRSmod 30, where the group hopping pattern fghl',ns is given by fghl',ns=i=07c8(l0+l'+ns∙Nsymbslot)+i∙2imod 30if grop hopping is enabled0 if grop hopping is disabled , - with l0+l'+ns∙Nsymbslot as the index of the starting SRS symbol, and c(i) is initialized with cinit=nIDRS 30 at the beginning of each radio frame. - v=c(l0+l'+ns∙Nsymbslot if sequence hopping is enabled and grop hopping is disabled0 otherwise where c(i) is initialized with cinit=nIDRS 30∙25+nIDRS+∆ssmod30 at the beginning of each radio frame, and ∆ss could be 0 or ∆ss∈{0,1,…29} configured by higher layers. |
| Ericsson | - Time-varying SRS sequences for multi-symbol SRS in normal UL subframes is not introduced in LTE Rel-16, unless the performance benefits of this feature are quantified via evaluations. |

Based on companies’ inputs, the following is proposed:

Proposal 9: The per-symbol group hopping or sequence hopping are supported. FFS details.

Application of virtual cell ID

On the application of virtual cell ID, there are following proposals:

| Sourcing | Proposal |
| --- | --- |
| Huawei, HiSilicon | Proposal 1: The eNB can configure to apply the virtual cell ID to: - Only legacy SRS - Or only additional SRS symbol(s) - Or both legacy and additional SRS symbol(s) |
| ZTE | Proposal 1: eNB has the flexibility to configure the virtual cell ID to legacy SRS symbol and/or additional SRS symbol(s). |
| Vivo | Proposal: alternative 1, i.e. when a virtual cell ID is configured, it applies to both legacy and additional SRS symbol(s), is supported. |
| LGE | Proposal 1: When a virtual cell ID is configured, it applies to both legacy and additional SRS symbol(s). |
| Samsung | Proposal: - We support Alt1: When a virtual cell ID is configured, it applies to both legacy and additional SRS symbol(s). - FFS: in which level the SRS symbols are to be initialized, in slot level or in symbol level. |
| QC | Proposal 1: When a virtual cell ID is configured, eNB has the flexibility to apply the virtual cell ID to - Only legacy SRS - Or only additional SRS symbol(s) - Or both legacy and additional SRS symbol(s) |
| Intel | Proposal 1: When a virtual cell ID is configured, it applies to both legacy and additional SRS symbol(s) |
| Nokia, NSB | Proposal 1: Alt 2 from the agreement in the last meeting is adopted: When a virtual cell ID is configured, eNB has the flexibility to apply the virtual cell ID to o Only legacy SRS o Or only additional SRS symbol(s) o Or both legacy and additional SRS symbol(s) |
| Ericsson | - When a virtual cell ID is configured, it applies to both legacy and additional SRS symbol(s). |

As agreed in last meeting, there are two options on applicability of virtual cell IDs:

Agreement

Down-select among the following alternatives in the next RAN1 meeting

- Alt1: When a virtual cell ID is configured, it applies to both legacy and additional SRS symbol(s)

- Alt2: When a virtual cell ID is configured, eNB has the flexibility to apply the virtual cell ID to

  - Only legacy SRS

  - Or only additional SRS symbol(s)

  - Or both legacy and additional SRS symbol(s)

If virtual cell ID is not configured, physical cell ID is used.

Based on the inputs, the views are summarized as:

- Alt 1: Vivo, LGE, Samsung, Intel, Ericsson

- Alt 2: ZTE, QC, Nokia, NSB, Huawei, HiSilicon

As the views are still diverse, the following is proposed:

Proposal 10: Down-select in this meeting among the following alternatives

- Alt1: When a virtual cell ID is configured, it applies to both legacy and additional SRS symbol(s)

- Alt2: When a virtual cell ID is configured, eNB has the flexibility to apply the virtual cell ID to

  - Only legacy SRS

  - Or only additional SRS symbol(s)

  - Or both legacy and additional SRS symbol(s)

If virtual cell ID is not configured, physical cell ID is used.

Conclusion

Proposal 1: Down-selection between following options for the time location of possible additional SRS symbols in one normal UL subframe for a cell:

- Option 1: All symbols in only one slot of one subframe can be used for SRS from cell perspective, and support

  - PUSCH/PUCCH rate matching to slot-level.

- Option 2: All symbols in one subframe can be used for SRS from cell perspective, and down-select between:

  - Alt 1:

    - Avoid collision between UCI/UL-SCH with additional SRS symbols by eNB configuration/scheduling.

    - Independent sPUSCH capability, and independent sPUCCH capability.

  - Alt 2:

    - Independent sPUSCH capability, and independent sPUCCH capability.

    - Priority rules, e.g., drop additional SRS transmission when it collides with HARQ-ACK feedback.

Proposal 2: Further study on periodic SRS triggering for additional SRS considering the gain and impacts to DL/UL performance.

Proposal 3: For intra-symbol frequency hopping and repetition of additional SRS symbols, for further study:

- For support of repetition and frequency hopping, , where l∈0,1,…,NsymbSRS-1  is the OFDM symbol number, NsymbSRS is the number of configured SRS symbols, and R is the repetition factor for the configured UE

- Consider whether legacy SRS and additional SRS symbols have the same hopping pattern.

- Consider whether flexible configuration (e.g., comb/comb offset configuration) is supported for repetition of additional SRS symbols.

- LTE Rel-15 framework for frequency hopping and antenna switching can be reused for supporting frequency hopping and antenna switching for additional SRS symbols with appropriate changes. FFS on the appropriate changes.

Proposal 4: Intra-subframe antenna switching and intra-subframe frequency hopping/repetition can be concurrently configured.

Proposal 5: On intra-symbol antenna switching of additional SRS symbols, further study on

- Whether the guard period of one symbol for antenna switching of the same UE is supported

- Whether frequency hopping/repetition is performed before switching antenna ports if Intra-subframe antenna switching and intra-subframe frequency hopping/repetition can be concurrently configured to a UE.

Proposal 6: The numbers of additional SRS symbols that can be configured to a UE

- Within a subframe is 1-13 if subframe-level SRS resources can be supported

- Within a slot is 1-6 for the second slot, and 1-7 for the first slot if slot-level SRS resources can be supported

Proposal 7: For further study on whether and the triggering on transmission of aperiodic legacy SRS and aperiodic additional SRS symbol(s) in the same subframes for a UE.

Proposal 8: Further study on power control enhancements of additional SRS symbols:

- Independent power control of additional SRS symbols from legacy SRS symbols and PUSCH, re-use SRS power control for carriers without PUSCH/PUCCH.

- Separate power control of additional SRS symbols.

- Consider SRS configurations to reduce the number of power changes in a subframe.

Proposal 9: The per-symbol group hopping or sequence hopping are supported. FFS details.

Proposal 10: Down-select in this meeting among the following alternatives

- Alt1: When a virtual cell ID is configured, it applies to both legacy and additional SRS symbol(s)

- Alt2: When a virtual cell ID is configured, eNB has the flexibility to apply the virtual cell ID to

  - Only legacy SRS

  - Or only additional SRS symbol(s)

  - Or both legacy and additional SRS symbol(s)

If virtual cell ID is not configured, physical cell ID is used.

- References

RP-181485, “DL MIMO efficiency enhancements for LTE”, 3GPP TSG RAN Meeting #80, La Jolla, USA, June 11 - 14, 2018.

R1-1901585	Introduction of additional SRS symbols in normal UL subframe	Huawei, HiSilicon

R1-1901586	On virtual cell ID for SRS sequence generation	Huawei, HiSilicon

R1-1901587	Discussion on power control for additional SRS	Huawei, HiSilicon

R1-1901644	Discussion on additional SRS symbols	ZTE

R1-1901645	Discussion on virtual Cell ID for SRS	ZTE

R1-1901649	Further discussion on SRS enhancements	vivo

R1-1901650	On virtual cell ID configuration for SRS	vivo

R1-1901730	Additional SRS symbols in LTE MIMO Enhancements	MediaTek Inc.

R1-1902075	Enhancements on SRS resource allocation	LG Electronics

R1-1902076	Discussion on VCID for SRS sequence	LG Electronics

R1-1902164	Discussion of additional SRS symbols	Lenovo, Motorola Mobility

R1-1902221	Discussion on additional SRS symbols	Samsung

R1-1902222	Discussions on virtual cell ID for SRS	Samsung

R1-1902380	Additional SRS symbols	Qualcomm Incorporated

R1-1902381	Virtual cell ID for SRS	Qualcomm Incorporated

R1-1902461	Additional SRS symbols	Intel Corporation

R1-1902462	Virtual cell ID support for SRS	Intel Corporation

R1-1902832	Views on additional SRS designs for LTE MIMO	Mitsubishi Electric Co.

R1-1902855	Discussion on introduction of additional SRS symbols	Nokia, Nokia Shanghai Bell

R1-1902856	Remaining issues on virtual cell ID for SRS	Nokia, Nokia Shanghai Bell

R1-1902914	On Rel-16 LTE SRS enhancements	Ericsson

R1-1902915	On virtual cell ID for SRS	Ericsson