3GPP TSG RAN WG1 #96	R1-1903328

Athens, Greece, 25th February – 1st March, 2019

Source:	Sharp

Title:	Views on potential enhancements to Scheduling/HARQ/CSI Processing timeline

Agenda Item:	7.2.6.1.4

Document for:	Discussion and Decision

Introduction

This is the revised contribution of R1-1902665.

In the RAN1 NR-AH1901 meeting, following was agreed [1]:

Agreements: - In Rel. 16 of NR, no PDSCH and PUSCH processing timing enhancement as compared to NR Rel. 15 is supported for at least SCS = 15KHz. Agreements: For supporting the out-of-order PDSCH-to-HARQ and PDCCH-to-PUSCH between two HARQ processes on the active BWP of a given serving cell, the companies are encouraged to perform further analysis, including at least the following aspects: - The details of the dropping rules if allowed - The conditions (if any) under which the UE is expected to process the out-of-order channels

In addition, on e-mail discussion [AH10-1-NR-01], the assumption to evaluate the necessity of introducing a new PDSCH and PUSCH processing timelines was agreed as below [2]:

Agreement: To further study the need for introducing a new PDSCH and PUSCH processing timelines, the following cases are used for calibration of the results amongst the companies: - For evaluating the impact of processing times on downlink latency: - The latency of the initial transmission must include the gNB processing time after receiving a packet from the higher layers and the alignment delay. - The alignment delay includes the gap between the two consecutive PDCCH monitoring occasions for FDD, the PDCCH transmission latency due to the UL/DL configuration for TDD, and the scheduling constraint due to the slot boundaries. - The alignment delay should also be considered for scheduling the later PDSCHs. - gNB’s processing time for transmission of the initial PDSCH and gNB’s PUCCH-to-PDCCH processing time for re-trasnmission of the PDSCH: - Case1: UE’s N2/2 + X for scheduling the initial PDSCH and UE’s N2 + X for re-transmission. - X = 2/4/8 symbols for SCS = 30/60/120KHz, respectively. - PDCCH duration = 1 symbol - 1-symbol overlap between PDCCH and PDSCH - Number of PDCCH monitoring occasions per slot = 4/7 - For the case of 4 monitoring occasions per slot, PDCCH monitoring occasions are given as [1,0,0,0,1,0,0,0,1,0,0,0,1,0]; - For the case of 7 monitoring occasions per slot, PDCCH monitoring occasions are given as [1,0,1,0,1,0,1,0,1,0,1,0,1,0]; - PDSCH duration: - 2 symbols - 4 symbols - 7 symbols - PDSCH with front-loaded DMRS is assumed. - PDSCH of mapping type B is assumed. - PUCCH duration = 1 symbol - Number of PUCCH carrying HARQ-ACK for URLLC per slot is 7 and using the following pattern: [1,0,1,0,1,0,1,0,1,0,1,0,1,0]; - UE decoding time for the last PDSCH: is N1 + d_1,1 - For evaluating the impact of processing times on uplink latency: - The latency of the initial transmission must include the alignment delay. - For the case of SR-based PUSCH, the alignment delay includes the gap between the two consecutive SR occasions for FDD, the SR transmission latency due to the UL/DL configuration for TDD, and the scheduling constraint due to the slot boundaries. - For the case of grant-free PUSCH, the alignment delay includes the transmission constraint due to the grant-free UL occasions for the initial transmission, and the scheduling constraint due to the slot boundaries for the grant-based re-transmission. - For both SR-based PUSCH and grant-free PUSCH, the alignment delay should also be considered for PUSCH re-transmission triggered by a dynamic grant. - The first symbol of PUSCH consists of only DMRS. - PUSCH with type-B mapping and no additional DMRS is assumed. - For the case of grant-free PUSCH, the latency of the initial transmission must also include the UE’s processing time given as UE’s N2/2 - gNB’s PUSCH-to-PDCCH processing time (note that PDCCH alignment has to be included separately) is UE’s N1 + X - X = 2/4/8 symbols for SCS = 30/60/120KHz, respectively. - gNB’s decoding time for the last PUSCH is UE’s N1/2 + X - X = 2/4/8 symbols for SCS = 30/60/120KHz, respectively. - PUSCH duration: - Case 1: 2 - Case 2: 4 - Case 3: 7 - For dynamic PUSCH, it is assumed that the TB cannot be repeated across the slot boundary. - PDCCH duration: 1 symbol - Number of PDCCH monitoring occasions per slot = 4/7 - For the case of 4 monitoring occasions per slot, PDCCH monitoring occasions are given as [1,0,0,0,1,0,0,0,1,0,0,0,1,0]; - For the case of 7 monitoring occasions per slot, PDCCH monitoring occasions are given as [1,0,1,0,1,0,1,0,1,0,1,0,1,0]; - For GF-PUSCH: - The re-transmission is triggered by a dynamic grant. - The number of PUSCH transmission occasions per slot: - 7 for the case of 2-symb PUSCH (i.e., the UL pattern is [2,2,2,2,2,2,2].) - 3 for the case of 4-symbol PUSCH (i.e., the UL pattern is [4,4,4,0].) - 2 for the case of 7-symb PUSCH (i.e., the UL pattern is [7,7].) - For SR-based PUSCH: - gNB’s processing time for SR is UE’s N1 - Duration of the PUCCH for SR: 1 symbol - Number of SR occasions per slot: 7 with [1,0,1,0,1,0,1,0,1,0,1,0,1,0] configuration. - For SCS = 30/60KHz, FDD is assumed. - The companies can additionally consider TDD; the assumed TDD UL/DL configuration should be reported. - For SCS = 120KHz, the companies report the considered TDD UL/DL configuration (e.g., [D,D,D,D,D,D,F,F,U,U,U,U,U,U] can be assumed, where ‘F’ indicates the semi-static flexible symbol.) - In this study, a timing advance is assumed to be 0. - The gNB processing times assumed in here are only for the purpose of this study, and are not necessarily indicative of actual gNB processing capabilities. - For each scenario, the following parameters are reported: - The worst-case latency for completing a single-shot transmission under NR Rel. 15 N1/N2 capabilities. - Cap#2 for SCS = 30/60KHz and Cap#1 for SCS = 120KHz are assumed. - The worst-case latency for completing two transmissions (i.e., the initial transmission and one HARQ-based re-transmission) under NR Rel. 15 N1/N2 capabilities. - Cap#2 for SCS = 30/60KHz and Cap#1 for SCS = 120KHz are assumed. - In case a single-shot transmission cannot be completed under (1), companies report the maximum required N1/N2 (smaller than those of the NR Rel. 15) to complete a single-shot transmission within 1ms. - Also, the latency reduction gains as compared to (1) above. - In case two transmissions cannot be completed under (2), companies report the maximum required N1/N2 (smaller than those of the NR Rel. 15) to complete two transmissions (i.e., the initial transmission and one HARQ-based re-transmission) within 1ms. - Also, the latency reduction gains as compared to (2) above. - Support/No support for introducing new processing timing capabilities for Rel. 16 eURLLC. - For the DL study, it is assumed that N2=N1 when calculating gNB processing time. This assumption applies only to the Rel. 16 based analysis. - For the UL study, it is assumed that N2=N1 when calculating gNB processing time. This assumption applies only to the Rel. 16 based analysis. - Besides the above mentioned values, the companies can consider other values for gNB’s processing time for transmission of the initial PDSCH and gNB’s PUCCH-to-PDCCH processing time for re-trasnmission of the PDSCH, gNB’s PUSCH-to-PDCCH processing time, and gNB’s decoding time for the last PUSCH. In case other values are considered, the assumption of N2 = N1 when calculating the gNB processing time for the Rel. 16 analysis is not required. - For the UL study, a solution with N2 of Rel. 15 > N2 of Rel. 16 = N1 of Rel. 16 > N1 of Rel. 15 is not valid. - The LLS and SLS evaluation results can be reported under the methodology agreed in RAN1 #95 for the scenarios identified above.

In this document, we show our view on potential enhancements to Scheduling/HARQ/CSI Processing timeline.

Discussions

Evaluation of the impact of processing times

Downlink latency

Based on the assumption which was agreed on the e-mail discussion, the latency of downlink is composed of following components (as shown in figure 1):

(1) gNB Tx processing time: UE’s N2/2 + X (X = 2/4/8 symbols for SCS = 30/60/120kHz)

(2) DL alignment time: based on the PDCCH periodicity, available DL symbols and slot boundary

(3) DL transmission time: PDCCH+PDSCH duration with 1-symbol overlap (2/4/7 symbols)

(4) UE Rx processing time (and Tx processing time for NACK): N1 + d_1,1

Following is only for the case of 2 PDSCH transmissions

(5) UL alignment time: based on the PUCCH locations with available UL symbols

(6) A/N transmission time: 1 symbol

(7) gNB Rx+Tx processing time: UE’s N2 + X (X = 2/4/8 symbols for SCS = 30/60/120kHz)

(8) DL alignment time: depend on the timing of PDCCH transmission occasion, available DL symbols and slot boundary

(9) DL re-transmission time: same as (3)

(10) UE Rx processing time: N1 + d_1,1

(a) single-shot PDSCH transmission

(b) two PDSCH transmissions

Figure 1: Processing timeline for DL latency

Table 1: Worst-case latency (ms) for a single-shot transmission under NR Rel. 15 N1/N2 capabilities

|  |  | 30 kHz SCS (capability #2) |  |  | 60 kHz case (capability #2) |  |  | 120 kHz case (capability #1) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PDSCH duration |  | 2-OS | 4-OS | 7-OS | 2-OS | 4-OS | 7-OS | 2-OS | 4-OS | 7-OS |
| FDD | 4 monitoring occasions | 0.58 | 0.72 | 0.94 | 0.46 | 0.53 | 0.63 | (0.47) | (0.51) | (0.56) |
|  | 7 monitoring occasions | 0.51 | 0.65 | 0.87 | 0.42 | 0.49 | 0.60 | (0.46) | (0.49) | (0.54) |
| TDD | 4 monitoring occasions | (0.79) | (0.87) | (1.08) | (0.56) | (0.60) | (0.71) | 0.53 | 0.54 | 0.60 |
|  | 7 monitoring occasions | (0.72) | (0.87) | (1.08) | (0.53) | (0.60) | (0.71) | 0.51 | 0.54 | 0.60 |

(TDD: [D,D,D,D,D,D,F,F,U,U,U,U,U,U])

Observation 1:

- For DL latency with single-shot PDSCH transmission

  - For 30/60 kHz case with FDD, all of the cases can be transmitted within 1ms

  - For 120 kHz case with TDD, all of the cases can be transmitted within 1ms

Table 2: Worst-case latency (ms) for two transmissions under NR Rel. 15 N1/N2 capabilities

|  |  | 30 kHz SCS (capability #2) |  |  | 60 kHz case (capability #2) |  |  | 120 kHz case (capability #1) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PDSCH duration |  | 2-OS | 4-OS | 7-OS | 2-OS | 4-OS | 7-OS | 2-OS | 4-OS | 7-OS |
| FDD | 4 monitoring occasions | 1.29 | 1.51 | 1.94 | 0.96 | 1.10 | 1.21 | (1.10) | (1.17) | (1.22) |
|  | 7 monitoring occasions | 1.15 | 1.37 | 1.87 | 0.92 | 1.03 | 1.17 | (1.08) | (1.13) | (1.21) |
| TDD | 4 monitoring occasions | (1.44) | (1.87) | (2.08) | (1.13) | (1.35) | (1.46) | 1.19 | 1.21 | 1.35 |
|  | 7 monitoring occasions | (1.37) | (1.87) | (2.08) | (1.13) | (1.35) | (1.46) | 1.17 | 1.21 | 1.35 |

Observation 2:

- For DL latency with single-shot PDSCH transmission with two shot transmissions

  - For 30/60 kHz case with FDD, only 2-OS PDSCH can be transmitted within 1ms

  - For 120 kHz case with TDD, all of the cases cannot be transmitted within 1ms

Uplink latency (SR-based PUSCH transmission)

The latency of uplink with SR-based PUSCH transmission is composed of following components (as shown in figure 2):

(1) UL alignment time: based on the PUCCH locations with available UL symbols

(2) SR transmission time: 1 symbol

(3) gNB Rx+Tx processing time: UE’s N1

(4) DL alignment time: depend on the timing of PDCCH transmission occasion and available DL symbols

(5) PDCCH transmission time: 1 symbol

(6) UE Rx+Tx processing time: N2

(7) UL alignment time: depend on the slot boundary with available UL symbols

(8) UL transmission time: PUSCH duration (2/4/7 symbols)

(9) gNB Rx processing time: UE’s N1/2 + X (2/4/8 symbols for 30/60/120kHz SCS) (and additional UE’s N1/2 for Tx processing time for PDCCH for two transmissions case)

Following is only for the case of 2 PUSCH transmissions

(10) DL alignment time: depend on the timing of PDCCH transmission occasion and available DL symbols

(11) PDCCH transmission time: 1 symbol

(12) UE Rx+Tx processing time: N2

(13) UL alignment time: depend on the slot boundary and available UL symbols

(14) UL transmission time: same as (8)

(15) gNB Rx processing time: UE’s N1/2 + X (2/4/8 symbols for 30/60/120kHz SCS)

(a) single-shot PUSCH transmission

(b) two PUSCH transmissions

Figure 2: Processing timeline for UL latency with SR-based PUSCH transmission

Table 5: Worst-case latency (ms) for a single-shot transmission under NR Rel. 15 N1/N2 capabilities

|  |  | 30 kHz SCS (capability #2) |  |  | 60 kHz case (capability #2) |  |  | 120 kHz case (capability #1) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PUSCH duration |  | 2-OS | 4-OS | 7-OS | 2-OS | 4-OS | 7-OS | 2-OS | 4-OS | 7-OS |
| FDD | 4 monitoring occasions | 0.83 | 1.01 | 1.12 | 0.65 | 0.69 | 0.81 | (0.75) | (0.77) | (0.81) |
|  | 7 monitoring occasions | 0.79 | 0.94 | 1.12 | 0.62 | 0.69 | 0.81 | (0.73) | (0.77) | (0.81) |
| TDD | 4 monitoring occasions | (1.04) | (1.12) | (1.22) | (0.76) | (0.83) | (0.88) | 0.79 | 0.81 | 0.88 |
|  | 7 monitoring occasions | (1.04) | (1.12) | (1.22) | (0.72) | (0.79) | (0.85) | 0.78 | 0.79 | 0.87 |

Observation 3:

- For UL latency with SR-based single-shot PUSCH transmission

  - For 30/60 kHz case with FDD, most of the cases except for 7-OS PUSCH with 30 kHz SCS can be transmitted within 1ms

  - For 120 kHz case with TDD, all of the cases can be transmitted within 1ms

Table 6: Worst-case latency (ms) for two transmissions under NR Rel. 15 N1/N2 capabilities

|  |  | 30 kHz SCS (capability #2) |  |  | 60 kHz case (capability #2) |  |  | 120 kHz case (capability #1) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PUSCH duration |  | 2-OS | 4-OS | 7-OS | 2-OS | 4-OS | 7-OS | 2-OS | 4-OS | 7-OS |
| FDD | 4 monitoring occasions | 1.47 | 1.72 | 1.97 | 1.15 | 1.26 | 1.42 | (1.38) | (1.39) | (1.47) |
|  | 7 monitoring occasions | 1.40 | 1.62 | 1.90 | 1.12 | 1.22 | 1.38 | (1.34) | (1.39) | (1.46) |
| TDD | 4 monitoring occasions | (1.69) | (1.87) | (2.22) | (1.26) | (1.51) | (1.63) | 1.42 | 1.44 | 1.63 |
|  | 7 monitoring occasions | (1.62) | (1.87) | (2.22) | (1.22) | (1.33) | (1.42) | 1.40 | 1.42 | 1.62 |

Observation 4:

- For UL latency with SR-based two PUSCH transmission

  - For 30/60 kHz case with FDD, all of the cases cannot be transmitted within 1ms

  - For 120 kHz case with TDD, all of the cases cannot be transmitted within 1ms

Uplink latency (Grant-free PUSCH transmission)

The latency of uplink with grant-free PUSCH transmission is composed of following components (as shown in figure 3):

(1) UE Tx processing time: UE’s N2/2

(2) UL alignment time: depend on the timing of grant-free PUSCH transmission with available UL symbols

(3) UL transmission time: PUSCH duration (2/4/7 symbols)

(4) gNB Rx processing time: UE’s N1/2 + X (2/4/8 symbols for 30/60/120kHz SCS) (and additional UE’s N1/2 for Tx processing time for PDCCH for two transmissions case)

Following is only for the case of 2 PUSCH transmissions

(5) DL alignment time: depend on the timing of PDCCH transmission occasion and available DL symbols

(6) PDCCH transmission time: 1 symbol

(7) UE Rx+Tx processing time: N2

(8) UL alignment time: depend on the slot boundary and available UL symbols

(9) UL transmission time: same as (8)

(10) gNB Rx processing time: UE’s N1/2 + X (2/4/8 symbols for 30/60/120kHz SCS)

Figure 3: Processing timeline for grant-free PUSCH transmission

Table 5: Worst-case latency (ms) for a single-shot transmission under NR Rel. 15 N1/N2 capabilities

|  |  | 30 kHz SCS (capability #2) |  |  | 60 kHz case (capability #2) |  |  | 120 kHz case (capability #1) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PUSCH duration |  | 2-OS | 4-OS | 7-OS | 2-OS | 4-OS | 7-OS | 2-OS | 4-OS | 7-OS |
| FDD | 4/7 monitoring occasions | 0.39 | 0.61 | 0.75 | 0.32 | 0.43 | 0.5 | (0.36) | (0.41) | (0.45) |
| TDD | 4/7 monitoring occasions | (0.61) | (0.89) | (1) | (0.43) | (0.57) | (0.63) | 0.41 | 0.48 | 0.51 |

Observation 5:

- For UL latency with grant-free single-shot PUSCH transmission

  - For 30/60 kHz case with FDD, all of the cases can be transmitted within 1ms

  - For 120 kHz case with TDD, all of the cases can be transmitted within 1ms

Table 6: Worst-case latency (ms) for two transmissions under NR Rel. 15 N1/N2 capabilities

|  |  | 30 kHz SCS (capability #2) |  |  | 60 kHz case (capability #2) |  |  | 120 kHz case (capability #1) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PUSCH duration |  | 2-OS | 4-OS | 7-OS | 2-OS | 4-OS | 7-OS | 2-OS | 4-OS | 7-OS |
| FDD | 4 monitoring occasions | 1.07 | 1.39 | 1.54 | 0.86 | 0.96 | 1.13 | (0.98) | (1.04) | (1.13) |
|  | 7 monitoring occasions | 1.04 | 1.29 | 1.54 | 0.82 | 0.96 | 1.1 | (0.96) | (1.04) | (1.13) |
| TDD | 4 monitoring occasions | (1.29) | (1.86) | (2) | (1.04) | (1.18) | (1.25) | 1.06 | 1.12 | 1.20 |
|  | 7 monitoring occasions | (1.21) | (1.61) | (2) | (1.04) | (1.18) | (1.25) | 1.06 | 1.12 | 1.20 |

Observation 6:

- For UL latency with grant-free two PUSCH transmissions

  - For 30/60 kHz case with FDD, some cases can be transmitted within 1ms

  - For 120 kHz case with TDD, all of the cases cannot be transmitted within 1ms

Required N1/N2

single-shot transmission case

As shown in above analysis, for the single-shot transmission case, DL transmission with NR Rel-15 N1/N2 can be transmitted within 1ms. On the other hand, for UL, 4/7-OS SR-based PUSCH transmission with 30 kHz requires latency reduction to perform within 1ms. If 7-OS PUSCH is required for the scenario, it requires N2 = 2 (OS) for 30 kHz SCS and the latency will be within 1 ms.

two transmissions case

In DL, at least 30 kHz and 120 kHz requires latency reduction to achieve less than 1ms. Following N1/N2 should be considered at least for 2 OS symbols PDSCH

30 kHz SCS: N1=N2=3 (OS) -> 0.98 ms for 2-OS PDSCH

120 kHz SCS: N1=N2=20 (OS) -> 0.99 ms for 2-OS PDSCH

In UL, each SCS requires latency reduction to achieve less than 1ms. Following N1/N2 should be considered at least for 2 OS symbols PUSCH.

30 kHz SCS: N1=N2=2 (OS) -> 0.93 ms for 2-OS SR-based PUSCH

60 kHz SCS: N1=N2=7 (OS) -> 0.96 ms for 2-OS SR-based PUSCH

120 kHz SCS: N1=N2=16 (OS) -> 0.99ms for 2-OS SR-based PUSCH

In Rel-16, UL repetition enhancement is also being considered in other topic and it may reduce the UL latency. Therefore, another enhancement such as mini-slot level repetition or multi-segment transmission may improve instead of N1/N2 reduction.

Proposal 1:

- For single-shot transmission, at least 2/4 OS PDSCH can satisfy Rel-16 latency requirement without any N1/N2 enhancement

- For two transmissions, following should be considered to satisfy Rel-16 latency requirement by 2-OS SR-based PUSCH transmission

  - N1 = N2 = 2 (OS) for 30 kHz SCS

  - N1 = N2 = 7 (OS) for 60 kHz SCS

  - N1 = N2 = 16 (OS) for 120 kHz SCS

- Other enhancement such as mini-slot level repetition or multi-segment transmission may improve instead of N1/N2 reduction

Conclusion

In this contribution, we have the following proposals:

Proposal 1:

- For single-shot transmission, at least 2/4 OS PDSCH can satisfy Rel-16 latency requirement without any N1/N2 enhancement

- For two transmissions, following should be considered to satisfy Rel-16 latency requirement by 2-OS SR-based PUSCH transmission

  - N1 = N2 = 2 (OS) for 30 kHz SCS

  - N1 = N2 = 7 (OS) for 60 kHz SCS

  - N1 = N2 = 16 (OS) for 120 kHz SCS

- Other enhancement such as mini-slot level repetition or multi-segment transmission may improve instead of N1/N2 reduction

References

“RAN1 Chairman’s Notes”, RAN1 NR-AH1901

R1-1901472 “Email discussion/approval on converging the proposals for eURLLC processing timeline”, Qualcomm, RAN1 #AH1901