3GPP TSG RAN WG1 Meeting #96	R1-1903350

Athens, Greece, February 25 – March 1, 2019

Agenda Item:	7.2.6.4

Source:	Huawei

Title:	Summary of remaining details on evaluation assumptions and evaluation results

Document for:	Discussion and decision

Introduction

This document summarizes the key issues discussed under agenda item 7.2.6.4 based on the views expressed in the contributions on evaluation results.

Evaluation results

In the RAN1#AH 1901 meeting, some simulations results and observations for power distribution, transport industry and Rel-15 enabled use case were agreed to be captured in the TR 38.824. Based on the contributions submitted to RAN1#96 meeting, some companies provide some additional simulation results for the three use cases. This contribution summarizes the update for these three use cases and aim to capture the update in the TR also. In addition, there was no conclusion on the evaluation results and observations for factory automation, the summary summarizes the evaluation results and the corresponding observations, aim to be captured in the TR 38.824.

Evaluation results for factory automation

[R1-1901775, ZTE][R1-1901555, Huawei][R1-1902009, CATT] and [R1-1902754/R1-1902755, Ericsson] provide some preliminary system level simulation results for factory automation:

Table 2.1-1 The percentage of UEs satisfying requirements for factory automation

| Source 1 (Huawei, R1-1901555): Factory automation Reliability of 99.9999%, 1 ms air interface, 4 GHz, FDD, 4Tx/4Rx at gNB and 2Tx/4Rx at UE side, realistic channel estimation, two UE groups, grant free for uplink data transmission, ITU channel model |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Percentage of UEs | Resource utilization | 5% Q-value |
| 10 users per cell | DL: 4 OS |  | 96.7% | 6.93% | -3.32 |
|  | DL: 2 OS |  | 100% | 7.1% | -3.32 |
|  | UL: 14 OS |  | 90.8% | 50% | - |
| 20 users per cell | DL: 4 OS |  | 88.3% | 13.2% | -3.32 |
|  | DL: 2 OS |  | 98.2% | 14.1% | -3.32 |
|  | UL: 14 OS |  | 68.8% | 50% | - |
| 40 users per cell | DL: 4 OS |  | 74.2% | 18.4% | -3.32 |
|  | UL: 14 OS |  | 40.8% | 50% | - |
| Source 2 (ZTE, R1-1901775): Factory automation Reliability of 99.9999%, 1 ms air interface, 4 GHz, FDD, 8Tx/8Rx at gNB side, 2 Tx/2 Rx at UE side, 5 users per cell, ideal channel estimation, one UE group, grant based and grant free for uplink data transmission, ITU channel model |  |  |  |  |  |
|  |  | Percentage of UEs |  | Offered cell load (Mbps) | 5% Q-value (dB) |
| DL |  | 100 % |  | 0.6108 | -1.35 |
| Grant based UL and grant free UL |  | 100 % |  | 0.6108 | -0.96 |
| Source 3 (CATT, R1-1902009): Factory automation Reliability of 99.9999%, 1 ms air interface, 4 GHz, FDD, 4Tx/4Rx at gNB side, 4 Rx at UE side, 2 OS TTI, 10 users per cell, realistic channel estimation, two UE groups, ITU channel model |  |  |  |  |  |
|  |  | Percentage of UEs |  | Resource utilization | 5% Q-value (dB) |
| DL |  | 82 % |  | 19.6% | -2.5 |
| Source 4 (Ericsson, R1-1902754): Factory automation Reliability of 99.9999%, 1 ms air interface, 4 GHz, FDD, 16Tx/16Rx at gNB side and 2Tx/4Rx at UE side, 4 OS TTI, ideal channel estimation, one UE group, random offset within a lot, modified indoor hotspot LOS model with blockers |  |  |  |  |  |
|  |  |  | Percentage of UEs | Resource utilization |  |
| 10 users per cell | DL |  | 100% | 6.09% |  |
|  | Grant based DL |  | 99.7% | 6.93% |  |
|  | Grant free UL |  | 100% | 6.93% |  |
| 15 users per cell | DL |  | 100% | 9.17% |  |
|  | Grant based DL |  | 98.54% | 10.42% |  |
|  | Grant free UL |  | 100% | 10.42% |  |
| 20 users per cell | DL |  | 100% | 12.24% |  |
|  | Grant based DL |  | 93.24% | 13.91% |  |
|  | Grant free UL |  | 99.99% | 13.91% |  |
| 25 users per cell | DL |  | 99.99% | 15.28% |  |
|  | Grant based DL |  | 81.24% | 17.37% |  |
|  | Grant free UL |  | 99.88% | 17.37% |  |
| 30 users per cell | DL |  | 99.9% | 18.34% |  |
|  | Grant based DL |  | 68.55% | 20.85% |  |
|  | Grant free UL |  | 99.36% | 20.86% |  |
| 35 users per cell | DL |  | 99.53% | 21.37% |  |
|  | Grant based DL |  | 58.95% | 24.31% |  |
|  | Grant free UL |  | 97.73% | 24.31% |  |
| 40 users per cell | DL |  | 98.34% | 24.38% |  |
|  | Grant based DL |  | 51.67% | 27.74% |  |
|  | Grant free UL |  | 93.94% | 27.74% |  |

Proposal 2.1-1: Capture Table 2.1-1 in TR 38.824.

Based on the evaluation results in Table 2.1-1, the following observations can be made:

Observation 2.1-1: One source (R1-1901555) shows that the percentage of UEs satisfying the latency (i.e. 1 ms air interface latency) and reliability (i.e. 99.9999%) requirements by Rel-15 NR is higher than 95% for downlink transmission for factory automation assuming 10 or 20 users per cell, realistic channel estimation, 2 OS TTI, 4 GHz, 4Tx/4Rx at gNB side and 2 Tx/4 Rx at UE side, and FDD. The same source shows that the percentage of UEs satisfying the latency (i.e. 1 ms air interface latency) and reliability (i.e. 99.9999%) requirements by Rel-15 NR is lower than 95% for downlink transmission for factory automation assuming 20 per cell, realistic channel estimation, 4 OS TTI, 21% DMRS and control overhead, 4 GHz and FDD.

Observation 2.1-2: One source (R1-1902009) shows that the percentage of UEs satisfying the latency (i.e. 1 ms air interface latency) and reliability (i.e. 99.9999%) requirements by Rel-15 NR is lower than 95% for downlink transmission for factory automation assuming 10 users per cell, realistic channel estimation, 2 OS TTI, 35% DMRS and control overhead, 4Tx/4Rx at gNB side and 2 Tx/4 Rx at UE side, 4 GHz and FDD.

Observation 2.1-3: One source (R1-1902754) shows that the percentage of UEs satisfying the latency (i.e. 1 ms air interface latency) and reliability (i.e. 99.9999%) requirements by Rel-15 NR is higher than 95% for downlink transmission for factory automation assuming 10 to 40 users per cell, ideal channel estimation, 4 OS TTI, 4 GHz, 16Tx/16Rx at gNB side and 2 Tx/4 Rx at UE side, and FDD.

Observation 2.1-4: One source (R1-1902754) shows that the percentage of UEs satisfying the latency (i.e. 1 ms air interface latency) and reliability (i.e. 99.9999%) requirements by Rel-15 NR is higher than 95% for grant based uplink transmission for factory automation assuming up to 15 users per cell, ideal channel estimation, 4 OS TTI, 4 GHz, 16Tx/16Rx at gNB side and 2 Tx/4 Rx at UE side, and FDD. The same source shows that the percentage of UEs satisfying the latency (i.e. 1 ms air interface latency) and reliability (i.e. 99.9999%) requirements by Rel-15 NR is higher than 95% for grant free uplink transmission for factory automation assuming up to 35 users per cell, ideal channel estimation, 4 OS TTI, 4 GHz, 16Tx/16Rx at gNB side and 2 Tx/4 Rx at UE side, and FDD

Observation 2.1-5: One source (R1-1901775) shows that the percentage of UEs satisfying the latency (i.e. 1 ms air interface latency) and reliability (i.e. 99.9999%) requirements by Rel-15 NR is higher than 95% with cell load of 0.6108 Mbps for both downlink and uplink transmission for factory automation assuming 5 users per cell, ideal channel estimation, 8Tx/8Rx at gNB side, 2 Tx/2 Rx at UE side, 4 GHz and FDD.

Observation 2.1-6: One source (R1-1901555) shows that the percentage of UEs satisfying the latency (i.e. 1 ms air interface latency) and reliability (i.e. 99.9999%) requirements by Rel-15 NR is lower than 95% with resource utilization of 50% for uplink transmission for factory automation assuming 10 users per cell, realistic channel estimation, 4 GHz, 4Tx/4Rx at gNB side and 2 Tx/4 Rx at UE side, and FDD.

In addition, [R1-1900170/R1-1900171, Ericsson], [R1-1900976, NTT DOCOMO] and [R1-1812996, Samsung] provide some link level simulation to show the performance of a single URLLC UE for factory automation as below:

Contribution [Ericsson, R1-1900170] With the system level simulation assumption in Table A-1 for factory automation, the 1%-tile SINR is -1.45 dB (DL) and -14.3 dB (UL). Figure 2: PDCCH BLER for different AL For data channels, we consider TBS = 256 bits (=32 bytes), transmission duration of 4 OFDM symbols with 1 DMRS symbol overhead. PDSCH/PUSCH BLER for different MCSs supported within 40 MHz BW (e.g., MCS1 to MCS6 from Table 5.1.3.1-3 [4]) are given in Fig. 3. Figure 3: PDSCH/PUSCH BLER (single transmission) for different MCSs It is possible to have single DL or single CG-UL transmission with 4os duration in a TDD configuration with 30 kHz SCS within 1 ms one-way latency. Reliability requirement of 99.9999% within 1 ms one-way latency for factory automation can be achieved with several transmission configurations.

Contribution [Ericsson, R1-1900171] With the system level simulation assumption in Table A-1 for factory automation, the 1%-tile DL SINR is -1.57 dB and the 1%-tile UL SINR is -1 dB. Figure 2: PDCCH BLER for different AL For data channel, we consider TBS = 256 bits (=32 bytes), transmission duration of 7 OFDM symbols with 1 DMRS symbol overhead. BLER for different MCSs supported within 160 MHz BW (e.g., MCS0 to MCS2 from Table 5.1.3.1-3 [4]) are given in Fig. 3 for both single transmission and one retransmission with soft combining. Figure 3: BLER of single transmission and one retransmissions for different MCSs With enhanced UE processing capability for 120 kHz SCS, it is possible to have one DL or one CG UL retransmission with 7os duration in a TDD configuration with 120 kHz SCS within 1 ms one-way latency. Reliability requirement of 99.9999% within 1 ms one-way latency for factory automation at 30 GHz carrier can be achieved if enhanced UE processing capability is adopted.

Contribution [NTT DOCOMO, R1-1900976] - Case 1: factory automation @ 30GHz It is observed from UL SINR CDF that inter-cell interference is negligible to many UEs. Besides, increasing P0 offers same/higher UL SINR. This implies that most of the UEs are neither in power-limited or in interference-limited. Looking at 5%-ile UL SINR, it is almost constant between P0=-92dBm and -90dBm. Therefore, we consider 5%-ile UL SINR is 9.3dB which is achievable with these P0 values. (a) DL SINR (b) UL SINR Fig. 1-1 DL/UL SINR CDF for Case 1. Table 1-3. Required SINR for achieving BLER = 10-6 Channel; No. of RBs; Repetition; Required SNR. PDSCH; 62 RBs; No; 18 dB. Rep over multiple TRPs; 4.5 dB. 48 RBs; No; 20 dB. 45 RBs; Rep over multiple TRPs; 6 dB. PUSCH; 36 RBs; No; 17 dB. Rep over multiple TRPs; 3 dB. 31 RBs; No; 17 dB. Rep over multiple TRPs; 3 dB. 8 RBs; No; 25 dB. Rep over multiple TRPs; 18 dB. Observation 1: - For Case 1 (factory automation @ 30GHz), - 5%-ile DL SINR is 8.7dB, and 5%-ile UL SINR is 9.3dB. - Repetition over multiple TRPs overcomes blockage and offers significant performance improvement (more than 10 dB) over single TRP transmission under the same spectral efficiency. - With the typical TDD UL-DL configurations, the number of UEs that can be accommodated in the cells are following: - TDD config FR2-A (DDDSU with S=10D:2G:2U): 35 UEs for DL, 6 UEs for UL - TDD config FR2-B (DSUU with S=12D;2G): 20 UEs for DL, 16 UEs for UL - Case 2: factory automation @ 4GHz Figure 2-1 illustrates DL/UL SINR CDF in case 2. Simulation assumptions are aligned with TR 38.824 Section A2.2 Table A2.2-1 with 10m BS antenna height. For UL power control, =1 is assumed and multiple P0 values are evaluated. For DL, 5%-ile DL SINR is about -2.84dB. For UL, 5%-ile UL SINR when P0 is -90dBm is about -1.9dB. It is observed from UL SINR CDF that P0 change does not impact the distribution a lot. This implies that UEs are interference-limited. (a) DL SINR (b) UL SINR Fig. 2-1 DL/UL SINR CDF for Case 2. Table 2-3. Required SINR for achieving BLER = 10-6 Channel; No. of RBs; Repetition; Required SNR. PDSCH; 101 RBs; No; -5.5 dB. 96 RBs; Rep over multiple TRPs; -6.8 dB. 48 RBs; No; -2.8 dB. 45 RBs; Rep over multiple TRPs; -4.2 dB. PUSCH; 36 RBs; No; -4 dB. Rep over multiple TRPs; -4.8 dB. 31 RBs; No; -3.5 dB. Rep over multiple TRPs; -4.5 dB. 8 RBs; No; 4.2 dB. Rep over multiple TRPs; 1.5 dB. Observation 2: - For Case 2 (factory automation @ 4GHz), - 5%-ile DL SINR is -2.84dB, and 5%-ile UL SINR is -1.9dB. - Repetition over multiple TRPs offers performance improvement (at least 1dB) over single TRP transmission under the same spectral efficiency. - With the typical TDD UL-DL configurations, the number of UEs that can be accommodated in the cells are following: - TDD config FR1-A (DDDDDDDSUU with S=6D:4G:4U): 17 UEs for DL, 2 UEs for UL - Note: UL cycle is beyond 2ms and hence UL latency requirement cannot be satisfied - TDD config FR1-B (SU with S=12D;2G): 4 UEs for DL, 6 UEs for UL - Repetition over multiple TRPs can further improve the number of accommodated UEs

Contribution [Samsung, R1-1812996] Observation #1: Slot-based PUSCH transmission (without repetition) with Rel-15 MCS table can achieve 10-6 BLER with required Q value for factory automation. Figure 1 PUSCH performance for Factory automation with slot-based transmission: 14 symbols Figure 1 shows the BLER performance for Case #1 Factory automation. 30PRBs with 30kHz subcarrier spacing are used to transmit 32 bytes with one slot (i.e. 14 symbols), where the code rate is similar to MCS =0 in Rel-15 MCS table with low code rate. The required SINR to achieve 10-6 BLER is about -4.7dB. Contribution [Samsung, R1-1813000] Figure 1 shows geometry SINR for 5 use cases: factory automation (4GHz and 30GHz), power distribution, Rel-15 indoor and Urban scenarios. Figure 1. Downlink geometry SINR for 5 use cases Table 1 shows 5th percentile downlink SINR per use cases from figure 1. It is shown that the range of 5th percentile downlink SINR is from about -4 dB to -2.9 dB for 4GHz and about -0.6 dB for 30GHz. Table 1. 5th percentile downlink SINR Factory automation; Power Distribution; Factory automation (30GHz); Rel-15 indoor; Rel-15 Urban. -2.914 (dB); -3.2433 (dB); -0.6345 (dB); -2.948 (dB); -4.0241 (dB). Figure 2 shows uplink geometry SINR for 3 use cases: factory automation (4GHz), power distribution, and Rel-15 Urban scenarios. Figure 2 shows uplink geometry SINR for 3 use cases: factory automation (4GHz), power distribution, and Rel-15 Urban scenarios. The evaluation assumptions, except the agreed parameters, are listed in Table 2. Table 2. evaluation assumptions Scenario; UE Power control; Downtilt (degrees); UE number per cell. Factory automation; P0 = -90dBm, α = 1; 180; 40. Power Distribution; P0 = -97.5dBm, α = 1; 102; 10. Rel-15 Urban; P0 = -97.5dBm, α = 1; 102; 20. Figure 2. Uplink geometry SINR for 2 use cases Based on the evaluation results, we can obtain the 5% SINR shown in Table 3. Table 3. 5th percentile uplink SINR Factory automation; Power Distribution; Rel-15 Urban. -2.401 (dB); -3.79 (dB); -3.86 (dB). The uplink geometry is sensitive to the power control parameters. One example for Rel-15 urban macro scenario is shown in Fig. 3. In Fig. 3, different P0 values can lead to different geometry, which means that by changing the parameters can result in different SINRs. Proper power control parameter should be selected for different scenarios. Figure 3. Uplink geometry SINR for different power control parameters

Based on the above inputs from link-level perspective, it seems the following observations are common points though the evaluation method for Ericsson and DOCOMO are different:

Observation 2.1-7: From link-level simulation perspective, reliability requirement of 99.9999% within 1 ms one-way latency for factory automation for a single UE by Rel-15 NR URLLC can be achieved under some certain transmission configurations at carrier frequency of 4 GHz.

Observation 2.1-8: From link-level simulation perspective, it seems challenging to meet the reliability requirement of 99.9999% within 1 ms one-way latency for factory automation for a single UE by Rel-15 NR URLLC with single-shot transmission at carrier frequency of 30 GHz.

| Company | View |
| --- | --- |

Evaluation results for power distribution

In the RAN1#AH 1901 meeting, some evaluation results and some observations were made based on the available simulation results. According to the contributions submitted to RAN1#96 meeting, [Ericsson, R1-1901599][Huawei, R1-1903233] provide some additional evaluations, which are summarized as shown in Table 2.2-1 below using change track based on the agreed table in RAN1#AH 1901 meeting.

Table 2.2-1 The percentage of UEs satisfying requirements for power distribution

| Source 1 (Huawei, R1-1901248): Differential protection Reliability of 99.999%, 6 ms air interface latency, 250 bytes, data arrival interval 0.833ms, 4 GHz, FDD, 4Tx/4Rx at gNB side and 2 Tx/4Rx at UE side, 10 users per cell, realistic channel estimation, grant based for data transmission |  |  |  |  |
| --- | --- | --- | --- | --- |
|  |  | Percentage of UEs | Resource utilization | 5% Q-value |
| DL |  | 100% | 27% | -2.48 |
| UL |  | 52.9% | 73.2% | - |
| Source 1 (Huawei, R1-1901248): Differential protection Reliability of 99.999%, 6 ms air interface latency, , 250 bytes, data arrival interval 0.833ms, 700 MHz, FDD, 4Tx/4Rx at gNB side and 2Tx/4Rx at UE side, 10 users per cell, realistic channel estimation, 20 MHz, grant based for uplink data transmission |  |  |  |  |
|  |  | Percentage of UEs | Resource utilization | 5% Q-value |
| DL |  | 78.1% | 64.9% | -3.1 |
| UL |  | 47.1% | 78.2% | - |
| Source 2 (ZTE, R1-1900077): Differential protection Reliability of 99.999%, 6 ms air interface latency, 250 bytes, data arrival interval 0.833ms, 4 GHz, FDD, 8Tx/8Rx at gNB side, 2 Tx/2 Rx at UE side, 5 users per cell, ideal channel estimation, grant based and grant free for uplink data transmission |  |  |  |  |
|  |  | Percentage of UEs | Offered cell load (Mbps) | 5% Q-value |
| DL |  | 98.1 % | 11.6197 | -0.06 |
| UL (grant based) |  | 98.1 % | 11.4572 | -0.07 |
| UL (grant free) |  | 96.2 % | 11.4442 |  |
| Source 3 (Ericsson, R1-1901350): Power Distribution Grid Fault and Outage Management Reliability of 99.9999%, 3 ms air interface, 100 bytes, data arrival interval 100 ms, 4 GHz, FDD, 8Tx/8Rx at gNB side, 2 Tx/4 Rx at UE side, 10 users per cell, ideal channel estimation, grant free for data transmission |  |  |  |  |
|  |  | Percentage of UEs | Resource utilization | 5% Q-value |
| DL |  | 99.8% | - | -0.35 |
| UL |  | 95.4% | - | -0.44 |
| Source 4 (Ericsson, R1-1901352): Power Distribution Grid Fault and Outage Management Reliability of 99.9999%, 3 ms air interface, 100 bytes, data arrival interval 100 ms, 700 MHz, FDD, 4Tx/4Rx at gNB side, 2 Tx/4 Rx at UE side, 10 users per cell, ideal channel estimation, grant free for data transmission |  |  |  |  |
|  |  | Percentage of UEs | Resource utilization | 5% Q-value |
| DL |  | 99% | - | - |
| UL |  | 95% | - | - |
| Source 4 (Ericsson, R1-1901352): Power Distribution Grid Fault and Outage Management Reliability of 99.9999%, 3 ms air interface, 100 bytes, data arrival interval 100 ms, 700 MHz, FDD, 4Tx/4Rx at gNB side, 2 Tx/4 Rx at UE side, 10 users per cell+10 eMBB users, ideal channel estimation, grant based for data transmission |  |  |  |  |
|  |  | Percentage of UEs | Resource utilization | 5% Q-value |
| DL |  | 95.8% | - | - |
| UL |  | 95.9% | - | - |
| Source 5 (Huawei, R1-1903233): Differential protection Reliability of 99.999%, 6 ms air interface latency, 250 bytes, data arrival interval 0.833ms, 4 GHz, TDD with TDD UL-DL configuration {SUDSU}, 4Tx/4Rx at gNB side and 2 Tx/4Rx at UE side, 5 users per cell, realistic channel estimation, grant based for data transmission |  |  |  |  |
|  |  | Percentage of UEs | Resource utilization | 5% Q-value |
| DL |  | 96.2% | 24.8% | -2.48 |
| UL |  | 38.9% | 67.9% | - |
| Source 6 (Ericsson, R1-1901599): Differential protection Reliability of 99.999%, 6 ms air interface latency, 250 bytes, data arrival interval 0.833ms, 4 GHz, FDD, 4Tx/4Rx at gNB side, 2 Tx/4 Rx at UE side, ideal channel estimation, grant based for data transmission, single shot transmission without HARQ/repetition |  |  |  |  |
|  |  | Percentage of UEs | Resource utilization | 5% Q-value |
| 4 users per cell | DL | 99.8% | 12.5% | - |
|  | UL | 96.2% | 8.7% | - |
| 6 users per cell | DL | 99.9% | 24.2% | - |
|  | UL | 95.7% | 16.3% | - |
| 8 users per cell | DL | 99.9% | 24.7% | - |
|  | UL | 95.2% | 20.6% | - |
| 10 users per cell | DL | 99.9% | 36.6% | - |
|  | UL | 94.4% | 24.8% | - |

Proposal 2.2-1: Capture the update in table 2.2-1 in TR 38.824.

Based on the above new additional simulation results, the following 3 additional observations can be made:

Observation 2.2-1: One source (R1-1903233) shows that the percentage of UEs satisfying the latency (i.e. 6 ms air interface latency) and reliability (i.e. 99.999%) requirements by Rel-15 NR is higher than 95% for downlink transmission for differential protection assuming 5 URLLC users per cell, realistic channel estimation, 4 GHz, 4Tx/4Rx at gNB side and 2 Tx/4 Rx at UE side, and TDD with TDD UL-DL configuration {SUDSU}. The same source shows that the percentage of UEs satisfying the latency (i.e. 6 ms) and reliability (i.e. 99.999%) requirements by Rel-15 NR is lower than 95% for uplink transmission for differential protection assuming 5 URLLC users per cell, realistic channel estimation, 4 GHz, 4Tx/4Rx at gNB side and 2 Tx/4 Rx at UE side, and TDD with TDD UL-DL configuration {SUDSU}.

Observation 2.2-2: One source (R1-1901599) shows that the percentage of UEs satisfying the latency (i.e. 6 ms air interface latency) and reliability (i.e. 99.999%) requirements by Rel-15 NR is higher than 95% for both downlink and uplink transmission for differential protection assuming 4 or 6 or 8 or 10 URLLC users per cell, ideal channel estimation, 4 GHz, 4Tx/4Rx at gNB side and 2 Tx/4 Rx at UE side, and FDD.

| Company | View |
| --- | --- |

Evaluation results for transport industry

In the RAN1#AH 1901 meeting, some evaluation results and some observations were made based on the available simulation results. According to the contributions submitted to RAN1#96 meeting, [Ericsson, R1-1901600][Huawei, R1-1901553] provide some additional evaluations, which are summarized as shown in Table 2.3-1 below using change track based on the agreed table in RAN1#AH 1901 meeting.

Table 2.3-1 The percentage of UEs satisfying requirements for transport industry

| Source 1 (Huawei, R1-1901247): Remote driving Reliability of 99.999%, 3 ms air interface latency, 4 GHz, FDD, 4Tx/4Rx at gNB side and 2Tx/4Rx at UE side, 10 users per cell, realistic channel estimation, grant free for uplink, periodic traffic model |  |  |  |  |
| --- | --- | --- | --- | --- |
|  |  | Percentage of UEs | Resource utilization | 5% Q-value |
| DL |  | 96.7% | 10.2% | -2.2 |
| UL |  | 60% | 91.8% | - |
| Source 1 (Huawei, R1-191553): Remote driving Reliability of 99.999%, 3 ms air interface latency, 700 MHz, FDD, 4Tx/4Rx at gNB side, 2Tx/4Rx at UE side, 10 users per cell, realistic channel estimation, grant free for uplink, periodic traffic model |  |  |  |  |
|  |  | Percentage of UEs | Resource utilization | 5% Q-value |
| DL |  | 100% | 18.3% | -2.72 |
| UL |  | 60% | 85.9% | - |
| Source 2 (ZTE, R1-1900080): Remote driving Reliability of 99.999%, 3 ms air interface latency, 4 GHz, FDD, 8Tx/8Rx at gNB side, 2 Tx/2 Rx at UE side, 2 users per cell, ideal channel estimation, grant based for uplink data transmission |  |  |  |  |
|  |  | Percentage of UEs | Offered cell load (Mbps) | 5% Q-value (dB) |
| DL |  | 97.62 % | 1.9081 | -0.39 |
| UL |  | - | - | -6.17 |
| Source 2 (ZTE, R1-1900080): ITS Reliability of 99.999%, 7 ms air interface latency, 4 GHz, FDD, 8Tx/8Rx at gNB side, 2 Tx/2 Rx at UE side, 2 users per cell, ideal channel estimation, grant based and grant free for uplink data transmission |  |  |  |  |
|  |  | Percentage of UEs | Offered cell load (Mbps) | 5% Q-value (dB) |
| DL |  | 100 % | 2.092 | -0.39 |
| UL (grant based) |  | 97.62 % | 2.0918 | -6.17 |
| UL (grant free) |  | 95.3 % | 2.0876 |  |
| Source 2 (ZTE, R1-1900238): Remote driving Reliability of 99.999%, 3 ms air interface latency, 700 MHz, FDD, 2Tx/2Rx at gNB side, 2 Tx/2 Rx at UE side, 2 users per cell, ideal channel estimation, grant based for uplink data transmission |  |  |  |  |
|  |  | Percentage of UEs | Offered cell load (Mbps) | 5% Q-value (dB) |
| DL |  | 95.2 % | 1.9081 | -0.44 |
| UL |  | - | - | -1.54 |
| Source 2 (ZTE, R1-1900238): ITS Reliability of 99.999%, 7 ms air interface latency, 700 MHz, FDD, 2Tx/2Rx at gNB side, 2 Tx/2 Rx at UE side, 2 users per cell, ideal channel estimation, grant based for uplink data transmission |  |  |  |  |
|  |  | Percentage of UEs | Offered cell load (Mbps) | 5% Q-value (dB) |
| DL |  | 100 % | 2.092 | -0.44 |
| UL |  | 95.2% | 2.0918 | -1.54 |
| Source 3 (Ericsson, R1-1901351): Remote driving Reliability of 99.999%, 3 ms air interface latency, 4 GHz, FDD, 8Tx/8Rx at gNB side, 2 Tx/4 Rx at UE side, 6 users per cell, ideal channel estimation, grant-free PUSCH |  |  |  |  |
|  |  | Percentage of UEs (Mean) | - | - |
| DL |  | 97 % | - | - |
| UL |  | 60 % | - | - |
| Source 3 (Ericsson, R1-1901351): Remote driving Reliability of 99.999%, 3 ms air interface latency, 4 GHz, FDD, 8Tx/8Rx at gNB side, 2 Tx/4 Rx at UE side, 6 users per cell+30 eMBB users per 21 cells, SR-based PUSCH, ideal channel estimation |  |  |  |  |
|  |  | Percentage of UEs (Mean) | - | - |
| DL |  | 81 % | - | - |
| UL |  | 53 % | - | - |
| Source 4 (Ericsson, R1-1901600): ITS Reliability of 99.999%, 7 ms air interface latency, 1370 bytes, 4 GHz, FDD, 8Tx/8Rx at gNB side, 2 Tx/4 Rx at UE side, ideal channel estimation, grant based for data transmission, single short transmission for uplink |  |  |  |  |
|  |  | Percentage of UEs (Mean) | Resource utilization |  |
| 4 users per cell | DL | 100 % | 0.6% |  |
|  | UL | 99.4 % | 0.6% |  |
| 6 users per cell | DL | 100 % | 0.7% |  |
|  | UL | 99.2 % | 0.7% |  |
| 8 users per cell | DL | 100 % | 0.7% |  |
|  | UL | 98.8 % | 0.8% |  |
| 10 users per cell | DL | 100 % | 0.8% |  |
|  | UL | 98.8 % | 1.0% |  |

Note: 5% Q-value is obtained by system-level simulation assuming full buffer for a given evaluation scenario

Proposal 2.3-1: Capture the update in table 2.3-1 in TR 38.824.

Based on the above new additional simulation results, the following 2 additional observations can be made:

Observation 2.3-1: One source (R1-191553) shows that the percentage of UEs satisfying the latency (i.e. 3 ms) and reliability (i.e. 99.999%) requirements by Rel-15 NR is lower than 95% for uplink transmission for remote driving assuming 10 URLLC users per cell, 4Tx/4Rx at gNB side, 2 Tx/4 Rx at UE side, realistic channel estimation, 700 MHz and FDD.

Observation 2.3-2: One source (R1-1901600) shows that the percentage of UEs satisfying the latency (i.e. 7 ms for ITS) and reliability (i.e. 99.999%) requirements by Rel-15 NR is higher than 95% for both downlink and uplink transmission for ITS assuming 4 or 6 or 8 or 10 users per cell, 8Tx/8Rx at gNB side, 2 Tx/4 Rx at UE side, ideal channel estimation, 4 GHz and FDD.

| Company | View |
| --- | --- |

Evaluation results for Rel-15 enabled use case

Case 1: Rel-15 enabled use case with urban macro

In the RAN1#AH 1901 meeting, some evaluation results and some observations were made based on the available simulation results. According to the contributions submitted to RAN1#96 meeting, [Intel, R1-1902895] provide some additional evaluations, which are summarized as shown in Table 2.4-2 below using change track based on the agreed table in RAN1#AH 1901 meeting.

Table 2.4-1 The percentage of UEs satisfying requirements for Rel-15 enabled use case with urban macro

| Source 1 (Huawei, R1-1901250): Rel-15 enabled use case with urban macro (32 bytes) Reliability of 99.999%, 1 ms air interface latency, 700 MHz, FDD, 2Tx/2Rx at gNB and 2 Tx/4 Rx at UE side, 10 users per cell, realistic channel estimation, grant based for uplink, aperiodic traffic model |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| =120 p/s |  |  | Percentage of UEs | Resource utilization | 5% Q-value |
|  |  | DL | 81.9% | 3.2% | -3.2 |
|  |  | UL | 15.7% | 7.3% | - |
| Source 1 (Huawei, R1-1901250): Rel-15 enabled use case with urban macro (32 bytes) Reliability of 99.999%, 1 ms air interface latency, 700 MHz, FDD, 4Tx/4Rx at gNB and 2 Tx/4 Rx at UE side, 10 users per cell, realistic channel estimation, grant based for uplink, aperiodic traffic model |  |  |  |  |  |
| =500 p/s |  |  | Percentage of UEs | Resource utilization | 5% Q-value |
|  |  | DL | 91.4% | 6.4% | -3.1 |
|  |  | UL | 45.3% | 16.2% | - |
| Source 2 (ZTE, R1-1900079): Rel-15 enabled use case with urban macro (32 bytes or 200 bytes) Reliability of 99.999%, 4 GHz, FDD, 8Tx/8Rx at gNB side, 2 Tx/2 Rx at UE side, 5 users per cell, ideal channel estimation, grant based and grant free for uplink data transmission |  |  |  |  |  |
| 32 bytes, 1 ms air interface latency =100 p/s |  |  | Percentage of UEs | Offered cell load (Mbps) | 5% Q-value |
|  |  | DL | 99.05% | 0.1222 | -1.04 |
|  |  | UL (grant based) | 100% | 0.1222 | -1.61 |
|  |  | UL (grant free) | 100% | 0.1221 |  |
| 200 bytes, 1 ms air interface latency =100 p/s |  |  | Percentage of UEs | Offered cell load (Mbps) | 5% Q-value |
|  |  | DL | 95.24% | 0.7635 | -1.04 |
|  |  | UL (grant based) | 98.1% | 0.7635 | -1.61 |
|  |  | UL (grant free) | 96.2% | 0.7633 |  |
| 200 bytes, 4 ms air interface latency =100 p/s |  |  | Percentage of UEs | Offered cell load (Mbps) | 5% Q-value |
|  |  | DL | 98.1% | 0.7635 | -1.04 |
|  |  | UL (grant based) | 100% | 0.7635 | -1.61 |
|  |  | UL (grant free) | 99.0% | 0.7627 |  |
| Source 3 (Intel, R1-1902895) Reliability of 99.999%, 1 ms air interface latency, 4 GHz, FDD, 4Rx at gNB side, 1 Tx at UE side, 10 users per cell, realistic channel estimation, grant free for uplink data transmission, 32 bytes, 7 OS TTI, aperiodic traffic model |  |  |  |  |  |
|  |  |  | Percentage of UEs | Offered cell load (Mbps) |  |
| =100 p/s | UL |  | 85% | 0.2560 |  |
| =250 p/s | UL |  | 74% | 0.64 |  |
| =400 p/s | UL |  | 70% | 1.024 |  |

Note: 5% Q-value is obtained by system-level simulation assuming full buffer for a given evaluation scenario

Note:   is the packet arrival rate

Proposal 2.4-1: Capture the update in table 2.4-1 in TR 38.824.

Based on the above new additional simulation results, the following 2 additional observations can be made:

Observation 2.4-1: One source (R1-1902895) shows that the percentage of UEs satisfying the latency (i.e. 1 ms air interface latency) and reliability (i.e. 99.999%) requirements by Rel-15 NR is lower than 95% for uplink transmission for Rel-15 enabled use case with urban macro assuming 10 URLLC users per cell, realistic channel estimation, 4Rx at the gNB size and 1Tx at the UE side, 4 GHz and FDD.

Case 2: Rel-15 enabled use case with indoor hot-spot

In the RAN1#AH 1901 meeting, some evaluation results and some observations were made based on the available simulation results. According to the contributions submitted to RAN1#96 meeting, [Intel, R1-1902895] provide some additional evaluations, which are summarized as shown in Table 2.4-2 below using change track based on the agreed table in RAN1#AH 1901 meeting.

Table 2.4-2 The percentage of UEs satisfying requirements for Rel-15 enabled use case with indoor hot-spot

| Source 1 (Huawei, R1-1901250) Reliability of 99.9%, 7 ms air interface latency, 4 GHz, FDD, 4Tx/4Rx at gNB and 4 Tx/4 Rx at UE side, 10 users per cell, realistic channel estimation, grant based for uplink, 4096 bytes |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| =60 p/s Periodic traffic model |  |  | Percentage of UEs | Resource utilization |  | 5% Q-value |
|  |  | DL | 100% | 23.6% |  | -3.73 |
|  |  | UL | 89.2% | 38.5% |  | - |
| =60 p/s Aperiodic traffic model |  |  | Percentage of UEs | Resource utilization |  | 5% Q-value |
|  |  | DL | 100% | 20.3% |  | -3.73 |
|  |  | UL | 82.5% | 36.5% |  | - |
| Source 2 (ZTE, R1-1900079) Reliability of 99.9%, 7 ms air interface latency, 4 GHz, FDD, 8Tx/8Rx at gNB side, 2 Tx/2 Rx at UE side, 5 users per cell, ideal channel estimation, grant based and grant free for uplink data transmission, 4096 bytes |  |  |  |  |  |  |
| =60 p/s Periodic traffic model |  |  | Percentage of UEs | Offered cell load (Mbps) |  | 5% Q-value |
|  |  | DL | 100 % | 9.3810 |  | -1.09 |
|  |  | UL (grant based) | 100 % | 9.3810 |  | -2.02 |
|  |  | UL (Grant free) | 100 % | 9.3727 |  |  |
| Source 3 (NTT DOCOMO, R1-1900976) Reliability of 99.9%, 7ms air interface latency, 4 GHz, TDD with TDD UL-DL configuration {SU}, S={D10, G2, U2}, 4Tx/4Rx at gNB side, 2 Tx/4 Rx at UE side, 5 users per cell, ideal channel estimation, grant based for uplink data transmission, 4096 bytes, aperiodic traffic model |  |  |  |  |  |  |
|  |  | Percentage of UEs |  | Resource utilization | 5% Q-value |  |
| =185 p/s | DL | 91.67 % |  | 33.8% | -3.13 |  |
| =145 p/s | UL | 76.67 % |  | 32.75 | -2.19 |  |
| Source 3 (NTT DOCOMO, R1-1900976) Reliability of 99.9%, 7 ms air interface latency 4 GHz, TDD with TDD UL-DL configuration {SU}, S={D10, G2, U2}, 4Tx/4Rx at gNB side, 2 Tx/4 Rx at UE side, 4 users per cell, ideal channel estimation, grant based for uplink data transmission, 4096 bytes, aperiodic traffic model |  |  |  |  |  |  |
|  |  | Percentage of UEs |  | Resource utilization | 5% Q-value |  |
| =185 p/s | DL | 96.43 % |  | 26% | -3.13 |  |
| =145 p/s | UL | 97.77 % |  | 24.93% | -2.19 |  |
| Source 4 (Intel, R1-1902895) Reliability of 99.999%, 1 ms air interface latency, 4 GHz, FDD, 4Rx at gNB side, 1 Tx at UE side, 10 users per cell, ideal channel estimation, grant free for uplink data transmission, 32 bytes, 7 OS TTI, aperiodic traffic model |  |  |  |  |  |  |
|  |  | Percentage of UEs |  | Offered cell load (Mbps) |  |  |
| UL |  | 95 % |  | 1.1 |  |  |

Note: 5% Q-value is obtained by system-level simulation assuming full buffer for a given evaluation scenario

Note:   is the packet arrival rate

Proposal 2.4-2: Capture the update in Table 2.4-2 in TR 38.824.

Based on the above new inputs, the following additional observation can be made:

Observation 2.4-2: One source (R1-1902895) shows that the percentage of UEs satisfying the latency (i.e. 1 ms air interface latency) and reliability (i.e. 99.999%) requirements by Rel-15 NR is 95% for uplink transmission with cell load of 1.1 Mbps for Rel-15 enabled use case with indoor hot-spot assuming 10 URLLC users per cell, 4 GHz, 32 bytes packet size, 1Tx at the UE side, 4Rx at gNB side, and FDD.

| Company | View |
| --- | --- |

- Reference

- R1-1901553	Baseline performance achievable with Rel-15 URLLC for transport industry	Huawei, HiSilicon

- R1-1903233	Baseline performance achievable with Rel-15 URLLC for power distribution	Huawei, HiSilicon

- R1-1901555	Baseline performance achievable with Rel-15 URLLC for factory automation	Huawei, HiSilicon

- R1-1901556	Baseline performance achievable with Rel-15 URLLC for R15 enabled cases	Huawei, HiSilicon

- R1-1901599	Evaluation of URLLC Power Distribution Scenario	Ericsson

- R1-1901600	Evaluation of URLLC Transport Industry Scenario	Ericsson

- R1-1901775	Performance evaluation for factory automation	ZTE

- R1-1902009	Performance evaluation of the factory automation scenario	CATT

- R1-1902754	Evaluation of URLLC Factory Automation Scenario at 4 GHz	Ericsson

- R1-1902755	Evaluation of URLLC Factory Automation Scenario at 30 GHz	Ericsson

- R1-1902756	Evaluation of Rel-15 Enabled URLLC Use Cases	Ericsson

- R1-1902757	Evaluation of URLLC Urban Macro Scenario at 700 MHz	Ericsson

- R1-1902895	System level evaluations of eURLLC use cases	Intel Corporation