3GPP TSG RAN WG1 #96					         R1-1903357

Athens, Greece, 25th February – 1st March, 2019

Agenda item: 7.2.8.6.

Source: LG Electronics

Title: Evaluation on quantization methods for Type II overhead reduction

Document for: Discussion and Decision

- Introduction

In RAN1#95 meeting, following guideline and agreement regarding quantization method used for Type II CSI overhead reduction are captured in the chairman’s note as:

Agreement:

For each layer, the following alternatives for quantizing each of the coefficients in  are to be studied for down selection in RAN1#96:

- Alt1A. Rel.15 3-bit amplitude; Rel.15 QPSK and 8PSK co-phasing

- Alt1B. Rel.15 3-bit amplitude; Rel.15 QPSK, Rel.15 8PSK, and new 16PSK co-phasing

- Alt2A. Rel.15 3-bit wideband amplitude for each beam, 2/3-bit differential amplitude for FD coefficients; Rel.15 QPSK and 8PSK co-phasing

- Alt2B. Rel.15 3-bit wideband amplitude for each beam, 2/3-bit differential amplitude for FD coefficients; Rel.15 QPSK, Rel.15 8PSK, and new 16PSK co-phasing

- Alt2C. Rel.15 3-bit wideband amplitude + Rel.15 QPSK and 8PSK wideband co-phasing for each beam, 2/3-bit differential amplitude and co-phasing for FD coefficients;

- Alt3. A-bit amplitude for each of 2L beams, B-bit amplitude for each of M FD components, 1-bit differential amplitude and 8PSK co-phasing for each of the 2LM FD coefficients

- Alt4. For each beam,

  - B0-bit amplitude and C0-bit phase for coefficients for the P0 strongest coefficients,

  - B1-bit amplitude and C1-bit phase for coefficients for the P1 2nd strongest coefficients, …

  - …

  - BQ-1-bit amplitude and CQ-1-bit phase for coefficients for the PQ-1 Qth strongest coefficients

  - Alternatively, amplitude/phase can be replaced with real/imaginary

- Alt5. Special case of Alt4: Q=2, B0=C0=3; B1=C1=2 on amplitude/phase

In this contribution, we provide our evaluation results for quantization methods for the overhead reduction for Type II CSI. This Tdoc is the revision of R1-1902096.

- Discussions on quantization methods for Type II CSI overhead reduction

In order to reduce the overhead for Type II CSI, DFT-based compression has been introduced. In this codebook framework, precoders for a layer is given by size-P×N3 matrix

W=W1W2WfH

where P=2N1N2, N3 is the size of frequency domain, W2 is size-2L×M matrix, L is the number of combining beams and M is the number of columns in Wf.

  - Quantization of the coefficients in W2

Through the email discussion, above agreed options are merged as follows:

- Alt1 (per coefficient analogous to Rel.15 Type II W2): Rel.15 3-bit amplitude, N-bit phase where N is configured to either 2 (QPSK), 3 (8PSK), or 4 (16PSK).

- Alt 2M:

- UE reports the following amplitudes of the coefficients in W2, for each spatial beam

- The amplitude of the strongest coefficient per beam

  - Quantized with A bits. A = 4 or 3

    - For A=4, the alphabet is {1,1214,1414,1814,…,121414 , 0} (-1.5dB step size)

    - For A=3, the alphabet is {1,1214,1414,1814,…,12614,0} (-1.5dB step size)

- The differential amplitudes of the coefficients w.r.t. the strongest coefficient in this beam

  - Quantized with B bits. B = 2 or 3

    - For B=2, the alphabet is {1,12,14,122} (-3dB step size)

    - For B=3, the alphabet is {1,12,12,122,14,142,18,182} (-3dB step size)

- (A, B) can be configurable from (4, 2), (4, 3), (3, 2)

- UE reports the phases of the coefficients in W2.

  - Phases are quantized with C = 3 bits (8PSK) or 4 bits (16PSK), which is configurable

- Alt 2’:

    - UE reports the following for the quantization of the coefficients in W2,

      - Strongest coefficient indicator: ceil(log2(K_NZ)) bits, where K_NZ  equals number of NZ coefficients reported using the bitmap

        - Strongest coefficient = 1 (hence its amplitude/phase are not reported)

      - Two reference amplitudes (for two polarizations):

        - For the polarization of the strongest coefficient, reference amplitude = 1 (hence it is not reported)

        - For the other polarization, reference amplitude is quantized with A bits. A = 4 or 3

          - For A=4, the alphabet is {1,1214,1414,1814,…,121414 , 0} (-1.5dB step size)

          - For A=3, the alphabet is {1,1214,1414,1814,…,12614,0} (-1.5dB step size)

      - For each polarization, the differential amplitudes of the coefficients w.r.t. the reference amplitude in this polarization

        - Quantized with B bits. B = 2 or 3

- For B=2, the alphabet is {1,12,14,122} (-3dB step size)

- For B=3, the alphabet is {1,12,12,122,14,142,18,182} (-3dB step size)

      - (A, B) can be configurable from (4, 2), (4, 3), (3, 2)

      - Each phase is quantized with C = 3 bits (8PSK) or 4 bits (16PSK), which is configurable

- Alt3 (ABC matrix): A and C are real-valued diagonal matrices and B is a coefficient matrix. The amplitude set for each element of B is either 0 or 1. The amplitude sets of A and C TBD

  - [Rel.15 3-bit amplitude for A/C and Rel.15 3-bit amplitude for A and {0, 1/4, 1/2, 1} for C were mentioned – please confirm the final choice]

- Alt4M: 4-bit amplitude and 16PSK phase for the nonzero coefficients of the FD component where the strongest coefficient is located; 3-bit amplitude and {8PSK,16PSK} phase for the remaining nonzero coefficients.

Figure 1 and 2 present the average UPT and 5% UPT various quantization schemes, respectively. It is assumed that 16-port CSI-RS and medium traffic load. Also, each UE is equipped with 2 Rx antenna ports and maximum rank 2 transmission is considered. Note that for rank 2 transmission of compressed schemes, additional layer orthogonality process such as Gram-Schmidt is applied after determining the codebook parameters. For payload comparison, we assumed beta as 1/2 so that LM coefficients are required for reporting of W2. Other simulation assumptions are listed in Annex. For fair comparison, we consider following quantization scheme. For all alternatives, we consider 8PSK for 3bit phase, and 16PSK for 4bit phase.

- Reference: Rel-15 Type II CSI with L=2 and CodebookMode=2

- Alt 1: 3bit SB Amplitude (1, 1/2,1/4,1/8,1/16,1/32,1/64,0)

- Alt 2M and Alt 2’:

  - 4bit WB Amplitude {1,1214,1414,1814,…,121414 , 0}

  - 3bit SB Amplitude {1,12,12,122,14,142,18,182}

  - 2bit SB Amplitude {1,12,12,122}

- Alt 2’’:

  - 4bit WB Amplitude {1,1214,1414,1814,…,121414 , 0}

  - 3bit SB Amplitude {1,12,12,122,14,142,18,0}

  - 2bit SB Amplitude {1,12,12,0}

- Alt 3:

  - 3bit Amplitude (1, 1/2,1/4,1/8,1/16,1/32,1/64,0) used for A matrix

  - 2bit Amplitude (1,12,14,0) used for C matrix

- Alt 4:

  - 4bit C1 Amplitude (1, 1/2,1/4,1/8,…,1/214,0)

  - 3bit C2 Amplitude (1, 1/2,1/4,1/8,1/16,1/32,1/64,0))

Figure 1. Performance comparison with various quantization schemes

Figure 2. Performance comparison with various quantization schemes

Observation 1: For average UPT performance with M=6, Alt 1 with 16PSK provides performance gain of 3%, 5%, and 1% compared to the Alt 2’, Alt 3 and Alt 4, respectively.

Observation 2: For 5% UPT performance with M=6, Alt 1 with 16PSK provides performance gain of 3%, 8.8%, and 3% compared to the Alt 2, Alt 3 and Alt 4, respectively.

Observation 3: For both average UPT and 5% UPT, Alt 1 provides best trade-off between performance and feedback overhead.

Observation 4: Although Alt 3 (W2=ABC) provides the lowest feedback overhead, Alt 3 requires additional iterative optimization complexity to obtain A and C where each element in matrix A or C is calculated based on the MSE optimization.

Observation 5: Among the configurable parameters, (A, B) in Alt 2’ and Alt 2’’, (A, B)=(4,3) provides the best performance and overhead trade-off.

Proposal 1. Support Alt 1 as a quantization method for W2.

- Conclusion

In this contribution, we discuss the Type II CSI enhancement in order to efficiently support MU-MIMO. Based on the discussion above, we have following observations and proposals as:

Observation 1: For average UPT performance with M=6, Alt 1 with 16PSK provides performance gain of 3%, 5%, and 1% compared to the Alt 2’, Alt 3 and Alt 4, respectively.

Observation 2: For 5% UPT performance with M=6, Alt 1 with 16PSK provides performance gain of 3%, 8.8%, and 3% compared to the Alt 2, Alt 3 and Alt 4, respectively.

Observation 3: For both average UPT and 5% UPT, Alt 1 provides best trade-off between performance and feedback overhead.

Observation 4: Although Alt 3 (W2=ABC) provides the lowest feedback overhead, Alt 3 requires additional iterative optimization complexity to obtain A and C where each element in matrix A or C is calculated based on the MSE optimization.

Observation 5: Among the configurable parameters, (A, B) in Alt 2’ and Alt 2’’, (A, B)=(4,3) provides the best performance and overhead trade-off.

Proposal 1. Support Alt 1 as a quantization method for W2.

Annex

Table A-1. Simulation assumptions

| Parameters | Value |
| --- | --- |
| Scenarios | Dense Urban (4GHz with 15kHz SCS), ISD=200m |
| BS Tx Power | 41 dBm |
| BS antenna configurations (M,N,P,Mg,Ng,Mp,Np) | Dense Urban: 16ports=(8,4,2,1,1,2,4), (dH,dV) = (0.5, 0.8)λ |
| MS antenna configurations | 2 Rx X-pol (0/+90) |
| Etilt angle | 102 degree |
| System bandwidth | 10MHz (52RBs), SB size = 4RBs |
| UE attachment | Based on RSRP |
| Duplex | FDD |
| UE speed | 3km/h for indoor, 30km/h for outdoor |
| Traffic model | FTP Model 1 with packet size 0.5 Mbytes (medium ~50% RU) |
| Receiver | Non-ideal channel estimation and interference modeling LMMSE-IRC receiver |
| Hybrid ARQ | Maximum 4 transmissions |
| Feedback | CQI, PMI and RI reporting triggered per 5ms Feedback delay is 5 ms |
| Transmission scheme | MU-MIMO with rank adaptation |
| Wrapping method | Geographical distance based |
| Metrics | Average UE throughput, 5% UE throughput vs. feedback overhead |
| Overhead | PDCCH (2 symbols), TRS (20ms period), DMRS Type 2, NZP CSI-RS for CM, ZP CSI-RS (4Port) for IM, 1 SSB / 20ms |