3GPP TSG RAN WG1 meeting #96	R1-1903358

Athens, Greece, 25th February – 1st March 2019

Agenda item:	7.2.8.6

Source: 	Samsung

Title: 	SLS evaluation on MU-MIMO CSI: LC coefficient quantization

Document for:	Discussion and Decision

# Introduction

Based on an offline email discussion on clarification for quantization schemes (after the agreement made in RAN#95 [1]), the following alternatives about LC coefficient quantization are clarified/simplified:

- Alt1 (per coefficient analogous to Rel.15 Type II W2): Rel.15 3-bit amplitude, N-bit phase where N is configured to either 2 (QPSK), 3 (8PSK), or 4 (16PSK) - Alt2A. Rel.15 3-bit wideband amplitude for each beam, 2/3-bit differential amplitude for FD coefficients; Rel.15 QPSK and 8PSK co-phasing - Alt2B. Rel.15 3-bit wideband amplitude for each beam, 2/3-bit differential amplitude for FD coefficients; Rel.15 QPSK, Rel.15 8PSK, and new 16PSK co-phasing - Alt3. The coefficient matrix W2 (2L-by-M matrix) is expressed by a product of three matrices (=ABC). A and C are real-valued diagonal matrices and B is a coefficient matrix. The amplitude set for B is {0,1}. For the amplitude sets of A and C: - Alt 3A: 3bit R15 amplitude set for A and C. - Alt 3B: 3bit R15 amplitude set for A and new 2bit amplitude set {0, 1/4, 1/2, 1} for C. - Alt4. For each beam: 4-bit amplitude and 4-bit phase for the first FD component’s coefficient; 3-bit amplitude and 3-bit phase for the remaining coefficients

During an offline discussion during RAN1 AH 1901, a new alternative (AltM), which tries to merge Alt1, Alt2A, Alt2B and Alt4, was proposed, which is copied below.

The following simplification of AltM is proposed in [3].

Alt 2’:

- UE reports the following for the quantization of the coefficients in W2,

  - Strongest coefficient indicator: ceil(log2(K_NZ)) bits, where K_NZ  equals number of NZ coefficients reported using the bitmap

    - Strongest coefficient = 1 (hence its amplitude/phase are not reported)

  - Two reference amplitudes (for two polarizations):

    - For the polarization of the strongest coefficient, reference amplitude = 1 (hence it is not reported)

    - For the other polarization, reference amplitude is quantized with A bits. A = 4 or 3

    - For A=4, the alphabet is {1,1214,1414,1814,…,121414 , 0} (-1.5dB step size)

    - For A=3, the alphabet is {1,1214,1414,1814,…,12614,0} (-1.5dB step size)

  - For each polarization, the differential amplitudes of the coefficients w.r.t. the reference amplitude in this polarization

    - Quantized with B bits. B = 2 or 3

    - For B=2, the alphabet is {1,12,14,122} (-3dB step size)

    - For B=3, the alphabet is {1,12,12,122,14,142,18,182} (-3dB step size)

  - (A, B) can be configurable from (4, 2), (4, 3), (3, 2)

  - Each phase is quantized with C = 3 bits (8PSK) or 4 bits (16PSK), which is configurable

Alt M:

- UE reports the following amplitudes of the coefficients in W2, for each spatial beam

- The amplitude of the strongest coefficient per beam

  - Quantized with A bits. A = 4 or 3

    - For A=4, the alphabet is {1,1214,1414,1814,…,121414 , 0} (-1.5dB step size)

    - For A=3, the alphabet is {1,1214,1414,1814,…,12614,0} (-1.5dB step size)

- The differential amplitudes of the coefficients w.r.t. the strongest coefficient in this beam

  - Quantized with B bits. B = 2 or 3

    - For B=2, the alphabet is {1,12,14,122} (-3dB step size)

    - For B=3, the alphabet is {1,12,12,122,14,142,18,182} (-3dB step size)

- (A, B) can be configurable from (4, 2), (4, 3), (3, 2)

- UE reports the phases of the coefficients in W2.

  - Phases are quantized with C = 3 bits (8PSK) or 4 bits (16PSK), which is configurable

There was also some discussion during RAN1 AH 1901 regarding whether the first component in Alt4 corresponds to the strongest FD component. An example of such a variation of Alt4 is discussed in [2], where the first (or strongest) FD component is indicated by the UE, and it equals to the FD component of the strongest coefficient (1 out of KNZ NZ coefficients indicated via a bitmap). This variation is referred to as Alt4S.

This contribution provides simulation results to compare some of these alternatives (Alt1, AltM, Alt2’, Alt4, and Alt4S) for LC coefficient quantization and makes conclusions in support of our proposal in [2].

# Simulation results for LC coefficient quantization

For performance evaluation, the non-full-buffer system-level evaluation is carried out for Dense Urban (Macro only) channel model in medium (50% target RU) traffic loading scenario, and dynamic switching between SU-MIMO and MU-MIMO is considered in the simulation. The results are provided for 16 antenna ports at the gNB. The relevant simulation assumptions and parameters are according to the agreed assumptions in RAN1#94bis, and are enlisted in Table 1 in Appendix. The results are provided in Figure 1 (10 MHz BW) and Figure 2 (40 MHz BW) for the following parameters.

- Spatial compression: L = 4

- Frequency compression: M = 7 for 10 MHz BW, and M = 4 for 40 MHz BW

- Rank 1 only

- Coefficient quantization: some of the LC quantization alternatives (Alt1, AltM, Alt2’, Alt4, and Alt4S) as explained in Section 1 are evaluated, where the first FD component in Alt4 is assumed to be the FD basis with index 0 (DC component) in this evaluation.

As reference, Rel. 15 Type II with L = 2, WB+SB amplitude, and 8-PSK phase is considered. We can observe the following.

Observation 1: The performance-overhead trade-offs achieved by Alt1, Alt2’, and Alt4S are close; in particular, they are in the following order Alt2’ > Alt1 > Alt4S.

- AltM performs poorly in low-overhead regime (for small β values), and performs competitively in high overhead regime (for large β values)

- Alt4 with FD component “0”, on the other hand, can suffer from significant performance degradation if coefficients (hence the FD basis) associated with FD component “0” are always chosen

Figure 1: Performance-overhead trade-off for different LC coefficient quantization for 10MHz BW

Figure 2: Performance-overhead trade-off for different LC coefficient quantization for 40MHz BW

Regarding Alt4S, it can be argued that the FD component of the strongest coefficient (across all the SD beams and FD units) may not be the strongest FD component for each SD beam. To assess how often this could happen, the probability of this event (FD component of the strongest coefficient = the strongest FD component for each SD beam) denoted as ‘0’ is shown in Figure 3.

Observation 2: For Alt4S, the probability of FD component of the strongest coefficient being equal to the strongest FD component for each SD beam is close to 90%.

Figure 3: Probability of strongest FD component being the same as FD component of the strongest coefficient

# Conclusions

In this contribution, simulation results are provided for LC coefficient quantization. The observations made are summarized as follows.

Observation

- The performance-overhead trade-offs achieved by Alt1, Alt2’, and Alt4S are close; in particular, they are in the following order Alt2’ > Alt1 > Alt4S.

  - AltM performs poorly in low-overhead regime (for small β values), and performs competitively in high overhead regime (for large β values)

  - Alt4 with FD component “0”, on the other hand, can suffer from significant performance degradation if coefficients (hence the FD basis) associated with FD component “0” are always chosen

- For Alt4S, the probability of FD component of the strongest coefficient being equal to the strongest FD component for each SD beam is close to 90%

# References

- Chairman’s notes, RAN1#95

- R1-1903359, “CSI enhancement for MU-MIMO,” Samsung

- R1-1903343, “CSI Enhancement for MU-MIMO Support,” ZTE

# Appendix

Table 1: Simulation assumptions

| Parameter |  | Value |
| --- | --- | --- |
| Duplex, Waveform |  | FDD, OFDM |
| Multiple access |  | OFDMA |
| Scenario |  | Dense Urban (Macro only) |
| Frequency Range |  | FR1, 4GHz with 13 SBs and 40 MHz with 7 SBs |
| Inter-BS distance |  | 200m |
| Channel model |  | According to the TR 38.901 |
| Antenna setup and port layouts at gNB |  | 16 ports: (M, N, P, Mg, Ng, Mp, Np) = (8,4,2,1,1,2,4), (dH,dV) = (0.5, 0.8)λ |
| Antenna setup and port layouts at UE |  | 2RX: (M, N, P, Mg, Ng, Mp, Np) = (1,1,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ |
| BS Tx power |  | 41 dBm |
| BS antenna height |  | 25m |
| UE antenna height & gain |  | Follow TR36.873 |
| UE receiver noise figure |  | 9dB |
| Modulation |  | Up to 256QAM |
| Coding on PDSCH |  | LDPC Max code-block size=8448bit |
| Numerology | Slot/non-slot | 14 OFDM symbol slot |
|  | SCS | 15kHz |
| Number of RBs |  | 52 for 15 kHz SCS SB size = 4 and #SBs = 13 |
| Simulation bandwidth |  | 10 MHz,15kHz SCS |
| Frame structure |  | Slot Format 0 (all downlink) for all slots |
| MIMO scheme |  | SU/MU-MIMO with rank adaptation |
| MIMO layers |  | Up to 4 MU layers |
| CSI feedback |  | Feedback assumption - CSI feedback periodicity (full CSI feedback) : 5 ms, - Scheduling delay (from CSI feedback to time to apply in scheduling) : 4 ms |
| Overhead |  | DMRS, CSI-RS, PDCCH |
| Traffic model |  | FTP model 1 with packet size 0.5 Mbytes |
| Traffic load (Resource utilization) |  | 50% |
| UE distribution |  | 80% indoor (3km/h), 20% outdoor (30km/h) |
| UE receiver |  | MMSE-IRC |
| Feedback assumption |  | Realistic |
| Channel estimation |  | Realistic |
| Evaluation Metric |  | Throughput vs CSI feedback overhead (bits) |
| Baseline for performance evaluation |  | Rel-15 Type II Codebook |