3GPP TSG RAN WG1 Meeting #96	R1-1903376

Athens, Greece, February 25th – March 1st, 2019

Source:	Ericsson

Title:	On CSI enhancements for MU-MIMO

Agenda Item:	7.2.8.1

Document for:	Discussion and Decision

# Introduction

In RAN1 1901 AH meeting, much progress was achieved on the codebook design for Type II CSI enhancements. However, a number of open issues remain to be decided in this meeting.

According to FL summary, the following open issues shall be discussed at RAN1#96:

- Finalize incomplete agreements from RAN1 NR-AH 1901:

  - Subset selection for 1st layer (subset design, )

  - Values of M (down select) and N3 (down select for larger # units)

- Agree on (finalize) the following components for DFT-based compression:

  - LC coefficient quantization

  - Basis/coefficient subset selection for RI=2

- SLS comparison and discussion for Type II rank >2 extension

  - Categorize proposed schemes

In this contribution, we present our view on the above open issues. This is a revision of R1-1903038.

# Basis subset selection

In RAN1 1901 AH meeting, the following agreements where achieved regarding basis subset selection:

Agreement

On basis/coefficient subset selection for the first layer, the following is supported:

- Common selection for all beams with size-K0 subset of 2LM reported

  - The value of K0 is configured via higher-layer signaling

    - The number of reported non-zero coefficients can be smaller than or equal to K0

Agreement

On basis/coefficient subset selection for the first layer, support the following:

- Size-K0K0K0 subset design: down select in RAN1#96 from the following alternatives

  - Alt1. Unrestricted subset (size=2LM)

  - Alt2. Polarization-common subset (size=LM)

  - Alt3. Restricted subset (for a given subset of beams and FD basis, size=2L+M)

- The value of K0:  K0=β×2LMK0=β×2LM where two values of β are supported

  - Down select in RAN1#96 from  β∈18,14,12,34β∈18,14,12,34

- The UCI consists of two parts:

  - Information pertaining to the number(s) of non-zero coefficients is reported in UCI part 1

    - Note: This does not imply whether this information consists of single or multiple values

  - The payload of UCI part 1 remains the same for different RI value(s)

- Bitmap is used to indicate non-zero coefficient indices

There are thus two issues requiring downselection in RAN1#96, size-K0 subset design and the supported values of beta.

Related to the first issue is if the same or different basis vectors are selected for the different layers, as captured by the RAN1#95 agreement:

Agreement:

In RAN1 NR-AH 1901, companies are encouraged to evaluate the following alternatives for compression basis (WfWf) subset selection scheme across different layers when RI=2. Select one of the following alternatives in RAN1#96:

- Alt1. Basis subset selection (WfWf) for the 1st is the same as that for the 2nd layer

- Alt2. Basis subset selection (WfWf) for the 1st can be different from 2nd layer

Assume Rel.15 3-bit amplitude and Rel.15 8PSK co-phasing for W2W2 quantization for evaluation purposes.

In fact, the subset design issue and basis selection issues can be considered jointly, and we have the following alternatives:

| Scheme | Basis subset and size-K0 coefficient sub-selection |  |  |  |
| --- | --- | --- | --- | --- |
|  | Basis subset selection for layers | Size-K0 coefficient sub-selection |  | Rank-2 Overhead for basis and subset indication NΔ [bits] |
|  |  | Layer-wise | Polarization-wise |  |
| A | Independent | Independent | Independent | 2⋅( log2O3+log2N3M)+4LM= 156 |
| B | Independent | Independent | Common | 2⋅( log2O3+log2N3M)+2LM= 100 |
| C | Common | Independent | Independent | ( log2O3+log2N3M)+4LM=134 |
| D | Common | Independent | Common | ( log2O3+log2N3M)+2LM=76 |
| E | Common | Common | Independent | ( log2O3+log2N3M)+2LM=76 |
| F | Common | Common | Common | ( log2O3+log2N3M)+LM=48 |

The total overhead is Ntot=Nbasic+NΔ, where Nbasic=231 bits for the considered systems, which is:

- L=4 beams

- M=7 FD-components

- R=1

- NSB=N3=13

- K0 =142LM=14 non-zero coefficients

The performance vs overhead trade-off for the different schemes are presented in Figure 1, where the average performance gain w.r.t Type I CSI is presented.

Figure 1: Performance of basis subset selection scheme

As can be observed from the Figure, the different schemes have different performance/overhead tradeoff, and so, it is difficult to decide what overhead is tolerable and which scheme should be selected.  However, we make these observations from the results:

Layer-common basis selection gives only a small performance loss compared layer-independent basis selection (and actually a small gain when polarization-common subset selection is used)

Similar rank-2 overhead for Schemes D & E but substantially larger performance for Scheme D

Common coefficient selection for layers give poor performance

Based on these observations, we can conclude that layer-common basis selection likely is a good design choice.

Support layer-common basis subset selection

The next issue is to determine whether subset selection should be completely independent, or one of polarization-common or layer-common. Based on the observations, we see that layer common coefficient subset selection performs poorly and should not be considered for the enhancement. Regarding polarization-common subset, even though some overhead can be saved with polarization-common subset selection compared to independent subset selection, the performance loss may be too large. The subset selection implementation at the UE also becomes more complex if polarization-common subset is used, which should be considered. Another thing to keep in mind is that simplistic polarization and antenna models which do not necessarily reflect a realistic antenna at the UE are used in RAN1. That is, the simulated antenna patterns may have much more similar spatial/polarization properties than a realistic antenna, implying that the performance of polarization-common subset selection may be lower in practice.

Support layer-independent and polarization independent coefficient selection

## Overhead calculation

For reference, we provide the assumed overhead calculation for the proposed schemes below.

- SB-basis overhead (for all layers)

  - L-beam selection: log2N1N2L bits

  - Rotation: log2O1O2=log216=4 bits

- FD-basis overhead (per layer or joint across layers)

  - Rotation:  log2O3 bits

  - Basis indication: log2N3M bits

- Coefficient feedback

  - WB Amplitude (assuming quantization Alt 2A):

    - Strongest coefficient indicator: log22L-1 bits

    - WB amplitude coefficients: 3⋅2L-1 bits

  - ”SB” phase and amplitude

    - Number of non-zero coefficient indicator: log2K0 bits

    - Subset indication:

      - Layer indep & pol indep:		rank⋅2LM bits

      - Layer indep & pol common:  	rank⋅LM bits

      - Layer common & pol indep:   	2LM bits

      - Layer common & pol common:	LM bits

    - Coefficients: 6K1 bits

## Value of K0

The other open issue is the value of K0 which needs to be settled. To evaluate this, we consider the following setup:

- Sweep K0=β×2LM, β={18,14,12,34}

- L={2, 4} beams, M={4, 7} FD-components

- 10 MHz BW, R=2, N3=26

- Independent basis/coefficient selection for layer/pol

The results are provided in Figure 2 below where each point on a curve corresponds to one β value. We first observe that in general

β=34 results in too high overhead which does not meet overhead reduction targets. Furthermore, too high value of β does not yield good performance/overhead tradeoff as many zero coefficients are included in the report

On a similar note, β=18 only includes around one coefficient per beam/polarization in most cases and so has a difficult in capturing any beam which experiences delay spread. This results in too low performance.

β=18 results in too low performance gain

On the other hand, the remaining two values of β seem to result in good performance/overhead tradeoff. Therefore, it is proposed that:

The allowed values of β is ½ and ¼

Figure 2: Performance vs overhead for different values of K0

# Quantization of LC coefficients

In RAN1#95, the following agreements were reached regarding LC coefficient quantization:

Agreement:

For each layer, the following alternatives for quantizing each of the coefficients in are to be studied for down selection in RAN1#96:

- Alt1A. Rel.15 3-bit amplitude; Rel.15 QPSK and 8PSK co-phasing

- Alt1B. Rel.15 3-bit amplitude; Rel.15 QPSK, Rel.15 8PSK, and new 16PSK co-phasing

- Alt2A. Rel.15 3-bit wideband amplitude for each beam, 2/3-bit differential amplitude for FD coefficients; Rel.15 QPSK and 8PSK co-phasing

- Alt2B. Rel.15 3-bit wideband amplitude for each beam, 2/3-bit differential amplitude for FD coefficients; Rel.15 QPSK, Rel.15 8PSK, and new 16PSK co-phasing

- Alt2C. Rel.15 3-bit wideband amplitude + Rel.15 QPSK and 8PSK wideband co-phasing for each beam, 2/3-bit differential amplitude and co-phasing for FD coefficients;

- Alt3. A-bit amplitude for each of 2L beams, B-bit amplitude for each of M FD components, 1-bit differential amplitude and 8PSK co-phasing for each of the 2LM FD coefficients

- Alt4. For each beam,

  - B0-bit amplitude and C0-bit phase for coefficients for the P0 strongest coefficients,

  - B1-bit amplitude and C1-bit phase for coefficients for the P1 2nd strongest coefficients, …

  - …

  - BQ-1-bit amplitude and CQ-1-bit phase for coefficients for the PQ-1 Qth strongest coefficients

  - Alternatively, amplitude/phase can be replaced with real/imaginary

- Alt5. Special case of Alt4: Q=2, B0=C0=3; B1=C1=2 on amplitude/phase

In our understanding, the coefficient calculation procedure for the different alternatives can be summarized as follows.  In all alternatives, the feedback coefficient cl,m for each SD- and FD-basis vector pair is calculated based on the unquantized coefficient  cl,m.

In Alt 1, the unquantized coefficients are first normalized with the strongest coefficient in the matrix cl,m/maxl,m |cl,m| and then the amplitude and phase of cl,m/maxl,m |cl,m| is quantized directly resulting in cl,m=pl,mφl,m. This is a quite simple scheme but may have issues with dynamic range of the amplitudes, since moderate taps of the moderate beams may lie outside of the dynamic range if normalization is done by the strongest coefficient.

To solve this issue, Alt 2 was proposed, where the amplitude coefficient is split into an per-beam average (“wideband”) amplitude pl1 and a per-coefficient amplitude pl,m(2) as  cl,m=1m=0M-1pl,m2 2 pl1 pl,m(2)φl,m. The WB amplitude per beam pl1 can be calculated exactly as for Rel-15 type II CSI, and to calculate pl,m(2), a row-wise normalization of the coefficient matrix can be done, so that the amplitude of the strongest coefficient for each beam is set to one and the remaining coefficients are quantized with respect to the strongest beam. By normalizing the coefficients withm=0M-1pl,m2 2 it is assured that each row of the coefficient matrix is normalized to the WB power of the beam.

Alt 3 can be seen as to be based on a Kronecker product between a beam amplitude vector and a FD-basis amplitude vector, with a bitmap in between where the K0 non-zero coefficient are selected.

Based on offline discussions, some alternatives have further been merged together or further refined. Based on our understanding, the following proposals are being discussed.

Alt 2M:

- UE reports the following amplitudes of the coefficients in W2, for each spatial beam

- The amplitude of the strongest coefficient per beam

  - Quantized with A=4 bits: {1,1214,1414,1814,…,121414 , 0}

- The differential amplitudes of the coefficients w.r.t. the strongest coefficient in this beam

  - Quantized with B bits. B = 2 or 3, which is configurable

    - For B=2, the alphabet is {1,12,14,122}

    - For B=3, the alphabet is {1,12,12,122,14,142,18,182}

- UE reports the phases of the coefficients in W2.

  - Phases are quantized with C = 3 bits (8PSK) or 4 bits (16PSK), which is configurable

Alt 4M:

Denote the LC coefficient associated with beam l ∈0,1,…,2L-1 and frequency unit m ∈0,1,…,M-1 as cl,m, and the strongest coefficient (out of the KNZ non-zero coefficients reported using the bitmap) as cl*,m*. The non-zero LC coefficients are quantized and reported as follows:

- cl*,m*=1: amplitude and phase are not reported. Instead, a log2KNZ-bit indicator for the strongest coefficient index (l*,m*) is reported

- cl,m*, l≠l*: 4-bit amplitude and 16PSK phase

  - Value set for the 4-bit amplitude: {1,1214,1224,1234,…, 12144,0} [-1.5dB step size]

- cl,m, m≠m*: 3-bit amplitude (using Rel.15 Type II amplitude value set), and either 8PSK or 16PSK phase (configurable)

## Comparison between the different schemes

To compare the different quantization schemes, we present evaluate using the following setup:

- Sweep K0=β×2LM, β={14,12,34}

- L=4 beams, M=7 FD-components

- 10 MHz BW, R=1, N3=13

- Independent basis/coefficient selection for layer/pol

In Figure 3, NFB evaluations are presented plotted against the rank-2 overhead. An immediate observation is that the different schemes perform roughly the same in the low overhead region but start to diverge in the high overhead region.

For β=1/4, all quantization schemes perform roughly the same

For β=12 and 3/4, the Alt 2M has superior performance

Figure 3: Performance of different quantization schemes

Therefore, Alt 2M is preferred.

For LC quantization, support Alt2M

# Segmentation or padding for larger number of FD-units

In RAN1 AH 1901, it was decided to, for larger number of FD-units, use a different approach for performing the FD DFT. Either by padding the subbands so that the number of FD-units are a multiple of 2,3, or 5, or to segment the bandwidth into two segments. There was also to alternatives for how to decide the number of FD-units M.

Agreement

Two values of M are supported. In RAN1#96, down select between the following alternatives (p∈14,12p∈14,12):

- Alt1. M=p×N3RM=p×N3R

- Alt2. M=p×N3M=p×N3

- FFS: support for p=1/8 and/or p=3/4 in addition to 1/4 and 1/2

Agreement:

The value of M is higher-layer configured

- FFS: Whether UE reporting smaller value of M (in addition to the configured M) is supported

Agreement

Values of N3: For R∈1,2R∈1,2 and NSB is # CQI subbands, when NSB×R≤19NSB×R≤19,

Values of N3: For R∈1,2R∈1,2 and NSB is # CQI subbands, when NSB×R>19NSB×R>19, downselect among the following alternatives in RAN1#96

- Alt1: N3N3N3 is smallest multiple of 2, 3, or 5 which is  ≥NSB×R≥NSB×R

- Alt2: N3N3N3 is a multiple of 2, 3, or 5. Segment into 2 parts with overlapping between 2 parts. Note: no padding is needed to align the DFT size with the multiple of 2, 3, or 5

For further study for evaluation of MU-CSI

Use the following criteria and guideline to compare Alt1 and Alt2 for down selection of N3 when NSB×R>13NSB×R>13.

Table 1 Criteria for evaluation

| Criteria | Comment |
| --- | --- |
| Performance-overhead tradeoff (UPT vs. overhead) | - On Alt1, whenever padding is necessary, companies should describe their (assumed) padding scheme/algorithm. - On Alt2, companies should describe their (assumed) segmentation scheme. - For both alternatives, gNB receiver operation (e.g. how to handle padding or segmentation and the associated precoders/PMIs) can also be stated. - Bandwidth = 20 MHz and/or 50 MHz, 30kHz SCS, R=1 and/or 2, different N3 values (ranging from small to large) |
| Specification and/or implementation complexity | - Additional specification components, e.g. for Alt2, consider the benefit(s) over using 2 reporting settings - Additional effort to implement PMI search algorithm and UCI processing |

In our understanding, the two alternatives of the value of M are coupled with the value of N3, i.e. if segmentation is used or not. If segmentation is not used, the maximum number of PMI subbands is N3=RmaxNSB,max=2⋅18=36 which would imply that either M=5,9 is selected if Alt 1 is assume, or M={9,18} if Alt 2 is used. Clearly, it would be difficult to achieve either overhead reduction or good performance/overhead tradeoff if 18 FD-components are used since the bitmap for coefficient sub-selection alone would constitute 2*4*18=144 bits. However, if segmentation is applied, according to the agreed expressions for calculating the codebook parameters, N3 is the number of PMI subbands in each segment, which in this example would be N3=18 which would result in M={3,5} for Alt 1 or  M={5,9} for Alt 2. I this case Alt 1 would result in too small FD-basis.

The number of basis vectors equal to  is appropriate if segmentation is not used and  is appropriate if segmentation is used

Thus, in order to determine the value of M, we first need to determine if padding or segmentation approach shall be used when .

To evaluate the two alternatives, we set up the following scenario.

For “20 MHz” and 30kHz SCS, we consider R=2 and 42 PRB CSI reporting band definition and evaluate 3 systems:

- No DFT padding:

  - N3=21 FD-units, which implies M=p×N3R =6

- DFT padding

  - N3=24 FD-units, which implies M=p×N3R =6

  - Last subband is duplicated 3 times to pad the DFT

- Segmentation

  - Two segments, using N3=10 and N3=11 FD-units, respectively

  - M=p×N3 =5 and M=p×N3 =6, respectivly

The value of K0 is swept for each scheme, taking the values K0=β×2LM, β={18,14,12}.

The results are presented in Error! Reference source not found., here the relative performance versus Rel-15 Type II with L=2 is shown plotted against the rank-2 overhead. Compared to “No DFT padding”, i.e. using the same approach as for <= 13 FD-units, the DFT padding approach results in some performance loss, around 2%. To us it is not clear that the resulting performance loss is proportionate to the reduction in UE complexity (which should be quite limited). For the segmentation approach, it is difficult to find good overhead / performance tradeoff, since two sets of coefficients needs to be include din the report instead of one, which essentially double the overhead for a given value of K0. To reach reasonable overhead, K0 needs to be configured with a smaller value. However, compared to the approaches without segmentation, the performance is lower for a given overhead and so the segmentation approach does not seem favorable, especially taking into account that it may increase UE complexity

DFT padding results in around 2% loss compared to No DFT padding

Segmentation approach has inferior performance/overhead tradeoff compared to approaches without segmentation

Figure 4: Performance vs overhead for padding and segmentation approaches

Based on the above observations, if downselection needs to be made, the DFT padding approach is the only viable choice. However, it would be more preferable if no DFT padding needs to be applie din order to maximize the attainable performance with Rel-16 Type II.

When NSB×R>19NSB×R>19,support either free selection of N3  or selecting N3N3N3 as the smallest multiple of 2, 3, or 5 which is  ≥NSB×R≥NSB×R

# Conclusion

Based on the discussion in this contribution we make these proposals:

Proposal 1	Support layer-common basis subset selection

Proposal 2	Support layer-independent and polarization independent coefficient selection

Proposal 3	The allowed values of β is ½ and ¼

Proposal 4	For LC quantization, support Alt2M

Proposal 5	When NSB×R>19,support either free selection of N3  or selecting N3N3 as the smallest multiple of 2, 3, or 5 which is  ≥NSB×R

# Appendix

Table 1: SLS assumptions for CSI enhancement

| Parameter | Value |
| --- | --- |
| Duplex, Waveform | FDD, OFDM |
| Scenario | Dense Urban (Macro only) |
| Carrier frequency | 4GHz |
| Inter-site distance | 200m |
| Channel model | According to the TR 38.901 |
| Antenna setup and port layouts at gNB | 32 ports: (8,4,2,1,1,4,4), (dH,dV) = (0.5, 0.8)λ 100 deg tilt |
| Antenna setup and port layouts at UE | 2RX: (1,1,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ for (rank 1,2) |
| BS Tx power | 41 dBm |
| BS antenna height | 25m |
| UE antenna height & gain | Follow TR36.873 |
| UE receiver noise figure | 9dB |
| Modulation | Up to 256QAM |
| Coding on PDSCH | LDPC Max code-block size=8448bit |
| Simulation bandwidth | Case 1: 100 MHz with 30kHz SCS Case 2: 10 MHz with 15kHz SCS |
| Frame structure | Slot Format 0 (all downlink) for all slots |
| MIMO scheme | SU/MU-MIMO with rank adaptation |
| MIMO layers | Maximum 8 layers |
| CSI feedback | - CSI feedback periodicity (full CSI feedback) : 5 ms, - Scheduling delay (from CSI feedback to time to apply in scheduling) : 4 ms |
| Overhead | Up to 2 port DMRS (pseudo-orthogonal DMRS ports used) CSI-RS overhead included TRS overhead included |
| Traffic model | FTP model 1 with packet size 0.5 Mbytes |
| Traffic load (Resource utilization) | - 70 % for CSI overhead reduction |
| UE distribution | - 80% indoor (3km/h), 20% outdoor (30km/h) |
| UE receiver | MMSE-IRC as the baseline receiver |
| Feedback assumption | Realistic |
| Channel estimation | Realistic |