3GPP TSG RAN WG1 Meeting 96	Tdoc R1-1903449

Athens, Greece, February 25 – March 1, 2019

Agenda Item:	7.2.6.4

Source:	Ericsson

Title:	Evaluation of Rel-15 Enabled URLLC Use Cases

Document for:	Discussion, Decision

# 1	Introduction

In RAN1#94bis [1], requirements and simulation assumptions for different URLLC use cases are defined.

Rel-15 enabled use case is one of the use cases with two sets of requirements, one for small packet size with strict reliability and latency requirements and another for large packet size with more relaxed requirements. In this contribution, we evaluate the Rel-15 enabled use case using the stricter requirement, i.e., reliability of 99.999% within 1ms latency for a small packet size of 32 bytes. We note that these requirements are also aligned with the ITU IMT 2020 reliability requirement [2].

# 2	Discussion

Reliability and latency requirements for Rel-15 enabled use case defined in [1] are shown in Table 1 below. In the following, we focus on the reliability requirement of 99.999% within 1ms latency for a small packet of 32 bytes.

Table 1. Requirements for Rel-15 enabled use case.

| Use case (Clause #) | Reliability (%) | Latency (ms) | Data packet size and traffic model |
| --- | --- | --- | --- |
| Rel-15 enabled use case (e.g. AR/VR) | 99.999 | 1ms (air interface delay) for 32 bytes 1 ms and 4 ms (air interface delay) for 200 bytes | DL & UL: 32 and 200 bytes FTP model 3 or periodic with different arrival rates |
|  | 99.9 | 7ms (air interface delay) | DL & UL: 4096, 10 K FTP model 3 or periodic with different arrival rates |

For calibration purposes first, we present DL and UL geometry in the Appendix A (Figure A-1) derived based on assumption from Table B-1 (appendix B). Moreover, the link level simulation results are presented in figures A-2 and A-3 of Appendix A for information.

## 2.1	System-Level Simulations (SLS)

We perform system level simulation based on the assumptions outlined in [1] and also shown in Table B-3 in appendix to this paper. The requirements here are 1 ms latency and 99.999% reliability. In addition to agreed parameters we use:

- Mini-slot length 7 OS.

- Periodic traffic with packet periodicity 1 ms

- Grant-based UL (but not SR-based)

Table 1. Percentage of users fulfilling reliability and latency requirements with given resource utilization for 4TX/4RX ports gNB antenna configuration.

| Number of UEs per cell (average) | 5 |  | 10 |  |
| --- | --- | --- | --- | --- |
|  | UE% | RU% | UE% | RU% |
| Uplink | 95.3% | 5.8% | 90.8% | 15.5% |
| Downlink | 99.3% | 8.6% | 96.3% | 16.9% |

Table 2. Percentage of users fulfilling reliability and latency requirements with given resource utilization for 8TX/8RX ports gNB antenna configuration.

| Number of UEs per cell (average) | 5 |  | 10 |  |
| --- | --- | --- | --- | --- |
|  | UE% | RU% | UE% | RU% |
| Uplink | 97.7% | 4.4% | 97.3% | 9.9% |
| Downlink | 98.7% | 8.6% | 95.4% | 16.9% |

The slight lower DL performance for 8TX/8RX ports may be due to statistical variations but also that scheduler prioritizes UL PDCCH allocation to improve UL performance. For 8TX/8RX, DL can reach 99.7% and 99.3% with just 0.1 ms relaxation of latency requirements. With CG PUSCH the competition of PDCCH resources will reduce and DL performance will likely improve.

- The reliability target of 99.999% (total BLER of 1E-5) can be met for more than 95% users with 5 URLLC UEs/cell for 4TX/4RX  and for more that 95% users with both 5 and 10 URLLC UEs/cell for 8TX/8RX having simulation assumption in Table B-3.

- URLLC Rel-15 enabled use case requirement can be met, i.e., 1E-5 block error rate within 1ms for a packet size of 32 bytes.

# Conclusion

In section 2 we made the following observations:

Observation 1	The reliability target of 99.999% (total BLER of 1E-5) can be met for more than 95% users with 5 URLLC UEs/cell having simulation assumption in Table B-3.

Observation 2	URLLC Rel-15 enabled use case requirement can be met, i.e., 1E-5 block error rate within 1ms for a packet size of 32 bytes.

# References

- 3GPP RAN1#94bis Chairman note

- 5D/TEMP/216-E, Preliminary draft new report ITU-R M., [IMT-2020.TECH PERF REQ], ITU-R Working Party 5D, (WP5D-TD-0300).

- 3GPP TS 38.214 V15.3.0

- R1-1900175 Latency Evaluation of Rel-15 URLLC, Ericsson, January 2019

# Appendix A

Figure A-1. Full buffer DL and UL SINR distribution for URLLC Rel-15 enabled use case.

For macro scenario, the 5th%-tile SINR is typically considered to representing a cell-edge user SINR. Here the 5th%-tile SINR is 1.69 dB (DL) and 0.92 dB (UL).

We evaluate link level performance for Urban macro scenario using the assumptions in Table B-2. For PDCCH a DCI of size 40 bits excluding CRC is assumed. For PUCCH we have assumed 1-bit UCI carried by PUCCH format 0 with 2os duration and frequency hopping (evaluated at 1% D2A rate). For data, different MCSs are evaluated for a small packet size of 32 bytes transmitted over 4os PDSCH/PUSCH.

The resulting BLER and error rate as a function of SNR for the DL and UL control channels are shown in Figure A-2, and for the data channels in Figure A-3.

Figure A-2. PUCCH format 0 error rate and PDCCH BLER as function of SNR.

Figure A-3. BLER for 4OS-data with different MCSs (from Table 5.1.3.1-3 [3]) as function of SNR.

# Appendix B

Table B-1. Assumptions for the system-level simulations for Rel-15 enabled use case (Urban macro).

| Parameters | Value |
| --- | --- |
| Layout | Single layer - Macro layer: Hex. Grid |
| Inter-BS distance | 500m |
| Carrier frequency | 4 GHz |
| Channel model | UMa in TR 38.901 |
| UE Tx power | 23dBm |
| BS antenna configurations | 16Tx/16Rx ports (M, N, P, Mg, Ng; Mp, Np) = (8, 8, 2, 1, 1; 1, 8), dH = 0.5λ, dV = 0.8λ; 9 degrees electrical antenna tilt |
| BS antenna height | 25m |
| BS antenna element gain + connector loss | 8 dBi |
| BS receiver noise figure | 5dB |
| UE antenna configuration | 2 Tx/4 Rx antenna ports |
| UE antenna height | Follow the modelling of TR 38.901 (e.g. 1.5m) |
| UE antenna gain | 3dBi |
| UE receiver noise figure | 9 dB |
| Total transmit power per TRxP | 49 dBm |
| Simulation bandwidth | 40 MHz DL and 40 MHz UL |
| Duplex mode | FDD |
| SCS | 30 kHz |
| UE distribution | 80% of users are outdoors and 20% of users are indoors Indoor penetration loss is modelled according to low loss model |
| Traffic | Full buffer |

Table B-2. Assumptions for the link-level simulations for Rel-15 enabled use case (Urban macro).

| Parameter | Value |
| --- | --- |
| Carrier frequency for evaluation | 4GHz |
| Channel model | TDL-C (delay spread: 300ns) as in 38.901 |
| UE speed | 3 km/h for Rel-15 enabled use case |
| BS antenna configuration | 2 Tx/4 Rx antenna ports |
| UE antenna configuration | 2 Tx/4 Rx antenna ports |
| System bandwidth | 40 MHz |
| Sub-carrier spacing | 30 kHz |
| Channel estimation | Practical |
| Receiver type | MMSE |

Table B-3: System level simulation assumption

| Parameters | Value |
| --- | --- |
| Layout | Single layer - Macro layer: Hex. Grid |
| Inter-BS distance | 500m |
| Carrier frequency | 4 GHz |
| Channel model | UMa in TR 38.901 |
| UE Tx power | 23dBm |
| BS antenna configurations | 4Tx/4Rx ports: (M, N, P, Mg, Ng; Mp, Np) = (8, 2, 2, 1, 1; 1, 2), 8Tx/8Rx ports: (M, N, P, Mg, Ng; Mp, Np) = (8, 4, 2, 1, 1; 1, 4), dH = 0.5λ, dV = 0.8λ; 3 degrees electrical antenna tilt |
| BS antenna height | 25m |
| BS antenna element gain + connector loss | 8 dBi |
| BS receiver noise figure | 5dB |
| UE antenna configuration | 2 Tx/4 Rx antenna ports |
| UE antenna height | Follow the modelling of TR 38.901 (1.5 m) |
| UE antenna gain | 0dBi |
| UE receiver noise figure | 9 dB |
| Total transmit power per TRxP | 49 dBm |
| Simulation bandwidth | 40 MHz DL and 40 MHz UL |
| Duplex mode | FDD |
| SCS | 30 kHz |
| UE distribution | 80% of users are outdoors and 20% of users are indoors |
| UE speed | 3 km/h |
| UE power control | alpha = 0.8, target SINR = 20 dB |
| HARQ/repetition | DL and UL: 7os (one-shot) |
| Channel estimation | Ideal |
| Traffic | Periodic, 1 ms packets periodicity, 32 Bytes TBS. |