3GPP TSG RAN WG1 Meeting #96	R1-1903477

# Athens, Greece, February 25th- March 1st, 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.213 | CR | 1229 | rev | - | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of SUO Case 1 for NE-DC as Rel-15 late drop |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-02-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | It has been agreed that Rel-15 NE-DC supports SUO Case 1 that have been defined for EN-DC, and functionality for EN-DC is reused for NE-DC. The UE behaviour for NE-DC SUO Case 1 should be captured into the specification. This CR is to capture the functionalities related to NE-DC as part of Rel-15 late drop. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce SUO Case 1 for NE-DC in TS 36.213. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The SUO Case 1 is not supported by NE-DC UE. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.2, 5.1, 6, 7.1, 7.3, 8, 8.3, 9, 10 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Isolated Impact Analysis: This CR captures an agreed UE feature of Rel-15 late drop into the specification. |  |  |  |  |  |  |  |  |

<Unchanged parts are omitted>

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. 
An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

ACK	Acknowledgement

AUL	Autonomous Uplink

AUL-DFI	AUL downlink feedback information

BCH	Broadcast Channel

CCE	Control Channel Element

CDD	Cyclic Delay Diversity

CG	Cell Group

CIF	Carrier Indicator Field

CQI	Channel Quality Indicator

CRC	Cyclic Redundancy Check

CRI	CSI-RS Resource Indicator

CSI	Channel State Information

CSI-IM	CSI-interference measurement

DAI	Downlink Assignment Index

DC	Dual Connectivity

DCI	Downlink Control Information

DL	Downlink

DL-SCH	Downlink Shared Channel

DTX	Discontinuous Transmission

EDT	Early Data Transmission

EN-DC	E-UTRA NR Dual Connectivity with MCG using E-UTRA and SCG using NR

EPDCCH	Enhanced Physical Downlink Control Channel

EPRE	Energy Per Resource Element

MCG	Master Cell Group

MCS	Modulation and Coding Scheme

NACK	Negative Acknowledgement

NE-DC 	NR E-UTRA Dual Connectivity with MCG using NR and SCG using E-UTRA

NPBCH	Narrowband Physical Broadcast CHannel

<Unchanged parts are omitted>

## 5.1	Uplink power control

<Unchanged parts are omitted>

For a UE configured with EN-DC/NE-DC and serving cell frame structure type 1, if the UE is configured with subframeAssignment-r15 for the serving cell, the UE is not expected to transmit any uplink physical channel or signal in the serving cell on subframes other than offset-UL subframes, where the offset-UL subframes are determined by applying an offset value given by harq-Offset-r15 to the subframes denoted as uplink in the UL/DL configuration subframeAssignment-r15.

<Unchanged parts are omitted>

# 6	Random access procedure

If the UE is configured with a SCG, the UE shall apply the procedures described in this clause for both MCG and SCG

-	When the procedures are applied for MCG, the terms 'secondary cell', 'secondary cells', 'serving cell', 'serving cells' in this clause refer to secondary cell, secondary cells, serving cell, serving cells belonging to the MCG respectively.

-	When the procedures are applied for SCG, the terms 'secondary cell', 'secondary cells', 'serving cell', 'serving cells' in this clause refer to secondary cell, secondary cells (not including PSCell), serving cell, serving cells belonging to the SCG respectively. The term 'primary cell' in this clause refers to the PSCell of the SCG

For a UE configured with EN-DC/NE-DC and serving cell frame structure type 1, if the UE is configured with subframeAssignment-r15 for the serving cell, the UE is not expected to transmit any uplink physical channel or signal in the serving cell on subframes other than offset-UL subframes, where the offset-UL subframes are determined by applying an offset value given by harq-Offset-r15 to the subframes denoted as uplink in the UL/DL configuration subframeAssignment-r15.

<Unchanged parts are omitted>

## 7.1	UE procedure for receiving the physical downlink shared channel

<Unchanged parts are omitted>

For a UE configured with EN-DC/NE-DC and serving cell frame structure type 1, if the UE is configured with subframeAssignment-r15 for the serving cell, the UE is not expected to receive PDSCH data transmissions signalled via PDCCH/EPDCCH in common search space for which HARQ-ACK response shall be provided in a subframe n, and PDSCH data transmissions signalled via PDCCH/EPDCCH in UE specific search space for which HARQ-ACK response shall be provided in the same subframe n.

<Unchanged parts are omitted>

## 7.3	UE procedure for reporting HARQ-ACK

<Unchanged parts are omitted>

For a UE configured with EN-DC/NE-DC and serving cell frame structure type 1, if the UE is configured with subframeAssignment-r15 for the serving cell, UE procedure for HARQ-ACK reporting for the serving cell is given in Subclause 7.3.4 assuming primary cell frame structure type 2 with "UL/DL configuration" given by subframeAssignment-r15. The UE shall apply an offset value given by harq-Offset-r15 to the subframe index in the UL/DL configuration for determining the HARQ-ACK reporting for the serving cell. The UE is not expected to transmit any uplink physical channel or signal in the serving cell on subframes other than the offset-UL subframes, where the offset-UL subframes are determined by applying the offset value to the subframes denoted as uplink in the UL/DL configuration. For a PDSCH data transmissions signalled via PDCCH/EPDCCH in common search space for which HARQ-ACK response shall be provided, the UE shall assume the value of the DAI field in the corresponding DCI format is equal to '1'.

For a UE configured with EN-DC/NE-DC and more than one serving cells with primary cell frame structure type 1, if the UE is configured with subframeAssignment-r15 for the primary cell, UE procedure for HARQ-ACK reporting of each secondary cell follows the procedure of the primary cell.

<Unchanged parts are omitted>

# 8	Physical uplink shared channel related procedures

<Unchanged parts are omitted>

For a UE configured with EN-DC/NE-DC and serving cell frame structure type 1, if the UE is configured with subframeAssignment-r15 for the serving cell, the UE is not expected to transmit any uplink physical channel or signal in the serving cell on subframes other than offset-UL subframes, where the offset-UL subframes are determined by applying an offset value given by harq-Offset-r15 to the subframes denoted as uplink in the UL/DL configuration subframeAssignment-r15.

For a UE configured with EN-DC/NE-DC and serving cell frame structure type 1, if the UE is configured with subframeAssignment-r15 for the serving cell, the UE is not expected to be configured with more than one serving cells in the uplink.

<Unchanged parts are omitted>

## 8.3	UE HARQ-ACK procedure

<Unchanged parts are omitted>

For a UE configured with EN-DC/NE-DC and serving cell frame structure type 1, if the UE is configured with subframeAssignment-r15 for the serving cell, the UE is not expected to receive PHICH corresponding to a transport block on the serving cell.

<Unchanged parts are omitted>

# 9	Physical downlink control channel procedures

If the UE is configured with a SCG, the UE shall apply the procedures described in this clause for both MCG and SCG

-	When the procedures are applied for MCG, the terms 'secondary cell', 'secondary cells' , 'serving cell', 'serving cells' in this clause refer to secondary cell, secondary cells, serving cell, serving cells belonging to the MCG respectively.

-	When the procedures are applied for SCG, the terms 'secondary cell', 'secondary cells', 'serving cell', 'serving cells' in this clause refer to secondary cell, secondary cells (not including PSCell), serving cell, serving cells belonging to the SCG respectively. The term 'primary cell' in this clause refers to the PSCell of the SCG.

If a UE is configured with a LAA Scell, the UE shall apply the procedures described in this clause assuming frame structure type 1 for the LAA Scell unless stated otherwise.

For a UE configured with EN-DC/NE-DC and more than one serving cells with primary cell frame structure type 1, if the UE is configured with subframeAssignment-r15 for the primary cell, the UE is not expected to be configured with carrier indicator field in any of the serving cells.

<Unchanged parts are omitted>

# 10	Physical uplink control channel procedures

<Unchanged parts are omitted>

For a UE configured with EN-DC/NE-DC and serving cell frame structure type 1, if the UE is configured with subframeAssignment-r15 for the serving cell, the UE shall apply the procedures described in this clause assuming FDD-TDD and primary cell frame structure type 2 with "UL/DL configuration" given by subframeAssignment-r15. The UE shall apply an offset value given by harq-Offset-r15 to the subframe index in the UL/DL configuration when applying the procedures in this clause. The UE is not expected to transmit any uplink physical channel or signal in the serving cell on subframes other than the offset-UL subframes, where the offset-UL subframes are determined by applying the offset value to the subframes denoted as uplink in the UL/DL configuration. The UE is configured by higher layers to use PUCCH format 3/4/5 for transmission of HARQ-ACK. The UE shall use  and  (defined in Subclause 10.1.3.2.2/10.1.3.2.3/10.2.3.2.4) when applying the procedures described in Subclause 10.1.3.2.2/10.1.3.2.3/10.2.3.2.4.

<Unchanged parts are omitted>