3GPP TSG RAN WG1 Meeting #96 R1-1903609 Athens, Greece, 25th February – 1st March, 2019 CR-Form-v11.2. CHANGE REQUEST. 38.213; CR; DRAFT; rev; -; Current version:; 15.4.0. For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests.. Proposed change affects:; UICC apps; ME; X; Radio Access Network; X; Core Network. Title:; Draft CR on identifying transmission occasion after resetting a PC closed loop. Source to WG:; ZTE. Source to TSG:; RAN1. Work item code:; NR_newRAT-Core; Date:; 2019-2-15. Category:; F; Release:; Rel-15. Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900.; Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16). Reason for change:; In TS38.213, if P0 or alpha which is related to j is re-configured by gNB, UE shall reset closed-loop power control value f(i=0, l)=0, where l and j are associated with the same SRI. f(i, l) is maintained independently for each of closed loop l, but uplink transmission occasion i is shared among all PUSCH transmissions in one BWP. If only one closed-loop power control value, say l=0, needs to be reset, it should be f(i, l=0)=0 rather than f(i=0, l=0)=0. - In this case, if transmission occasion index i is also reset, it also affects the other closed loop process f(i, l=1). There is no description about how to determine the value of f(i=0, l=1) in TS 38.213. Straightforwardly, f(i, l=1) should maintanin its historical value and is decoupled with the resetting process of f(i, l=0). - Besides, even for closed loop l=0, resetting the transmission occasion i is still meaningless. Therefore it is proposed to remain the transmission occasion index i when resetting f(i,l). PUCCH transmission also support up to 2 closed-loop power control loops. So the similar changes are recommended. SRS transmission only supports 1 closed-loop power control loop, but for consistency, the transmission index is suggested not to be reset when resetting h(i).. Summary of change:; When resetting closed-loop power control value f(i,l),g(i,l) or h(i), do not reset transmission index i to 0, and just keep it as naturally accumulated.. Consequences if not approved:; In case of two closed-loop power control loops, resetting the transmission occasion index i of one closed loop causes unclear procedure for the other loop.. Clauses affected:; 7, 7.1.1, 7.2.1, 7.3.1. Y; N. Other specs; X; Other core specifications; TS/TR ... CR .... affected:; X; Test specifications; TS/TR ... CR .... (show related CRs); X; O&M Specifications; TS/TR ... CR .... Other comments:.

### 7.1.1	UE behaviour

<Unchanged part omitted>

-	A UE resets accumulation of a PUSCH power control adjustment state  for active UL BWP  of carrier  of serving cell  to

-	If a configuration for a corresponding  value is provided by higher layers

-	If a configuration for a corresponding  value is provided by higher layers

-	If  and the PUSCH transmission is scheduled by a DCI format 0_1 that includes a SRI field, and the UE is provided higher SRI-PUSCH-PowerControl, the UE determines the value of  from the value of  based on an indication by the SRI field for a sri-PUSCH-PowerControlId value associated with the sri-P0-PUSCH-AlphaSetId value corresponding to  and with the sri-PUSCH-ClosedLoopIndex value corresponding to

-	If  and the PUSCH transmission is scheduled by a DCI format 0_0 or by a DCI format 0_1 that does not include a SRI field or the UE is not provided SRI-PUSCH-PowerControl,

-	If ,  is provided by the value of powerControlLoopToUse

<Unchanged part omitted>

### 7.2.1	UE behaviour

<Unchanged part omitted>

-	If a configuration of a  value for a corresponding PUCCH power control adjustment state  for active UL BWP  of carrier  of serving cell  is provided by higher layers,

-

If the UE is provided PUCCH-SpatialRelationInfo, the UE determines the value of  from the value of  based on a pucch-SpatialRelationInfoId value associated with the p0-PUCCH-Id value corresponding to  and with the closedLoopIndex value corresponding to ; otherwise,

<Unchanged part omitted>

### 7.3.1	UE behaviour

<Unchanged part omitted>

-	If a configuration for a  value or for a  value for a corresponding SRS power control adjustment state  for active UL BWP  of carrier  of serving cell  is provided by higher layers

-

<Unchanged part omitted>