3GPP TSG-RAN WG1 Meeting #96	R1-1903665

Athens, Greece, February 25th – March 1st, 2019

Source:	WI rapporteur (Huawei)

Title:	RAN1 agreements for Rel-16 Additional Enhancements for NB-IoT

Agenda Item:	6.2.2

Document for:	Information

# Introduction

This contribution lists RAN1 agreements made for the Rel-16 WI on ‘Additional Enhancements for NB-IoT’ (WI code NBIOTenh3-Core; WID in RP-182902), until and including RAN1#96.

Notes:

- The WI objective from the WID in RP-182902 is inserted below for convenience.

- The list of submitted feature lead summaries is included.

- A WI work plan for the Core part was provided by the WI rapporteur in R1-1901501.

- RAN2 agreements are summarised in R2-1902405.

# WI objective

Core part WI objective:

The objective is to specify the following set of improvements for machine-type communications for NB-IoT FDD. Improved DL transmission efficiency and/or UE power consumption: - Specify support for mobile-terminated (MT) early data transmission (EDT) [RAN2, RAN3] - Specify support for UE-group wake-up signal (WUS) [RAN1, RAN2, RAN4] Improved UL transmission efficiency and/or UE power consumption: - Specify support for transmission in preconfigured resources in idle and/or connected mode based on SC-FDMA waveform for UEs with a valid timing advance[RAN1, RAN2, RAN4] - Both shared resources and dedicated resources can be discussed - Note: This is limited to orthogonal (multi) access schemes Scheduling enhancement: - Specify scheduling multiple DL/UL transport blocks with or without DCI for SC-PTM and unicast [RAN1, RAN2] - Enhancement of SPS can be discussed. Network management tool enhancement: - SON support for reporting of [RAN2, RAN3] - Cell Global Identity and strongest measured cell(s) (ANR) - Random access performance - Radio link failure (RLF), if needed Improved multi-carrier operation: - Specify support of Msg3 quality reporting for non-anchor access [RAN1, RAN2] - Specify signalling to indicate on a non-anchor carrier for paging a set of subframes which will contain NRS even when no paging NPDCCH is transmitted [RAN1, RAN2, RAN4] Mobility enhancement: - Specify power efficient NB-IoT mechanism which would assist idle mode inter-RAT cell selection for NB-IoT to and from LTE, LTE-MTC and GERAN [RAN2] Coexistence with NR - Study NR and LTE specifications to identify possible issues related to coexistence of FDD/TDD NB-IoT with NR [RAN4, RAN1, RAN2] Connection to 5GC: - Specify support for the following features [RAN2, RAN3] - Support of extended DRX in CM-IDLE - Support of extended DRX in CM-CONNECTED with RRC_INACTIVE (support of sleep cycles up to the NAS and SMS retransmission timers) - Support of EDT for Data over NAS and UP solution (see Note) - Support of Inter-UE QoS for data over NAS (resource prioritisation between different NB-IoT UEs) - Support of restriction of use of Enhanced Coverage - Delivery of Expected UE Behaviour information to the RAN - Additional information in SIB to indicate supported CIoT features; indication of CIoT features supported by the UE in RRC Note: Based on the outcome of RAN2/SA2 liaison exchanges, UP solution to be supported for connection to 5G-CN may be later updated.

Performance part WI objective:

Specify necessary performance requirements, measurement accuracy requirements and test cases related to the above-mentioned enhancements and core requirements.

# UE-group wake-up signal

RAN1#94:

R1-1809532 Feature summary of 6.2.2.1 UE-group wake-up signal Ericsson Agreement: UE-group WUSs are only multiplexed in the same carrier as associated PO - FFS TDM/CDM for UE-group NWUS multiplexing Agreement Rel-16 UE-group NWUS sequence should consider at least - Fallback to legacy UE behavior - Inter-cell interference randomization - UE group ID for different UE-group NWUS - Reuse of Rel-15 sequences is not precluded - Effect of sequence detection on UE complexity Agreement Study the RAN1 consequence of UE-grouping on the following basis: - UE ID - Coverage - DRX/eDRX - Gap configuration - Services

RAN1#94bis:

R1-1811664 Feature summary of UE-group wake-up signal for NB-IoT Ericsson R1-1811679 Further topics for UE-group wake-up signal for NB-IoT Ericsson Agreement UE-group NWUS is supported based on eNB’s and UE’s capability. Whether the network supports UE-group NWUS is done by higher layer signaling. FFS: The number of UE groups is configured by SIB. Note that the UE-group NWUS is UE optional Agreement From a RAN1, perspective, UE grouping is based on at least UE ID or some function of UE ID. Agreement The legacy UE should not be prevented from using legacy WUS even in the case of Rel-16 group WUS is enabled - Performance impact on legacy WUS should be carefully considered Agreement Group WUS is based on at least legacy WUS and UE-group ID. Agreement Configuration of group WUS is at least signaled in SI Agreement A Rel-16 group WUS capable UE shall also be capable of Rel-15 legacy WUS Agreement Rel-16 UE-groups WUS with same DRX/eDRX gap configuration are multiplexed with TDM and/or single sequence CDM - FFS: Whether single sequence CDM can include legacy WUS - FFS: Multiplexing between Rel-15 and Rel-16 UE groups - Note: Single sequence CDM is where different sequences share the same resource and only one sequence is transmitted at a given time

RAN1#95:

R1-1813714 Summary of 6.2.2.1 Group WUS for NB-IoT Ericsson R1-1813756 Further topics on 6.2.2.1 Group WUS for NB-IoT Ericsson Agreement UE group ID is used as a parameter to generate WUS UE group sequence(s) Agreement One group WUS is designed as a single sequence Agreement Further study false detection (cross/auto correlation) performance properties for the following designs: - legacy WUS + cover codes, - legacy WUS + shifted scrambling codes, - legacy WUS + phase shift + cover code + scrambling bits - Including combinations of phase shift, cover code, and/or scrambling bits Other designs are not precluded. Agreement The number of UE groups is configurable and broadcasted in SIB. - FFS: Further details on the number of UE groups. For example, whether it is per PO or per gap configuration of a PO. Agreement Multiplexing of Rel-16 UE groups is down-selected among - Single sequence CDM is supported - Single sequence CDM is supported and TDM can be additionally applied - FFS: Configurability of CDM and/or TDM Agreement A common WUS is used to wake up all group WUS UEs monitoring the same WUS (time-frequency) resource if more than one UE group occupies the WUS resource. - FFS: Whether the above is also applicable for Rel-15 WUS UEs - FFS: Whether to support waking up a subset of all WUS UE groups Agreement For multiplexing between Rel-16 UE-group WUS and Rel-15 legacy WUS within the same carrier, further evaluate and down select among the following options - TDM - single-seq. CDM - single-seq. CDM+TDM FFS whether legacy WUS is the common WUS for all new UEs or only a part of new UEs. Agreement Rel-16 group WUS uses the same gap configurations as for Rel-15 legacy WUS except for differences from possible TDM. - No new gap higher layer signaling will be introduced for TDM

RAN1#96:

R1-1903247 Feature lead summary for Group wake-up signal for NB-IoT Ericsson R1-1903508 Further topics for 6.2.2.1 Group WUS for NB-IoT Ericsson For further evaluation and study (especially from UE implementation perspective): Benefits and costs with more grouping levels, i.e., subgroups of subgroups. Agreement A UE is required to monitor WUS(s) in only one WUS (T/F) resource location Agreement Up to 2 time-multiplexed WUS resources may be configured. FFS whether a group WUS resource may be shared with legacy WUS or not. For further study until next meeting Companies are encouraged to provide results for the different sequence candidates, taking into account, e.g., - Detection window lengths - Frequency errors - Channel model where a starting point for evaluations is the Rel-15 WUS assumptions in R1-1714993 and Rel-15 WUS functionality (w.r.t. frequency error and detection window lengths).

# Support for transmission in preconfigured UL resources

RAN1#94:

R1-1809571 Feature lead summary of Support for transmission in preconfigured UL resources Huawei, HiSilicon Agreement Idle mode based pre-configured UL resources is supported for UEs in possession of a valid TA - FFS: Validation mechanism for TA - FFS: How the pre-configured UL resources is acquired Agreement For transmission in preconfigured UL resources, the UE may use the latest TA of which its validity can be confirmed Agreement Study both shared and dedicated resource for preconfigured UL resources. If both shared and dedicated resources are supported, strive for commonality in design of both resource types. Agreement HARQ procedures for transmission in preconfigured UL resources should be studied and the following aspects should be considered: - Whether to support HARQ; - If supported, details of HARQ design including the number of HARQ processes; - Whether ACK/NACK is necessary Fallback mechanisms should be considered, e.g. fallback to legacy RACH/EDT procedures.

RAN1#94bis:

R1-1811697 Feature lead summary of Support for transmission in preconfigured UL resources Huawei, HiSilicon R1-1811678 Summary of offline discussions on PUR for eMTC and NB-IoT Huawei, HiSilicon Agreement In idle mode, the UE will at least consider one or more of the following attributes when validating TA (combination of multiple attributes is allowed): - Serving cell changes (serving cell refers the cell that the UE is camping on) - Time Alignment Timer for idle mode - Serving cell NRSRP changes (serving cell refers the cell that the UE is camping on) - FFS Other attributes: - Neighbour cell NRSRP change - TDOA of >=2 eNBs - TA History - Subscription based UE differentiation - Others not precluded (for example, attributes that need to be considered for high mobility UEs) Note that UE power consumption should be taken into account for the FFS attributes Agreement For transmission in preconfigured UL resources, an RRC idle UE may use the latest TA that passed the validation criteria Agreement Pre-configured UL resources for transmission of data are indicated by RRC signaling. At least UE-specific RRC signaling is supported. Agreement The resource configuration includes at least the following - Time domain resources including periodicity(s) - Frequency domain resources - TBS(s)/MCS(s) Agreement For UL transmission in preconfigured resource, fallback mechanism to RACH/EDT procedures is supported. Agreement Dedicated preconfigured UL resource is defined as an NPUSCH resource used by a single UE - NPUSCH resource is time-frequency resource - Dedicated PUR is contention-free Contention-free shared preconfigured UL resource (CFS PUR) is defined as an NPUSCH resource simultaneously used by more than one UE - NPUSCH resource is at least time-frequency resource - CFS PUR is contention-free Contention-based shared preconfigured UL resource (CBS PUR) is defined as an NPUSCH resource simultaneously used by more than one UE - NPUSCH resource is at least time-frequency resource - CBS PUR is contention-based (CBS PUR may require contention resolution) Agreement In IDLE mode, HARQ is supported for transmission in dedicated PUR - A single HARQ process is supported, - FFS whether two HARQ processes are supported - FFS: The design of the corresponding NPDCCH search space Agreement In idle mode, dedicated PUR is supported. - Support for CFS PUR is FFS. - Support for CBS PUR is FFS.

RAN1#95:

R1-1813717 Feature lead summary of Support for transmission in preconfigured UL resources Huawei, HiSilicon R1-1813762 Further topics for transmission in preconfigured UL resources Huawei, HiSilicon - Agreement - In idle mode, at least the following TA validation attributes are supported: - Serving cell changes (serving cell refers the cell that the UE is camping on) - Time Alignment Timer for idle mode - Serving cell NRSRP changes (serving cell refers the cell that the UE is camping on) - Based on NRSRP measurement definition in existing Rel-15 TS36.214 Send LS to RAN2, RAN4 to consider in their work. All the agreements with ‘include in LS to RAN2, RAN4’ for NB-IoT and eMTC should be captured in this LS. The LS is endorsed in R1-1813778 - Agreement - The UE can be configured to use at least these TA validation attributes: - Time Alignment Timer for idle mode - Serving cell NRSRP changes - Note: the configuration shall support disabling of the TA validation attributes Include in LS to RAN2, RAN4. For further study: TA validation attributes: - Subscription based UE differentiation (or Stationary indication in held in subscription) - Cell specific indication where TA is valid within that cell Agreement Include in LS to RAN2, RAN4: RAN1 assumes that a UE transitioning from EDT/connected to idle mode can use the valid TA that was used while in EDT/connected mode. Agreement For dedicated PUR in idle mode, the UE may skip UL transmissions. - FFS: Resource release mechanism - FFS: Whether or not to support mechanism to disable skipping by eNB Agreement In idle mode, only one HARQ process is supported for dedicated PUR Agreement For dedicated PUR in idle mode, UL grant for HARQ retransmission is transmitted in search space - FFS: Details on the search space (for example USS, CSS)

RAN1#96:

R1-1903254 Feature lead summary of Support for transmission in preconfigured UL resources Huawei, HiSilicon R1-1903513 Updated feature lead summary of Support for transmission in preconfigured UL resources Huawei, HiSilicon Agreement When the UE is configured to use several TA validation criteria, the TA is valid only when all the configured TA validation criteria are satisfied. Agreement For dedicated PUR in idle mode, the PUR search space configuration shall be included in the PUR configuration. - PUR search space is the search space where UE monitors for NPDCCH - FFS: Whether PUR search space is common or UE specific Agreement In idle mode, the TA validation configuration can include “PUR Time Alignment Timer” - Where the UE considers the TA as invalid if the (current time – time at last TA update) > the PUR Time Alignment Timer - Details on how to specify the “PUR Time Alignment Timer” is up to RAN2 Agreement In idle mode, when the UE validates TA, the UE considers the TA for the previous serving cell as invalid if the serving cell changes - Above applies for the case where the UE is configured to use the serving cell change attribute Agreement For dedicated PUR in idle mode, the dedicated PUR ACK is at least sent on NPDCCH - FFS: Whether to introduce new field in DCI or reuse existing field - RAN2 can decide if a higher layer PUR ACK is also supported Agreement When the TA is validated and found to be invalid and the UE has data to send, the UE can obtain a valid TA and may send data via legacy RACH or EDT procedures - FFS whether only TA is acquired and then data sent on PUR is supported - FFS other approaches to obtain a valid TA Agreement In idle mode, at least the following PUR configurations and PUR parameters may be updated after a PUR transmission: - Timing advance adjustment - UE TX power adjustment - FFS: Repetition adjustment for NPUSCH FFS: Whether the above update is done in L1 and/or higher layer Agreement In idle mode, the PUR search space configuration includes at least the following: - NPDCCH repetitions and aggregation levels - NPDCCH starting subframe periodicity (variable G) - Starting subframe position (alpha_offset) Agreement For dedicated PUR, in idle mode, the PUR resource configuration includes at least the following - Time domain resources including periodicity(s) - Note: also includes number of repetitions, number of RUs, starting position - Frequency domain resources - TBS(s)/MCS(s) - Power control parameters - Legacy DMRS pattern

# Scheduling of multiple DL/UL transport blocks

RAN1#94:

R1-1809523 Feature lead summary of scheduling enhancement for NB-IoT ZTE Agreement For unicast, scheduling multiple DL/UL transport blocks with single DCI is supported. Agreement - One DCI to schedule multiple TBs for SC-MCCH is not supported Agreement For Unicast, the possibility of scheduling multiple DL/UL transport blocks is configured via RRC. Details TBD Agreement For unicast, the number of TBs scheduled should be dynamically indicated in the DCI, the maximum number of TBs is FFS

RAN1#94bis:

R1-1811660 Summary on Multiple TB scheduling enhancement for NB-IoT ZTE Agreement The UE should only monitor one DCI size in the UE specific search space. Working Assumption For UE supporting multiple TBs, the soft buffer size stays the same as that of the legacy UE Agreement Individual feedback for each HARQ process is supported. FFS if HARQ bundling/multiplexing can be optionally supported. Agreement Using one DCI to schedule multiple TBs for SC-MTCH is supported, and it is configured and enabled per SC-MTCH via SC-PTM configuration message in SC-MCCH.

RAN1#95:

R1-1813740 Summary on Multiple TB scheduling enhancement for NB-IoT ZTE R1-1813757 Summary on Multiple TB scheduling enhancement for NB-IoT ZTE Agreement For multi-TBs scheduling - UL: I_sc for each TB is same Agreement Confirm the working assumption that for UE supporting multiple TBs, the soft buffer size stays the same as that of the legacy UE. Agreement For UL/DL unicast, at least consecutive resource allocation in time is supported when multiple TBs are scheduled by one single DCI. - ‘consecutive resource allocation in time’ means no new scheduling gap between the end of previous TB and the start of the next TB FFS: Whether scheduling gaps is also supported FFS: How to schedule repetitions within the consecutive resource allocation Agreement For unicast, when multiple DL/UL transport blocks are assigned by a single DCI, the relationship(s) between HARQ process and TB is/are selected from the following two candidates(multiple choices are allowed) - Relationship 1: 1 HARQ process corresponds to 1 TB - Relationship 2: 1 HARQ process corresponds up to 2 TBs Agreement Maximum UL HARQ process supported is 2. Agreement Maximum DL HARQ process supported is 2. Agreement The maximum number of TBs for multicast is one of [4, 8] - FFS: Whether the TBs are back to back without gap

RAN1#96:

R1-1903257 Summary on Multiple TB scheduling enhancement for NB-IoT ZTE Agreement One DCI can be used to schedule both initial and retransmission of different HARQ processes. Agreement For unicast, when all the TBs are scheduled by one DCI - MCS, repetition number, resource allocation, are common across all UL transport blocks - There is a single field for each of the following as in Rel-15: Scheduling delay, DCI subframe repetition number, Flag for differentiation - MCS, repetition number, resource assignment are common across all DL transport blocks - There is a single field for each of the following as in Rel-15: Scheduling delay, DCI subframe repetition number, NPDCCH order indicator, Flag for differentiation - FFS: HARQ-ACK resource Agreement For unicast, relationship 1 is supported: 1 HARQ process corresponds to 1 TB - FFS: Whether to support relationship 2 (1 HARQ process corresponds up to 2 TBs) in addition to relationship 1 - RAN1 will make decision on the support for the FFS part in RAN1#96bis Agreement For unicast, scheduling gaps between TBs scheduled by one single DCI are not supported for relationship 1 Agreement For TBs scheduled by one DCI that are contiguous, the ACK/NACK resources are back-to-back. FFS details. Agreement For SC-MTCH, the maximum number of TBs scheduled is 8. Agreement For SC-MTCH, all the TBs scheduled by one DCI use the same resource assignment, MCS and repetition number. Agreement For SC-MTCH multiple TBs scheduling, down-select from the following options: - Modify existing DCI to indicate the number of scheduled TBs (e.g. by adding new field) - Reuse Rel-15 DCI and use SC-MCCH to indicate TB numbers. - Support both a) and b)

# Coexistence of NB-IoT with NR

RAN1#94:

R1-1809572 Feature lead summary of Coexistence of NB-IoT with NR Huawei, HiSilicon Observation From RAN1 perspective, no issues were identified that would prevent the coexistence of NR and NB-IoT. Agreement RAN1 studies additional specification enhancement for improving the performance of coexistence of NB-IoT with NR.

RAN1#94bis:

R1-1811698 Feature lead summary of Coexistence of NB-IoT with NR Huawei, HiSilicon Agreement The following aspects are studied to improve the performance of coexistence of NB-IoT with NR - Resource reservation in NB-IoT - Overlap of NR SSB with NB-IoT

RAN1#95:

R1-1813718 Feature lead summary of Coexistence of NB-IoT with NR Huawei, HiSilicon Agreement RAN1 continues to study the following techniques for performance improvements of NB-IoT resource allocation until the next meeting: - Resource reservation at symbol level/slot level/subframe level/subcarrier level - Whether the resource reservation for NB-IoT is dynamic or semi-static (if supported) - Whether and how to support NB-IoT transmission in a portion of the subframe - Impact of resource reservation to legacy UEs - Whether NB-IoT transmission is postponed or dropped in reserved resources - Whether resource reservation is used for anchor/non-anchors Conclusion Overlap of NR SSB with NB-IoT anchor carrier can be avoided.

RAN1#96:

R1-1903255 Feature lead summary of Coexistence of NB-IoT with NR Huawei, HiSilicon For further study in future meeting (including): - NB-IoT resource reservation on non-anchor carrier to improve the performance of coexistence of FDD/TDD NB-IoT with NR - Subframe-level resource reservation in NB-IoT - Symbol-level resource reservation in NB-IoT - Slot-level resource reservation in NB-IoT For further study in future meeting (including):: NB-IoT transmission is dropped/postponed in reserved resources. Conclusion From RAN1 perspective, no issues were identified that would prevent the coexistence of NR and FDD/TDD NB-IoT.

# Support of Quality report in Msg3 for non-anchor access

RAN1#94:

R1-1809600 Feature lead summary on support of quality report in Msg3 for non-anchor access Huawei, HiSilicon Agreement For channel quality report in Msg3 on non-anchor access, the channel quality definition is denoted by the number of repetitions that the UE needs to decode hypothetical NPDCCH with BLER of 1% - FFS: Whether the details on the hypothetical NPDCCH are specified or not Working Assumption For channel quality report in Msg3 on non-anchor access, UE performs the channel quality measurement on the carrier it monitors to receive Msg2 (i.e. RAR) - FFS: Whether the UE performs measurement on other carriers Agreement For non-anchor access, RAN1 further studies how UEs report the measured channel quality.

RAN1#94bis:

R1-1811699 Feature lead summary of support of quality report in Msg3 for non-anchor access Huawei, HiSilicon Agreement RAN1 does not define search space for hypothetical NPDCCH for channel quality report in Msg3 on non-anchor access. Agreement From RAN1 point of view, specification support for measurement period for non-anchor access in RAN1 specifications is not needed Agreement RAN1 does not define measurement reference resource for non-anchor access. For further study: The following scenarios with regards to downlink channel quality reporting in msg3 for non-anchor carrier access. - For EDT/non-EDT, msg3 associated with PDCCH order PRACH, IDLE - PUR

RAN1#95:

R1-1813719 Feature lead summary of support of quality report in Msg3 for non-anchor access Huawei, HiSilicon Agreement In case 4 bits is used for a non-anchor carrier, all repetition i.e. 12 candidate values {1,2,4,8,16,32,64,128,256,512,1024,2048} can be reported in Msg3. Agreement In case of 2 bits is used for a non-anchor carrier, 3 candidate values can be reported in Msg3. Select one of the following alternatives for determining the 3 values: - Depending on Rmax, the maximum number of repetitions for NPDCCH Type 2 CSS. - Depending on R, "DCI subframe repetition number" indicated in DCI format N1 for Msg2 scheduling. - Depending on Rdecoded, based on the number of repetitions for NPDCCH scheduling Msg2 where UE decodes successfully.

RAN1#96:

R1-1903256 Feature lead summary of Support of Quality report in Msg3 for non-anchor access Huawei, HiSilicon For further study in future meetings: For channel quality report in Msg3 on non-anchor access, RAN1 identified the following scenarios in which UE may monitor a carrier that is different from the carrier in which the UE receives Msg2 (i.e. RAR): - Idle mode - Non-EDT random access procedure - EDT procedure - Connected mode - NPDCCH ordered NPRACH procedure - Random access procedure to apply for UL-SCH resource Agreement For the measurement on carrier(s) other than the one UE is receiving RAR for non-anchor access, if supported, RAN1 to select one or more among the following candidates: - Paging carrier - Anchor carrier - Carrier(s) configured by CarrierConfigDedicated-NB in connected mode - Other carriers configured by network with implicit or explicit signaling The following issues are identified for RAN1 further study - Whether amount of time or gap is needed for the measurement - Which carrier(s) UE reported if more than one carrier is measured - What kind of NRS UE can use for the measurement and whether needs indication Agreement In case of 2 bits are used for a non-anchor carrier, one of 3 candidate values can be reported in Msg3 - Which depends on Rmax, the maximum number of repetitions for NPDCCH Type 2 CSS. For further study in future meetings: If supported, RAN1 further discuss how to derive the candidate values reported in Msg3 if no NPDCCH Type2 CSS on the carrier.

# Presence of NRS on a non-anchor carrier for paging

RAN1#94:

R1-1809522 Feature lead summary of presence of NRS on a non-anchor carrier for paging Qualcomm Agreement (confirmed by Friday) The presence of NRS on subframes which will contain NRS even when no paging NPDCCH is transmitted is enabled by broadcast signalling. - Above applies only for non-anchor carriers Agreement (confirmed by Friday) For the objective on presence of NRS on a non-anchor carrier for paging, consider the following use case: - Early termination of NPDCCH / WUS Above is not intended to restrict the study of other use cases in other WGs which may or may not have RAN1 specification impact.

RAN1#94bis:

R1-1811326 Presence of NRS on a non-anchor for paging (feature lead summary) Qualcomm Europe Inc. (Spain) Agreement The presence of NRS on subframes which will contain NRS even when no paging NPDCCH is transmitted is enabled by SIB. Agreement Subframes which will contain NRS even when no paging NPDCCH is transmitted are associated to a PO. - FFS: Whether PO is from UE perspective or NW perspective Agreement A subset of the POs have associated subframes which will contain NRS even when no paging NPDCCH is transmitted. FFS: Details of the subset, whether it depends on some existing parameters such as DRX cycle or new parameters by configuration FFS: Whether the subset can be the whole set of POs by configuration.

RAN1#95:

R1-1813716 Presence of NRS on a non-anchor for paging (feature lead summary) Qualcomm Agreement RAN1 to discuss the issue of NRS presence for NPDCCH early termination when NWUS is enabled. Down-select among: - Alt1: If NWUS is enabled (from network perspective), there is no NRS for NPDCCH early termination, but there may be NRS for NWUS early termination. - Alt2: If NWUS is enabled (from UE perspective, i.e., enabled by network and supported by UE), there is no NRS for NPDCCH early termination, but there may be NRS for NWUS early termination. - Alt3: Decouple configuration of NRS for NPDCCH early termination and NWUS early termination - e.g. eNB can enable NRS for NWUS and disable for NPDCCH, or vice versa. - Alt4: NRS is always associated with NPDCCH for paging Agreement In the POs that have associated subframes which will contain NRS even when no paging NPDCCH is transmitted for NPDCCH early termination: - NRS is present in the first M subframes out of the 10 NB-IoT DL subframes before the PO, and the N first NB-IoT DL subframes of NPDCCH search space - FFS value(s) of N, M Agreement The subset of POs that have associated subframes which will contain NRS even when no paging NPDCCH is transmitted can be the whole set of POs.

RAN1#96:

R1-1903251 Presence of NRS on a non-anchor for paging (feature lead summary) Qualcomm Incorporated R1-1903515 Presence of NRS on a non-anchor for paging (feature lead summary) Qualcomm Incorporated Agreement For the issue of NRS presence for NPDCCH early termination when NWUS is enabled, down-select among: - Alt3: Decouple configuration of NRS for NPDCCH early termination and NWUS early termination - e.g. eNB can enable NRS for NWUS and disable for NPDCCH, or vice versa. - Alt4: NRS is always associated with NPDCCH for paging For further study in future meetings: RAN1 to consider enabling presence of CRS in non-anchor carriers in subframes not containing NRS. Agreement The decimation pattern (i.e., the pattern that determines which POs have subframes with NRS even when no NPDCCH is transmitted) is designed at least with the following principles: - P1: The decimation pattern shall be fair across UEs, i.e., all UEs see the same/similar percentage of POs with NRS. - FFS: the case for eDRX - P2: A UE belonging to a given UE group (being a UE group the group of UEs that monitor paging in the same PO) can use NRS belonging to a PO of a different group in addition to the NRS of its own UE group. - The maximum gap between the PO with NRS and the PO the UE monitors is not larger than X (to ensure that the UE can reliably estimate the SNR for NPDCCH early termination) - P3: The POs with NRS are quasi-uniformly/uniformly distributed from UE perspective. - P4: The POs with NRS are quasi-uniformly/uniformly distributed from network perspective. NOTE: It was already agreed in previous meetings that a subset of the POs have associated subframes which will contain NRS even when no paging NPDCCH is transmitted. Further study details of decimation pattern until RAN1#96bis.