3GPP TSG RAN WG1 96	R1- 1903689

Athens, Greece, February 25th – 1st March 2019

Agenda Item:	7.2.10.1.4

Source:	Ericsson

Title:	offline session notes for 7.2.10.1.4 on evaluations of positioning solutions (Wednesday)

Document for:	Discussion, Decision

# 1 Session notes

Offline consensus:

---------begin TP for section 8.4.1.2--------------------------

For the Umi scenario, channel models and simulation assumptions, the following can be observed for horizontal accuracy as described in section 5 of the TR for regulatory and commercial requirements:

- For scenario 2 the evaluations from 12 out of 12 sources showed that DL-TDOA can meet the regulatory requirements for FR1 and FR2.

- For scenario 2, the evaluations from 12, out of 12 sources showed that DL-TDOA can meet the commercial requirement, when no synchronization error is included in the evaluation for FR1 and FR2.

- When synchronization error is modeled, the DL-TDOA evaluations from 0 out of 6 sources showed the commercial requirements as described in section 5 of the TR can be met for FR1 and FR2.

- For scenario 2, 1 source evaluated AoD, and observed that it can meet the regulatory requirements for FR2.

- For scenario 2, 1 source evaluated  AoD+ZoD positioning method , and observed that it can not meet the regulatory requirements for FR1 or FR2.

- For AoD+ZoD+serving cell TA positioning method described in [polaris], the regulatory requirements for FR1 or FR2 were met.

-----End TP------

Offline consensus proposal:

---------begin TP for section 8.4.1.3--------------------------

For the UMa scenario, channel models and simulation assumptions, the following can be observed for horizontal accuracy as described in section 5 of the TR for regulatory and commercial requirements:

- For scenario 3, the evaluations from 12 out of 12 sources showed that DL-TDOA can meet the regulatory requirements.

- For scenario 3, and for outdoor UEs the evaluations from 8 out of 12 sources showed that DL-TDOA can meet the commercial requirements, when no synchronization error is included in the evaluation.

- For scenario 3, and for outdoor UEs, when synchronization error is modeled, the evaluations from 0 out of 6 ources showed that for DL-TDOA the commercial requirements as described in section 5 of the TR can be met.

-----End TP------

Offline consensus proposal:

---------begin TP for section 8.4.2.1--------------------------

For the indoor open office  scenario, channel models and simulation assumptions, the following can be observed for horizontal accuracy as described in section 5 of the TR for regulatory and commercial requirements:

- For scenario 1, the evaluations from 8 out of 8 sources showed that UTDOA can meet the regulatory requirements.

- For scenario 1, the evaluations from 8 out of 8 sources showed that UTDOA can meet the commercial requirements, when no synchronization error is included in the evaluation.

- For scenario 1, When synchronization error is modeled, the evaluations from 1 out of 4 sources showed that for UTDOA the commercial requirements as described in section 5 of the TR can be met.

- For scenario 1, 3 sources evaluated UL AoA and observed that it can meet the regulatory requirements.

- For scenario 1, 3 sources evaluated UL AoA , and observed that it can meet the commercial requirements.

-----End TP------

Offline consensus proposal:

---------begin TP for section 8.4.2.2--------------------------

For the Umi scenario, channel models and simulation assumptions, the following can be observed for horizontal accuracy as described in section 5 of the TR for regulatory and commercial requirements:

- For scenario 2, the evaluations from 6 out of 6 sources showed that UTDOA can meet the regulatory requirements.

- For scenario 2, the evaluations from 5 out of 7 sources showed that UTDOA can meet the commercial requirements, when no synchronization error is included in the evaluation.

- For scenario 2, When synchronization error is modeled, the evaluations from 0 out of 4 sources showed that for UTDOA the commercial requirements as described in section 5 of the TR can be met.

- For scenario 2, 1 source evaluated UL AoA and observed that it can meet the regulatory requirements.

- For scenario 2, 1 source evaluated UL AoA , and observed that it can not meet the commercial requirements.

-----End TP------

Offline consensus proposal:

---------begin TP for section 8.4.2.3--------------------------

For the UMa scenario, channel models and simulation assumptions, the following can be observed for horizontal accuracy as described in section 5 of the TR for regulatory and commercial requirements:

- For scenario 3, the evaluations from 3 out of 5 sources showed that UTDOA can meet the regulatory requirements.

  - 1 source showed that indoor and outdoor UEs could not meet the regulatory requirements, even under perfect network synchronization.

  - 1 source showed that indoor UEs could not meet the regulatory requirements, even under perfect network synchronization.

- For scenario 3, and for outdoor UEs the evaluations from 3 out of 5 sources showed that UTDOA can meet the commercial requirements, when no synchronization error is included in the evaluation.

- For scenario 3, and for outdoor UEs, when synchronization error is modeled, the evaluations from 0 out of 4 sources showed that for UTDOA the commercial requirements as described in section 5 of the TR can be met.

- For scenario 3, 1 source evaluated UL AoA and observed that it can not meet the regulatory requirements.

-----End TP------

Offline consensus proposal:

---------begin TP for section 8.4.3-1--------------------------

For the indoor open office scenario, channel models and simulation assumptions, the following can be observed for horizontal accuracy as described in section 5 of the TR for regulatory and commercial requirements:

- For scenario 1, the evaluations from 5 out 5 sources showed that multi-cell RTT can meet the regulatory requirements.

- For scenario 1, the evaluations from 5 out 5 sources showed that multi-cell RTT can meet the commercial requirements.

- Note: There are multiple solutions under the multi cell RTT definition

- For scenario 1, the evaluations from 1 out 1 source showed that E-CID (single cell RTT + UL AOA as in [HW]) can meet the regulatory requirements.

- For scenario 1, the evaluations from 1 out 1 source showed that E-CID (single cell RTT + UL AOA as in [HW]) can meet the commercial requirements.

-----End TP------

Offline consensus proposal:

---------begin TP for section 8.4.3-1--------------------------

For scenario 2, channel models and simulation assumptions, the following can be observed for horizontal accuracy as described in section 5 of the TR for regulatory and commercial requirements:

- For scenario 2, the evaluations from 3 out 3 sources showed that multi-cell RTT can meet the regulatory requirements.

- For scenario 2, the evaluations from 3 out 3 sources showed that multi-cell RTT can meet the commercial requirements.

- For scenario 2, the evaluations from 1 out 1 source showed that E-CID (single cell RTT + UL AOA as in [HW]) can meet the regulatory requirements.

- For scenario 2, the evaluations from 1 out 1 source showed that E-CID (single cell RTT + UL AOA as in [HW]) can meet the commercial requirements.

-----End TP------

Offline consensus proposal:

---------begin TP for section 8.4.3-1--------------------------

For scenario 3, channel models and simulation assumptions, the following can be observed for horizontal accuracy as described in section 5 of the TR for regulatory and commercial requirements:

- For scenario 3,  and outdoor UEs the evaluations from 2  sources out of 2 showed that multi cell RTT can meet the regulatory requirements.

  - Indoor UEs do not meet the regulatory requirements according to 1 sources, while 1 source showed that the regulatory requirements were met.

- For scenario 3, the evaluations from 2 sources out of 2 showed that multi cell RTT cannot meet the commercial requirements.

- For scenario 3, the evaluations from 1 source showed that E-CID (single cell RTT + UL AOA as in [HW]) can meet the regulatory requirements.

- For scenario 3, the evaluations from 1 out 1 source showed that E-CID (single cell RTT + UL AOA as in [HW]) cannot meet the commercial requirements.

-----End TP------