3GPP TSG RAN WG1 #96 		       R1- 1903704

Athens, Greece, Feb 25th – March 1st, 2019

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.885 | CR |  | rev | - | Current version: | 1.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Text proposal on 2 Rx Antennas for NR V2X |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | General Motors, Volkswagen AG |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_NR_V2X |  |  |  |  |  |  | Date: |  | 2019-2-25 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In the recent RAN plenary meeting (RAN #82) it has been decided to grant a waiver for vehicular UEs that allows the use of 2Rx antennas for the Uu link. This waiver is based on the work jointly done by the automotive industry, 5GAA and 3GPP to explain the challenges automakers face in deploying multiple antennas on a vehicle. RAN1 simulation assumptions for NR V2X (sidelink) at FR1, however, include 4Rx antennas as a baseline. For the reason of implementation challenges (also outlined in TR38.826), it is requested that 2RX antennas are also considered for the sidelink at FR1 in Rel-16 NR V2X. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add 2RX antennas to the same extend as 4Rx antennas for the development of the NR V2X sidelink at FR1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Significant adoption challenges of the technology by some automotive companies |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

-----------Text Proposal for 38.214 Subclause 6.1--------

## A.1	Simulation profiles

Simulation profiles are defined in Table A.1-1.

Table A.1-1: Simulation profiles

|  | Unicast | Multicast | Broadcast | Mixture |
| --- | --- | --- | --- | --- |
| SL frequency (GHz) | 6, 30 | 6, 30 | 6, 30 | 6, 30 |
| Traffic models | Periodic: Medium intensity; [50] ms inter-packet arrival, [50]% vehicles generate packets. Aperiodic: Medium intensity, 100% vehicles generate packets. Periodic and aperiodic traffic are simulated separately. | Periodic: Medium intensity; [50] ms inter-packet arrival, [50]% vehicles generate packets. Aperiodic: Medium intensity, 100% vehicles generate packets. Periodic and aperiodic traffic are simulated separately. | Periodic: Medium intensity; [50] ms inter-packet arrival, [50]% vehicles generate packets Aperiodic: Medium intensity, 100% vehicles generate packets. Periodic and aperiodic traffic are simulated separately. | 33%, 33%, 34% vehicles generate unicast, multicast, broadcast packets, respectively. For each traffic type, 50% is periodic and 50% is aperiodic. Periodic: Medium intensity; 100 ms inter-packet arrival Aperiodic: Medium intensity |
| Simulation environment, UE drop and mobility | Highway: Option A Urban: Option A | Highway: Option A Urban: Option A | Highway: Option A Urban: Option A | Highway: Option A Urban: Option A |
| Number of Tx/Rx antenna elements for vehicle UE1 | 2Tx/4Rx for 6 GHz 2Tx/2Rx for 6GHz2 FFS for 30 GHz | 2Tx/4Rx for 6 GHz 2Tx/2Rx for 6GHz2 FFS for 30 GHz | 2Tx/4Rx for 6 GHz 2Tx/2Rx for 6GHz2 FFS for 30 GHz | 2Tx/4Rx for 6 GHz 2Tx/2Rx for 6GHz2 FFS for 30 GHz |
| Antenna model for vehicle UE | Option 1 | Option 1 | Option 1 | Option 1 |
| Channel model | As defined | As defined | As defined | As defined |
| SL simulation bandwidth (MHz) | 20 MHz for 6 GHz 100 MHz for 30 GHz | 20 MHz for 6 GHz 100 MHz for 30 GHz | 20 MHz for 6 GHz 100 MHz for 30 GHz | 20 MHz for 6 GHz 100 MHz for 30 GHz |
| NOTE 1: The number of antennas can be increased for the evaluations for transmissions using more than 2 layers. NOTE 2: The combination of 2Tx/2Rx antennas was only introduced at the end of the study by the automotive industry (5GAA LS in R1-1903333). Due to the late introduction no simulations with this scenario are expected to be conducted during the study phase. |  |  |  |  |

The transmitter-receiver association model in [5] is used, with setting X and Y to [150 meters for Urban and 320 meters for Freeway] as baseline. Other values of X and Y are not precluded. When evaluating V2I based on Uu, it is assumed that the packet is generated at the location where UE-type RSU is placed.

-----------End of Text Proposal--------