3GPP TSG RAN WG1 Meeting #96 R1-1903750 Athens, Greece, 25th February – 1st March, 2019 CR-Form-v11.2. CHANGE REQUEST. 38.213; CR; 0027; rev; -; Current version:; 15.4.0. For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests.. Proposed change affects:; UICC apps; ME; X; Radio Access Network; X; Core Network. Title:; CR on identifying transmission occasion after resetting a PC closed loop. Source to WG:; ZTE. Source to TSG:; R1. Work item code:; NR_newRAT-Core; Date:; 2019-2-28. Category:; F; Release:; Rel-15. Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900.; Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16). Reason for change:; In TS38.213, if P0 or alpha which is related to j is re-configured by gNB, UE shall reset closed-loop power control value f(i=0, l)=0, where l and j are associated with the same SRI. f(i, l) is maintained independently for each of closed loop l, but uplink transmission occasion i is shared among all PUSCH transmissions in one BWP. If only one closed-loop power control value, say l=0, needs to be reset, it should be f(k, l=0)=0, k=0,1,…,i rather than f(i=0, l=0)=0. There are the same issues for PUCCH and SRS transmission. Summary of change:; When resetting closed-loop power control value f(i,l),g(i,l) or h(i), do not reset transmission index i to 0, and just keep it as naturally accumulated.. Consequences if not approved:; In case of two closed-loop power control loops, resetting the transmission occasion index i of one closed loop causes unclear procedure for the other loop.. Clauses affected:; 7.1.1, 7.2.1, 7.3.1. Y; N. Other specs; X; Other core specifications; TS/TR ... CR .... affected:; X; Test specifications; TS/TR ... CR .... (show related CRs); X; O&M Specifications; TS/TR ... CR .... Other comments:; Isolated impact analysis: This CR fixes the issue of the value of identifying transmission occasion after resetting a PC closed loop and aligns the specification to RAN1 agreements. It is expected UEs and networks are implemented in accordance to this CR and therefore no change is required on UE and network implementation..

### 7.1.1	UE behaviour

<Unchanged part omitted>

-	A UE resets accumulation of a PUSCH power control adjustment state  for active UL BWP  of carrier  of serving cell  to

-	If a configuration for a corresponding  value is provided by higher layers

-	If a configuration for a corresponding  value is provided by higher layers

-	If  and the PUSCH transmission is scheduled by a DCI format 0_1 that includes a SRI field, and the UE is provided higher SRI-PUSCH-PowerControl, the UE determines the value of  from the value of  based on an indication by the SRI field for a sri-PUSCH-PowerControlId value associated with the sri-P0-PUSCH-AlphaSetId value corresponding to  and with the sri-PUSCH-ClosedLoopIndex value corresponding to

-	If  and the PUSCH transmission is scheduled by a DCI format 0_0 or by a DCI format 0_1 that does not include a SRI field or the UE is not provided SRI-PUSCH-PowerControl,

-	If ,  is provided by the value of powerControlLoopToUse

<Unchanged part omitted>

### 7.2.1	UE behaviour

<Unchanged part omitted>

-	If a configuration of a  value for a corresponding PUCCH power control adjustment state  for active UL BWP  of carrier  of serving cell  is provided by higher layers,

-

If the UE is provided PUCCH-SpatialRelationInfo, the UE determines the value of  from the value of  based on a pucch-SpatialRelationInfoId value associated with the p0-PUCCH-Id value corresponding to  and with the closedLoopIndex value corresponding to ; otherwise,

<Unchanged part omitted>

### 7.3.1	UE behaviour

<Unchanged part omitted>

-	If a configuration for a  value or for a  value for a corresponding SRS power control adjustment state  for active UL BWP  of carrier  of serving cell  is provided by higher layers

-

<Unchanged part omitted>