3GPP TSG RAN WG1 #98bis		                                                    R1-1910176

Chongqing, China, October 14th – 20th, 2019

Agenda item:	7.2.10.1

Source: 	CMCC

Title: 	Discussion on DL positioning reference signal design

Document for:	Discussion and Decision

Introduction

At RAN#83 meeting, a work item for NR positioning was approved [1], in which an issue to define new DL positioning reference signal applicable for NR positioning techniques, such as DL-TDOA, DL-AoD and multi-RTT, was included on RAN1 scope. In RAN1#98 meeting, design aspects including DL PRS periodicity and staggering pattern were agreed [2].

In this contribution, we discuss further the design issues on the DL PRS resource repetitions and the DL PRS multiplexing with other signals/channels.

DL PRS design

- 2.1 DL PRS resource repetitions

The LTE PRS can be transmitted in  consecutive DL subframes, where  is configured by higher layers. The enumerated values of  are specified as 1, 2, 4, or 6 subframes. If the value sf-add is further defined in this field, then the value of  can be configured as 1, 2, 3, …, 160 consecutive DL subframes [3].

In NR, in order to ensure accurate positioning, the flexibility to configure DL PRS resource repetition values should be supported as well. In RAN1#98 meeting, the following agreement was made on the bandwidth of the DL PRS:

Agreement: The following parameters describing a DL PRS Resource are defined: - Bandwidth of DL PRS Resource - FFS granularity of bandwidth configuration which is to be down-selected at the next meeting among the following options: - Option 1. One PRB - Option 2. Four PRBs - Option 3. RBG granularity - Option 4. One of values configurable from the set 24, 48, 96 192, 264 PRBs - Option 5: Option 2, 3 or 4 combined with a possibility to blank PRBs at the band edges - Option 6: Option 2, 3 or 4 combined with the maximum PRBs per carrier bandwidth

It is noted that if smaller DL PRS bandwidth is configured (e.g., 24 PRBs), allowing to configure larger number of consecutive DL slots helps attain the required positioning accuracy. On the other hand, supporting configurable DL PRS repetitions is feasible to implement continuous DL PRS transmission. For instance, in RAN1#98 meeting, the lowest periodicity of the DL PRS resource was approved as P = 4 slots. In such a case, by setting = 4 slots, the UE can track the DL PRS continuously, which is able to achieve high positioning accuracy.

Proposal 1: The DL PRS resource repetitions (the number of consecutive DL slots) should be configurable. At least support = 1, 2, 4, 6 slots per DL PRS resource.

- 2.2 DL PRS multiplexing with other signals/channels

In LTE, considering the PRS multiplexing with other signals/channels, it has the following limitations:

- The positioning reference signals shall not be mapped to resource elements  allocated to the core part of the PBCH, PSS or SSS regardless of their antenna port  [4];

- A UE may assume that positioning reference signals are not present in resource blocks in which it shall decode PDSCH according to a detected PDCCH with CRC scrambled by the SI-RNTI or P-RNTI with DCI format 1A or 1C intended for the UE [5].

In a similar way, the NR DL PRS time/frequency resource allocation should avoid the collision with the SS-PBCH block and the PDSCH scheduled by the PDCCH scrambled with SI-RNTI, P-RNTI, RA-RNTI from the serving TRP. Considering the DL PRS multiplexing with PDSCH scheduled by UE data services, the limitation should be precluded. Supporting the DL PRS sharing time/frequency resources with PDSCH carrying data allows the continuous DL PRS transmissions and measurements/tracking, which is beneficial for improving the positioning accuracy and reducing the PRS overhead.

Proposal 2: The DL PRS shall not be mapped to resource elements allocated to the SS-PBCH block and the PDSCH scheduled by the PDCCH scrambled with SI-RNTI, P-RNTI, RA-RNTI from the serving TRP.

Proposal 3: The DL PRS multiplexing with PDSCH scheduled by UE data services should be supported.

Conclusions

In this contribution, we discuss the DL PRS designs and make the following proposal:

Proposal 1: The DL PRS resource repetitions (the number of consecutive DL slots) should be configurable. At least support = 1, 2, 4, 6 slots per DL PRS resource.

Proposal 2: The DL PRS shall not be mapped to resource elements allocated to the SS-PBCH block and the PDSCH scheduled by the PDCCH scrambled with SI-RNTI, P-RNTI, RA-RNTI from the serving TRP.

Proposal 3: The DL PRS multiplexing with PDSCH scheduled by UE data services should be supported.

References

- RP-190752, “New WID: NR Positioning Support”, Intel Corporation, Ericsson, RAN#83, Shenzhen, China.

- Draft Report of 3GPP TSG RAN WG1 #98 v0.2.0, Prague, Czech Republic.

- TS 36.355, LTE Positioning Protocol (LPP), v15.3.0.

- TS 36.211, Physical channels and modulation, v15.5.0

- TS 36.213, Physical layer procedures, v15.5.0