3GPP TSG RAN WG1 #80 LAA ad-hoc	R1-151058

March 24th  – 26th , 2015

Paris, France

Agenda item:	2.1

Source: 	Cisco Systems

Title: 	Simulation results for Downlink Coexistence of LAA and Wi-Fi

Document for:	Discussion and Decision

# Introduction

According to [1], the design targets of an LAA system are

-	A single global solution framework allowing compliance with any regional regulatory requirements

A single global solution framework for LAA should be defined to ensure that LAA can be operated according to any regional regulatory requirements. Furthermore, LAA design should provide sufficient configurability to enable efficient operation in different geographical regions.

-	Effective and fair coexistence with Wi-Fi.

The LAA design should target fair coexistence with existing Wi-Fi networks to not impact Wi-Fi services more than an additional Wi-Fi network on the same carrier, with respect to throughput and latency.

-	Effective and fair coexistence among LAA networks deployed by different operators

The LAA design should target fair coexistence among LAA networks deployed by different operators so that the LAA networks can achieve comparable performance, with respect to throughput and latency.

It has been agreed in [2] that the Listen-before-talk (LBT) courtesy mechanism is an important component for LAA in order to achieve a fair co-existence not only with other Wi-Fi devices, but also with other LAA devices. Simulations have shown that the presence of LAA without LBT can cause significant degradation to the Wi-Fi system [3]. LBT is defined as [1]

…a mechanism by which an equipment applies a clear channel assessment (CCA) check before using the channel. The CCA utilizes at least energy detection to determine the presence or absence of other signals on a channel in order to determine if a channel is occupied or clear, respectively. European and Japanese regulations mandate the usage of LBT in the unlicensed bands. Apart from regulatory requirements, carrier sensing via LBT is one way for fair sharing of the unlicensed spectrum and hence it is considered to be a vital feature for fair and friendly operation in the unlicensed spectrum in a single global solution framework.

As specified in the European regulations EN 301.893 [4], the LBT mechanism can be divided into two types: Frame Based Equipment (FBE) and Load Based Equipment (LBE). In FBE, the transmit/receive structure is not based on the instantaneous traffic. Instead, a clear channel assessment (CCA) is performed over a channel observation period ≥20μs at the beginning of every fixed frame period. If the channel is clear, the node can transmit over a duration of a given channel occupancy time. If the channel is busy, the node will have to wait until the end of the fixed frame before a new CCA is performed again. However, an IEEE 802.11 Wi-Fi AP accesses the channel continuously without a fixed frame structure. Rather, a random number is defined based on a certain contention window size.  The Wi-Fi AP transmits whenever this number of idle occurrences has passed. As pointed out in [8], if an LAA-LTE eNB were to adopt this approach, the performance of the eNB would suffer, as the eNB can only sense the channel once every frame while the Wi-Fi node does not have such a restriction. In fact, as discussed in [8], this approach is detrimental even in the case of LAA-LTE to LAA-LTE co-existence, as the transmitting node can potentially capture the channel for a long time.

In the case of LBE, the transmit/receive structure is demand-driven. A CCA is performed, and if the channel is clear, the node would transmit over a given channel occupancy time that is no greater than  ms, where is selected by the vendor in the range between 4 and 32  If the channel is not clear, an extended CCA is performed where the channel is observed over a period of , where is selected uniformly between 1 and , and is the CCA observation time of no less than 20μs [4]. Similar to Wi-Fi, CCA is continuously performed without any restriction to frame boundaries. Although the extended CCA mentioned above is similar to the backoff mechanism in Wi-Fi, [4] does not mandate the backoff to be exponential as in the case of Wi-Fi. Thus, if the LAA-LTE backoff mechanism is fixed (i.e. as outlined in [4]) while that of Wi-Fi is exponential, the Wi-Fi performance will be unfairly impacted as numerically confirmed in [5], [6], and [7]. Additionally, as pointed out in contributions such as [9], [10], [11], and [12], the LBE is inherently unfair to Wi-Fi.

Simulations [3] have shown that LAA can have a significant impact on the performance of Wi-Fi in the absence of LBT. Further simulations [15] have examined the potential improvement of the exponential back-off as outlined in the option A of [14] relative to the fixed back-off in [4]. The results show that the use of a fixed back-off mechanism as described in [4] is not enough for LAA-LTE to provide a fair coexistence with Wi-Fi. However, the exponential back-off scheme described in [15] may produce unfair results against LAA. Based on the simulation results, further investigations need to be done in order to assess the fairness issue in LAA/Wi-Fi coexistence. According to [15], suggestions to further investigations include:

- Using a larger CCA interval than 20μs

- Using an adaptive backoff window mechanism (such as exponential backoff) in ECCA

- LAA nodes should consider a CTS-to-Self mechanism to enable Wi-Fi nodes to detect the start of a LAA transmission burst.

- LAA nodes should consider including Wi-Fi Preamble Detection and Virtual Carrier Sensing to improve co-existence with Wi-Fi.

In this contribution, the previous study [15] on downlink LAA/Wi-Fi co-existence is extended by taking into account the above conclusions. Specifically, the main objective of this study is to refine the initial and extended CCA durations as well as back-off rate during the extended CCA in order to explore the possibility of achieving fairness in LAA/Wi-Fi coexistence.

# Simulation results and discussions

In this contribution, four different versions of the back-off mechanisms are used.  The first one is labelled as ‘Scheme A (fixed)’, which corresponds to the LBE LBT Option B scheme specified in [14], with a ‘q’ value that corresponds to 10ms maximum channel occupancy time (MCOT). The second version is ‘Scheme A (exp)’, which is based on the LBE LBT Option A scheme as outlined in [14]. The third version is labelled as ‘Scheme B (fixed)’, which corresponds to the back-off mechanism similar to that in [16] with a fixed back-off as in Option B of [14]. The fourth version is labelled as ‘‘Scheme B (exp)’, which corresponds to the back-off mechanism similar to that in [16] but with an exponential back-off as in Option A of [14]. In all cases, transmissions are only allowed at the sub-frame boundary. The schematic diagrams for scheme A (fixed) and scheme A (exp) are shown in Figure 1 and Figure 2 respectively. On the other hand, scheme B (fixed) and scheme B (exp) are shown in Figure 3 and Figure 4. The major difference between scheme A and B is that the latter combines the ICCA and ECCA even when the channel is idle, whereas scheme A allows the transmission as soon as the ICCA is passed. The extra ECCA is introduced after the ICCA in scheme B in order to mimic the extra defer period during the ECCA in Wi-Fi.

In this contribution, both energy detection and preamble detection schemes are assumed in channel sensing. Also, virtual carrier sensing is included. The CTS-to-self mechanism is assumed for channel reservation purposes. More details regarding these assumptions can be found in [15].   In this table, T0 and T1 correspond to the slot size of the initial CCA (ICCA) and extended CCA (ECCA) respectively. The ‘Exp rate’ r refers to the rate at which the observation period is multiplied when N unoccupied ECCA slots were available within the observation period q.  Further simulation assumptions and parameters are given in the Appendix.

Figure 1 Flow diagram for scheme A (fixed) back-off mechanism.

Figure 2 Flow diagram for scheme A (exp) back-off mechanism.

Figure 3 Flow diagram for scheme B (fixed) back-off mechanism.

Figure 4 Flow diagram for scheme B (exp) back-off mechanism.

## Scheme A (fixed) results

Figure 5 and Figure 6 shows the network throughput for the victim Wi-Fi and the aggressor LAA network respectively in the Wi-Fi/LAA scenario with different combinations of (T0,T1) for scheme A (fixed). As the results show, the fairness is relatively insensitive to T0 and T1. One potential explanation is that the ECCA is not invoked frequently enough. However, due to the exponential back-off nature of Wi-Fi, the results suggest that a fixed back-off is not enough to provide fairness under scheme A.

Figure 5  Network throughput for Wi-Fi victim in the Wi-Fi/LAA scenario for scheme A (fixed) with different (T0,T1) values.

Figure 6 Network throughput for the LAA aggressor in the Wi-Fi/LAA scenario for scheme A (fixed) with different T0,T1 values.

## Scheme B (fixed) results

Figure 7 and Figure 8 show the network throughput for the victim Wi-Fi and aggressor LAA network respectively in the Wi-Fi/LAA scenario with different combinations of (T0,T1) for scheme B (fixed).Now, when the ICCA is combined with the ECCA, the combined mechanism mimics the initial CCA + defer + ECCA, and so scheme B resembles more closely to Wi-Fi. The results suggest that T0,T1=(34μs, 18μs) is the best one among all tested combinations. While a perfect fairness is not achieved, the results suggest that a good combination of (T0,T1) is able to stabilize the fairness at various offered load levels.

In fact, fairness can be further improved by fine-tuning the (T0,T1) values. For example, assuming T0=34μs, Figure 9 shows the network throughput ratio between Wi-Fi and LAA as a function of T1. The results show that perfect fairness can potentially be achieved at around T0,T1=(34μs,14μs), which occurs at the intersection of all three load lines in Figure 9. At this point, fairness can be achieved independent of traffic load.

Figure 7  Network throughput for victim Wi-Fi in the Wi-Fi/LAA scenario for scheme B (fixed) with different (T0, T1) values.

Figure 8 Network throughput for the LAA aggressor in the Wi-Fi/LAA scenario for scheme B (fixed) with different T0, T1 values.

Figure 9 Network throughput ratio between Wi-Fi and LAA for different offered load as a function of T1 at T0=34μs for scheme B (fixed).

## Scheme A (exp) results

Figure 10 and Figure 11 show the network throughput for the victim Wi-Fi and aggressor LAA network respectively in the Wi-Fi/LAA scenario with different combinations of (T0,T1) for scheme A (exp) at a back-off rate of 2. The results show that fairness can be improved markedly with the introduction of the exponential back-off mechanism. Amongst all tested combinations, T0,T1=(20μs,120μs) appears to achieve the best compromise of fairness over a range of offered loads. On the other hand, increasing T0 to 34μs at T1=120μs does not seem to provide further improvement. Again, at small values of T0 and T1, little improvement is made due to the fact that the exponential back-off is rarely triggered. It is important to note that fairness is not stabilized with increasing traffic loads with all selected combinations of (T0,T1).

Figure 12 and Figure 13 show the network throughput for the victim Wi-Fi and aggressor LAA network in the Wi-Fi/LAA scenario with different values of  the exponential back-off rate r for scheme A (exp), at T0,T1=(20μs, 120μs). The results suggest fairness does not improve significantly due to the changing back-off rate. Amongst all tested values, a back-off rate of r=1.5 seems to be reasonable, although further improvements can still be made.

Figure 10 Network throughput as a function of offered load for scheme A (exp) with various T0and T1 values for the Wi-Fi victim at an exponential back-off rate r=2.

Figure 11 Network throughput as a function of offered load for scheme A (exp) with various T0and T1 values for the LAA aggressor (right) at an exponential back-off rate r=2.

Figure 12 Network throughput as a function of offered load for scheme A (exp) with various exponential back-off rate r values for Wi-Fi victim at T0,T1=(20μs,120μs).

Figure 13 Network throughput as a function of offered load for scheme A (exp) with various exponential back-off rate r values for the LAA aggressor at T0,T1=(20μs,120μs).

It may be tempting to conclude that, with a better parameterization, increased fairness for scheme A (exp) can be achieved. However, the final verdict in fairness for scheme A (exp) is far from over. As shown in Figure 14, while the network throughput between Wi-Fi and LAA show potential signs of stability with improved parameterization as suggested in Figure 12 and Figure 13, the 50%-tile User Perceived Throughput (UPT) can still be very unfair. The reason for the discrepancy is that it is possible for the 5%-tile UPT for Wi-Fi to be higher than that for LAA. Due to the large value of T1, some LAA users who are under a very disadvantageous channel condition may get stuck and would not be able to transmit, while, on average, LAA users enjoy a higher UPT. On the other hand, it is possible for disadvantageous Wi-Fi users to still get some lower data rate. This can be observed in Figure 15.

Figure 14 50%-tile User perceived throughput (UPT) ratio between Wi-Fi and LAA for various back-off rate at T0,T1=(20μs,120μs) for scheme A (exp).

Figure 15 CDF of mean UPT across UEs across all drops for Wi-Fi and LAA at a load of 2.5 Hz (zoomed in on 0 to 20Mbps).

## Scheme B (exp) results

Figure 16 and Figure 17 show the network throughput for the victim Wi-Fi and aggressor LAA network respectively in the Wi-Fi/LAA scenario with different combinations of (T0,T1) for scheme B (exp) at a back-off rate of 2. The results suggests that the fairness can be stabilized across a wide range of loads if a reasonable combination of T0,T1 is selected. In this particular example, the best combination amongst those tested is (T0,T1)=(34μs, 9μs).

Figure 18 and Figure 19 show the network throughput for the victim Wi-Fi and aggressor LAA network respectively in the Wi-Fi/LAA scenario with different values of the exponential back-off rate r for scheme B (exp) at T0,T1=(34μs, 9μs). The results suggest that a back-off rate of r=1.5 seems to produce the fairest performance across a wide range of loads.

Figure 16 Network throughput as a function of offered load for scheme B (exp) with various T0 and T1 values for victim Wi-Fi (left) and aggressor LAA (right) at an exponential back-off rate r=2.

Figure 17 Network throughput as a function of offered load for scheme B (exp) with various T0 and T1 values for victim Wi-Fi (left) and aggressor LAA (right) at an exponential back-off rate r=2.

Figure 18 Network throughput as a function of offered load for scheme B (exp) with various exponential back-off rate r values for the Wi-Fi victim at T0,T1=(34μs,9μs).

Figure 19 Network throughput as a function of offered load for scheme B (exp) with various exponential back-off rate r values for the LAA aggressor at T0,T1=(34μs,9μs).

Figure 20 and Figure 21 show the network throughput and the 50%-tile user perceived throughput (UPT) ratio between Wi-Fi and LAA for various back-off rate at T0,T1=(34μs,9μs) for scheme B (exp). It can be seen that a stable ratio of very close to unity for both network throughput as well as the 50%-tile UPT can be achieved for all traffic load when a back-off rate of 1.5 is used. These results have a significant implication as they allude to the possibility that once fairness is achieved, it can be maintained across a wide range of traffic loads. It is important to note that while the results suggest that fairness can be achieved once a particular combination of parameters is employed, such a conclusion is only conditional on a particular deployment scenario. In other words, if the traffic ratio between Wi-Fi and LAA is altered, or if the uplink traffic for Wi-Fi is also assumed, the optimal parameter setting may need to be modified. This observation alludes to the potential need for a more adaptive parameter setting in maintaining fairness between Wi-Fi and LAA.

2.0

1.7

1.5

1.2

Wi-Fi Higher Tput than LAA

Wi-Fi Lower Tput than LAA

Figure 20 Network throughput ratio between Wi-Fi and LAA for various back-off rate at T0,T1=(34μs,9μs) for scheme B (exp). The best value for r seems to be 1.5.

2.0

1.7

1.5

1.2

Wi-Fi Higher Tput than LAA

Wi-Fi Lower Tput than LAA

Figure 21 50%-tile User perceived throughput (UPT) ratio between Wi-Fi and LAA for various back-off rate at T0,T1=(34μs,9μs) for scheme B (exp). The best value for r seems to be 1.5.

# Conclusion

Simulation results show that scheme A with fixed back-off does not provide reasonable fairness. While the exponential back-off can improve the fairness for scheme A dramatically with a good combination of (T0,T1,r), much improvement can still be made. On the other hand, with a good choice of T0,T1, even a fixed back-off for scheme B can potentially achieve a stable level of fairness between Wi-Fi and LAA which is relatively insensitive to the traffic load. With the introduction of exponential back-off, the extra degree of freedom, i.e. the exponential back-off rate r, on top of the (T0,T1) it is possible for scheme B to achieve a good level of fairness which is insensitive to the traffic load.

Despite the fact that a good and stable level of fairness can be achieved via scheme B with either fixed or exponential back-off over a wide range of offered load, their respective optimal combination of parameters may still be dependent on the nature of the deployment scenario. For example, the optimal parameters may vary depending on the presence of uplink Wi-Fi traffic, as well as the nature of the traffic mix or deployment topology. Thus, more investigations are still required to gain a better of picture on the issue of fair coexistence between Wi-Fi and LAA.

Proposal 1:  Scheme B with both 1) fixed and 2) exponential back-off can be potential candidates for further investigation in the context of fair coexistence between Wi-Fi and LAA if a good combination of parameters such as (T0,T1). In the case of exponential back-off, the back-off rate r can be used as an extra degree of freedom to fine tune the level of fairness between Wi-Fi and LAA.

Proposal 2: Further investigation is needed to examine the sensitivity of the optimal parameters such as initial CCA slot duration, extended CCA slot duration, and exponential back-off rate in different deployment scenarios including

- Different proportions of downlink and uplink traffic for Wi-Fi

- Different proportions of traffic mix in both Wi-Fi and LAA

# References

- TR 36.889, Study on Licensed-Assisted Access to Unlicensed Spectrum, V0.1.1, (2014-11).

- RAN1 Chairman’s Notes, 3GPP TSG RAN WG1 Meeting #79, San Francisco, USA, 17th – 21st November 2014.

- R1-144603, “Initial Coexistence Evaluations comparing LAA Versus Wi-Fi”, Cisco Systems.

- ETSI EN 301 893 V1.7.1.

- A. Babaei, J. Padden, J. Andreoli-Fang, “Overview of EU LBT and its Effectiveness for Coexistence of LAA LTE and Wi-Fi”, IEEE 802.19-14/0082r0, Nov., 2014.

- BRAN(14)000098r1, “5 GHz Coexistence Simulations”, Broadcom, BRAN#81, 2014-12-10.

- BRAN(14) 000095a1, “5 GHz Coexistence Simulations LAA-LTE (LTE-U) versus Wi-Fi”, CableLabs, BRAN#81, 2014-12-10.

- R1-144000, “Solutions for required functionalities and design targets”, Qualcomm.

- R1-145132, “Discussion on the comparison of LBE and FBE for LBT”, Coolpad.

- R1-145193, “Details of Listen-Before-Talk for LAA”, Ericsson.

- BRAN(14) 000097, “Let’s make equal sharing in 5GHz a reality”, Cisco, BRAN#81, 2014-12-10.

- BRAN(14) 000102, “WFA proposal for addressing comments on clause 4.8.3.2 on LBT in EN 301 893 v1.7.2”, WiFi Alliance, BRAN#81, 2014-12-10.

- R1-150549, “Proposal for achieving fair coexistence between LAA and Wi-Fi”, Cisco, Feb. 9-13, 2015.

- ETSI EN 301 893 V1.8.0.

- R1-150712, “Simulation results for Coexistence of LAA and Wi-Fi”, Cisco, Feb. 9-13, 2015.

- R1-150584, “Further details on LBT for LAA”, Ericsson, Feb. 9-13, 2015.

# Appendix:

# Detailed Simulation Parameters and Assumptions

The working assumption and agreements from RAN1#79 on detailed coexistence evaluation assumptions in LAA are reproduced in the tables below with the addition of a column to report our implementation against each of these.

## Indoor scenario for LAA

| Indoor scenario for LAA | Licensed cell | Unlicensed cell | Current assumption |
| --- | --- | --- | --- |
| Layout for nodes | For DL-only coexistence evaluations: Two operators deploy 4 small cells each in the single-floor building. The small cells of each operator are equally spaced and centered along the shorter dimension of the building. The distance between two closest nodes from two operators is random. The set of small cells for both operators is centered along the longer dimension of the building. |  | Only unlicensed cell is modelled The location of the APs of one operator is equi-distance from APs of the second operator (with the exception of the edge APs). |

| Indoor scenario for LAA | Licensed cell | Unlicensed cell | Current assumption |
| --- | --- | --- | --- |
| System bandwidth per carrier | 10MHz | 20MHz | Only a 20MHz unlicensed cell is explicitly modelled |
| Carrier frequency | 3.5 GHz | 5.0GHz | Use 5.3 GHz which is the centre frequency of the 5.0GHz unlicensed band (channel 60) |
| Number of carriers | 2 (one for each operator) | For DL-only LAA coexistence evaluations: 1, 4 (to be shared between two operators) | One 20MHz channel for the unlicensed LAA-LTE cells to be modelled. This is shared between the two operators. |
| Total BS TX power | 24dBm (Ptotal per carrier) | 18 dBm across aggregated carriers Optional: 24 dBm | 18dBm |
| Total UE TX power | Total UE TX power: 23dBm across aggregated cells Max total UE TX power per cell in licensed spectrum: 23dBm Max total UE TX power across aggregated cells in unlicensed spectrum: 18 dBm |  | 18dBm |
| Distance-dependent path loss | Small cell-to-Small cell, Small cell-to-UE: ITU InH [referring to Table B.1.2.1-1 in TR36.814] Indoor UE-to-indoor UE: 3GPP TR 36.843 (D2D). (3D distance between an eNB and a UE is applied. Working assumption is that 3D distance is also used for LOS probability and break point distance) |  | Same |
| Penetration | 0dB |  | Same |
| Shadowing | ITU InH [referring to Table A.2.1.1.5-1 in TR36.814] Working assumption is that 3D distance is used for shadowing correlation distance |  | Same as the ITU InH model with 1dB indoor handover margin (TR 36.814) |
| Antenna pattern | 2D Omni-directional is baseline; directional antenna is not precluded |  | Same |
| Antenna Height: | 6m |  | Same |
| UE antenna Height | 1.5m |  | Same |
| Antenna gain + connector loss | 5dBi |  | Antenna + connector: 5dBi 2dB feeder loss to calibrate against TR36.814 InH model |
| Antenna gain of UE | 0 dBi |  | Same |
| Fast fading channel between eNB and UE | ITU InH |  | Ped A 3kmh assumed for LTE-LAA link level |
| Number of clusters/buildings per macro cell geographical area | N/A |  | N/A |
| Number of small cells per cluster | N/A |  | N/A |
| Number of small cells per Macro cell | N/A |  | N/A |
| Number of UEs | 10 UEs per unlicensed band carrier for DL-only LAA coexistence evaluations |  | Same |
| UE dropping per network | All UEs should be randomly dropped and be within coverage of the small cell Example of a dropping method to achieve this with N=10 UEs: - Drop a large enough number of UEs, so that at least 10 UEs are covered by the small cell. Randomly select 10 UEs from the UEs that have coverage. |  | Same |
| Radius for small cell dropping in a cluster | N/A |  | N/A |
| Radius for UE dropping in a cluster | N/A |  | N/A |
| Minimum distance (2D distance) | 3m |  | Assume 3m is the minimum distance for AP to UE not UE to UE or AP to AP distance. Assume Local Area Base Station type with 45dB MCL as specified in 36.104 V11.9.0 (2014-07) |
| Traffic model | FTP Model 3: Based on FTP model 2 as in TR 36.814 with the exception that packets for the same UE arrive according to a Poisson process and the transmission time of a packet is counted from the time instance it arrives in the queue. FTP Model 1 as in TR 36.814 FTP model file size: 0.5 Mbytes. Optional: Mixed traffic model with each UE carrying only VoIP traffic or only FTP traffic in the Wi-Fi network that is not replaced by LAA. - Two UEs with VoIP traffic in addition to UEs with FTP traffic - The VoIP traffic model is based on G.729A (data rate is 24 kbps) - Packet inter-arrival time: 20 ms - Packet size: 60 bytes (payload plus IP header overhead) - Voice activity is assumed to be 100% statistics are independently reported in each direction No associated control plane traffic is modelled |  | All LAA-LTE UEs will use: - FTP 3 traffic - With 0.5MB file sizes Wi-Fi victim network modelling considers the scenario of FTP3 in DL traffic alone The aggressor {LAA, Wi-FI} and victim Wi-Fi network uses loading levels of Lambda={ 0.5,1.0,1.4, 1.75, 2.0 2.5, 3.5} Hz |
| UE receiver | MMSE-IRC as baseline |  | Rel 8 UE receiver (as in [3]) |
| UE noise figure | 9dB |  | Same |
| UE speed | 3km/h |  | Same |
| Cell selection criteria | For LAA UEs, cell selection is based on RSRP in the unlicensed band. For WiFi STAs, cell selection is based on RSS (Received signal power strength) of WiFi APs. RSS threshold is -82 dBm. |  | Same |
| UE Bandwidth | UE bandwidth for LAA: 10 MHz licensed + 20 MHz unlicensed - CA scheduling assumptions stated when reporting results - Served traffic per small cell per carrier can be reported UE bandwidth for Wi-Fi: 20 MHz unlicensed |  | Licenced portion of LAA-LTE not modelled. 20 MHz bandwidth for unlicensed carrier assumed throughout |
| Network synchronization | For the same operator, the network can be synchronized and the assumed synchronization accuracy in such simulations should be stated. Small cells of different operators are not synchronized. |  | Synchronized |
| Backhaul assumptions | Dropped in R1-145453 |  | Not modelled |
| Performance metrics | Performance metric - User perceived throughput (UPT) UPT CDF File throughput is calculated per file Unfinished files should be incorporated in the UPT calculation. The number of served bits (possibly zero) of an unfinished file by the end of the simulation is divided by the served time (simulation end time – file arrival time). User throughput is the average of all its file throughputs - Latency (From packet arrival in devices (eNB, AP, UE, STA) MAC buffer to successful transmission (including retransmission) of packet) Latency CDF If VoIP users are included, number of VoIP users with 98%ile latency greater than 50 ms should be reported Note: DL and/or UL can be reported when applicable |  | FTP network throughput and UPT Latency and UPT CDFs as well as mean UPT and latency per users are collected and presented in a separate template. |

## Additional LAA assumptions

|  | 3GPP value as per R1-145453 | Current assumption |
| --- | --- | --- |
| PCI planning for each NW | Planned | Not explicitly modelled (as in [3]) |
| Antenna configuration | 2Tx2Rx in DL, Cross-polarized. Optional: 1Tx2Rx in DL. 1Tx2Rx in UL (should be the same as for Wi-Fi) | 2x2 Cross polarized (as in [3]) TM4 (closed loop spatial multiplexing, which falls back to TM6 for low SINR/scattering) |
| Transmission schemes | Based on TM4 or TM10, QPSK/16QAM/64QAM Optional: include 256QAM (should be the same as for Wi-Fi) | Not explicitly modelled but implied in SINR to Tput mapping used. Includes 256 QAM |
| Turbo code block interleaving depth | Per LTE specs (1-14 LTE OFDM symbols dependent on MCS and PRB allocation) | Not explicitly modelled |
| Scheduling | Proportional fair | Fair (as in [3]) |
| Link adaptation | Realistic | Ideal (as in[3]) |
| CCA-ED | Up to each company; should state assumption when reporting results | Assume an LAA energy detection threshold of -62dBm for CAA-ED and -82dBm for CCA-PD where a recognizable preamble is transmitted (in line with Wi-Fi) |
| Channel selection | Up to each company; should state assumption when reporting results | Not implemented (as in[3]) |
| Cyclic Prefix | Normal | Not explicitly modelled but assumed in SINR to Tput mapping used. |

## Additional Wi-Fi assumptions

| Parameter |  | 3GPP value as per R1-145453 | Current assumption |
| --- | --- | --- | --- |
| MCS |  | 802.11ac MCS table without 256 QAM Optional: include 256QAM (should be the same as for LAA) | Yes, 256 QAM included for Wi-Fi |
| Antenna configuration |  | 2Tx2Rx in DL, Cross-polarized Optional: 1Tx2Rx in DL. UL: 1Tx2Rx (should be the same as for LAA) Baseline: open loop Company should state assumptions if assumed otherwise | As in phase 1: 2Tx2Rx in DL, Cross-polarized UL 1x2 (which is a change from 2x2 in phase 1) |
| Channel coding |  | BCC Optional: LDPC code | LDPC |
| Frame aggregation |  | A-MPDU | Yes |
| MPDU size |  | Up to each company | Fixed 1500B MPDU size (variable transmission duration) as in [3] |
| Max PPDU duration |  | Baseline:< 4 ms (Asynchronous to LTE timing) Company should state assumptions if assumed otherwise | 4.096ms maximum PPDU applied. |
| MAC | Coordination | DCF If VoIP users are included, EDCA can be used | DCF (no EDCA) |
|  | SIFS, DIFS | SIFS, DIFS | Yes |
|  | Detection | Energy detection & preamble detection | Yes |
|  | RTS/CTS | Optional | Modelled |
|  | Contention window | Per DCF If VoIP users are included, per EDCA can be used | Yes – EDCA not being used for VoIP users, as only FTP simulations are involved |
| CCA-PD |  | -82dBm and preamble decoding (Note preamble occupies the 20MHz system bandwidth with rate 1/2 coding and BPSK modulation) | Yes |
| CCA-ED |  | -62dBm | Yes |
| ACK Modeled (successful reception, resources utilized) |  | Yes | Yes |
| DL/UL Duplexing |  | DL traffic only for DL-only LAA coexistence evaluation | DL only for victim network DL only for aggressor network |
| Rate control |  | Up to each company; should state assumption when reporting results | Same rate adaption as in [3] |
| Channel selection |  | Up to each company; should state assumption when reporting results | Congested scenario: Single 20MHz channel for all APs |
| OFDM symbol length |  | 4 micro second | As in [3], short guard interval is adopted therefore OFDM symbol length is 3.6 micro second |

In option A of [14], r is set to be 2.