3GPP TSG-RAN WG2 #100	Tdoc R2-1712955

# Reno, Nevada, 27th November – 1st December 2017

Agenda Item:	9.19

Source:	Ericsson

Title:	Solutions for LCID space extension

Document for:	Discussion, Decision

# Introduction

In the end of Rel-14 it was proposed that the LCID space in MAC shall be extended since there are soon no more LCIDs left. However this was postponed to Rel-15.

It should be noted that it is discussed to extend the number of DRBs for LTE and that may require an extension of LCIDs.

# Discussion

As explained in our previous contribution [1] we are running out of LCIDs to use for new MAC CEs as shown in Table 1. Currently there are only 7 LCIDs left for the UL based on TS 36.321 version 14.4.0.

Table 1 – Remaining LCIDs in MAC per release.

| Release | Remaining LCIDs |  |
| --- | --- | --- |
|  | DL | UL |
| Rel-11 | 16 | 14 |
| Rel-12 | 15 | 10 |
| Rel-13 | 13 | 10 |
| Rel-14 | 10 | 7 |

While RAN2 can choose to postpone this problem until all LCIDs are used we think there are good arguments to extend the LCID space now. As seen in Rel-14 there has been a need to use different values for the CCCH when transmitting message 3 in the random access procedure. A UE of Category 0 uses the LCID value “01011” for CCCH (unless it supports frequency hopping in which case it uses value "01100") whereas other UEs use value “00000”. We think this is an example when the UE must convey a certain capability very early in the connection procedure to the eNB. We think this possibility should be open also in future releases for scenarios where no other feasible solution can be found. If RAN2 decides to correct the problem of the LCID space later with a solution which increases the size of the MAC subheader for message 3, the random access procedure can no longer be flexibly used to convey early capabilities.

The current LCID value can be used to convey UE capabilities before the RRC connection has been setup.

Future extensions of the LCID space should support the UE to transmit UE capabilities before the RRC connection has been setup.

## Solution 1 – Use of the R-bit to indicate a new LCID table

Figure 1 shows the existing R/F2/E/LCID subheader. The R-field is currently set to 0 and has no meaning today. It can be used to introduce additional LCID values.

Figure 1 – R/F2/E/LCID sub-header

If the R-bit is set equal to 1 a different table of LCID values would be used. This table would not contain any LCIDs for the existing MAC CEs or for logical channels, but instead be for 32 new MAC CEs only. Strictly speaking, the R-bit is turned to a “V-bit” (V for version), making the new sub-header look like in Figure 2 (corresponding changes can be made to the other MAC sub-headers).

Figure 2 – V/F2/E/LCID sub-header

If the receiver (eNB or UE) gets a MAC sub-header with the V-bit set to 0 it would use the existing tables 6.2.1-1 and 6.2.1-2 for DL and UL. If the V-bit is set to 1 the receiver would use new tables.

The drawback with this solution is that no new reserved bits which can be used for future extensions are introduced so there are no R-bits left. Any future extensions must be made using other techniques (e.g. using a special value of LCID). An advantage is that this solution does not increase the size of the MAC sub-header so it is particularly useful for scenarios when overhead is critical.

## Solution 2 – Use of the R-bit to indicate a new LCID field

Solution 2 is similar to solution 1 in that the R-bit is set equal to 1. However, unlike solution 1 for solution 2 the R-bit (or V-bit) indicates presence of 1 additional byte which allows for more R-bits and a new longer LCID field. This is shown in Figure 3.

Figure 3 – V/F2/R/R/R/R/R/R/E/LCID sub-header

If the receiver (eNB or UE) gets a MAC sub-header with the V-bit set to 0 it would use the existing tables 6.2.1-1 and 6.2.1-2 for DL and UL. If the V-bit is set to 1 the receiver would use the new longer LCID field and new tables.

An advantage of this solution is that it introduces new R-bits which allows for future extensibility. It also introduces a longer LCID field allowing for 128 new LCID values, but 32 of these must be used to cater for the LCID values existing today, so in principle only 96 LCID values remain. However, if this is deemed too small the LCID field can be extended at the expense of fewer R-bits. The drawback is that the sub-header size is increased with one byte. However as explained in [1] RAN2 could assign LCIDs for MAC CEs used in overhead critical scenarios to the existing (5-bit) tables.

If this solution is chosen RAN2 should decide whether to move the LCID values of MAC CEs introduced in Rel-15 to the new LCID table to keep some reserved LCID values for overhead-critical scenarios.

## Solution 3 – Special LCID value

This solution introduces a special LCID value among the existing values (see Index 01011 in Table 2) as an indicator of presence of a new LCID-field in the header which has a new mapping table. The example is for the downlink, but the principle is the same for the uplink.

Table 2 (Table 6.2.1-1 Values of LCID for DL-SCH)

| Index | LCID values |
| --- | --- |
| 00000 | CCCH |
| 00001-01010 | Identity of the logical channel |
| 01011 | Presence of new LCID field |
| 01100-10100 | Reserved |
| 10101 | Activation/Deactivation of CSI-RS |
| 10110 | Recommended bit rate |
| 10111 | SC-PTM Stop Indication |
| 11000 | Activation/Deactivation (4 octets) |
| 11001 | SC-MCCH, SC-MTCH (see note) |
| 11010 | Long DRX Command |
| 11011 | Activation/Deactivation (1 octet) |
| 11100 | UE Contention Resolution Identity |
| 11101 | Timing Advance Command |
| 11110 | DRX Command |
| 11111 | Padding |
| NOTE: Both SC-MCCH and SC-MTCH cannot be multiplexed with other logical channels in the same MAC PDU except for Padding and SC-PTM Stop Indication |  |

If the LCID is set to the “special value” the MAC subheader could look like in Figure 4 and the UE reads LCID2 from a new table to find out which MAC CE is included.

Figure 4 – R/F2/E/LCID/R/R/R/LCID2 sub-header

This way we introduce 32 new LCID values, but we could also make use of the 3 new R-bits and have an 8-bit LCID2 field introducing 256 new LCID values. The possibility to add 256 new LCIDs together with the fact the R-bits are left for future extensions are the advantages of this solution. A drawback is that the solution adds one extra byte making it difficult to use in overhead-critical scenarios.

If this solution is chosen RAN2 should decide whether to move the LCID values of MAC CEs introduced in Rel-15 and onwards to the new LCID2 table to keep some reserved LCID values for overhead-critical scenarios.

## Evaluation

Solution 1 introduces no additional size while solutions 2 and 3 introduce one additional byte. Solutions 2 and 3 allow for introduction of more than 32 new LCID values whereas solution 1 is restricted to 32 values.

Table 3 – Evaluation result of the solutions

|  | Solution 1 | Solution 2 | Solution 3 |
| --- | --- | --- | --- |
| Additional size | 0 bytes | 1 byte | 1 byte |
| Amount of new LCIDs | 32 | More than 32 | More than 32 |
| R-bits left | No | Yes | Yes |

Even though solution 1 has the lowest additional size we think the lack of R-bits limits future extensibility. Therefore, it is ruled out. When selecting between solutions 2 and 3 it can be understood that the exact number of new LCIDs in both solution 2 and solution 3 can be traded for R-bits. If solution 3 is setup with 6 R-bits (as shown in figure 5) to allow for a fair comparison with solution 2, only three bits are left for the LCID2 field. Hence only 8 new LCID values can be introduced in this case. As solution 2 allows for 96 new LCID values and 6 R-bits we think solution 2 should be chosen.

Figure 5 – Solution 3 with 6 R-bits

In conclusion, solution 2 has 13 bits to spend for LCID field and R-bits, while solution 3 has 9 bits for LCID field and R-bits.

Change the R-bit in the MAC sub-headers for UL-SCH and DL-SCH to a V-bit.

When the V-bit is equal to zero the existing tables for LCID values are used.

Introduce a new V/F2/R/R/R/R/R/R/E/LCID MAC sub-header with 7-bit LCID field and new corresponding sub-headers for other values of F2 and E.

Introduce new LCID tables for 7-bit LCID values to be used when the V-bit is equal to 1.

Out of the added MAC CEs added in Rel-15 and onwards the non-overhead critical MAC CEs should be assigned to the new LCID table.

## Need for a capability bit

There is no need for a capability bit for this change as such. Instead we can stick to the current scheme where individual MAC CEs require a capability bit.

No need to introduce a capability bit to show that a UE supports this solution.

# Conclusion

In section 2 we made the following observations:

Observation 1	The current LCID value can be used to convey UE capabilities before the RRC connection has been setup.

Observation 2	No need to introduce a capability bit to show that a UE supports this solution.

Based on the discussion in section 2 we propose the following:

Proposal 1	Future extensions of the LCID space should support the UE to transmit UE capabilities before the RRC connection has been setup.

Proposal 2	Change the R-bit in the MAC sub-headers for UL-SCH and DL-SCH to a V-bit.

Proposal 3	When the V-bit is equal to zero the existing tables for LCID values are used.

Proposal 4	Introduce a new V/F2/R/R/R/R/R/R/E/LCID MAC sub-header with 7-bit LCID field and new corresponding sub-headers for other values of F2 and E.

Proposal 5	Introduce new LCID tables for 7-bit LCID values to be used when the V-bit is equal to 1.

Proposal 6	Out of the added MAC CEs added in Rel-15 and onwards the non-overhead critical MAC CEs should be assigned to the new LCID table.

# References

[1]	R2-1712956, "LCID space extension", Ericsson, RAN2#100