3GPP TSG-RAN WG2 #101	Tdoc R2-1803531

Athens, Greece, 26th February – 2nd March 2018

Agenda Item:	10.4.3.2

Source:	Ericsson

Title:	Definitions for PSCell/Pcell/SpCell (38331 RIL F008)

Document for:	Discussion, Decision

# Introduction

This document discusses PSCell/SCell terminology as discussed in RAN2 Ad Hoc in Vancouver. One main part of the discussion was if PSCell is considered as Scell or not. The following agreements where made:

R2-1801665	Offline discussion PSCell/Scell terminology in NR	Ericsson

=>	PSCell is not defined to be an SCell.

=>	Refined set of definitions can be brought to the next meeting.

This contribution intends to propose definitions as those were postponed in Vancouver meeting.

## Background

As a comparison, in LTE RRC, there are following definitions:

Primary Cell: The cell, operating on the primary frequency, in which the UE either performs the initial connection establishment procedure or initiates the connection re-establishment procedure, or the cell indicated as the primary cell in the handover procedure.

Primary Secondary Cell: The SCG cell in which the UE is instructed to perform random access or initial PUSCH transmission if random access procedure is skipped when performing the SCG change procedure.

Primary Timing Advance Group: Timing Advance Group containing the PCell or the PSCell.

PUCCH SCell: An SCell configured with PUCCH.

Secondary Cell: A cell, operating on a secondary frequency, which may be configured once an RRC connection is established and which may be used to provide additional radio resources.

Secondary Cell Group: For a UE configured with DC, the subset of serving cells not part of the MCG, i.e. comprising of the PSCell and zero or more other secondary cells.

Serving Cell: For a UE in RRC_CONNECTED not configured with CA/ DC there is only one serving cell comprising of the primary cell. For a UE in RRC_CONNECTED configured with CA/ DC the term 'serving cells' is used to denote the set of one or more cells comprising of the primary cell and all secondary cells.

These definitions need update due to agreements form the last meeting but can be still taken as starting point.

## Text proposal for 38.331

In this document following definitions are proposed:

Primary Cell: The MCG cell, operating on the primary frequency, in which the UE either performs the initial connection establishment procedure or initiates the connection re-establishment procedure.

Primary SCG Cell: The SCG cell in which the UE is instructed to perform random access when performing the Reconfiguration with Synch procedure.

Secondary Cell: For a UE configured with CA, a cell providing additional radio resources on top of Special Cell.

Secondary Cell Group: For a UE configured with dual connectivity, the subset of serving cells comprising of the PSCell and zero or more secondary cells.

Serving Cell: For a UE in RRC_CONNECTED not configured with CA/DC there is only one serving cell comprising of the primary cell. For a UE in RRC_CONNECTED configured with CA/ DC the term 'serving cells' is used to denote the set of cells comprising of the Special Cell(s) and all secondary cells.

Special Cell: For Dual Connectivity operation the term Special Cell refers to the PCell of the MCG or the PSCell of the SCG, otherwise the term Special Cell refers to the PCell.