3GPP TSG-RAN WG2 Meeting #102                                                       R2-1808302

Busan, Korea, May 21– 25, 2018                                                  (Revison of R2-1806056)

Agenda item:	10.2.6

Source:	Samsung

Title:	MR-DC bearer management in inactive state

Document for:	Discussion & Decision

1 Introduction

Previously in RAN2#99 meeting, RAN2 agreed not to support resume of DC configurations from suspend /inactive state [1]:

Agreements: For MR-DC and NR-DC we will not support that the UE can resume the DC configuration after the UE returns from suspended/inactive in Rel-15.

In RAN2#99bis meeting, SA2 requests to RAN2 in LS (S2-176158) [2].

SA2 prefers that dual connectivity configuration is maintained while the UE is in RRC inactive in order to potentially avoid unnecessary signalling between the RAN and CN, and within the CN, whenever the UE moves between RRC connected and RRC inactive states. In this respect it is ideal if the UE resumes the RRC connection in the Master RAN node, and if based on RAN conditions the Secondary RAN node configuration remains valid, the Secondary RAN node configuration is expected to be resumed too. SA2 requests RAN2 and RAN3 to study and provide specification for the signalling between the Secondary RAN node and Master RAN node if dual connectivity has been established for the UE for which there is at least one N3 tunnel termination to the Secondary RAN node for at least one 5G QoS Flow.

Then RAN2 replied to SA2 in R2-1712063 [3] as below.

RAN2 thanks SA2 for the LS on coexistence between RRC inactive and dual connectivity (S2-176158). RAN2 had discussed this in RAN2#99 and agreed not to maintain the dual connectivity configuration when UE returns from suspended/inactive in Rel-15. RAN2 is still discussing what this implies in terms of SDAP/PDCP configurations in the SCG.

With related discussion, RAN2 discussed in RAN2#100 as follows:

R2-1712613 Handling of EN-DC with suspend/resume ZTE Corporation, Sane Chips discussion Rel-15 - Intel thinks the latest RRC CR addresses this. The SCG configuration is always released for re-establishment and resume. For the radio bearer configurations they need to be kept and the MN can keep them or reconfigure them at re-establishment. For suspend case there are more cases that need to be discussed. - Ericsson think it is important to keep the PDCP configuration of the bearer. - Samsung think that for release 13 suspend the S1 connection is released completely so why is this needed. - Ericsson think in R13 the SCG config is released when the UE resumes, but now for R15 we can keep the bearer configuration but release the SCG configuration. Intel think this is sufficient from the UE point of view. - Samsung think keeping the bearer configuration only makes sense if the S1 is kept. => Noted

Although RAN2 clearly agreed to “not support that the UE can resume the DC configuration after the UE returns from suspended/inactive in Rel-15”, there’s SA2’s preference “that dual connectivity configuration is maintained while the UE is in RRC inactive” to reduce the signalling overhead.  Hence, now RAN2 have the further study on “what this implies in terms of SDAP/PDCP configurations in the SCG”.

In this contribution, we discuss when and how SN (SCG bearer) can be released and what shall be done for the SN context while DC configuration is not supported for resumption from suspend/ inactive in Rel-15.

2 Discussion

From the previous agreements and LSs, we can summarize the agreements from RAN2 and SA2 as follows:

- RAN2 agreed not to maintain the dual connectivity configuration when UE returns from suspended/inactive in Rel-15

- RAN2 is still discussing what this implies in terms of SDAP/PDCP configurations in the SCG.

- SA2 prefers that dual connectivity configuration is maintained while the UE is in RRC inactive in order to potentially avoid unnecessary signalling

Considering the timeline of EN-DC which needs to be closed already, we cannot afford of large change of specification which may require both RAN2 and RAN3 involvements (e.g. to enable suspend resume signaling over the Xn interface, handling the resume behavior of SCG part, etc).

Therefore, in this contribution, we would like to discuss the SDAP/PDCP configurations in the SCG for UE transition to inactive state while DC is configured, without requiring large specification change. Upon transition to the INACTIVE state it is possible to consider UE re-configuration scenarios as exemplified in Figure 1.

- Keep SDAP/PDCP configurations of SN: Figure 1(a) shows that SDAP/PDCP configuration of SN (SCG bearer) is maintained while RLC/MAC/PHY are released, and the both of MN and SN are connected to CN.

- Release all the configurations of SN: Figure 1(b) shows that all the configurations of SN are released. The bearer on SN can be re-located to MN before suspension using bearer type change changing, or just be released. For the bearer change, there could be two different approaches: (b-1) Bearer change from SN PDCP to MN PDCP and (b-2) Bearer type change from split bearer to MCG bearer before suspension.

| - Keep SDAP/PDCP configurations of SN while releasing RLC/MAC/PHY layer configurations | - Release all the configurations of SN |
| --- | --- |

Figure 1: Example of two options (a) keep SN SDAP/PDCP configuration or (b) release full SN configuration.

Then the additional signaling for suspension and resumption are summarized as the following Table 1.

Table 1. Comparison of required Xn/CN signals for the two suspension options.

| Options |  | (a) Keep SN SDAP/PDCP configuration | (b) Release full SN configuration |  |
| --- | --- | --- | --- | --- |
|  |  |  | (b-1) Bearer change from SN PDCP to MN PDCP before suspension | (b-2) Bearer type change from (MCG) split bearer to MCG bearer before suspension |
| Required signalling for suspension | Xn | Marginal | Marginal | Marginal |
|  | CN | Not required : SDAP/PDCP configuration is saved but PHY channel is released when suspension | Required : the SN bearer change requires CN signalling. | Not required : the split bearer change does not require CN signalling. (from MCG split bearer to MCG bearer) |
| Required signalling For DC (SN) configuration after resumption | Xn | Not required : no SN capability negotiation needed. (reuse saved SN context) | Required : MN needs to add SN and SN capability negotiation needed | Required : MN needs to add SN and SN capability negotiation needed |
|  | CN | Not required : no CN signalling is needed | Required : when the SN bearer is released or changed bearer type, CN signalling is needed for PDU session reestablishment. | Not required : the split bearer change does not require CN signalling. (from MCG bearer to split bearer) |

According to the above Table 1, the option (a) requires the least additional signalling, and the option (b-1) requires the most additional signalling.

Observation 1: Keeping SDAP/PDCP configuration upon transition to INACTIVE can save additional signalling for suspension and DC configuration after resumption.

With option (a), the DRBs in SN are not released but suspended and the connection between SGW/UPF and SN is maintained. In order to realize the option (a), NR needs new RAN features including RAN-based paging from SN, mobility support by RLAU, and UE context saving/ retrieve procedures as follows:

- RAN-based paging initiated from SN: if there are DL packets arriving for these DRBs towards the SN, then INACTIVE UE shall be paged. This would require SN also maintaining the UE context and SN needs to indicate the MN to page the UE.

- Mobility support by RLAU: if UE moves out of RAN Notification Area (RNA), RAN3 need to support new RLAU procedures for mobility support considering MN/SN architecture, e.g., MN change while maintaining the same SN.

- UE context saving and retrieve: the UE context with both of MN and SN needs to be saved and UE context retrieve procedures between MN and SN is also needed.

Even though one could argue that all the aforementioned RAN enhancements to keep the SN context are fully constrained in RAN3 and are not visible to UE, it anyway requires non-trivial amount of standardization efforts. Nevertheless, it could be premature to exclude immediately a possibility to keep the SN context, especially accounting for the fact that it is completely transparent to the UE. Based on these considerations we propose that it could be up to the network deployment and implementation to choose which option to follow. If the network deployment and implementation does not allow for keeping the SN context upon transition to INACTIVE, the network will command UE to change/release the corresponding bearers. If RAN3 related enhancements become available and are supported by the network, the latter can always ask a UE to keep the current bearer configuration.

It is also worth noting that suspension of the SN bearers upon transition to INACTIVE is feasible only if the SN node supports the INACTIVE state. In other words, it is not possible to keep SN bearers in option 3 because both MN and SN are connected to the EPC core that does not support a concept of the INACTIVE state. On the contrary to it, suspension of the SN bearers should be feasible in option 2 (NR-NR DC). Option 4 and option 7 need further discussions because even though both the MN and SN are connected to 5GC, final design of the INACTIVE state for LTE connected to 5GC is not clear yet.

Proposal 1: Both options of keeping SN SDAP/PDCP configuration and release full SN configuration should be supported for the future flexibility (to account further decision in RAN3).

Proposal 2: It is up to the network implementation whether to re-configure SCG or MCG split bearers or to keep them.

Some additional RAN2 enhancements were proposed to reconfigure SCG radio configuration early at connection resumption:

- Early measurement report to help gNB configure SCG leg at connection resumption

- Indication on whether some of the last SCG cells at connection suspension are still the best cells on SN layers from UE

The enhancements will require substantial amount of meeting time in RAN2 and also RAN4 to discuss brand new RRC procedures and corresponding performance requirements. Considering that the late drop has wide work scope (i.e. architecture option 4, 7 and probably NR-DC) with limited number of meetings, it is proposed not to pursue these enhancements in rel.15. Blind SCG addition that has been available and supported from legacy LTE-DC can be used instead.

Proposal 3:	In MR-DC, SCG radio configuration can be reconfigured blindly at connection resumption. No additional enhancement is considered in rel.15.

3 Conclusion

RAN2 is requested to discuss and if possible agree on the following proposal for suspension and resumption in MR dual connectivity:

Observation 1: Keeping SDAP/PDCP configuration upon transition to INACTIVE can save additional signalling for suspension and DC configuration after resumption.

Proposal 1: Both options of keeping SN SDAP/PDCP configuration and release full SN configuration should be supported for the future flexibility (to account further decision in RAN3).

Proposal 2: It is up to the network implementation whether to re-configure SCG or MCG split bearers or to keep them.

Proposal 3: In MR-DC, SCG radio configuration can be reconfigured blindly at connection resumption. No additional enhancement is considered in rel.15.

References

- RAN2#99 Chairman’s report

- S2-176158 LS on coexistence between RRC inactive and dual connectivity Qualcomm

- R2-1712063 Reply LS on coexistence between RRC inactive and dual connectivity Intel Corporation

- R2-1708808 Inactive and dual connectivity Intel Corporation

- R2-1711665 Suspension to INACTIVE in NR Dual connectivity Samsung