# 3GPP TSG-RAN2 Meeting #103bis	R2-1815861

# Chengdu, China, 2018-10-08 to 2018-10-12

Title:	Reply LS on bandwidth configuration for initial BWP

Response to:	LS on bandwidth configuration for initial BWP (R2-1813522)

Release:	Release 15

Work Item:	NR_newRAT-Core

Source:	RAN2

To:	RAN1

Cc:	-

Contact Person:

#### Name:	Henning Wiemann

Tel. Number:	+49-2407-575-0

E-mail Address:	Henning [dot] Wiemann [at] Ericsson [dot] com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1. Overall Description:

RAN2 would like to thank RAN1 for their LS on bandwidth configuration for initial BWP. RAN2 discussed how to capture the RAN1 agreements provided therein and reached the following agreements:

Agreements

1	Capture in the field description of the initialDownlinkBWP in DownlinkConfigCommonSIB that “The network configures the  locationAndBandwidth so that the initial downlink BWP contains the entire CORESET#0 of this serving cell in the frequency domain.”

2	Capture in the field description of the commonControlResourceSet that “The network configures the commonControlResourceSet in SIB1 so that it is confined in the bandwidth of CORESET#0”.

3	Capture in the field description of initialDownlinkBWP that “The  locationAndBandwidth is applicable after reception of Msg4.”

4	The rules how the UE determines the DCI sizes based on CORESET#0 are not captured in the RRC specification since this is clearly the scope of the L1 specs.

5	The problem that the UE is not configured with CORESET#0 should be resolved by RAN1 and captured in their specs, e.g. UE determines DCI 1-0 size by CORESET#0 if configured and by the width of the initial DL BWP otherwise.

RAN2 would like to highlight in particular agreement 5 and provide the following background information:

RAN2 discussed that there are cases in which the UE is not configured with a CORESET#0. In those cases, the UE cannot determine the DCI size from the size of CORESET#0. RAN2 discussed different options how to resolve this problem and agreed that RAN1 should resolve this case in the L1 specifications e.g. by determining the DCI-size from the size of the initial BWP if a CORESET#0 is not configured.

2. Actions:

To RAN1

ACTION: 	RAN2 respectfully asked RAN1 to take above agreements into account and to adjust the L1 specifications as indicated in agreement 5.

3. Date of Next TSG-RAN WG2 Meetings:

TSG-RAN WG2 Meeting #104 	2018-11-12 to 2018-11-16	Spokane, US.

TSG-RAN WG2 Meeting #105	2019-02-25 to 2019-03-01	Athens, Greece