3GPP TSG-RAN WG2 Meeting #104	DRAFT R2-1817499

Spokane, USA, 12 - 16 November 2018

Title:	[DRAFT] LS on UL HARQ operation for SUO case 1

Response to:	-

Release:	Release 15

Work Item:	NR_newRAT

Source:	Nokia [TSG RAN WG2]

To:	TSG RAN WG1

Cc:

Contact Person:

#### Name:	Chunli Wu

E-mail Address:	Chunli.wu@nokia-sbell.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

RAN2 discussed UL HARQ operation for SUO case 1 following the RAN1 agreement of 10 ms PUSCH HARQ RTT and understood that it is still synchronous HARQ operation with the timing defined that UL grant in subframe n scheduled PUSCH in subframe n+4, and the UL grant for the same UL HARQ process occurs in subframe n+10.

It was captured in MAC specification as UL HARQ RTT Timer of 6ms for this case which has now been removed as the UL HARQ RTT Timer is only used for asynchronous HARQ operation which is not the case for SUO case 1, hence not applicable here.

For synchronous HARQ operation, RAN2 specification has not captured any timing information on at which subframe the PDCCH and PUSCH for retransmission will happen. RAN2 expected RAN1 will capture it.

2. Actions:

To RAN1 group.

ACTION: 	RAN2 respectfully asks RAN1 to take the RAN2 agreement into account and update the specification if seen needed.

3. Date of Next TSG-RAN WG2 Meetings:

3GPP RAN2#105		25 Feb - 01 Mar 2018	Athens

3GPP RAN2#105bis		08 - 12 Apr 2018		China