3GPP TSG-RAN WG2 Meeting #105                                                 	R2-1900210

Athens, Greece, 25th Feb – 1st Mar 2019

Source:	CATT

Title:	Enhancements for initial cell setup

Agenda Item:	11.10.4

Document for:	Discussion and Decision

Introduction

In RAN#81 meeting, the objective of “Efficient and low latency serving cell configuration/activation/setup” is included in the DC/CA enhancement WID[1], and the context is listed as below:

- Efficient and low latency serving cell configuration/activation/setup: Minimizing signalling overhead and latency needed for initial cell setup, additional cell setup and additional cell activation for data transmission. [RAN2, RAN1, RAN4, RAN3] - This objective applies to MR-DC, NR-NR DC and CA - The objective should consider enhancements when starting from IDLE, INACTIVE mode and CONNECTED mode

The objective could be divided into three scenarios and should be considers for all the RRC states of IDLE, INACTIVE and CONNECTED mode:

- Initial cell setup;

- Additional cell setup;

- Additional cell activation for data transmission.

In this contribution, we intend to discuss how to reduce the signalling overhead and latency for the first scenario of initial cell setup.

Discussion

  - Minimize the signaling overhead and latency for CA

In NR, it is convenient to reuse the euCA solution [2] in LTE to reduce the signaling overhead and the latency. The procedure for initial CA setup could be performed for UE status change of “CONNECTED – IDLE/INACTIVE - CONNECTED” as below:

Figure1 Initial Cell Setup for CA Scenario

1) Using the procedure above, the latency could be saved:

- The steps 1~4 include a whole procedure of IDLE/INACTIVE mode measurement, based on the euCA IDLE mode measurement. The measurement configuration could be sent to the UE by dedicated signalling e.g. RRCRelease message or by system information. To achieve a faster measurement report, the step4 may be merged into the step3 procedure of initial cell setup (e.g. indicate measurement result available/report the measurement in msg5 of initial access procedure).

- In step 5, when the UE connected to the network, all the “active” cells(including Pcell and SCells) could be configured with “short CQI report” which CQI reporting cycle and the latency between CA setup and large data transmission will be reduced;

- In step6, a dormant SCell state could be introduced same as LTE euCA, characterized by the UE not monitoring PDCCH but continuing with CQI/RRM measurements and reporting. The CQI reporting for dormant Scell may be configured separately from the CQI reporting configuration of the Scell in activated state. And to configure the UE with direct Scell state configuration, the overhead of subsequent SCell activation by MAC CE is not required.  Timers for implicit SCell state transitions from activated to dormant and dormant to deactivated are introduced.

2) Using the procedure above, the signaling overhead could be saved:

- And also in step6, a common (S)cell configuration could be provided to the UE to minimize the signalling overhead. In euCA, only the common Scell configuration are focused, because of the different structure of configurations of PCell and SCell. But in NR, all cells(SPCells and SCells) are configured with the same structure(ServingCellConfigCommon IE and ServingCellConfig IE) as the figure below:

Figure2 Structure of CellGroupConfig IE

Due to the unified structure, the common SCell configuration solution in LTE could be extended to common cell configuration of all kinds of cells in NR, and the PCell/SCell could use the common cell configuration as a reference to perform a delta configuration.

Observation 1: Some methods of euCA in LTE could be reused in NR for minimizing signalling overhead and latency.

Proposal 1: Reuse the euCA solution in LTE for minimizing signaling overhead and latency in CA scenario, for initial cell setup:

- IDLE mode measurement, and extend to INACTIVE mode;

- Short CQI report, and extend to all active cells(PCell, SCell);

- Dormant SCell state and Direct Scell state configuration;

- Common Scell configuration, and extend to delta configuration for all kinds of cells.

  - Minimize the signaling overhead and latency for DC

In NR, it is convenient to reuse the euCA solution in LTE to reduce the signaling overhead and the latency. The procedure for initial DC setup could be performed for UE status change of “CONNECTED – IDLE/INACTIVE - CONNECTED” as below:

Figure3 Initial Cell Setup for DC Scenario

1) Using the procedure above, the latency could be saved:

- If euCA solution in LTE is reused for NR DC, the MN will send IDLE/INACTIVE mode measurement configuration to the UE. Before this, a negotiation (step0 in figure3) may be need between the MN and SN. A simplest method is that SN sends its interested carriers to the MN, and then MN sends carriers interested by both network nodes to the UE.

- The steps 1~4 include a whole procedure of IDLE/INACTIVE mode measurement, based on the euCA IDLE mode measurement. All steps are same with the steps in figure1. After step4, the MN has already received the measurement results of carriers it was configured before RRC connection release.

- In step 5~6, immediately after UE connect to the network, MN could decide to which SN to request a dual connection by existing IDLE/INACTIVE mode measurement result.

- In step7~10, subsequent steps of adding cell(s) in the SN for the UE same as current procedure could be performed.

During the procedure, all the “active” cells(including Pcell, PSCell and SCells) could be configured with “short CQI report” which CQI reporting cycle could be reduced  and the latency between DC setup and large data transmission will be reduced;

In step7, a dormant SCell state could be introduced same as LTE euCA for all SCells in MN and in SN. And to configure UE with direct Scell state configuration, the overhead of subsequent SCell activation by MAC CE is not required.

2) Using the procedure above, the signaling overhead could be saved:

- And also in step7 of RRCReconfiguration message, common (S)cell configuration in SN could be introduced based on the CA solution in section2.1 above. Since all CGs have the same structure, the common SCell configuration solution in LTE could be extended to common cell configuration of all kinds of cells in NR, and the PCell/PSCell/SCell could use the common cell configuration as a reference to perform a delta configuration.

Observation 2: Some methods of euCA in LTE could be reused in NR for minimizing signalling overhead and latency.

Observation 3: Besides the reused solution, negotiation between MN and SN may be needed.

Proposal 2: Reuse the euCA solution in LTE for minimizing signaling overhead and latency in DC scenario, for initial cell setup:

- IDLE mode measurement, and extend to INACTIVE mode;

- Short CQI report, and extend to all active cells(PCell, PSCell, SCell);

- Dormant SCell state and Direct Scell state configuration;

- Common Scell configuration, and extend to delta configuration for all kinds of cells.

Proposal 3: MN and SN negotiate the carriers to be measured in IDLE/INACTIVE mode.

Conclusion

According to the analysis in section 2, we have some observations as below:

Observation 1: Some methods of euCA in LTE could be reused in NR for minimizing signalling overhead and latency.

Observation 2: Some methods of euCA in LTE could be reused in NR for minimizing signalling overhead and latency.

Observation 3: Besides the reused solution, negotiation between MN and SN may be needed.

And according to the analysis, we propose:

Proposal 1: Reuse the euCA solution in LTE for minimizing signaling overhead and latency in CA scenario, for initial cell setup:

- IDLE mode measurement, and extend to INACTIVE mode;

- Short CQI report, and extend to all active cells(PCell, SCell);

- Dormant SCell state and Direct Scell state configuration;

Common Scell configuration, and extend to delta configuration for all kinds of cells.

Proposal 2: Reuse the euCA solution in LTE for minimizing signaling overhead and latency in DC scenario, for initial cell setup:

- IDLE mode measurement, and extend to INACTIVE mode;

- Short CQI report, and extend to all active cells(PCell, PSCell, SCell);

- Dormant SCell state and Direct Scell state configuration;

- Common Scell configuration, and extend to delta configuration for all kinds of cells.

Proposal 3: MN and SN negotiate the carriers to be measured in IDLE/INACTIVE mode.

Reference

- RP-182076 WID on Multi-RAT Dual-Connectivity and Carrier Aggregation enhancements

- R2-1809270 Signalling for euCA, Nokia, Nokia Shanghai Bell