3GPP TSG-RAN WG2 Meeting #105	R2-1901104

Athens, Greece, 25 Feb - 1 Mar 2019

| CR-Form-v11.2.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.340 | CR |  | rev | 0100 | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on bearer type selection for MR-DC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-02-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In TS 37.340, it is not much clear which node determines the bearer type for offloaded QoS flows whose SDAP entity are at the SN during the SN addition/modification procedure. The MN can determine the bearer type for those offloaded QoS flows, and the SN can accept or reject it. The MN already decides the bearer type for the offloaded QoS. When a QoS flow is offloaded to SN, it can be mapped to SN terminated SCG bearer, MCG bearer, or split bearer. Only SN terminated SCG bearer doesn’t need MN resources. By telling SN how much MN can contribute to carry this QoS flow – none or partially or completely - MN already decides the bearer type to be SN terminated SCG bearer, or split bearer, or MCG bearer, respectively Otherwise, a set of wrap round and redundant signalling is needed. The exemplary example is given as follows, especially for non GBR QoS flows. - The MN requests to offload a set of QoS flows; - The SN decides to setup the split bearer(s), and sends back to the MN; - However the MN finds it can not meet the split bearer requirement, and rejects the response. Then the MN may send another round of offloading request. Hence it can be concluded that the MN can determine the bearer type for those offloaded QoS flows, and the SN can accept or reject it. This can reuse the principle for EN-DC. The detailed analysis can be found in [R2-1901103]. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The MN can determine the bearer type for those offloaded QoS flows, and the SN can accept or reject it; Impact Analysis: Impacted functionality: Bearer type decision for MR-DC with 5GC Interoperability issue: - No interoperability issue. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The Bearer type decision in case of QoS flow offloading is not clear |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

Start of change

8.2	Bearer type selection

In EN-DC, for each radio bearer the MN decides the location of the PDCP entity and in which cell group(s) radio resources are to be configured. Once an SN terminated split bearer is established, e.g. by means of the Secondary Node Addition procedure or MN initiated Secondary Node Modification procedure, the SN may removeSCG resources for the respective E-RAB, as long as the QoS for the respective E-RAB is guaranteed.

In MR-DC with 5GC, the following principles apply:

-	The MN decides per PDU session the location of the SDAP entity, i.e. whether it shall be hosted by the MN or the SN or by both (split PDU session);

-	If the MN decides to host an SDAP entity it may decide some of the related QoS flows to be realized as MCG bearer, some as SCG bearer, and others to be realized as split bearer;

-	If the MN decides that an SDAP entity shall be hosted in the SN, the MN decides that some of the related QoS flows may be realized as SCG bearer, some as MCG bearer, while others may be realized as split bearer. The SN is responsible to assign the corresponding DRB IDs, based on the DRB IDs indicated by the MN. The SN may remove or add SCG resources for the respective QoS flows, as long as the QoS for the respective QoS flow is guaranteed

-	For each PDU session, including split PDU sessions, at most one default DRB may be configured (see [3]).

End change