3GPP TSG-RAN WG2 Meeting # 105	     R2-1901325

Athens, Greece, 25th February - 1th  March, 2019

Agenda Item:	11.1.3

Source:	Ericsson

Title:	Adaptation Layer Header Content

Document for: Discussion, Decision

# 1	Introduction

As part of the IAB WI [1], 3GPP has agreed to enhance the L2 wireless transport by introducing a new layer called Adaptation layer, which will be located above RLC. In the IAB SI [2], the following possible set of functions were identified to be provided/supported by the Adaptation layer:

- Identification of the UE- associated bearer for the PDU;

- Forwarding across the wireless backhaul topology;

- QoS-enforcement by the scheduler on DL and UL on the wireless backhaul link;

- Mapping of UE associated user-plane PDUs to backhaul RLC channels;

- Potentially other functions.

Furthermore, the IAB SI [2] listed the following content/identifiers possibly to be carried/included in the Adaptation Layer header:

-	UE-bearer-specific Id;

-	UE-specific Id;

-	Route Id, IAB-node or IAB-donor address;

-	QoS information;

-	Potentially other information (e.g. VLAN-ids, Ethertypes, etc.).

The list of functionalities and identifiers listed above were addressing all the protocol/architectural options that were discussed in the SI phase. However, only one architecture option (option 1a), and within that architecture, only one UP and CP protocol options are going to be standardized in Rel-16 according to the agreements made during RAN2#104. Thus, some of the above-mentioned identifiers and functionalities may not be relevant anymore, while new identifier(s) may need to be included for other potential functions provided by the Adaptation layer. This contribution discusses these aspects in detail.

# 2 	Discussion

Figure 1 shows the UP architecture recommended for the IAB WI, where the full F1-U header (GTP/UDP/IP) terminates at the IAB DU. For this architecture, the GTP TEID and IP header can be used for mapping UE user-plane PDUs to BH RLC channels and identifying the UE-bearer for the PDU at the Access IAB node as explained in [3], the Donor DU and the Access IAB node will perform this mapping based on the configuration done by the CU-CP, since the CU-CP is responsible for the DRB management of all the UEs served by Donor DU as well as downstream IAB nodes. Consequently, for the first and fourth functions listed above, the Adaptation layer header does not need to carry any specific identifier.

Figure 1: User Plane (UP) architecture recommended for Rel-16 WI on IAB.

For forwarding packets across the wireless backhaul topology, either a path ID or an IAB-node L2 address can be used. As explained in [4], forwarding based on path ID is more beneficial than using access IAB-node or IAB-donor addresses as it provides extra information to the intermediate IAB nodes that can be used for enforcing fairness and QoS. However, irrespective of the forwarding mechanism, the Adaptation layer header will carry an identifier for packet forwarding across the backhaul topology.

Regarding static QoS information of UE bearers, the Adaptation layer does not need any explicit identifier to carry in its header for appropriate QoS treatment of packets by the network nodes. The CU-CP will implement/enforce the QoS requirements for applications/services used by end-user by appropriately configuring the mapping rules between the ingress and egress BH RLC channels. Thus, the intermediate IAB nodes will simply use these lookup tables to forward packets to egress BH RLC channels suited to packets/data QoS profile. However, some dynamic information, such as the time a packet has already spent in the network, could be useful for enforcing the QoS requirements of the bearers [5]. Optional fields in the adaptation layer can be used to convey such information to intermediate IAB nodes.

When it comes to other potential information/identifier carried in the Adaptation header, one possible field can be the marker flag used by IAB nodes for marking the packets that experienced queueing delay beyond a threshold value, an approach proposed in [6] for DL flow control in IAB network. Moreover, some control elements might be defined for the Adaptation layer that can be used for L2 communication between IAB nodes, such as indicating backhaul RLF or recovery to child IAB nodes [7]. For this purpose, the Adaptation header needs to carry flag(s)/indicator(s) for differentiating control packets from data packets. Lastly, some flags might be included in the Adaptation header for future compatible extension.

The Adaptation header does not need to carry any specific identifier for the UE, UE-bearer, and static bearer QoS information.

The Adaptation header will carry a path identifier for packet forwarding.

The Adaptation header would need some flags/fields for flow control, differentiating Adaptation layer control packets from data packets, and optional fields enabling dynamic QoS enforcement.

The Adaptation header would need some flags for future compatible extension.

Finally, the Adaptation layer header will be counted as overhead incurred in RAN. To keep the overhead low, it will be useful to adopt a minimum set of identifiers/fields needed to provide the essential functions.

The Adaptation Layer should carry a path identifier for packet forwarding and a flag indicating user data or control, other elements are FFS.

# 3	Conclusion

In this paper we made the following observations:

Observation 1	The Adaptation header does not need to carry any specific identifier for the UE, UE-bearer, and static bearer QoS information.

Observation 2	The Adaptation header will carry a path identifier for packet forwarding.

Observation 3	The Adaptation header would need some flags/fields for flow control, differentiating Adaptation layer control packets from data packets, and optional fields enabling dynamic QoS enforcement.

Observation 4	The Adaptation header would need some flags for future compatible extension.

Leading to the following proposal:

Proposal 1	The Adaptation Layer should carry a path identifier for packet forwarding and a flag indicating user data or control, other elements are FFS.

# 4		Reference

[1] R2-182882; WID: Integrated Access and Backhaul for NR, Qualcomm, RAN#82, Sorrento, Italy, December 2018.

[2] TR 38.874; Study on Integrated Access and Backhaul.

[3] R2-1901323; QoS Mapping to Backhaul Bearers in IAB Networks, Ericsson, RAN2#105, Athens, Greece, February 2019.

[4] R2-1901326; Forwarding Packet in IAB Network, Ericsson, RAN2#105, Athens, Greece, February 2019.

[5] R2-1901330; Timestamping for User Plane Latency Control, Ericsson, RAN2#105, Athens, Greece, February 2019.

[6] R2-1901387; Flow Control in IAB, Ericsson, RAN2#105, Athens, Greece, February 2019.

[7] R2-1901386; Handling of link failures in IAB networks, Ericsson, RAN2#105, Athens, Greece, February 2019.