3GPP TSG-RAN WG2 Meeting #106	R2-1906501

# Reno, Nevada, 13 - 17 May 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 1048 | rev | - | Current version: | 15.5.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Per-PLMN Implicit Barring Exemption |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Perspecta Labs, OEC, AT&T, Sprint |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-05-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | TS 38.331 is unclear on how to support barring exemption when the per-PLMN implicit barring list is used. Since the per-PLMN implicit barring list includes all access categories and each access category needs to point to an entry into the UAC barring info set list, it is not possible to exempt one or more access categories from barring when all 8 entries in the UAC barring info set list are populated. Barring exemption for an access category when the per-PLMN explicit barring list is used or when the barring for common list is used does not have this problem as barring exemption can be done by not populating an access category in the list. Barring exemption per access category should be possible in all cases. As such, a note should be added to clarify that an entry in the UAC barring info set list should be reserved and not populated when barring exemption is needed and the per-PLMN implicit barring list is used. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a note in TS 38.331 below the ASN1 UAC-BarringInfoSetIndex field descriptions to clarify how to support barring exemption when the per-PLMN implicit barring list is used. To support barring exemption for one or more access categories when the per-PLMN implicit barring list is used, one of the 8 index values of the UAC barring info set index should refer to an entry not included in the UAC barring info set list and the access category(ies) not to be subject to barring should use this index value. Impact analysis: Impacted 5G architecture options: Standalone No impact to the UE Impacted Functionality: Per – PLMN gNB barring Info SIB broadcast Interoperability: A gNB which does not implement this CR may broadcast different barring info than a gNB which does implement the CR. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is unclear how barring exemption will be supported in all cases. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR … CR … |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

#### –	UAC-BarringInfoSetIndex

The IE UAC-BarringInfoSetIndex provides the index of the entry in uac-BarringInfoSetList. Value 1 corresponds to the first entry in uac-BarringInfoSetList, value 2 corresponds to the second entry in this list and so on. An index value referring to an entry not included in uac-BarringInfoSetList indicates no barring.

UAC-BarringInfoSetIndex information element

-- ASN1START

-- TAG-UAC-BARRINGINFOSETINDEX-START

UAC-BarringInfoSetIndex ::=                INTEGER (1..maxBarringInfoSet)

-- TAG-UAC-BARRINGINFOSETINDEX-STOP

-- ASN1STOP

NOTE:	To support barring exemption for one or more access categories when the uac-ImplicitACBarringList is used, one of the maxBarringInfoSet index values of the UAC-BarringInfoSetIndex should be reserved and refer to an entry not included in uac-BarringInfoSetList, and the access category(ies) not to be subject to barring in the uac-ImplicitACBarringList should use this index value.

END OF CHANGES