3GPP TSG RAN WG2#107	R2-1908625

Prague, Czech Republic, 26 – 30 August, 2019

3GPP TSG RAN WG1 Meeting #97			R1-1907956

Reno, USA, May 13th – 17th, 2019

Title:	LS on PDCCH monitoring for NR-DC

Response to:	-

Release:	Rel-15

Work Item:	NR_newRAT-Core

Source:	RAN WG1

To:	RAN WG2

CC:	RAN WG3

Contact Person:

#### Name:	Huilin Xu

E-mail Address:	huilinxu@qti.qualcomm.com

1. Overall Description:

In [1], RAN1 informed RAN2 the following agreements from RAN1 #96bis for PDCCH monitoring for NR-DC:

Agreements: - For NR-DC (including late drop UEs), ask RAN2 if it is possible to introduce RRC parameters pdcch-BlindDetectionMCG and pdcch-BlindDetectionSCG, which configures the reference number of cells for blind detection for MCG and SCG, respectively, and RRC parameters pdcch-BlindDetectionMCG-UE and pdcch-BlindDetectionSCG-UE for optional new UE capability signalling that informs the maximum values for pdcch-BlindDetectionMCG and pdcch-BlindDetectionSCG, respectively. If yes, - The value range of pdcch-BlindDetectionMCG-UE and pdcch-BlindDetectionSCG-UE is - [1, …, pdcch-BlindDetectionCA-1] and pdcch-BlindDetectionMCG-UE + pdcch-BlindDetectionSCG-UE >= pdcch-BlindDetectionCA if the UE reports pdcch-BlindDetectionCA, and - FFS: [1, 2, 3] and pdcch-BlindDetectionMCG-UE + pdcch-BlindDetectionSCG-UE >= the maximum number of DL serving cells over CGs that UE can support if the UE does not report pdcch-BlindDetectionCA. - If the UE does not report pdcch-BlindDetectionMCG-UE and pdcch-BlindDetectionSCG-UE, - pdcch-BlindDetectionCA for NR-CA is re-used as the UE capability signalling for NR-DC to determine BD/CCE limit across serving cells over CGs if the UE reports pdcch-BlindDetectionCA, and - the number of configured DL serving cells over CGs is used to determine BD/CCE limit across serving cells over CGs if the UE does not report pdcch-BlindDetectionCA. - If the UE reports pdcch-BlindDetectionMCG-UE or pdcch-BlindDetectionSCG-UE, both of them are reported (i.e., not either of them). - pdcch-BlindDetectionMCG-UE and pdcch-BlindDetectionSCG-UE are per UE capability signalling. - FFS: If the UE does not report pdcch-BlindDetectionCA, the UE must support at least one NR-DC band combination where the UE behaviour of blind decoding operation is the same as in Rel.15 NR-CA. - For MCG and SCG, the UE calculates BD/CCE limit for the CGs by using pdcch-BlindDetectionMCG and pdcch-BlindDetectionSCG, respectively. - The value range of pdcch-BlindDetectionMCG and pdcch-BlindDetectionSCG is [1, 2, …, 15]. - pdcch-BlindDetectionMCG + pdcch-BlindDetectionSCG <= pdcch-BlindDetectionCA if the UE reports pdcch-BlindDetectionCA, and pdcch-BlindDetectionMCG + pdcch-BlindDetectionSCG <= the number of configured DL serving cells over CGs if the UE does not report pdcch-BlindDetectionCA. - For a UE configured with NR-DC, if the UE reports pdcch-BlindDetectionCA, the UE is always configured with pdcch-BlindDetectionMCG and pdcch-BlindDetectionSCG. - FFS: if the UE does not report pdcch-BlindDetectionCA. - The information exchange between MCG and SCG (if any) is up to RAN2/RAN3 to discuss.

In the RAN1 #97 meeting, RAN1 resolved the FFS aspects in the above RAN1 #96bis agreements. RAN1 also clarified the UE behaviour for the fallback NR-CA band combination derived from a NR-DC band combination. The updated agreements are as follows and the updated parts are highlighted:

Agreements: - For NR-DC (including late drop UEs), ask RAN2 if it is possible to introduce RRC parameters pdcch-BlindDetectionMCG and pdcch-BlindDetectionSCG, which configures the reference number of cells for blind detection for MCG and SCG, respectively, and RRC parameters pdcch-BlindDetectionMCG-UE and pdcch-BlindDetectionSCG-UE for optional new UE capability signalling that informs the maximum values for pdcch-BlindDetectionMCG and pdcch-BlindDetectionSCG, respectively. If yes, - The value range of pdcch-BlindDetectionMCG-UE and pdcch-BlindDetectionSCG-UE is - [1, …, pdcch-BlindDetectionCA-1] and pdcch-BlindDetectionMCG-UE + pdcch-BlindDetectionSCG-UE >= pdcch-BlindDetectionCA if the UE reports pdcch-BlindDetectionCA, and - [1, 2, 3] and pdcch-BlindDetectionMCG-UE + pdcch-BlindDetectionSCG-UE >= the maximum number of DL serving cells over CGs that UE can support if the UE does not report pdcch-BlindDetectionCA. - If the UE does not report pdcch-BlindDetectionMCG-UE and pdcch-BlindDetectionSCG-UE, - pdcch-BlindDetectionCA for NR-CA is re-used as the UE capability signalling for NR-DC to determine BD/CCE limit across serving cells over CGs if the UE reports pdcch-BlindDetectionCA, and - the number of configured DL serving cells over CGs is used to determine BD/CCE limit across serving cells over CGs if the UE does not report pdcch-BlindDetectionCA. - If the UE reports pdcch-BlindDetectionMCG-UE or pdcch-BlindDetectionSCG-UE, both of them are reported (i.e., not either of them). - pdcch-BlindDetectionMCG-UE and pdcch-BlindDetectionSCG-UE are per UE capability signalling. - If the UE does not report pdcch-BlindDetectionCA, and if X is the maximum number of CCs supported by the UE across all NR-DC band combinations then there is at least one parameter pair (X1, X2) such that X1 + X2 = X and the UE supports at least one NR-DC band combination with X1 CCs in MCG and X2 CCs in SCG and for which X1 <= pdcch-BlindDetectionMCG-UE and X2 <= pdcch-BlindDetectionSCG-UE. - For MCG and SCG, the UE calculates BD/CCE limit for the CGs by using pdcch-BlindDetectionMCG and pdcch-BlindDetectionSCG, respectively. - The value range of pdcch-BlindDetectionMCG and pdcch-BlindDetectionSCG is [1, 2, …, 15]. - pdcch-BlindDetectionMCG + pdcch-BlindDetectionSCG <= pdcch-BlindDetectionCA if the UE reports pdcch-BlindDetectionCA, and pdcch-BlindDetectionMCG + pdcch-BlindDetectionSCG <= the number of configured DL serving cells over CGs if the UE does not report pdcch-BlindDetectionCA. - For a UE configured with NR-DC, the UE is always configured with pdcch-BlindDetectionMCG and pdcch-BlindDetectionSCG. - The information exchange between MCG and SCG (if any) is up to RAN2/RAN3 to discuss. - For a NR-CA operation with only the fallback NR-CA band combination derived from a NR-DC band combination, the determination of PDCCH blind decoding capability follows the existing mechanism for NR-CA without involvement of any pdcch-BlindDetectionMCG-UE or pdcch-BlindDetectionSCG-UE.

2. Actions:

To RAN WG2

ACTION: RAN1 kindly would like to ask RAN2 to take the above updated agreements into consideration for the specification work for NR-DC (including late drop UEs).

3. Date of Next RAN WG1 Meetings:

TSG-RAN WG1 Meeting #98				26th – 30th August 2019		Prague.

TSG-RAN WG1 Meeting #98bis				14th – 18th October 2019		China.

4. Reference

[1] R1-1905689, ‘LS on PDCCH monitoring for NR-DC’, NTT DOCOMO, INC.