3GPP TSG-RAN WG2 Meeting #109bis-e	R2-2003338

# Online, 20 – 30 April 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | - | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR 38.331 Remaining issues on RRC for V2X Sidelink |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL-Core |  |  |  |  |  | Date: |  |  | 2020-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Give the reasons for change: - To take into account the radio conditions when performing a RRC reconfiguration on sidelink interface in NR |  |  |  |  |  |  |  |  |
| Summary of change: |  | Explain the changes: - Adds a conditional statement to the criterias on when T400 timer is started, to ensure it is not started while neither T311 nor T301 is running Impact analysis Impacted functionality: NR sidelink RRC reconfiguration Inter-operability: - T400 is a sidelink related timer, and should not have an impact on Uu |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | RRC reconfiguration Timer T400 will not behave as intended. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.x.9.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First Modified Subclause

### 5.X.9	Sidelink RRC procedure

#### 5.X.9.1	Sidelink RRC reconfiguration

##### 5.x.9.1.1	General

Figure 5.x.9.1.1-1: Sidelink RRC reconfiguration, successful

Figure 5.x.9.1.1-2: Sidelink RRC reconfiguration, failure

The purpose of this procedure is to establish/modify/release sidelink DRBs or configure NR sidelink measurement and report for a PC5-RRC connection.

The UE may initiate the sidelink RRC reconfiguration procedure and perform the operation in sub-clause 5.x.9.1.2 to its peer UE in the following cases:

1>	If UE is configured with sl-ScheduledConfig; and

1> If sl-TxPoolExceptional is not included in sl-ConfigDedicatedNR

2> if T311 is not running; and

2> T301 is not running;

3>	the release of sidelink DRBs associated with the peer UE, as specified in sub-clause 5.x.9.1.4;

3>	the establishment of sidelink DRBs associated with the peer UE, as specified in sub-clause 5.x.9.1.5;

3>	the modification for the parameters included in SLRB-Config of sidelink DRBs associated with the peer UE, as specified in sub-clause 5.x.9.1.5;

1> Else

2>	the release of sidelink DRBs associated with the peer UE, as specified in sub-clause 5.x.9.1.4;

2>	the establishment of sidelink DRBs associated with the peer UE, as specified in sub-clause 5.x.9.1.5;

2>	the modification for the parameters included in SLRB-Config of sidelink DRBs associated with the peer UE, as specified in sub-clause 5.x.9.1.5;

##### -	the configuration of the peer UE to peform NR sidelink measurement and report.

##### 5.x.9.1.2	Actions related to transmission of RRCReconfigurationSidelink message

The UE shall set the contents of RRCReconfigurationSidelink message as follows:

1>	for each sidelink DRB that is to be released, according to sub-clause 5.x.9.1.4.1, due to configuration by sl-ConfigDedicatedNR, SIBX, SidelinkPreconfigNR or by upper layers:

2>	set the slrb-PC5-ConfigIndex included in the slrb-ConfigToReleaseList corresponding to the sidelink DRB;

1>	for each sidelink DRB that is to be established or modified, according to sub-clause 5.x.9.1.5.1, due to receiving sl-ConfigDedicatedNR, SIBX, SidelinkPreconfigNR:

2>	set the SLRB-Config included in the slrb-ConfigToAddModList, according to the received sl-RadioBearerConfig and sl-RLC-BearerConfig corresponding to the sidelink DRB;

1>	for each NR sidelink measurement and report that is to be configured:

2>	set the sl-MeasConfig according to the stored NR sidelink measurement configuration information;

1>	start timer T400 for the destination associated with the sidelink DRB;

The UE shall submit the RRCReconfigurationSidelink message to lower layers for transmission.

Next Modified Subclause