3GPP TSG-RAN WG2 #109bis-e	R2-2003581

Electronic, 20 Apr – 30 Apr 2020

Agenda Item:		6.9.4.2	UE capabilities for Conditional PSCell change for intra-SN

Source:		Huawei, HiSilicon

Title:				Discussion on UE capabilities for CPC

WI code(s):		NR_Mob_enh-Core

Document for:	Discussion and Decision

Introduction

At RAN2-109-e meeting, RAN2 made some agreements on UE capabilities [1][2]. However, it seemed not much discussions on UE capabilties related to CPC. So this paper is to provide our understandings and analysis on CPC part.

Discussion

Based on [1], there were the following agreements:

1: Agree the capabilities (x1-1, x1-3, x2, x3) including the revisions as indicated in the table for NR. May be revised based on further inputs

2: Agree the capabilities (x1-1, x1-3) including the revisions as indicated in the table for LTE. May be revised based on further inputs

These agreements are from [2], and the capabilities related to CHO are listed as below:

- X1-1: Handling of CHO configuration, including execution condition and candidate cell configuration

- X1-3: CHO based failure handling

In the tables “RAN2 UE feature list on NR mobility enhancement” and “RAN2 UE feature list on LTE mobility enhancement” in [2], CPC has not been mentioned in the column Feature group. So there may be the two options on UE capabilities for CPC:

Option 1: No need to introduce extra capabilities. If the UE only supports SA, the X1-1 and X1-3 indicates that the UE supports CHO. If the UE supports both SA and NE-DC/NG-EN-DC/NR-DC, the X1-1 and X1-3 indicates that the UE supports both CHO and CPC.

Option 2: Introduce new capabilities for CPC and they are similar as X1-1 and X1-3.

Even if CHO and CPC have lots of commonilities, we think they are still two features with different purposes, complexities and other apsects, so we prefer option 2.

Proposal 1: It is proposed RAN2 to discuss the two options for UE capabilities for CPC:

Option 1: re-use X1-1 and X1-3 for CPC and no need to introduce extra capabilitiy bits.

Option 2: introduce new capabilities for CPC, i.e. X1-1 for CPC, X1-3 for CPC.

Conclusion

In this paper, we think that RAN2-109-e meeting did not explicitly discuss and conclude on UE capabilities for CPC, so it is proposed:

Proposal 1: It is proposed RAN2 to discuss the two options for UE capabilities for CPC:

Option 1: re-use X1-1 and X1-3 for CPC and no need to introduce extra capabilitiy bits.

Option 2: introduce new capabilities for CPC, i.e. X1-1 for CPC, X1-3 for CPC.

Reference

- Draft_RAN2#109-e_MeetingReport_v1

- R2-2000459	UE feature list for LTE and NR mobility	Intel Corporation	discussion