# 3GPP TSG-RAN WG2 Meeting #109bis-e					  R2-2002934

# E-meeting, April 20 – April 30, 2020

Agenda item:		6.7.4.1 (NR_IIOT-Core)

Source:	LG Electronics Inc.

Title: 	Use of Rel-15 Duplication MAC CE

Document for:	Discussion and Decision

1.	Introduction

Whether to use Rel-15 Duplication MAC CE for Rel-16 Duplication configuration is still open in RAN2. At the RAN2#109e meeting, this issue was discussed several times by e-mail, but RAN2 failed to reach an agreement due to quite evenly split views from companies. This issue should be concluded to finalize the Rel-16 IIOT WI.

2.	Discussion

The Rel-16 Duplication MAC CE is used for a single DRB indicating which RLC entities of the DRB should be activated or deactivated. On the other hand, the Rel-15 Duplication MAC CE is used for multiple DRBs indicating whether PDCP duplication should be activated or deactivated for each DRB.

Therefore, the pros and cons of each MAC CE are quite obvious, as shown below.

|  | Pros | Cons |
| --- | --- | --- |
| Rel-15 Duplication MAC CE | Multiple DRB control | Only indicating PDCP duplication ON/OFF |
| Rel-16 Duplication MAC CE | Provide finer granularity of PDCP duplication | Single DRB control |

The benefit of using Rel-15 Duplication MAC CE over Rel-16 Duplication MAC CE is reduced signaling overhead. For example, if the network wants to deactivate PDCP duplication for multiple DRBs at the same time, with Rel-15 Duplication MAC CE, the network can send only one Rel-15 Duplication MAC CE to achieve that purpose. However, without the Rel-15 Duplication MAC CE, the network has to send multiple Rel-16 Duplication MAC CEs, one for each DRB, to achieve that purpose.

Based on this benefit, we thought that using the Rel-15 Duplication MAC CE is beneficial and should be used for Rel-16 Duplication MAC CE.

However, we acknowledge that the Rel-16 is almost closed, and use of Rel-15 Duplication MAC CE is not essential for finalizing the IIOT WI. Using Rel-15 Duplication MAC CE is a singaling optimization, and Rel-16 IIOT can live without the Rel-15 Duplication MAC CE. Moreover, if the Rel-15 Duplication MAC CE is agreed to be used, RAN2 needs further discusison on other issues such as what the RLC entity status is when Rel-15 MAC CE indicates Duplication activation. It should also be noted that additional specification effort is needed if the Rel-15 Duplication MAC CE is agreed to be used.

Though we see the benefit of using Rel-15 Duplication MAC CE for Rel-16 Duplication configuration, considering the limited time in Rel-16, we propose not to use Rel-15 Duplication MAC CE for Rel-16 Duplication configuration.

Proposal: Rel-15 Duplication MAC CE is not used for Rel-16 Duplication configuration.

3.	Conclusion

To finalize the IIOT WI in Rel-16 time frame, we propose following:

Proposal: Rel-15 Duplication MAC CE is not used for Rel-16 Duplication configuration.