3GPP TSG RAN WG2 Meeting #109bis-e                                              R2-2003608

e-Meeting, April 20th – 30th, 2020

Agenda item:	6.18.3

Source:	Samsung

Title:	Remaining issues related to Manual CAG Selection

Document for:	Discussion & Decision

# Introduction

In this contribution we discuss remaining issues related to manual CAG selection which were not discussed in email discussion [Post109e#18][PRN].

# Discussion

An LS was sent to SA2/CT1 [1] to seek their opinion on following questions raised in the LS:

Question 1.1; TO: SA2; CC: CT1: If a UE performs manual CAG selection and a successful registration, then whether the UE shall stay on cells supporting the manually selected CAG ID in RRC_CONNECTED state especially in the case when after registration the Allowed CAG List in the UE does not contain the manually selected CAG ID? Question 1.2; TO: SA2; CC: CT1 Shall a UE prioritize for cell reselection the cells supporting the manually selected CAG ID over other suitable cells that do not support the manually selected CAG ID after a successful registration?

These issues related to manual CAG selection were deferred in email discussion [Post109e#18][PRN] citing response from SA2/CT1 to be referred.

## Manual CAG selection and a successful registration

The UE NAS layer is allowed to manually select a CAG ID which is not present in the current Allowed CAG list. In such a scenario the UE NAS layer triggers the registration procedure on the cell broadcasting the CAG ID manually selected by the UE. The registration request to forwarded to AMF by NG-RAN. Upon receiving the registration request the AMF checks with the UDM whether the Allowed CAG list of the UE is updated. If the UDM finds the ’CAG information Subscription Change Indication’ in the UE subscription context i.e. the Allowed CAG list is just updated, then the UDM indicates to the AMF that the CAG information in the subscription data has changed. Then, the AMF should provision the CAG information (including the up-to-date Allowed CAG list) to the UE via user configuration update procedure. The registration is successful wherein the Allowed CAG list of the UE is updated. If the updated Allowed CAG list contains the CAG ID manually selected by the UE, the UE can stay in RRC_CONNECTED state and the mobility of the UE is controlled based on the updated Allowed CAG list. If the AMF determines that it has already successfully delivered the up-to-date CAG information, then it may not update the UE. If the CAG ID that was manually selected is not part of the Allowed CAG list, then the UE shall not stay on the cell. Since the UE allowed CAG list is not updated, there is no point in keeping the UE in RRC_CONNECTED. Assume NG-RAN will release the RRC connection of the UE to IDLE.

Observation#1: If the updated Allowed CAG list contains the CAG ID manually selected by the UE, the UE can stay in RRC_CONNECTED state and the mobility of the UE is controlled based on the updated Allowed CAG list.

Observation#2: If the CAG ID that was manually selected is not part of the Allowed CAG list, then the UE shall not stay on the cell, and NG-RAN will release the RRC connection of the UE to IDLE.

## Prioritizing the Manually Selected CAG ID

After successful registration if UE allowed CAG list is updated with the manually selected CAG ID, then UE NAS will provide the updated allowed CAG list to AS. For cell reselection then it is business as usual and nothing special. After successful registration if UE allowed CAG list is NOT updated with the manually selected CAG ID, the UE is released to RRC_IDLE. The allowed CAG list with UE AS is not updated. For cell reselection then it is business as usual and nothing special. Therefore, there is no prioritization requirement among the CAG IDs in the allowed CAG ID list.

Observation#3: There is no prioritization requirement among the CAG IDs in the allowed CAG ID list.

# Conclusion

Based on the above, RAN2 is requested to discuss the observations in the context of the questions sent in the LS to SA2/CT1:

Observation#1: If the updated Allowed CAG list contains the CAG ID manually selected by the UE, the UE can stay in RRC_CONNECTED state and the mobility of the UE is controlled based on the updated Allowed CAG list.

Observation#2: If the CAG ID that was manually selected is not part of the Allowed CAG list, then the UE shall not stay on the cell, and NG-RAN will release the RRC connection of the UE to IDLE.

Observation#3: There is no prioritization requirement among the CAG IDs in the allowed CAG ID list.

# References

[1] R2-2002417, LS on Manual CAG ID selection and granularity of UAC parameters for PNI-NPNs.