3GPP TSG-RAN WG2 #109bis-e	 TDoc R2-2003779

# Electronic, 20th – 30th April 2020

Agenda Item:	6.4.2.2

Source:	ZTE Corporation, Sanechips

Title:	Summary of NR V2X UE cell (re-)selection related contribution

Document for:	Discussion, Decision

Introduction

- In this contribution, we will give a summarize of all contributions related to NR V2X cell selection/ reselection, which is based on the following contributions.

| - TDoc No. | - Title | - Source |
| --- | --- | --- |
| - R2-2002829 | - Discussion on inter-RAT Cell Selection/Reselection | - CATT |
| - R2-2003097 | - Remaining issues of cell (re)selection for NR V2X | - Lenovo, Motorola Mobility |
| - R2-2003515 | - Remaining issues on cell reselection for sidelink in TS 38.304 | - Huawei, HiSilicon |
| - R2-2003721 | - Finalising cell reselection for V2X | - Samsung |

- In details, all proposals raised in those contributions are quite essential at this stage. Thus, all issues listed out in the following are marked with essential.

Discussion

- [Essential] Rel-16 V2X UE performing cell (re-)selection in RRC connected state

- According to the description in [1], it clarifies that in LTE sidelink and V2X, UE is allowed to perform cell selection/reselection under RRC connected mode. The reason is that the SL/V2X UE may also work as a normal Uu UE, then it may access into RRC connected mode due to Uu services, but the eNB which it accesses into may not support SL/V2X SL features. As the consequence, the UE may still need to choose another cell for SL configurations if it has the SL traffic. Thus, in their opinion, the same principle should be applied to NR V2X as well. Thus,

- Proposal 1: RAN2 should discuss whether the procedure of cell selection and reselection should apply to RRC_CONNECTED UE as well.

- [Essential] On-demand SI providing anchor carrier configuration

- In LTE, SIBs will only be configured to the camping UE via broadcast communication. Thus, the UE can recognize whether the cell can provide sidelink configuration or only anchor carrier configuration via reading the related SIB message. However, when it comes to NR V2X, the SIB is allowed to be configured to the UE not only via broadcast, but also can be through on-demand SI. If the V2X SIB is configured to the UE through on-demand SI, the UE cannot recognize whether there is detailed V2X configuration or only anchor carrier configuration until it performing random access and acquire the on-demand SI, where it is somehow a waste of time. Therefore, to avoid this issue, the following alternatives are proposed, as concluded in [2],[3] and [4]:

- Alt 1: If a cell providing only anchor carrier configuration, the cell should always broadcast the V2X SIB.

- Alt 2: If the NR V2X SI is provided on-demand, then the UE does not prioritize this cell.

- Alt 3: If a carrier doesn’t broadcast the V2X SIB but provide it by on-demand, then this frequency should be indicated by other frequency as anchor frequency in the V2X SIB.

- Alt 4: Introduce an IE in SIB 1,2/4 to indicate whether the cell will only providing anchor carrier configuration.

- Proposal 2: Targeting on the issue that cell broadcasting only anchor carrier configuration via on demand SI, the following solutions should be discussed in RAN2:

- Alt 1: If a cell providing only anchor carrier configuration, it should always broadcast the V2X SIB.

- Alt 2: If the NR V2X SI is provided on-demand, then the UE does not prioritize this cell.

- Alt 3:If a carrier doesn’t broadcast the V2X SIB but provide it by on-demand, then this frequency should be indicated by other frequency as anchor frequency in the V2X SIB.

- Alt 4:Introduce an IE in SIB 1,2/4 to indicate whether the cell will only providing anchor carrier configuration.

- [Essential] How to use pre-configuration[4][5]

- Currently, there are three types of sidelink carrier:

- -Type 1:Frequency which can provide both NR sidelink communication configuration and LTE sidelink communication configuration.

- -Type 2:Frequency which can provide either NR sidelink communication configuration and LTE sidelink communication configuration.

- -Type 3: Frequency which cannot provide any sidelink communication configuration.

- If there are only type 2 frequencies. Specifically, if some type 2 frequencies can provide only NR SL configuration, the other type 2 frequencies

- Proposal 3: For the case that if UE supports both NR and LTE SL, but UE’s camped cell can only provide on SL RAT configuration, RAN2 should discuss whether pre-configured SL resource can be used on the other RAT if the UE is also in-coverage of that RAT.

- Moreover, if pre-configuration is not allowed to use in the above mentioned case, then several alternatives to help UE acquire V2X configuration are provided in [4],[5].

- Option 1: UE performs cell reselection to select the carrier which provides the SL configuration of the other RAT.

- Option 2: UE requests the SL configuration of the other RAT from the camped cell.

- Option 3: UE a quires the SL configuration of the other RAT by reading the broadcast V2X SIB on the concerned carrier.

- Then RAN2 should filter out one workable solution to help UE acquire V2X related configuration.

- Proposal 4: If pre-configuration is not allowed for the UE to use which is in coverage, then RAN2 should filter out one possible solution to help the UE acquire V2X related configuration.

- Option 1: UE performs cell reselection to select the carrier which provides the SL configuration of the other RAT.

- Option 2: UE requests the SL configuration of the other RAT from the camped cell.

- Option 3: UE a quires the SL configuration of the other RAT by reading the broadcast V2X SIB on the concerned carrier.

Conclusion

In this contribution, we have summarized all principle opinions from NR V2X cell selection/reselection contributions, a brunch of proposals have been provided in the following:

- Proposal 1: RAN2 should discuss whether the procedure of cell selection and reselection should apply to RRC_CONNECTED UE as well.

- Proposal 2: Targeting on the issue that cell broadcasting only anchor carrier configuration via on demand SI, the following solutions should be discussed in RAN2:

- Alt 1: If a cell providing only anchor carrier configuration, it should always broadcast the V2X SIB.

- Alt 2: If the NR V2X SI is provided on-demand, then the UE does not prioritize this cell.

- Alt 3:If a carrier doesn’t broadcast the V2X SIB but provide it by on-demand, then this frequency should be indicated by other frequency as anchor frequency in the V2X SIB.

- Alt 4:Introduce an IE in SIB 1,2/4 to indicate whether the cell will only providing anchor carrier configuration.

- Proposal 3: For the case that if UE supports both NR and LTE SL, but UE’s camped cell can only provide on SL RAT configuration, RAN2 should discuss whether pre-configured SL resource can be used on the other RAT if the UE is also in-coverage of that RAT.

- Proposal 4: If pre-configuration is not allowed for the UE to use which is in coverage, then RAN2 should filter out one possible solution to help the UE acquire V2X related configuration.

- Option 1: UE performs cell reselection to select the carrier which provides the SL configuration of the other RAT.

- Option 2: UE requests the SL configuration of the other RAT from the camped cell.

- Option 3: UE a quires the SL configuration of the other RAT by reading the broadcast V2X SIB on the concerned carrier.

References

- R2-2003515	Remaining issues on cell reselection for sidelink in TS 38.304	Huawei, HiSilicon

- R2-2001974	Report of offline discussion 709		ZTE Corporation, Sanechips

- R2-2003097	Remaining issues of cell (re)selection for NR V2X	Lenovo, Motorola Mobility

- R2-2002829	Discussion on inter-RAT Cell Selection/Reselection		CATT

- R2-2003721	Finalising cell reselection for V2X	Samsung