3GPP TSG-RAN WG2 Meeting #111 electronic	R2-200XXXX

Online, August 17 – 28, 2020

Agenda item:	6.5.3

Source:	Samsung

Title:	Report of Offline 033: IIOT MAC Corrections II

Document for:	Discussion & Decision

# Introduction

This document provides a summary report of the following offline discussion:

- [AT111-e][033][IIOT] MAC Corrections II (Samsung)

Scope: HARQ PID for SPS: Treat R2-2006712/7527 (related to RRC discussion), and R2-2007136. UE autonoumous retransmission: Treat R2-2007147, 7530, 6863, 7389, 8055

Determine agreeable parts in a first phase, Agree CRs in a second phase

Deadline: Aug 27 0900 UTC, Intermediate deadlines by Rapporteur if needed.

This discussion covers the following tdocs:

HARQ PID for SPS

R2-2006712	Correction on the calculation of HARQ Process ID for SPS	Huawei, HiSilicon	CR	Rel-16	38.321	16.1.0	0774	-	F	NR_IIOT-Core

R2-2007527	CR on 38.321 for SPS resources and HARQ process ID calculation	ZTE Corporation, Sanechips	CR	Rel-16	38.321	16.1.0	0828	-	F	NR_IIOT-Core

R2-2007136	Clarification on HARQ process ID determination for SPS	OPPO, Samsung	CR	Rel-16	38.321	16.1.0	0804	-	F	NR_newRAT-Core, NR_IIOT-Core

UE autonomous retransmissions

R2-2007147	UE autonomous retransmission considering the processing time	vivo,Samsung, Lenovo	CR	Rel-16	38.321	16.1.0	0805	-	F	NR_IIOT-Core

R2-2007530	Considieration on the lack of time to process the autonomous transmission	ZTE Corporation, Sanechips	discussion	Rel-16	NR_IIOT-Core

RACH collisions

R2-2006863	CR to PUSCH duration comparision with MSGA transmission	Fujitsu	CR	Rel-16	38.321	16.1.0	0791	-	F	NR_IIOT-Core

R2-2007389	Correction on resource overlapping with grants addressed to T-C-RNTI	Huawei, HiSilicon	CR	Rel-16	38.321	16.1.0	0821	-	F	NR_IIOT-Core

Other

R2-2008055	Miscellaneous corrections for IIOT MAC	Samsung	CR	Rel-16	38.321	16.1.0	0876	-	F	NR_IIOT-Core

# Phase-1 Discussion

## 2.1 Issues covered by other discussions (No Phase-1 discussion here)

### 2.1.1 Note on HARQ Process ID Determination for SPS

R2-2007136	Clarification on HARQ process ID determination for SPS	OPPO, Samsung	CR	Rel-16	38.321	16.1.0	0804	-	F	NR_newRAT-Core, NR_IIOT-Core

This contribution discusses the same issue with R2-2007135, submitted to Rel-15 UP correction:

R2-2007135	Clarification on HARQ process ID determination for SPS	OPPO, Samsung	CR	Rel-15	38.321	15.9.0	0803	-	F	NR_newRAT-Core

Note 1. R2-2007136 will be discussed after [AT111-e][002][NR15] NR MAC corrections (Samsung) is concluded.

### 2.1.2 Clarification on Uplink Grant Addressed to Temporary C-RNTI

R2-2007389	Correction on resource overlapping with grants addressed to T-C-RNTI	Huawei, HiSilicon	CR	Rel-16	38.321	16.1.0	0821	-	F	NR_IIOT-Core

This contribution discusses the same issue with R2-2007861, submitted to Rel-15 UP correction:

R2-2007861	Clarification on collision between uplink grant for MSG3 retransmission and DG	Huawei, HiSilicon	CR	Rel-15	38.321	15.9.0	0843	-	A	NR_newRAT-Core

Note 2. R2-2007389 will be discussed after [AT111-e][002][NR15] NR MAC corrections (Samsung) is concluded.

### 2.1.3 Unit of SPS Periodicity and Formula

R2-2006712	Correction on the calculation of HARQ Process ID for SPS	Huawei, HiSilicon	CR	Rel-16	38.321	16.1.0	0774	-	F	NR_IIOT-Core

R2-2007527	CR on 38.321 for SPS resources and HARQ process ID calculation	ZTE Corporation, Sanechips	CR	Rel-16	38.321	16.1.0	0828	-	F	NR_IIOT-Core

R2-2008055	Miscellaneous corrections for IIOT MAC	Samsung	CR	Rel-16	38.321	16.1.0	0876	-	F	NR_IIOT-Core

An issue on SPS periodicity in three contributions above is related to RRC Correction. Those contributions will be treated in the offline discussion [AT111-e][031][IIOT] RRC Corrections (Ericsson). Depending on the RRC discussion, corresponding MAC CR will be updated.

Note 3. Issue on Unit of SPS periodicity in R2-2006712, R2-2007527, and R2-2008055 will be discussed in [AT111-e][031][IIOT] RRC Corrections (Ericsson).

## 2.2 Discussion

### 2.2.1 AutonomousTx with lack of preparation time

In RAN2#110-e meeting, RAN2 discussed how to capture an RAN2#109-e agreement “When the CG is de-prioritized, it is up to the UE implementation to determine the processing time restriction determining whether the very next CG resource with same HARQ process can be used, or the/a following one, for an autonomous transmission.” Despite the large support, it was not agreed due to lack of time. R2-2007147 (vivo, Samsung, Lenovo) proposed to add a NOTE to capture the agreement.

NOTE: It is up to the UE implementation to determine the closest available next configured grant configured with autonomousTx for the transmission of the MAC PDU of the same HARQ process of the previously de-prioritized uplink grant.

Otherwise, the UE may not have sufficient preparation time for the PUSCH transmission when the UE is required to perform the autonomous retransmission.

On the other hand, R2-2007530 (ZTE) pointed out that an uplink grant which does not have sufficient processing time cannot be a prioritized uplink grant, because a condition “for each uplink grant whose associated PUSCH can be transmitted by lower layers” prohibits prioritizing very next CG. Thus, they think further clarification is not necessary.

< Companies’ view >

1) NOTE is necessary.

2) NOTE is not necessary.

| Company | 1) or 2) | Comments |
| --- | --- | --- |
| Fujitsu | 1 | We can accept to capture the NOTE as RAN2 agreed in the last meeting: “A NOTE for RAN2#109-e agreement on next CG selection for autonomous retransmission will be added”. |
| Lenovo | 1 | Regarding R2-2007530 (ZTE), the issue is not that an uplink grant having not sufficient processing time cannot be a prioritized grant. The problem is rather that the current spec mandates the UE for triggering an autonomous Retransmission to check whether the previous configured uplink grant for this HARQ process was deprioritized. As shown in the figure, for cases when the very next uplink CG following a deprioritized CG cannot be used for transmission due to processing timing constraints, the immediately preceding uplink grant (when checking the condition for autonomous transmission at the next but one CG) may be a prioritized grant though, leading to a situation that autonomous retransmission is not allowed according to current specified behaviour. |
| Nokia (Wallace, Email: Ping-Heng.Kuo@nokia.com) | 1, But … | We think the note is needed, but the wording can be improved. With the current proposed text, it sounds like the UE has to choose the nearest available CG resource, but whether the word “available” means a resource beyond the processing constraint is debatable. We suggest change the wording as following to make it simpler: NOTE: It is up to the UE implementation to select a determine the closest available next configured grant configured with autonomousTx for the transmission of the MAC PDU of the same HARQ process of the previously de-prioritized uplink grant. Samsung: Let’s discuss the detail wording after we agree to add it. |
| ZTE (Fei Dong; dong.fei@zte.com.cn) | 2 | Thanks for explain from Intel. We totally understand the intention of capturing this note. And the reason why we assert that the note may have a redundant specification with the current description is what if depriotized Configured grant can not be prioritized for transmission , the transmission cannot be processed at all. I would like to correct your figure based on the current text procedure as below: Lenovo (jlohr@lenovo.com): Just to clarify: the point is that the immediately preceding CG (orange color) is not a deprioritized grant. Therefore the condition for triggering an autonomous transmission, (which is checked at the CG following the orange one) can never be true 3> if the previous configured uplink grant, in the BWP, for this HARQ process was de-prioritized; and Hence UE would not be allowed to trigger autonomous transmission according to current spec. From above figure, we can see that the very near CG for autonomous transmission cannot be processed by MAC layer, it is naturally this CG cannot be transmitted by lower layer, and it won’t be indicated as “prioritized UL grant” at all. Thus that’s is our understanding the current spec have already captured the note in the text procedure. In addition, from the note, it will give UE so much loosen behavior for determining the “very closed configured grant”. UE can assume every CG with the same HARQ process ID as the “very closed configured grant”. From the current text procedure, it will restrict UE to use the real “closest available configured grant” in a more accurate way. Compare between two methods, we think the current text procedure is our intention of auto-transmission with the configured grant. |
| Ericsson | 2 | Companies should first confirm if the latest MAC spec mandates or does not mandate UE to transmit autonomously always in the next CG occasion. In our understanding, it is not mandated. As explained in R2-2007530, due to the following condition, for each uplink grant whose associated PUSCH can be transmitted by lower layers, such a grant cannot be declared as prioritized. In clause 5.4.2.1, the transmission on this grant is only initiated when it is declared as prioritized. By this reasoning, UE is not mandated to transmit autonomously always in the next CG occasion (which cannot be transmitted by lower layers). Not sure I understand the arguments above from Lenovo. The CG in the middle of the above figure cannot be prioritized, as its PUSCH cannot be transmitted by lower layers (due to processing time constraint) and so no declaration that the grant is prioritized. Lenovo: Please see comments made above to ZTE. More importantly, in Ericsson’s view, a reasonable gNB does not configure a CG configuration intended for eMBB data and with a very short interval (close to T_proc,2) between the two transmissions with the same HARQ process. This basically leaves no re-transmission opportunity, not only for the data, but also for the MAC CEs that might be multiplexed on the grants. Keep in mind that network also needs time to process and send a retransmission grant if needed. |
| LG (Seong Kim; sj117.kim@lge.com) | 2 | Even without the NOTE, the UE would select the closest CG. |
| vivo | 1 | The current specification is mandating the UE to use the immediate next CG for autonomous retransmission even though the UE is not able to use it due to the processing delay. The ZTE’s proposal does not resolve the issue in the current specification, as the immediate CG can also be considered as the prioritized grant as explained by Lenovo. |
| CATT (pierrebertrand@catt.cn) | 1 | We don’t think the newly added condition "whose associated PUSCH can be transmitted by lower layers" covers the processing time discussed here since such processing time involves both MAC and PHY layers, not “lower layers” only. So interpreting such condition as covering this issue is not straightforward at all for us. Hence we would prefer capturing something explicitly. |
| OPPO （fuzhe@oppo.com） | 1 | If I remembered correctly, it is already agreed in RAN2 to capture a Note. |
| MediaTek (pradeep[dot]jose[at]mediatek[dot]com) | 1 | Agree with others that the newly added condition does not cover processing time, as RAN1 have not defined processing time for CG anywhere. The NOTE is needed to capture our agreements. |
| Huawei, Hisilicon (Zhenzhen, caozhenzhen@huawei.com) | 2 | No strong view, but it seems proponents of the NOTE also want to further polish the wording, which we think would be time consuming, not only in this meeting but also in future. We think companies have already sufficiently aligned on the intended UE behaviour. |
| Samsung | 1 | The note was agreed in RAN2#109-e and it was missing in the spec. We also share the view with Lenovo. - The current normative text does not allow the autonomous transmission when the very next CG is not used due to lack of processing time. The condition “if the previous configured uplink grant, in this BWP, was de-prioritized” is only for the previous configured grant, not the de-prioritized CG. In this context, we need to modify the normative text as well as the NOTE. |
| CMCC | 1 | Slightly prefer option 1 |
| DOCOMO | 1 |  |
| Intel (Yujian Zhang; yujian.zhang@intel.com) | 1 | Agree with CATT and we prefer to capture the note explicitly. |
| Sharp | 2 | We agree with ZTE that condition “for each uplink grant whose associated PUSCH can be transmitted by lower layers” prohibits prioritizing very next CG. |
| Futurewei | 2 | No strong view. Slightly prefer option 2. |
| Sequans | 1 | No strong view, but given RAN2 had an earlier agreement to have a NOTE we prefer to capture it. |
| Apple | 1 | Agree with CATT’s views |

< Summary >

13 companies think the note is necessary.

- This NOTE is what RAN2 already agreed to add. Without this NOTE, UE’s autonomous transmission in the very next CG occasion is mandated.

6 companies think the note is NOT necessary.

- The newly added condition “for each uplink grant whose associated PUSCH can be transmitted by lower layers” covers the case that there is not sufficient processing time.

Although clear majority prefer to add the NOTE, the rapporteur found some technical arguments on the current specification text.

- Discussion #1. The newly added condition “for each uplink grant whose associated PUSCH can be transmitted by lower layers” includes the case that UE does not have the sufficient processing time. It seems logically true but some companies do not agree or prefer to capture the situation explicitly.

- Discussion #2. As pointed out by Lenovo, another problem could be the normative text. One of the current conditions of autonomousTx is “3> if the previous configured uplink grant, in the BWP, for this HARQ process was de-prioritized; and”. In this procedure, the MAC entity checks only the previous CG. If the previous CG was not used due to the lack of processing time, the “previous CG” was neither prioritized or de-prioritized.

The rapporteur would suggest one more round of technical discussions on the above two points..

### 2.2.2 “PUSCH duration of a MSGA payload”

In the current MAC specification, “PUSCH duration of a MSGA payload” modified by 2-step RACH CR (#0714r5) and “transmission of MSGA payload” introduced by IIOT CR (#0712r3) have the same meaning but are used in different places. Since it is compared with PUSCCH resource of an SR transmission or PUSCH duration of an uplink grant, “PUSCH duration of a MSGA payload” is a correct expression to avoid a confusion. R2-2006863 (Fujitsu) and R2-2008055 (Samsung) propose to use a single consistent expression “PUSCH duration of a MSGA payload” for all cases.

In 5.4.1 (R2-2008055):

1> if the MAC entity is configured with lch-basedPrioritization, and the PUSCH duration of the configured uplink grant does not overlap with the PUSCH duration of an uplink grant received in a Random Access Response for this Serving Cell or with the PUSCH duration of a MSGA payload; or

In 5.4.4 (R2-2006863, R2-2008055):

3> if the MAC entity is configured with lch-basedPrioritization, and the PUCCH resource for the SR transmission occasion does not overlap with an uplink grant received in a Random Access Response nor with the PUSCH duration of a MSGA payload, and the PUCCH resource for the SR transmission occasion for the pending SR triggered as specfied in clause 5.4.5 overlaps with any other UL-SCH resource(s), and the priority of the logical channel that triggered SR is higher than the priority of the uplink grant(s) for any UL-SCH resource(s) where the uplink grant was not already de-prioritized, and the priority of the uplink grant is determined as specified in clause 5.4.1; or

< Companies’ view >

| Company | Agree/ Not agree | Comments |
| --- | --- | --- |
| Fujitsu | Agree | This was a bit discussed in [AT109bis-e][028][IIOT], and the suggestion from the rapporteur was to wait for MAC discussed in 2sRA. Now, the MAC text in 2sRA uses the terminology “PUSCH duration of a MSGA payload”, so that the current IIoT text should be corrected accordingly as proposed above. |
| Lenovo | Agree | Agree with Fujitsu |
| Nokia (Wallace, Email: Ping-Heng.Kuo@nokia.com) |  | No strong view, the current text should be quite clear already. |
| ZTE |  | Agree with Nokia |
| Ericsson | Agree |  |
| LG (Seong Kim; sj117.kim@lge.com) | Agree |  |
| vivo | Agree |  |
| CATT | Agree |  |
| OPPO |  | Agree with Nokia |
| MediaTek | Agree |  |
| Huawei, Hisilicon | Agree | No strong view. Can go with majorities. |
| Samsung | Agree |  |
| CMCC | Agree |  |
| DOCOMO | Agree |  |
| Intel | Agree |  |
| Sharp | Agree |  |
| Futurewei | Agree |  |
| Sequans | Agree |  |
| Apple | Agree |  |

< Summary >

16 companies agree to have the consistency.

2 companies are ok with the current text.

Proposal 1. “transmission of a MSGA payload” is replaced by “PUSCH duration of a MSGA payload” in 5.4.1 and 5.4.4. TP in R2-2006863, R2-2008055 is a baseline.

### 2.2.3 Addition of “Considered as” for Cancelled Uplink Grant

When an uplink grant becomes a prioritized uplink grant or de-prioritized uplink grant, the grant is “considered” as either prioritized or de-prioritized. For cancelled uplink grant, the expression is not consistent. Thus, R2-2008055 (Samsung) proposed to add “considered as”.

If the corresponding PUSCH transmission of a configured uplink grant is cancelled by CI-RNTI, as specified in clause 11.2A of TS 38.213 [6], this uplink grant is considered as a de-prioritized uplink grant.

< Companies’ view >

| Company | Agree/ Not agree | Comments |
| --- | --- | --- |
| Fujitsu | Agree | It looks good to see alignment with other parts. |
| Lenovo | Agree |  |
| Nokia (Wallace, Email: Ping-Heng.Kuo@nokia.com) | Agree | More consistent |
| ZTE | Agree |  |
| Ericsson | Agree |  |
| LG (Seong Kim; sj117.kim@lge.com) | Agree |  |
| vivo | Agree |  |
| CATT | Agree |  |
| OPPO | Agree |  |
| MediaTek | Agree |  |
| Huawei, Hisilicon | Agree |  |
| Samsung | Agree |  |
| CMCC | Agree | It looks better than before. |
| DOCOMO | Agree |  |
| Intel | Agree |  |
| Sharp | Agree |  |
| Futurewei | Agree |  |
| Sequans | Agree |  |
| Apple | Agree |  |

< Summary >

All companies agree.

Proposal 2. “considered as” is inserted in the description of prioritization of cancelled uplink grant.

### 2.2.4 Clarification of harq-ProcID-Offset2

harq-ProcID-Offset2 can be configured only for operation in licensed spectrum. However, it is not specified in MAC specification whereas it clearly mention that harq-ProcID-Offset can be configured for operation with shared spectrum channel access. It may give misunderstanding that harq-ProcID-Offset2 can be configured irrespective of spectrum. R2-2008055 (Samsung) proposed to clarify as follows:

RRC configures the following parameters when the configured grant Type 1 is configured: - cs-RNTI: CS-RNTI for retransmission; - periodicity: periodicity of the configured grant Type 1; - timeDomainOffset: Offset of a resource with respect to SFN = timeReferenceSFN in time domain; - timeDomainAllocation: Allocation of configured uplink grant in time domain which contains startSymbolAndLength (i.e. SLIV in TS 38.214 [7]) or startSymbol (i.e. S in TS 38.214 [7]); - nrofHARQ-Processes: the number of HARQ processes for configured grant; - harq-ProcID-Offset: offset of HARQ process for configured grant for operation with shared spectrum channel access; - harq-ProcID-Offset2: offset of HARQ process for configured grant for operation in licensed spectrum; - timeReferenceSFN: SFN used for determination of the offset of a resource in time domain. The UE uses the closest SFN with the indicated number preceding the reception of the configured grant configuration. RRC configures the following parameters when the configured grant Type 2 is configured: - cs-RNTI: CS-RNTI for activation, deactivation, and retransmission; - periodicity: periodicity of the configured grant Type 2; - nrofHARQ-Processes: the number of HARQ processes for configured grant; - harq-ProcID-Offset: offset of HARQ process for configured grant for operation with shared spectrum channel access; - harq-ProcID-Offset2: offset of HARQ process for configured grant for operation in licensed spectrum.

< Companies’ view >

| Company | Agree/ Not agree | Comments |
| --- | --- | --- |
| Fujitsu | Agree, but… | The text can be further improved: “for operation in licensed spectrum channel access”. |
| Lenovo |  | No strong opinion. However this clarification is in our view not really necessary |
| Nokia (Wallace, Email: Ping-Heng.Kuo@nokia.com) | Not agree | Don’t think this is necessary. Typically we do not explicitly indicate whether a parameter is used in licensed band – by default it can be used in licensed band. |
| ZTE |  | No strong opinion |
| Ericsson | Not agree | This is covered by the RRC field description harq-ProcID-Offset2 Indicates the offset used in deriving the HARQ process IDs, see TS 38.321 [3], clause 5.4.1. This field is not configured for operation with shared spectrum channel access. harq-ProcID-Offset For operation with shared spectrum channel access, this configures the range of HARQ process IDs which can be used for this configured grant where the UE can select a HARQ process ID within [harq-procID-offset, .., (harq-procID-offset + nrofHARQ-Processes – 1)] On a separate note, by a simple cross-checking, one can realize that these two RRC fields functional-wise are the same. They both give the lower end of the HARQ process pools configured for a configured grant configuration, while the higher end is procID-Offset + numberOfHARQ-Processes. If the spec readability is a concern here, a merge of these two fields seems to be the best way-forward. |
| LG (Seong Kim; sj117.kim@lge.com) | Agree |  |
| vivo | Agree |  |
| CATT | Not agree | We agree with Ericsson that it is already clear from RRC spec, so this change is not essential. |
| OPPO | Not agree | As mentioned by Ericsson, the configuration restrictions for harq-ProcID-Offset2 is already captured in RRC field description. |
| MediaTek | Agree | We prefer consistency in the specifications and therefore support this clarification. |
| Huawei, Hisilicon | Not agree | The spec is sufficiently clear. The change is not essential. |
| Samsung | Agree | We prefer consistency in the MAC specification. |
| Agree | Agree | This looks clearer. |
| DOCOMO |  | No strong opinion |
| Intel | Not agree | We agree with Ericsson that it is clear from RRC field description, therefore no need to capture in MAC again. |
| Sharp |  | No strong opinion. |
| Futurewei | Not agree | The proposed additions are unnecessary. |
| Sequans |  | No strong view |
| Apple | No strong view | Clarification is not really necessary for licensed band scenarios. |

< Summary >

6 companies agree

9 (incl. Lenovo and Apple) companies disagree, or indicated that clarification is not necessary

4 companies did not express their preference.

Slightly majority does not touch the current text. One reason is that RRC spec clearly specifies the restriction and we do not need to do the same thing in the MAC spec. The rapporteur would suggest to follow the majority view.

Proposal 3. MAC spec does not clarify that harq-ProcID-Offset2 is configured only for operation in licensed spectrum.

# Phase-2 Discussion

## 3.1 Confirmation of Phase-1 Proposals

Proposal 1. “transmission of a MSGA payload” is replaced by “PUSCH duration of a MSGA payload” in 5.4.1 and 5.4.4. TP in R2-2006863, R2-2008055 is a baseline. Proposal 2. “considered as” is inserted in the description of prioritization of cancelled uplink grant. Proposal 3. MAC spec does not clarify that harq-ProcID-Offset2 is configured only for operation in licensed spectrum.

If there is any unacceptable proposal, please provide your view with sufficient technical reasons:

| Company | Unacceptable proposal | Reason (technical reason to revert the majority view) |
| --- | --- | --- |

## 3.2 AutonomousTx with lack of preparation time

During Phase-1 discussion, some companies mentioned that newly added condition “for each uplink grant whose associated PUSCH can be transmitted by lower layers” covers all the cases that UE cannot use the uplink grant, e.g. PHY does not support the transmission, lack of processing time, etc. This condition has been added last meeting, even after we agreed to add the NOTE. If companies could agree this, the note may not be necessary. However, some companies think it would be better to specify clear condition to avoid the confusion.

< Condition that MAC entity performs the prioritization >

When the MAC entity is configured with lch-basedPrioritization, for each uplink grant whose associated PUSCH can be transmitted by lower layers, the MAC entity shall:

< Proposed NOTE >

NOTE: It is up to the UE implementation to determine the closest available next configured grant configured with autonomousTx for the transmission of the MAC PDU of the same HARQ process of the previously de-prioritized uplink grant.

< Companies’ view >

1) NOTE is necessary. The condition does not cover the behaviour of the NOTE.

2) NOTE is necessary. The condition covers the behaviour of the NOTE. It is better to capture the NOTE explicitly.

3) NOTE is not necessary. The condition covers the behaviour of the NOTE.

| Company | 1), 2), 3) | Comment |
| --- | --- | --- |
| Samsung | 2) or 3) | No strong view among 2) and 3). The new condition does not specify any particular case. The lack of processing time is also the case that PHY cannot transmit it. We think technically NOTE may not be needed. However, we tend to agree that it would be better to specify that UE can select the next available CG, for clarity. |
| Lenovo | 2) | No strong view. We think it is more important to correct the normative text such that the specification supports autonomous transmissions also on CG PUSCHs not immediately following a deprioritized CG. If normative text is corrected, there is no NOTE required. |
| Qualcomm rprakash@qti.qualcomm.com | 2 |  |
| Nokia | 2 or 3 | Agree with Samsung. |
| Ericsson | 3) | If RAN2 agrees that the condition covers the behavior of the NOTE, Ericsson does not see the necessity of a NOTE in the spec. If needed, it can be captured in the chairman notes. |
| LG | 3 |  |

Another problem pointed out by Lenovo is that the current normative text only allows autonomousTx only if “previous CG” was de-prioritized. In the figure below, autonomous Tx is NOT performed at the very next CG occasion of CG1.

According to the current MAC specification, the MAC entity checks if the “previous” CG was de-prioritized as follows.

3> else if this uplink grant is a configured grant configured with autonomousTx; and 3> if the previous configured uplink grant, in the BWP, for this HARQ process was de-prioritized; and 3> if a MAC PDU had already been obtained for this HARQ process; and 3> if the uplink grant size matches with size of the obtained MAC PDU; and 3> if a transmission of the obtained MAC PDU has not been performed: 4> consider the MAC PDU has been obtained.

If autonomousTx at the very next CG occasion is not performed, the previous CG was not used, i.e. neither prioritized nor de-prioritized. Therefore the condition of autonomous Tx is not met. The autonomousTx is prohibited, according to the current text in MAC specification

< Companies’ view >

1) Agree the problem in the current normative text (TBD: how to fix it)

2) Disagree

| Company | 1), 2) | Comment |
| --- | --- | --- |
| Samsung | 1) | Even with the NOTE, the autonomous Tx is supported with checking the previous CG. However, we need to check the CG which was actually de-prioritized. So, the “previous” needs to be modified, to indicate the actually de-prioritized CG. |
| Lenovo | 1) | Agree with Samsung that current normative text prohibits autonomous transmission for cases when a CG is skipped due to processing time constraints. Basically, UE needs to check the priority status (deprioritized/prioritized) of the CG on which the last transmission attempt for the MAC PDU was made, i.e. don’t check the priority status of the CG which was skipped. Our suggestion would be to have following small fix: 3> else if this uplink grant is a configured grant configured with autonomousTx; and 3> if the previous configured uplink grant, in the BWP, for this HARQ process for which the last transmission attempt of the MAC PDU was made was de-prioritized; and 3> if a MAC PDU had already been obtained for this HARQ process; and 3> if the uplink grant size matches with size of the obtained MAC PDU; and 3> if a transmission of the obtained MAC PDU has not been performed: 4> consider the MAC PDU has been obtained |
| Qualcomm | 1 (soft view) | Simply changing “the previous” to “a previous” would be ok. We prefer to not add too much new language, as the intent of the current language is clear anyway. |
| Nokia | 1 | We tend to agree with Qualcomm’s suggestion on the text. It is better to keep the text simple while capturing the correct behavior. |
| Ericsson | 1 | A simple fix is preferred and one proposal from Ericsson is 3> if the previous configured uplink grant, in the BWP, for this HARQ process was not prioritized de-prioritized; and The previous configured uplink grant is not used and, thus, declared as neither “prioritized” nor “de-prioritized”. A simple change of the wording from “de-prioritized” to “not prioritized” would fix this issue. |
| LG | 1 | To fix this issue, our suggestion is to remove the line starting with ‘3> if the previous configured uplink grant …’ and move ‘in the BWP’ to the next line as follows. … 3> if the previous configured uplink grant, in the BWP, for this HARQ process was de-prioritized; and 3> if a MAC PDU had already been obtained for this HARQ process in the BWP; and … We think the following lines of the current text can indicate the situation that the previous CG is de-prioritized. Furthermore, they can indicate this problem situation (deprioritized CG, then skipped (undefined status) CG). 3> if a MAC PDU had already been obtained for this HARQ process; and … 3> if a transmission of the obtained MAC PDU has not been performed: |

# Conclusion